3GPP TSG-RAN WG2 Meeting #129	R2-25xxxxx

Athens, Greece, 17-21 February 2025

Source: 	Session Chair (MediaTek)

Title:	Report from session on positioning and sidelink relay

# 4	EUTRA Rel-17 and earlier

Only essential corrections. No documents should be submitted to 4. Please submit to 4.x

## 4.3	Positioning corrections Rel-16 and earlier

(LTE_NavIC-Core, LTE TEI16 Positioning), REL-15 and Earlier WIs related to positioning are in scope but not listed explicitly (long list).

Tdoc Limitation: 1 tdoc

# 5	NR Rel-15 and Rel-16

Essential corrections only.

Tdoc Limitation: 2 tdocs in total for all sub agenda items NOTE: some agenda items have additional Tdoc limits.

In case a correction need to be reflected in both NR TS and LTE TS, the corrections should be submitted under one single AI (so the NR and LTE correction can be treated together), the sub-Ais below this

## 5.3	NR Positioning Support

(NR_newRAT-Core; leading WG: RAN1; REL-15; started: Mar. 17; closed: Jun. 19: WID: RP-191971)

(NR_pos-Core; leading WG: RAN1; REL-16; started: Mar 19; target; Jun 20; WID: RP-200218).

(NR TEI16 Positioning)

Stage 2 corrections shall be discussed with the specification rapporteur (Sven Fischer sfischer@qti.qualcomm.com) before submission. Stage 2 CRs not discussed with the specification rapporteur will not be treated.

Tdoc Limitation: 1 tdoc

R2-2500817	Correction on SP positioning SRS (de-)activation MAC CE	Huawei, HiSilicon	CR	Rel-16	38.321	16.18.0	2034	-	F	NR_pos-Core

- Not pursued

Discussion:

vivo think the first change is incorrect because C indicates the BWP ID if it is different from the serving cell, E is always mandatory if the extension is there, and the two should not be linked.  The second part they think is already clear in the current spec.

Xiaomi tend to agree with vivo and do not see the need to describe the E=0 case; they also think there is no case where E is absent.

Huawei indicate that when BWP is missing, the E field will also not be populated, hence the linkage; on the second part, they think the spec is open if E=0 now.

Samsung are fine with the second change, but they think the first part is already clear that C indicates the presence of the octet including E.  CATT have the same understanding.

Ericsson think the change is not an essential correction, and we should not be doing clarifications on Rel-16 at this point.  They agree that the meaning of the E bit is already clear.

Qualcomm agree with Ericsson and note that E was a reserved bit before, so it should be quite clear what the UE does when it is 0, and it cannot be absent as such.

R2-2500818	Correction on SP positioning SRS (de-)activation MAC CE	Huawei, HiSilicon	CR	Rel-17	38.321	17.11.0	2035	-	A	NR_pos-Core

- Not pursued

R2-2500819	Correction on SP positioning SRS (de-)activation MAC CE	Huawei, HiSilicon	CR	Rel-18	38.321	18.4.0	2036	-	A	NR_pos-Core

- Not pursued

# 6	NR Rel-17

Essential corrections only.  Editorial/clarifications should be sent to be reviewed and approved by spec rapporteurs prior to submission.  Editorials should only be submitted by spec rapporteurs.

Tdoc limitation: 4 Tdocs

## 6.2	NR Sidelink relay

(NR_SL_Relay-Core; leading WG: RAN2; REL-17; WID: RP-212601)

CRs

R2-2500908	Miscellaneous corrections for Rel-17 SL relay	ZTE Corporation, Sanechips	CR	Rel-17	38.331	17.11.0	5243	-	F	NR_SL_relay-Core

- Not pursued

Discussion:

CATT think this is more category D.  ZTE indicate that the third correction is substantive and they think it justifies category F.

Huawei agree that the corrections are editorial, and they think adding the field description is not needed because the procedural text for the events is already clear.  They do not think the CR is wrong but think the bar for Rel-17 should be higher than this.  Nokia agree with Huawei.

R2-2500909	Miscellaneous corrections for Rel-17 SL relay	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.4.0	5244	-	A	NR_SL_relay-Core

- Not pursued

New proposals outside WI scope

R2-2500316	RRC Connection Establishment for Multihop-Parallel Relay	IIT, Kharagpur	discussion	Rel-17

R2-2500325	Data distribution and HARQ management for multihop-parallel relay topology in  5G NR 	IIT, Kharagpur	discussion	Rel-17

## 6.3	NR positioning enhancements

(NR_pos_enh-Core; leading WG: RAN1; REL-17; WID: RP-210903)

R2-2500602	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R17)	ZTE Corporation, Ericsson, Qualcomm, CATT, Samsung, vivo, Nokia, Xiaomi	CR	Rel-17	38.321	17.11.0	1977	2	F	NR_pos_enh-Core	R2-2410985

- Agreed

R2-2500603	Correction on spatial relation info in SP SRS activation deactivation MAC CE (R18)	ZTE Corporation, Ericsson, Qualcomm, CATT, Samsung, vivo, Nokia, Xiaomi	CR	Rel-18	38.321	18.4.0	1978	2	A	NR_pos_enh-Core	R2-2410986

- Agreed

R2-2500813	Correction to BDS issue of data indication	Huawei, HiSilicon	CR	Rel-17	37.355	17.9.0	0546	-	F	NR_pos_enh-Core

- Not pursued

Discussion:

CATT would prefer not to modify the earlier releases and we can start from the new signal in Rel-19.  Ericsson agree with CATT and think there is not a real risk of confusion.

Qualcomm agree with the CR and think the issue arose from the B1c introduction; they understand that the Toe field just identifies the instance of ephemeris and does not need explicit units.  They also have some concern that this happened in NavIC too.

Xiaomi agree with Qualcomm but also agree with CATT and Ericsson’s point that there is no critical change, so they would be OK with fixing it from Rel-19.  They think it can be handled in the BDS and NavIC WIs.

R2-2500814	Correction to BDS issue of data indication	Huawei, HiSilicon	CR	Rel-18	37.355	18.4.0	0547	-	A	NR_pos_enh-Core

- Not pursued

R2-2500815	Correction to PRS priority subset for DL-AoD	Huawei, HiSilicon, Ericsson, VIVO, Samsung	CR	Rel-17	38.305	17.7.0	0182	-	F	NR_pos_enh-Core

- Title of affected line in table to change to “PRS priority list for measurement reporting”

- Agreed with this change as R2-2501422

Discussion:

ZTE wonder if the UE-based mode can provide measurements along with the location estimate; if so, they see that the priority subset could also apply to UE-based.  Huawei understand that LPP supports reporting both, but the priority subset is only useful for measurements.

CATT wonder why the LMF should be forbidden to provide the subset for UE-based; they understand that all assistance information could be made available.

Ericsson see that the CR does not prohibit signalling the information, but it clarifies that it is designed for UE-assisted mode.

Qualcomm think this is a problem with the unified PRS assistance data; they think the stage 2 clarification is fine as long as LPP works properly, and they understand that the intention was to inform the UE of the priority of reporting the measurements.

CATT understand from RAN1 colleagues that there was no discussion there of whether to exclude the UE-based case, and they think that UE-based can also use the information subject to UE implementation.

vivo agree with the CR and think in the UE-based case, the UE may have its own preference for the measurements and the network does not need to provide it.

Xiaomi agree with Qualcomm that the intention of the priority is to control reporting, and the UE implementation can control how it performs the actual measurements, so they think the intention of the CR is correct.  However, they do not think there is anything broken in Rel-17 and the RAN1 description is already clear.  They think TEI19 would be appropriate.

Huawei think there is consensus that the current spec is wrong.

Lenovo think it would be a bit inconsistent to leave it in Rel-17/18 but change it in Rel-19.

Xiaomi support a perfect specification but think we should not take unnecessary CRs for issues that are not critical.  Huawei think this is more than just a clarification because something is actually wrong in the spec.

ZTE understand that the issue is what the UE would do with the information in UE-based mode, and maybe a NOTE could be captured saying that it is up to UE implementation what to do if it is provided.

Qualcomm would prefer to take the CR but have some modifications to clarify the table in general; they think it is not clear for a reader.  They think we could change to “PRS priority list for measurement reporting”, and they think changing from Rel-19 would create confusion.

R2-2500816	Correction to PRS priority subset for DL-AoD	Huawei, HiSilicon, Ericsson, VIVO, Samsung	CR	Rel-18	38.305	18.4.0	0183	-	A	NR_pos_enh-Core

- Title of affected line in table to change to “PRS priority list for measurement reporting”

- Agreed with this change as R2-2501423

# 7	Rel-18

## 7.1	Expanded and improved NR positioning

(NR_pos_enh2-Core; leading WG: RAN1; REL-18; WID: RP-232670)

Time budget: 0 TU

Tdoc Limitation: 1 tdoc

Minor and editorial issues should be coordinated with the appropriate spec rapporteur and submitted by rapporteur company together with any additional corrections the rapporteur company may have.    Larger issues can be discussed based on contributions/individual CRs.

### 7.1.1	Organizational

Including incoming LSs and rapporteur inputs.

R2-2500007	Reply LS on CBR range (R1-2410708; contact: CATT)	RAN1	LS in	Rel-18	NR_pos_enh2-Core	To:RAN2

- Noted

Discussion:

Huawei think the issue is not so critical.

R2-2500276	Corrections of SL CBR Range and level related parameters	CATT, Ericsson	CR	Rel-18	38.331	18.4.0	5204	-	F	NR_pos_enh2-Core

- Not pursued (related agreement in minutes)

Discussion:

Huawei think this issue arises elsewhere and we should not specify restrictions that are already there in the ASN.1 ranges.

Xiaomi wonder about the consequence if we do not change: Can the network still indicate 64 to the UE, and if so, what happens?  They understand that we should capture the change if it affects UE behaviour.

ZTE agree with the CR because there was an error during the running CR implementation, and the alternative as seen last meeting was an extensive ASN.1 change.

Xiaomi note that this is a UE specification, and we should capture what we need the UE behaviour to reflect.  They want to clarify that this CR does not force a change to UE implementation, and they think we could capture in the minutes that this case is up to UE implementation.

Ericsson understand that we should address the recommendations of the RAN1 LS, but they agree we could capture something in the notes.

Agreement:

If the NW configures a maximum value for sl-CBR-ConfigIndexDedicatedSL-PRS-RP, sl-DefaultTxConfigIndexDedicatedSL-PRS-RP, or sl-PRS-TxConfigIndex that goes beyond the actual list sizes, the handling is up to UE implementation.

### 7.1.2	Stage 2

Impact to 38.300, 37.340, and 38.305.

This agenda item may be handled at lower priority.

### 7.1.3	SLPP corrections

Impact to 38.355.

R2-2500845	Missing additional measurements for SL-TDOA and SL-TOA	Qualcomm Incorporated	CR	Rel-18	38.355	18.4.0	0009	1	F	NR_pos_enh2-Core	R2-2408513

- Request field name should include “perARP” and be clarified in the field description

- -r18 suffix to be removed

- Coversheet issues to be addressed

- Revised in R2-2501424

Discussion:

Huawei think the CR is fine in general, but they think the request should be per ARP.  Qualcomm understand that the same number of measurements for each ARP is correct.  Huawei agree but think the request field name could be changed to indicate per ARP.

Xiaomi agree with Huawei’s point.

Lenovo wonder if we should have the -r18 suffix since it is the first release, and if we should have the affected architecture options on the coversheet.

[AT129][402][POS] CR check on additional measurements for SL positioning (Qualcomm)

Scope: Revise and check the CR in R2-2500845.

Intended outcome: Agreed CR (without CB if possible) in R2-2501424

Deadline: Wednesday 2025-02-19 1900 EET

### 7.1.4	LPP corrections

Impact to 37.355.

### 7.1.5	RRC corrections

Impact to 38.331 and 38.306.

R2-2500166	RRC Sidelink Positioning Correction	Fraunhofer IIS, Fraunhofer HHI, Ericsson	CR	Rel-18	38.331	18.4.0	5201	-	F	NR_pos_enh2-Core

Discussion:

vivo understand that we have not discussed IUC for SL positioning and the IUC configuration should not be modified to support SL positioning now.  Ericsson think in this case we would need the CR but not the second change.

ASUSTeK think IUC should be applicable, so they support the intention of the change, but they think the first sentence needs clarification to say it applies for data transmission generically.

Huawei support the change because the previous text clearly restricts the field only to shared resource pool; on the use of IUC, they checked the WID and they understand that it should be supported.

Samsung agree with Ericsson and think the first change is enough, with no need to add the additional sentence.  CATT agree.

Xiaomi agree with Samsung, but they think there is an interoperability concern.  Ericsson indicate that there is no UE-to-NW interoperability issue, but there can be problems if one UE implements and the other does not, and this is reflected in the coversheet.

vivo note that we did not discuss IUC in this context, and they would like to postpone to check with RAN1 colleagues.  Huawei think the CR is correct anyway, because the restriction to shared resource pool was wrong.

Ericsson understand that we need to take the first change to restore legacy operation, and the second part can be postponed.

ZTE think we could postpone the whole CR.  They are also concerned about the mention of SL-PRS in the unchanged sentence at the end of the field description.

[AT129][403][POS] IUC for SL-PRS (Ericsson)

Scope: Check the need for the CR in R2-2500166 and update if necessary.

Intended outcome: Agreeable CR in R2-2501425

Deadline: Wednesday 2025-02-19 1900 EET

R2-2500277	Corrections on activation of non-preconfigured SRS with the type of semi-persistent	CATT, Ericsson, Qualcomm Incorporated, ZTE Corporation, Samsung	CR	Rel-18	38.331	18.4.0	5205	-	F	NR_pos_enh2-Core

Discussion:

Huawei think we need to clarify where the trigger comes from: If it is from upper layers, they think the change is in the wrong place.  They agree that some change is needed, but they think this change is written as if the trigger comes from RRC itself.

Ericsson think this issue was discussed during drafting of the RRC CR, and they are not sure how upper layers would learn about the SRS configuration.  They understand that the spec currently has support for both cases, but how the RRC gets the information could be up to UE implementation.

Huawei think there is confusion about the procedure at a stage 2 level and we may need to consult the SA2 spec; they would be willing to have an offline discussion to check this.

Xiaomi are not sure there is a stage 2 issue; they understand the trigger does come from upper layer to do the positioning operation and the RRC layer decides if resume is needed, but they see the interaction as up to implementation and think the current CR is sufficient.

CATT understand the divergence is about the interaction between AS and upper layer, and if there is doubt, an offline might be worthwhile.

Ericsson would prefer not to have an offline discussion about functional issues for a frozen release; they would rather postpone and allow companies to check offline.

Xiaomi think the issues being discussed are UE implementation.  CATT think we could confirm this offline, and if we just postpone the CR we will come back to the same discussion next meeting.

[AT129][404][POS] CR check on non-preconfigured SP-SRSp activation (CATT)

Scope: Check the CR in R2-2500277 and revise if necessary, taking into account procedural requirements from upper layers to evaluate what needs to be specified vs. left to UE implementation.

Intended outcome: Agreeable CR in R2-2501426

Deadline: Wednesday 2025-02-19 1900 EET

R2-2500812	Correction to sidelinkUEInformation for SL-PRS shared resource pool	Huawei, HiSilicon, Vivo	CR	Rel-18	38.331	18.4.0	5230	-	F	NR_pos_enh2

- “-Core” to be added to WI code

- Agreed

Discussion:

InterDigital wonder if we need to specify something here or the NW implementation can handle it.  Huawei think it is possible that the UE requests a set of frequencies larger than the frequencies requested for SL communication.

Xiaomi think the CR is OK, but the WI code should include “-Core” on the coversheet.

R2-2500971	Low Power High Accuracy Positioning Correction	Ericsson	CR	Rel-18	38.331	18.4.0	5252	-	F	NR_pos_enh2-Core

- Not pursued

Discussion:

vivo have some sympathy for the intention but think the LMF could also ensure that the cells in the validity area support the feature.

Huawei think this issue is similar to other features that can be requested by ResumeRequest, for which we do not set a barring bit but use other mechanisms within UAC and reselection to keep the UE from multiple requests; for instance, the NW could respond with RRCReject with a wait time or change the reselection priorities.

Xiaomi understand that we guide the network not to reject connections for an unknown cause, but they agree that there are ways for the network implementation to resolve the issue.

Ericsson think there will be latency issues if the UE tries and fails, even if the network avoids the loop, and it would be more efficient to enable a different approach like MO-LR.

Xiaomi think the network could guarantee that the feature is supported within the RNA.

Huawei think the issue only occurs in case of a configuration update request; for the initial configuration, the UE will request by MO-LR, and the network can guarantee that the cells in the RNA support it.  They see the problem as a corner case and think there are other solutions.

Xiaomi think it is not a correction but an optimization.

### 7.1.6	MAC corrections

Impact to 38.321.

R2-2500513	Miscellaneous corrections on SL-PRS	ASUSTeK	CR	Rel-18	38.321	18.4.0	2026	-	F	NR_pos_enh2-Core

Discussion:

Huawei think most of the changes are OK, but on the first change, there is a corrected field that is not used anywhere, and it could just be removed.  They also agree with a comment received by the proponent that “with PRS” vs. “and PRS” does not make much difference.

ZTE think the fifth change is not needed (“with PRS” vs. “and PRS”), but they agree with the intention that both cases are decremented, so they suggest using another bullet with an “or” structure to cover both the Rel-17 and Rel-18 cases.

[AT129][405][POS] Revision of miscellaneous SL-PRS MAC changes (ASUSTeK)

Scope: Revise the CR in R2-2500513 to break up the bullet in the fifth change into two parts linked with “or”, and adjust the NOTE accordingly.  Also remove the unused field from the first change.

Intended outcome: Agreed CR (without CB if possible) in R2-2501427

Deadline: Wednesday 2025-02-19 1900 EET

R2-2500545	Correction on parameters for SL-PRS for configured grant Type 1	Sharp	CR	Rel-18	38.321	18.4.0	2030	-	F	NR_pos_enh2-Core

- Not pursued

R2-2500604	Correction on SRS hopping in positioning	ZTE Corporation	CR	Rel-18	38.321	18.4.0	2031	-	F	NR_pos_enh2

Discussion:

Lenovo want to check that there is no related procedural text in RRC, only the ASN.1 to configure the fields.  ZTE indicate that there is text in the PHY spec referring to configuration by RRC signalling.

vivo think the question is whether the MAC should maintain the time window, and they think the RAN1 description is sufficient and the MAC does not need to do anything.  They also note that the MAC does not describe the expiration of the timer, so they agree that the spec is not complete and it makes sense to remove the clause.

Huawei indicate that the current MAC spec does not mean the UE does not transmit SRS outside the window; it follows legacy operation, and the SRS configuration is given directly to the physical layer.  So they understand that the MAC spec just says that the MAC layer indicates to PHY when the collision resolution shall happen.  They agree that there is misalignment about parameter names, but they do not think it is OK to remove the procedural text in 5.32 completely.

ZTE understand that the RRC configures the window, the UE knows where the window is, and the UE just does collision handling during this window; they do not see that the MAC layer needs to do anything.  They see it as analogous to the PPW, where we do not have MAC behaviour specified.

Lenovo understand that we have no procedural text in RRC for the UTW at all, and it looks a bit strange, so we should check before removing it from MAC.

Ericsson have some sympathy for the CR; we have implemented the RRC parameter list to support procedures specified in L1, and it is not clear if we need procedural text of our own for everything.

Huawei indicate that we did not define additional procedures, and MAC controls PHY normally; they see the analogy to PPW as not right because PPW affects RRC, not MAC.

Lenovo think some revision is needed anyway, at least to align parameter names.

Xiaomi agree with Lenovo.

[AT129][406][POS] MAC spec impact for UTW (ZTE)

Scope: Further discuss the CR in R2-2500604 and determine what MAC spec is needed to clarify the operation of the UTW.

Intended outcome: Agreeable CR in R2-25001428

Deadline: Wednesday 2025-02-19 1900 EET

### 7.1.7	Corrections to other specifications

Impact to any specifications not identified above.

## 7.5	Enhanced NR Sidelink Relay

(NR_SL_relay_enh-Core; leading WG: RAN2; REL-18; WID: RP-223501)

Time budget: 0TU

Tdoc Limitation: 1 tdoc

1 additional tdoc on top of the limit is allowed for co-sourced contribution with 3 or more companies.

Minor and editorial issues should be coordinated with the appropriate spec rapporteur and submitted by rapporteur company together with any additional corrections the rapporteur company may have.    Larger issues can be discussed based on contributions/individual CRs.

### 7.5.1	Organizational

Including incoming LSs and rapporteur inputs.

### 7.5.2	Stage 2 corrections

Impact to 38.300.

### 7.5.3	Control plane corrections (including UE capabilities)

Impact to 38.331, 38.304, and 38.306.

Discussed jointly

R2-2500411	Corrections to SidelinkUEInformationNR setting and E2E SL DRB release	ASUSTeK	CR	Rel-18	38.331	18.4.0	5208	-	F	NR_SL_relay_enh-Core

Discussion:

Huawei agree with the first change, but for the e2e DRB handling, several companies identify some missing cases in related papers, but the solutions are different, and they think we should discuss the alternatives together.

Apple also agree with the first change; for the second and third changes, they understand that all the contents of the RRCReconfigurationSidelink should be processed, and we should be able to capture the behaviour without a long list of conditions about the current configuration, so they think we could simplify a bit.  ASUS understand that this comment refers to the Huawei proposal.

OPPO understand that for the fourth change, there is a sentence in the specification saying that when the e2e release or failure occurs, the UE discards the related configurations, which seems already to cover the change.  ASUS understand that in the per-hop cases the addition/modification/release procedure cannot be triggered at the relay UE without the change.

Huawei understand that the point of the fourth change is to add handling of the per-hop cases, but we agreed last meeting that these behaviours will be triggered based on the QoS information, so the relay UE will know the QoS flows are gone and will re-derive the RLC channels for the affected hop, including detecting that some RLC channels are no longer needed.  They understand that the sentence pointed out by OPPO is not intended to cover the per-hop RLC channel release.

Apple think there is some overlap between the changes in terms of functionality and the fourth change may not be strictly needed.

[AT129][408][Relay] Revision of R2-2500411 on Rel-18 relay (ASUSTeK)

Scope: Revise the CR in R2-2500411 to have only the first change (as numbered on the coversheet).

Intended outcome: Revised CR (without CB if possible) in R2-2501430

Deadline: Wednesday 2025-02-19 1900 EET

R2-2501236	Correction to UE information transfer on sidelink	Huawei, HiSilicon	CR	Rel-18	38.331	18.4.0	5270	-	F	NR_SL_relay_enh-Core

Discussion:

OPPO think there is yet another option: to rely on upper layers to release the link if there is no remaining QoS flow.  Huawei’s intention here is to release the RLC channels when all the QoS flows are gone.

Xiaomi think the whole link should be released.  Nokia have the same view and understand that this will have the effect of releasing the RLC channels.

Huawei think when to establish/release the link has generally been left to UE implementation, and there is no guarantee that it will be released, so we cannot take such a decision in RAN2.

Apple consider that whether or not the link is released, the upper layers will be informed that all the DRBs are gone.  They think we could do some clarification, but we do not need to send an empty message for information that the upper layer already knows.

Huawei think if we have no changes, the UE implementation can remove or keep the RLC channels, and when a new set of QoS flows comes to the relay UE, it will re-derive the RLC channels anyway.  They could accept this outcome but think we should document something to avoid further corrections to the same places.

OPPO understand that if the QoS list is empty, the UE will already re-derive the RLC channels, resulting in releasing them all.

Huawei understand that the relay UE cannot release the RLC channels by itself, because there is a peer entity in the target remote UE, but here there is no trigger specified that would cause that release.

Agreement:

RAN2 understand that when all the e2e connections using a particular first-hop link fail or are released for an idle/inactive/OOC relay UE, the handling of the second-hop RLC channels is based on the information provided by upper layers.  No specification impact is expected.

[AT129][407][Relay] Revision of R2-2501236 on Rel-18 relay (Huawei)

Scope: Check and update the CR in R2-2501236, excluding change 3 (as numbered on the coversheet) which was handled in online discussion.

Intended outcome: Agreed CR (without CB if possible) in R2-2501429

Deadline: Wednesday 2025-02-19 1900 EET

Other CRs

R2-2500567	Correction on indirect path failure information	China Telecom, Huawei, HiSilicon	CR	Rel-18	38.331	18.4.0	5214	-	F	NR_SL_enh2-Core

- Merged into R2-2501431

R2-2500907	Corrections for Rel-18 SL relay	ZTE Corporation, Sanechips	CR	Rel-18	38.331	18.4.0	5242	-	F	NR_SL_relay_enh-Core

- Revised in R2-2501431

Discussion:

OPPO wonder if the fourth change is correctly captured; ZTE clarify that the clause number is different.

OPPO are not sure the last change covers the case of three legs (split bearer on Uu + multi-path relay).  Nokia think we should also mention that the secondary split cannot be on the indirect path.

[AT129][409][Relay] Revision of R2-2500907 on Rel-18 relay (ZTE)

Scope: Revise the CR in R2-2500907 to merge in the change from R2-2500567, and check the details of the last change.

Intended outcome: Agreed CR (without CB if possible) in R2-2501431

Deadline: Wednesday 2025-02-19 1900 EET

R2-2500802	Correction on T400 configuration	Xiaomi, Apple	CR	Rel-18	38.331	18.4.0	5229	-	F	NR_SL_relay_enh-Core

- Not pursued (related agreement in minutes)

Discussion:

Huawei recall that we agreed to introduce new values for the U2U cases, so they think that if the network supports U2U, it will always provide t400-u2u, but the CR seems to imply that it is allowed not to provide it.

Ericsson agree with Huawei that the field logically would be provided, but it is an OPTIONAL field; however, they think this situation occurs in other places and they are not sure that anything needs to change.

Xiaomi could accept specifying that the network always provides the U2U field.

Ericsson understand that the t400-u2u will always be present if the feature is supported.

Qualcomm agree with Xiaomi’s intention, but they think maybe something should be clarified in the procedural text rather than the field description.

Apple think t400-u2u is something of an optimization and it would be possible in principle for the network not to set it.

Nokia understand that the value is optional for the network, and the CR clarifies what the UE should do if both are provided.  They think the current text is clear enough, but could be misunderstood with effort, so they can live with the change.

Xiaomi think if there is an implementation that does not comply with this CR, there will be extra SL RLFs, so it has real functional impact.

Huawei think the existing text already specifies that the UE shall use t400-u2u for U2U.

Agreement:

RAN2 understand that if the field t400-u2u is provided, the UE does not apply the value of the field t400 for end-to-end PC5 connection in UE-to-UE relaying.

### 7.5.4	User plane corrections (including SRAP)

Impact to 38.351, 38.321, 38.322, and 38.323.

# 8	Rel-19

## 8.13	NR sidelink multi-hop relay

(NR_SL_relay_multihop; leading WG: RAN2; REL-19; WID: RP-242349)

Time budget: 1 TU

Tdoc Limitation: 3 tdocs

### 8.13.1	Organizational

LSs and rapporteur input, including workplan, etc.

Incoming LSs with “take into account” action only

R2-2500064	LS on Authorization information for Layer-2 multi-hop U2N relaying to NG-RAN (S2-2501296; contact: LGE)	SA2	LS in	Rel-19	5G_ProSe_Ph3	To:RAN2, RAN3

R2-2500068	Reply LS on relay discovery announcement (S2-2501334; contact: LGE)	SA2	LS in	Rel-19	NR_SL_relay_multihop-Core, 5G_ProSe_Ph3	To:RAN2

Rapporteur CR

R2-2500866	Introduction of multi-hop U2N relay in TS 38.323	Ericsson	draftCR	Rel-19	38.323	18.4.0	NR_SL_relay_multihop

### 8.13.2	Relay discovery and (re)selection

Enhancements to relay dscovery and (re)selection to support one additional hop relay (remote UE  first relay UE  last relay UE  gNB). Extensibility to a second additional hop in this WI is considered as a design criterion.

P3 (cross-cell cases)

R2-2500307	Considerations on relay discovery and (re)selection	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 3: RAN2 is kindly asked to agree that the last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell.

P4-P5 (L2IDs)

R2-2500508	Discussion on the discovery and relay (re)selection for multi-hop U2N relay	LG Electronics Inc.	discussion

Proposal 4: In Figure 2.5A-2, the L2 ID(D) and LD ID(D’) can be the same or a difference value. If the last Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

Proposal 5: In Figure 2.5B-1, the L2 ID(B) and LD ID(B’) can be the same or a difference value. if the intermediate Relay UE reports the source L2 IDs and destination L2 IDs by using SUI, the last Relay UE should report the source-destination L2 ID pair to identify other links.

P4-P5 (discovery thresholds and resource pools)

R2-2500865	Discussion on relay discovery and relay (re)selection	Ericsson	discussion	Rel-19	NR_SL_relay_multihop

Proposal 4	Reuse the same Uu RSRP thresholds as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay.

Proposal 5	Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay.

P9-P11 (AS conditions for discovery forwarding)

R2-2500905	Discussion on multi-hop Relay discovery and (re)selection	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 9: For Model A discovery, the intermediate relay UE forwards/propagates the discovery message when the PC5 RSRP between the relay UE and its parent relay UE is above a threshold.

Proposal 10: For Model B discovery, the intermediate relay UE forwards the discovery solicitation/response message when the PC5 RSRP between the relay UE and the UE from which the discovery solicitation/response message is received is above a configured threshold.

Proposal 11: For Model B discovery, after receiving a discovery solicitation message from an intermediate relay, the last relay UE transmits the discovery response message when the PC5 RSRP between the last relay UE and the intermediate relay is above a configured threshold.

P1 (FFS on upper RSRP threshold for intermediate relay UE)

R2-2500724	Relay discovery and (re)selection	Nokia	discussion	NR_SL_relay_multihop	R2-2410288

Proposal 1. In Rel-19 multi-hop relay, RAN2 prioritizes the same cell scenario, and the last relay UE should not operate as an intermediate relay UE.

P8-P11 (selection/reselection criteria, reselection by remote and intermediate relay UEs)

R2-2500192	Discovery and relay (re)selection for multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 8	For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE.

Proposal 9	For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition:

-	PC5 signal strength of current direct connected multi-hop U2N Relay UE is below a (pre)configured signal strength threshold.

-	When U2N Remote UE receives a PC5-S link release message from current direct connected multi-hop U2N Relay UE.

-	When U2N Remote UE detects PC5 RLF with the current direct connected multi-hop U2N Relay UE.

-	Indicated by upper layer.

Proposal 10	Multi-hop U2N Remote UE triggers relay reselection upon PC5-RRC signaling indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the multi-hop U2N Relay UE(s) and network.

-	PC5 RLF/release between the multi-hop U2N Relay UE(s).

Proposal 11	RAN2 to discuss intermediate relay UE performs relay (re)selection with the same trigger condition defined for the remote UE. Upon relay reselection, it sends notification to the remote UE to indicate PC5 link release with its parent relay.

Other contributions

R2-2500122	Discussion on multi-hop U2N relay discovery and relay selection	NEC	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500187	Discussion on Multi-hop Discovery and (Re)selection	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500420	One remaining issue on multi-hop U2N Relay Discovery message forwarding	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2500432	Relay discovery and selection for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2500497	Discovery and Relay (Re)Selection for Multi-hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

R2-2500570	Consideration on multi-hop relay discovery and reselection	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500632	Relay (re)selection in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2500700	Relay discovery and (re)selection for multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500723	[Draft] LS on legacy UE participation in multi-hop UE communication	Nokia	LS out	NR_SL_relay_multihop	To:SA2

R2-2500753	Multi-hop relay selection/re-selection	Sony	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500933	Discovery and (re)selection under multihop relay 	Kyocera	discussion

R2-2501118	Discussion on topology and intermediate relay UE (re)selection	vivo	discussion	Rel-19

R2-2501174	Relay discovery and (re)selection	TCL	discussion

R2-2501183	discussion on Relay discovery and (re)selection for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2501259	Discovery and Relay (re)selection for multi-hop U2N relay	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

### 8.13.3	Control Plane Procedures and SRAP impact

Contributions should focus on control plane procedures and can include SRAP impact and QoS handling to support additional hops.

Including outcome of email discussion [Post128][401][Relay] Control plane baseline solution (InterDigital)

[AT129][401][Relay] SRAP for Rel-19 multihop relay (OPPO)

Scope: F2F offline for advance discussion of header format in SRAP.  Related proposals from R2-2500194 / R2-2500434 / R2-2500701 / R2-2500913 / R2-2501184 can be taken into account but need not be treated exhaustively.

Intended outcome: Report to Wednesday session in R2-2501421

Time: Monday 2025-02-17 1630-1700 EET, in Brk3

Deadline: Wednesday 2025-02-19 1000 EET (for tdoc availability—rapporteur is asked to provide the report earlier for company comments)

Email discussion summary

R2-2500496	Report of [Post128][401][Relay] Control Plane Baseline Solution	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

Easy Proposals (at most 1 company disagrees strongly)

Proposal 1: RAN2 assumes that discovery and PC5 connection establishment can be performed in each intermediate UE prior to processing the received RRCSetupRequest by the remote UE.  The related FFS can be removed from the stage 2 description.

Proposal 2: For the baseline solution, the last relay sending SUI on behalf of other relay UEs is not supported.  The related FFS can be removed from the stage 2 description.

Proposal 3: For the baseline solution, Rel17 SUI message and format is re-used.

Proposal 4: No further clarification in stage 2 description is needed to clarify that a relay UE can establish its RLC channel for relaying of SRB1 during its own connection establishment and details will be clarified in stage 3.  The related FFS can be removed from the stage 2 description.

Proposal 5:  For system information acquisition in multi-hop, the remote UE:

o	When RRC_CONNECTED, uses Uu RRC signaling to obtain its system information directly from its connected cell.

o	When RRC_IDLE/RRC_INACTIVE, can request SI using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

o	When in RRC_IDLE/RRC_INACTIVE, receives the required SI from PC5-RRC signaling (e.g., UuMessageTransferSidelink)

Proposal 6:  For system information acquisition by the remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

o	when there is a change in the required SI while in RRC_IDLE/RRC_INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

o	when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to cancel a previously sent required SI

Proposal 7:  The last relay UE in multihop can forward SI (e.g., in a UuMessageTransferSidelink) to an intermediate Relay upon:

•	acquisition of the SIB(s) requested (in a hop by hop manner) by a connected child node (intermediate node and/or remote UE)

•	reception of updates of any SIBs requested by a remote UE or another a child relay UE (in a hop-by-hop manner), including SIB1

•	deciding to perform unsolicited SIB1 forwarding

Proposal 8:  An intermediate relay UE that is RRC_CONNECTED uses Uu RRC signaling to obtain its system information directly from its connected cell (i.e., behaving as a remote UE).

Proposal 12:  The intermediate UE, when it receives SI from a parent relay, forwards the SI message only to the child UEs which requested the SI (i.e., the intermediate UE keeps track of the required SI for each child node).  FFS on the need for including a “remote UE ID” in the request.

Proposal 13:  The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list.

Proposal 14:  Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to request the required SI.

Proposal 15:  The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE that provides the SI to a child UE contains at least containers with SIB1 and other system information requested by the child UE.

Proposal 16:  Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides SI to the child UE.

Proposal 18:	The remote UE in multi-hop:

o	When RRC_IDLE/RRC_INACTIVE, can request to receive paging by sending its paging information using PC5-RRC signaling (e.g., RemoteUEInformationSidelink message)

o	When in RRC_IDLE/RRC_INACTIVE, can receive paging record from PC5-RRC signaling (e.g., UuMessageTransferSidelink)?

Proposal 19:	For a remote UE in multi-hop, the same triggers as Rel17 are supported for sending the PC5-RRC message (e.g., RemoteUEInformationSidelink) namely:

o	when there is a change in the paging information while in IDLE/INACTIVE, or when entering RRC_IDLE/RRC_INACTIVE

o	when it entering RRC_CONNECTED, a PC5-RRC message (e.g., RemoteUEInformationSidelink) is sent to release the paging information

Proposal 20:	The last relay UE in multihop can forward paging to an intermediate Relay upon receiving paging message related to a multihop remote UE, or intermediate relay UE.

Proposal 25:	When the intermediate UE receives a paging message from a parent relay on PC5, it forwards the paging message only to the remote UE/intermediate UE being paged or the intermediate UE serving a UE being paged.

Proposal 26:	The PC5-RRC message containing the paging information that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least paging ID and paging cycle of the remote UE and any serving (parent) intermediate relay UEs.

Proposal 27:	Re-use RemoteUEInformationSidelink as the PC5-RRC message transmitted by the remote UE or by the intermediate relay UE to the parent node (intermediate relay or last relay) to provide the paging information.

Proposal 28:	The PC5-RRC message transmitted by the last relay UE or by the intermediate relay UE contains at least one or multiple paging record(s) associated with intermediate relay UE(s) and/or remote UE(s).

Proposal 29:	Re-use UuMessageTransferSidelink as the PC5-RRC message transmitted by the Last relay or by the intermediate relay UE that provides paging record to the child UE(s).

Proposals requiring further discussion (more than one company having different views on some aspects)

Proposal 9:  When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly from SIB broadcast by the cell on Uu (when the the intermediate relay UE is in coverage).

Proposal 10:  The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay):

•	when there is a change in the SI required by the intermediate UE

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel a previously sent SI request)

•	upon reception of new/changed required SI received from a remote UE/child relay UE

•	FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

Proposal 11:  The intermediate relay UE can send SI (e.g., in UuMessageTransferSidelink) to a child node:

a)	Upon reception of SI received from a parent node (intermediate relay or last relay) containing SI requested by a child node (intermediate relay or remote UE)

b)	Upon acquisition (from the network) of SI requested by a child node (intermediate relay or remote UE)

c)	Upon receiving updated SIBs from the network which have been requested by a child node (intermediate relay or remote UE)

d)	Upon reception of SIB1 received from a parent relay (i.e., this case may correspond to SIB1 update detected by the last relay, or unsolicited SIB1 forwarding by the last relay)

e)	Upon receiving updated SIB1 from the network (as in Rel17)

f)	Upon unsolicited SIB1 forwarding to a connected child node (intermediate relay UE or remote UE)

FFS if b), c), and e) are limited to RRC_CONNECTED relay UE or apply in all states.  FFS whether to consolidate some of the conditions in stage 3 specification text.

Proposal 17:	RAN2 discusses/confirms that if multiple remote UEs connected to a single intermediate relay UE, the multiple remote UEs must connect to the same cell.

Proposal 21:	An intermediate relay UE in RRC_CONNECTED releases its paging-related information in the parent UE at least for itself.  FFS whether it also releases it for the child UEs and how the child UEs receive the paging when the intermediate relay UE is in RRC_CONNECTED.

Proposal 22:	When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain paging from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink).  FFS if it can also receive it directly on Uu (when the the intermediate relay UE is in coverage).

Proposal 23:	What triggers the intermediate relay UE to request paging monitoring by the parent relay (intermediate relay or last relay) in PC5-RRC (e.g., in RemoteUEInformationSidelink)?

•	when there is a change in the paging information of the intermediate UE or child UE

•	when the intermediate UE enters RRC_IDLE/RRC_INACTIVE

•	when the intermediate UE enters RRC_CONNECTED (to cancel paging monitoring request)

•	upon reception of paging monitoring request from a remote UE/child relay UE?

•	FFS upon change in the ability of the intermediate UE to monitor paging on Uu (e.g., moving in/out of coverage) to initiate/cancel paging monitoring by the parent relay).

Proposal 24:	The intermediate relay UE sends paging message (e.g., in UuMessageTransferSidelink) to a child node

•	Upon reception of paging message received from a parent node (intermediate relay or last relay) that is intended for a child node (intermediate relay or remote UE)

•	FFS upon acquisition (from the network) of paging message that is for a child node (intermediate relay or remote UE)

P1-P3 (baseline/Approach 2)

R2-2500864	Discussion on control plane procedures	Ericsson, Apple, AT&T, InterDigital Inc, FirstNet, Qualcomm Incorporated	discussion	Rel-19	NR_SL_relay_multihop

Proposal 1	RAN2 to further work on the approach that allows a RRC_IDLE/RRC_INACTIVE intermediate UE in the path, aiming for limited additional design complexity and specification efforts for RAN2.

Proposal 2	RAN2 to work on mechanisms to allow an intermediate relay UE to stay in RRC_INACTIVE when the remote UE triggers RRC connection establishment.

Proposal 3	RAN2 to send an LS to SA2 and SA3 to query whether there is any authorization concern to support an intermediate relay UE to stay in RRC_IDLE when the remote UE triggers RRC connection establishment.

P1 (message forwarding before transition to connected)

R2-2500934	Control Plane under multihop L2 U2N relaying 	Kyocera	discussion

Proposal 1	The intermediate relay UE should be allowed to forward the remote UE’s RRC Setup Request in case while it is still in RRC IDLE/INACTIVE.

P5b-P5c, P7 (details of setup procedure, local ID allocation)

R2-2500701	Control plane procedures for multi-hop relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 5b. The Last Relay UE cannot send the SUI on behalf of all other relay UEs. Each relay UE shall report the SUI and indicate the L2 ID of its adjacent child UE to the gNB.

Proposal 5c. RAN2 can discuss the necessity to enable the parallel RLC channel establishment for relaying of SRB1 at each relay UE.

Proposal 7: Reusing the single-hop relay mechanism to support the Local ID allocation for multi-hop relay:

•	Parent relay UE of the remote UE reports the L2 ID to the gNB to request the local ID allocation, the uniqueness of the local ID can be guaranteed by the gNB by implementation.

•	The remote UE local ID is 8 bits.

P1-P2, P11-P12 (protocol stacks, RLC channel configuration)

R2-2500898	Discussion on control plane procedures for multi-hop SL Relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop

Proposal 1.	Adopt the following CP and UP protocol stacks for multi-hop L2 U2N relay.

Proposal 2.	Capture following protocol stack for discovery:

Proposal 11.	In multi-hop L2 U2N relay, SL-RLC0/1 defined in R17 can be reused for multi-hop remote UE’s SRB0/1 forwarding over the first PC5 hop. PC5 RLC channels for remote UE’s SRB2/DRBs forwarding over the first PC5 hop are configured by gNB via dedicated signalling.

Proposal 12.	For each intermediate PC5 hops, the PC5 RLC channels used to forward remote UE’s SRBs/DRBs are configured by gNB via dedicated signalling.

P11-P15 (RLF handling)

R2-2500188	Discussion on the Control Plane Procedures	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 11: In approach 1, when Remote UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Remote UE.

Proposal 12: In approach 1, when the first Relay UE detects PC5 RLF between the Remote UE and the first Relay UE, its behaviors are same as Rel-17 U2N Relay .

Proposal 13: In approach 1, when the first Relay UE detects PC5 RLF between the first Relay UE and Intermediate Relay/last Relay UE, it can send NotificationMessageSidelink to the Remote UE to indicate the failure between first Relay UE and Intermediate Relay/last Relay UE.

Proposal 14: In approach 1, when the last Relay UE detects PC5 RLF, its behaviors are same as Rel-17 U2N Relay UE.

Proposal 15: In approach 1, when the last Relay UE detects Uu RLF, it can send NotificationMessageSidelink to its child Relay UE, and its child Relay UE can forward it to the Remote UE.

P16-P17 (further details on notification message)

R2-2501184	discussion on C-plane procedure for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 16. NotificationMessageSidelink should be sent hop-by-hop if needed.

Proposal 17. RAN2 discusses whether the remote UE including intermediate UEs should perform RRC re-establishment (, unicast link release or cell re-selection) upon reception of NotificationMessageSidelink.

SRAP aspects (TBD following offline)

Other contributions

R2-2500194	Control plane procedures of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500300	Control Plane aspects for Multi-hop Relay	NEC	discussion	Rel-19	NR_SL_relay_multihop

R2-2500308	Consideration on CP issues for multi-hop SL relay	Samsung	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500433	Discussion on Control Plane for Multi-hop UE-to-NW Relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2500434	Discussion on SRAP for Multi-hop Layer-2 U2N Relay	Apple	discussion	Rel-19	NR_SL_relay_multihop

R2-2500498	Control Plane Handling for Multi-hop U2N Relays	InterDigital	discussion	Rel-19	NR_SL_relay_multihop

R2-2500509	Discussion on the control plane procedure for multi-hop U2N relay	LG Electronics Inc.	discussion

R2-2500561	Discussion on control plane aspects for NR sidelink multi-hop relay	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500633	Control plane in Multi-hop relay	Lenovo	discussion	Rel-19

R2-2500913	SRAP design for R19 multi-hop SL relaying	Samsung R&D Institute UK	discussion

R2-2500953	Discussion on control plane procedure for SL multi-hop relay	KT Corp.	discussion

R2-2501119	Discussion on CP and SRAP impact for baseline procedure	vivo	discussion	Rel-19

R2-2501260	Open issue for control plane approach 1	Qualcomm Incorporated	discussion	NR_SL_relay_multihop-Core

Withdrawn/Not available

R2-2500725	SRAP impacts on MH relay	Nokia	discussion	NR_SL_relay_multihop	R2-2410290	Withdrawn

### 8.13.4	Service continuity

First priority scenarios: (A) intra-gNB multi-hop indirect to direct path switch, (B) intra-gNB multi-hpo indirect to single-hop indirect path switch.  Second priority scenarios: (C) intra-gNB direct to multi-hop indirect path switch, (D) intra-gNB single-hop indirect to multi-hop indirect path switch.

Cases A/B (m2x, higher priority):

P1-P2, P5-P6 (measurement report contents)

R2-2501120	Discussion on Service continuity for multi-hop relay	vivo	discussion	Rel-19

Proposal 1: For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 2: For multi-hop i2d, first relay UE/ Intermediate relay UE evaluates adjacent relay link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

Proposal 5: For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information.

Proposal 6: For multi-hop i2i, first/ intermediate relay UE evaluates adjacent relay links, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result.

P1-P2 (reuse of reconfigurationWithSync)

R2-2500733	Considerations on Service Continuity of Multi-hop Relay	NEC	discussion

Proposal 1:	Using reconfigurationWithSync without sl-PathSwitchConfig for path switching of Scenario A and an added step to release the first relay UE of the source indirect path.

Proposal 2:	Using reconfigurationWithSync including sl-PathSwitchConfig for path switching of Scenario B and an added step to release the first relay UE of the source indirect path.

P2 (reconfiguration of relays)

R2-2500927	Intra-gNB Service Continuity for Multi-Hop Relays	Ericsson	discussion	Rel-19

Proposal 2	For both mh2d and mh2i, it is up to network implementation when the first/last/intermediate relay UEs are reconfigured after the path switch command is sent to the remote UE.

Cases C/D (x2m, lower priority):

P2, P4 (events for cases C and D)

R2-2500562	Discussion on service continuity for multi-hop relay	China Telecom	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 2: Event Y1/ Y2 could be used for intra-gNB direct to multi-hop indirect path switching. The PC5 quality between the remote UE and candidate U2N relay/candidate first relay is considered for evaluating Event Y1/ Y2.

Proposal 4: Event Z1/ Y2 could be used for intra-gNB multi-hop indirect to direct path switching. The remote UE shall consider the first relay as serving L2 U2N relay for evaluating Event Z1.

P2, P6 (no special handling for relays already in path)

R2-2500193	Service continuity of multi-hop U2N relay	OPPO	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 2	RAN2 to confirm for scenario B, no spec impact is needed for the remote UE to identify whether the U2N Relay at the target side is the intermediate or last relay at the source side.

Proposal 6	RAN2 to confirm for scenario D, no specific handling is needed for the case that the either intermediated relay or last relay at the target side is the U2N relay UE at the source side compared with the normal case.

P3 (procedure for C), P6-P7 (message contents for C), P10-P12 (procedure and message contents for D)

R2-2500189	Intra-gNB Service Continuity for Multi-hop U2N Relay	CATT	discussion	Rel-19	NR_SL_relay_multihop-Core

Proposal 3: Suggest RAN2 to adopt Figure-2 as the baseline signaling flow for intra-gNB direct to multi-hop indirect path switching.

Proposal 6: The UE will perform the legacy measurement event Y1/Y2 except the description on candidate L2 U2N Relay UE in event Y1/Y2 should be revised to include the candidate First Relay UE.

Proposal 7: For Scenario C, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

Proposal 10: Suggest RAN2 to adopt Figure-3 as the baseline signaling flow for intra-gNB single-hop indirect to multi-hop indirect path switching.

Proposal 11: The UE will perform the legacy measurement event X2/Y2/Z1 except the description on candidate L2 U2N Relay UE in event Y2/Z1 should be revised to include the candidate First Relay UE.

Proposal 12: For Scenario D, the network will send RRC reconfiguration message to the target Last/Intermediate/First Relay UE, which includes at least the Ingress/Egress RLC channel and mapping configuration for L2 U2N Remote UE.

P2 (state of target relay UE), P3 (T420 reuse)

R2-2500634	Service continuity for Multi-hop system	Lenovo	discussion	Rel-19

Proposal 2: RAN2 confirm that the target relay UE as first intermediate relay UE should be RRC connected state for the path switch towards multi-hop indirect path.

Proposal 3: Timer T420 can be reused for path switching towards multi-hop indirect path.

R2-2500421	RRC states of candidate Relay UEs for path switching	ASUSTeK	discussion	Rel-19	NR_SL_relay_multihop

R2-2500510	Discussion on service continuity for multi-hop U2N relay	LG Electronics Inc.	discussion

R2-2500702	Discussion on service continuity for Multi-hop Relay	Huawei, HiSilicon	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500906	Discussion on service continuity for multi-hop SL relay	ZTE Corporation, Sanechips	discussion	Rel-19	NR_SL_relay_multihop-Core

R2-2500935	Service Continuity for U2N multihop relay 	Kyocera	discussion

R2-2501185	discussion on service continuity for multi-hop relay	Sharp	discussion	Rel-19	NR_SL_relay_multihop-Core

## 8.15	NavIC L1 SPS A-GNSS support

(LCS_NAVIC_L1_SPS_NR_LTE-Core; leading WG: RAN2; REL-19; WID RP-243247)

Time budget: 0.5 TU

Tdoc Limitation: 1 tdoc

Including outcome of email discussion [Post128][403][POS] NavIC L1 stage 3 CR check (Reliance Jio)

Email discussion report

R2-2500361	[Post128][403][POS] NavIC L1 stage 3 CR check (Reliance Jio)	Reliance Jio	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core	Late

Rapporteur CR

R2-2500108	Introduction of NavIC L1 SPS A-GNSS in LPP	Reliance Jio, ISRO, Ericsson, MediaTek, CEWiT	CR	Rel-19	37.355	18.4.0	0532	1	B	LCS_NAVIC_L1_SPS_NR_LTE-Core	R2-2409726	Late

Other contributions

R2-2500811	Discussion on the support of NavIC L1 SPS	Huawei, HiSilicon	discussion	Rel-19	LCS_NAVIC_L1_SPS_NR_LTE-Core

R2-2500972	NavIC broadcast ephemeris to refer to for GNSS SSR corrections	Ericsson, Reliance Jio	discussion

## 8.16	BDS B2b in A-GNSS

LCS_BDS_B2b_LTE_NR; leading WG: RAN2; REL-19; WID RP-242459)

Time budget: 0.25 TU

Tdoc Limitation: 1 tdoc

Including outcome of email discussion [Post128][402][POS] BDS B2b stage 3 CR check (CATT)

Email discussion report

R2-2500278	[Post128][402][POS] BDS B2b stage 3 CR check (CATT)	CATT	discussion	Rel-19	LCS_BDS_B2b_LTE_NR

Rapporteur CR

R2-2500279	Introduction of B2b signal in BDS system in A-GNSS	CATT, CAICT, Ericsson, Huawei, HiSilicon	CR	Rel-19	37.355	18.4.0	0545	-	B	LCS_BDS_B2b_LTE_NR

Other contributions

R2-2500810	Discussion on the remaining issues for BDS B2b	Huawei, HiSilicon	discussion	Rel-19	LCS_BDS_B2b_LTE_NR

R2-2500973	Remaining issues of BDS B2b Signal Addition	Ericsson	discussion

## 8.18	TEI19

Time budget: 1 TU

Tdoc Limitation: 1 tdoc for new proposals and 1 tdoc for old proposals.

1 additional tdoc for primary co-sourcing company on top of the limit is allowed for co-sourced contribution with 4 or more companies.

Companies are encouraged to submit co-sourced contributions, which will have priority for discussion in RAN2#129.

Positioning: on-demand posSIB

R2-2500820	Discussion on the control parameters for on-demand posSIB request [PosOdSIB-Req]	Huawei, HiSilicon, CATT, Ericsson, Qualcomm	discussion	Rel-19	TEI19

Positioning: coordinate reference systems

R2-2500970	Introduction of Location Coordinate Reference Systems	Ericsson, AT&T, FirstNet, Deutsche Telekom, MediaTek Inc.	discussion

Revised in R2-2501329

R2-2501329	Introduction of Location Coordinate Reference Systems	Ericsson, AT&T, FirstNet, Deutsche Telekom, MediaTek Inc., ESA	discussion

## 8.19	NR Others

Tdoc limit: 1

Contributions addressing LS from RAN4 R4-2420410 and any RAN4 LSs not related to any of the AIs above.

R2-2500047	LS on Location service of UEs served by MWAB (S2-2412625; contact: Huawei)	SA2	LS in	Rel-19	VMR_Ph2	To:RAN3	Cc:RAN2