3GPP TSG-RAN WG2 Meeting #129                                                               R2-2500077

Athens, Greece, February 17th– 21st, 2025

Agenda item:	7.3.2

Source:	vivo

Title:	Discussion on SA2 LS on UE Location Information for NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

Due to the lack of AS security, the UE coarse location for NB IoT UE is reported via NAS procedure and then based on eNB request, the MME provides the corresponding UE coarse location to eNB [1].

RAN3#125bis RAN3 agrees for NB-IoT NTN UE, the coarse UE location reporting from MME to eNB is performed upon eNB request, per UE, and only once for UE during RRC connected state. Based on eNB request, MME will provide Coarse UE Location Information to the eNB for a given UE.

However, the UE coarse location request and the response between eNB and MME are only supported once at the initial access stage. It is asked by SA2 whether the coarse location acquisition is beneficial for UE in the later state after initial access, i.e., S1-U bearer establishment for CP CIoT, MT-EDT for UP CIoT, and connection resume [2].

SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

In the contribution, we will analyze the issue and give our views.

- Discussion

Currently, UE coarse location can only be reported during the initial access. However, there are also some NB-IoT device that are not stationary, i.e., smart wearable devices. The updated UE coarse location reporting to the network is useful,  especially from the network perspective. There are some use cases where the updated UE coarse location can be useful.

Case 1: the network can use the coarse UE location to decide the load control

When the load for the network is high, to make the network capacity usage more efficient, the network can decide to prioritize the UE near the network center, and release the UE in the cell edge to the IDLE state or redirect the UE in the cell edge to other carriers.

Case 2: the network can use the coarse UE location to  perform satellite beam management

Based on the updated coarse UE location, network or satellite can decide how to redistribute the satellite beam, or adjust the satellite beam angle, etc., to make the UE in good coverage.

Observation 1: From RAN2 perspective, the latest UE position may be useful to eNB, e.g., for load control, for satellite beam management.

Moreover, the procedure, e.g., “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation”, “Connection Resume procedure” and “MT-EDT procedure for User Plane CIoT EPS Optimisation”, indicated by SA2 in the LS are performed after the AS security is activated. Thus, it is feasible for UE to report the coarse location via the AS procedure to eNB.

Observation 2: It is feasible for UE to report the UE coarse location via the AS solution in the procedure as mentioned by SA2 in LS.

Moreover, currently, the spec supports the UE coarse location report for the eMTC scenario via the RRC procedure [3], i.e., the NW requests the UE coarse location via the UEInformationRequest message, and then UE reports the coarse location via the UEInformationResponse message.

UEInformationRequest-v1710-IEs ::= SEQUENCE { coarseLocationReq-r17 ENUMERATED {true} OPTIONAL, -- Need ON nonCriticalExtension UEInformationRequest-v1800-IEs OPTIONAL } Skip unrelated part…… UEInformationResponse-v1710-IEs ::= SEQUENCE { coarseLocationInfo-r17 OCTET STRING OPTIONAL, nonCriticalExtension UEInformationResponse-v1800-IEs OPTIONAL }

If there is a need to support such the coarse UE location report, a similar procedure can be reused for NB-IoT.

However, since the procedure referred by SA2 is related to the NG procedure and is within RAN3's scope, the final decision on whether to support the coarse UE location report and the updated coarse UE location report is up to RAN3's decision.

Proposal 1: From RAN2 perspective, the updated UE coarse location report is feasible. The final decision is up to RAN3.

Proposal 2: If the updated UE coarse location report should be supported for NB-IoT, a similar method like eMTC can be reused, i.e, NW requests the UE coarse location via the UEInformationRequest-NB message, and then UE report the coarse location via the UEInformationResponse-NB message.

- Conclusion

In this contribution, we have further discussed the updated UE coarse location report issue. And we make the following observations and proposals:

Observation 1: From RAN2 perspective, the latest UE position may be useful to eNB, e.g., for load control, for satellite beam management.

Observation 2: It is feasible for UE to report the UE coarse location via the AS solution in the procedure as mentioned by SA2 in LS.

Proposal 1: From RAN2 perspective, the updated UE coarse location report is feasible. The final decision is up to RAN3.

Proposal 2: If the updated UE coarse location report should be supported for NB-IoT, a similar method like eMTC can be reused, i.e, NW requests the UE coarse location via the UEInformationRequest-NB message, and then UE report the coarse location via the UEInformationResponse-NB message.

- Reference

- draft_MeetingReport_RAN3#125-bis_vOct25.

- S2-2412665, Reply LS on UE Location Information for NB-IoT NTN.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.