3GPP TSG-RAN WG2 Meeting #129                                                                   R2-2500078

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.4.2

Source:	vivo

Title:	Discussion on RAN4 LS on Simultaneous Operation between GNSS and NR NTN

Document for:	Discussion and Decision

- Introduction

RAN4 sent an LS to ask for introducing improvements, if necessary, to existing IDC solution(s) and related IDC capabilities to address the in-device coexistence issue between GNSS reception and NR NTN transmission [1].

In this contribution, we would like to discuss the RAN2 understanding of IDC solutions for the support of simultaneous operation between GNSS and NR NTN.

- Discussion

In order to resolve the UE’s IDC problem/internal interference (i.e. from the UE’s Tx to the UE’s Rx) between NR and other RATs (e.g. GPS), NR has introduced the IDC enhancement with using FDM solution, TDM solution, and autonomous denial solution.

  - 2.1	FDM solution

For the FDM solution, when the gNB enables the FDM solution for the UE via RRC configuration, the UE is allowed to report its affected NR frequency(ies), which can be either attacking frequency or victim frequency, via UEAssistanceInformation. When the affected NR frequency is the attacking frequency, the UE can indicate the victim system type (e.g. GPS and GLONASS) of other RATs. When the interference type from NR to other RATs is harmonic interference, the UE can indicate a list of affected NR frequencies, and each entry of the list includes only one NR frequency and the bandwidth around the NR frequency. When the interference type from NR to other RATs is the IMD (Inter-Modulation Distortion) interference, the UE can indicate a list of affected NR frequency combinations, and each NR frequency combination includes more than one NR frequencies and the bandwidth around the NR frequency. It is up to the network implementation on how to use the frequency information reported by the UE.

In our understanding, the existing FDM solution can be reused for the simultaneous operation between GNSS and NR NTN case. For example, upon detecting IDC problem, the UE can report the corresponding NR frequency (either attacking frequency or victim frequency) information to the network. Then the network, in turn, may reconfigure the BWP or hand over the UE to another cell of a different frequency. Moreover, we fail to see the motivation to do further enhancement since all the necessary frequency-related parameters (e.g. affected frequency/frequency combinations/affected bandwidth) have been specified.

Based on the above, RAN2 can conclude it is not necessary to consider any enhancement to IDC FDM solution for the simultaneous operation between GNSS and NR NTN.

Proposal 1: For simultaneous operation between GNSS and NR NTN, RAN2 concludes that it is not necessary to consider any enhancement to the existing IDC FDM solution.

  - TDM solution & Autonomous denial solution

For the TDM solution, when the gNB enables the TDM solution for the UE via RRC configuration, the UE is allowed to report its preferred TDM pattern, which includes the UE’s preference on the active duration, cycle and starting offset for the affected frequency. It is up to the network implementation on how to stop/start scheduling the UE’s transmission/reception for the affected frequency, by using the TDM pattern reported by the UE.

For simultaneous operation between GNSS and NR NTN, to resolve the potential IDC issue, the UE should indicate a TDM pattern that covers at least the GNSS fix duration and GNSS update period, as illustrated in Figure 1. The existing TDM solution can be reused but the existing TDM assistance parameters might not be suitable for some cases of the GNSS implementation. For example, if the UE is moving at low or medium speed (e.g. 80 km/h or lower value), the UE does not need to update GNSS frequently. The UE may update it every 40 seconds, with which the UE can sufficiently guarantee the frequency pre-compensation accuracy (i.e. controlling the frequency pre-compensation error within 0.03ppm (about 630 Hz frequency offset at 2.1GHz with 15kHz SCS) during the 40s period [2]).

Figure 1: Time pattern of simultaneous operation between GNSS and NR NTN

Specifically, the candidate values of  cyclelength-r18 and activeDuration-r18 seem a bit small for GNSS implementation.

IDC-TDM-Assistance-r18 ::=            SEQUENCE {

cycleLength-r18                       ENUMERATED {ms2, ms3, ms4, ms5, ms6, ms7, ms8, ms10, ms14, ms16, ms20, ms30,

ms32, ms35, ms40, ms60, ms64, ms70, ms80, ms96, ms100, ms128, ms160,

ms256, ms320, ms512, ms640, ms1024, ms1280, ms2048, ms2560, ms5120, ms10240},

startOffset-r18                       INTEGER (0..10239),

slotOffset-r18                        INTEGER (0..31),

activeDuration-r18                    CHOICE {

subMilliSeconds INTEGER (1..31),

milliSeconds    ENUMERATED {

ms1, ms2, ms3, ms4, ms5, ms6, ms8, ms10, ms20, ms30, ms40, ms50, ms60,

ms80, ms100, ms200, ms300, ms400, ms500, ms600, ms800, ms1000, ms1200,

ms1600, spare8, spare7, spare6, spare5, spare4, spare3, spare2, spare1 }

},

...

}

We think if enhancement is needed, RAN2 should consider extending the value range of cyclelength-r18 and activeDuration-r18. For example, cycle length with values of 40s, 80s, and 90s can be considered and active duration with values of 10s, 20s, and 60s etc, considering the specified range of GNSS fix duration is [1s, 31s].

-- ASN1START

GNSS-PositionFixDuration-r18 ::= ENUMERATED{

s1, s2, s3, s4, s5, s6, s7, s13, s19, s25, s31}

-- ASN1STOP

Moreover, considering the Rel-18 spec has been frozen for a while, it is not preferable to introduce new TDM parameters for some simultaneous operation implementation cases between GNSS and NR NTN in the Rel-18 stage. Since there existing real implementation issues in field tests, RAN2 should consider doing some enhancements for the IDC TDM solution in TEI19 and further check with RAN4 whether introducing extended values for TDM assistance parameters is beneficial and sufficient for resolving the mentioned IDC issue.

For the autonomous denial solution, the gNB can provide the autonomous denial configuration to the UE per cell group. Based on the autonomous denial configuration, the UE is allowed to deny the uplink transmission in some slots for a cell group (e.g. MCG or SCG), without exceeding the maximum number of uplink transmission slots permitted by the network. The UE by implementation can decide which uplink transmission can be denied so that the interference to other RATs can be mitigated.

For simultaneous operation between GNSS and NR NTN, to resolve the potential IDC issue, the UE with autonomous denial configuration may deny some UL transmissions within the denial validity period when updating the GNSS position, as illustrated in Figure 2.

Figure 2: Autonomous denial for simultaneous operation between GNSS and NR NTN

Similar to IDC TDM solutions, the existing autonomous denial solution can be reused but the existing related parameters might not be suitable for some cases of the GNSS implementation. For example, the maximum value range of autonomousDenialValidity and autonomousDenialSlots cannot fully cover the duration of GNSS measurement gap. As a result, in some cases, the existing IDC autonomous denial solution cannot completely resolve the IDC issue. To this end, RAN2 may consider doing some enhancements for the IDC autonomous denial solution in TEI19 and further check with RAN4 whether introducing extended values for TDM autonomous denial parameters is beneficial and sufficient for resolving the mentioned IDC issue.

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

Based on the above, we have the following observation and proposals,

Observation: The candidate values of  cyclelength-r18, activeDuration-r18, autonomousDenialValidity-r18, and autonomousDenialSlots-r18 seem a bit small for representing the GNSS implementation.

Proposal 2: For simultaneous operation between GNSS and NR NTN, RAN2 concludes that enhancement to the existing IDC TDM solution or IDC autonomous denial solution is not considered in Rel-18.

Proposal 3: For simultaneous operation between GNSS and NR NTN, RAN2 can work on value range extension of TDM assistance parameters or autonomous denial parameters as part of TEI19 work (check with RAN4 whether value range extension is beneficial and sufficient).

3 Conclusion

In this contribution, we have provided our understanding of IDC solutions for the support of simultaneous operation between GNSS and NR NTN. And we make the following observation and proposals,

FDM solution:

Proposal 1: For simultaneous operation between GNSS and NR NTN, RAN2 concludes that it is not necessary to consider any enhancement to the existing IDC FDM solution.

TDM solution & Autonomous denial solution:

Observation: The candidate values of  cyclelength-r18, activeDuration-r18, autonomousDenialValidity-r18, and autonomousDenialSlots-r18 seem a bit small for representing the GNSS implementation.

Proposal 2: For simultaneous operation between GNSS and NR NTN, RAN2 concludes that enhancement to the existing IDC TDM solution or IDC autonomous denial solution is not considered in Rel-18.

Proposal 3: For simultaneous operation between GNSS and NR NTN, RAN2 can work on value range extension of TDM assistance parameters or autonomous denial parameters as part of TEI19 work (check with RAN4 whether value range extension is beneficial and sufficient).

4 Reference

[1] R2-2500033, LS on simultaneous operation between GNSS and NR NTN, RAN4.

[2] R4-2419063, Discussion on the NR NTN simultaneous operation with GNSS, Huawei, HiSilicon.