3GPP TSG-RAN WG2 Meeting #129                                                               R2-2500081

Athens, Greece, February 17th– 21st, 2025

Agenda item:	8.8.6

Source:	vivo

Title:	Discussion on Redirection from E-UTRA TN to NR-NTN

Document for:	Discussion and Decision

- Introduction

In the LTE TN to NR NTN idle mobility discussion in RAN2#127bis [1], whether to support the redirection from LTE TN to NR NTN was discussed. However, due to some controversy, the issue was postponed.

Proposal 3: RAN2 confirms the understanding that dedicated NR NTN frequency is not configured in the RRCConnectionRelease message, and confirms that this can be left to NW implementation (w/o Spec impact). - QC is ok with this but thinks it’s fine to clarify this somewhere - Samsung thinks we should separate the case of providing dedicated frequencies from the redirection case. Xiaomi agrees, redirection could be performed providing assistance information in the release message. LG shares similar view as Samsung but thinks the NW does not need to provide assistance info - China Telecom thinks satellite information would be needed to support redirection to NR NTN - QC thinks that either we go for proposal 3 or we need to introduce a UE capability for this. Come back in the next meeting

In RAN2#128 [2], the issue was discussed and the following agreements were made. However, the UE behaviour is still unclear.

RAN2 understands that the UE behaviour is currently not clear if the network includes a NR NTN frequency in redirectedCarrierInfo in the RRCConnectionRelease message in a TN cell. We can further discuss whether we want to clarify the UE behaviour and in case whether we need to introduce a new UE capability.

In the contribution, we will analyse the issue and give our view.

- Discussion

According to TS 36.331 [3], the NR frequency can be configured by NW in the RRCConnectionRelease message [3], and there is no restriction on whether the NR frequency is for TN or NTN. This also aligns with the agreements made in the last meeting [2].

RedirectedCarrierInfo ::= CHOICE { eutra ARFCN-ValueEUTRA, geran CarrierFreqsGERAN, utra-FDD ARFCN-ValueUTRA, utra-TDD ARFCN-ValueUTRA, cdma2000-HRPD CarrierFreqCDMA2000, cdma2000-1xRTT CarrierFreqCDMA2000, ..., utra-TDD-r10 CarrierFreqListUTRA-TDD-r10, nr-r15 CarrierInfoNR-r15, nr-r17 CarrierInfoNR-r17 }

Observation 1: Up to implementation, NW configures the NTN frequency within the redirection information within the RRCConnectionRelease message.

Besides, according to RAN4 spec [4] [5], the TN carrier frequency may overlap with the NTN frequency range. And since the network only provides the NR frequency range to UE in the redirection information, UE may not distinguish whether this is an NR TN frequency or an NR NTN frequency.

Table 5.2-1: NR operating bands in FR1 NR operating band; Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high; Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high; Duplex Mode. n1; 1920 MHz – 1980 MHz; 2110 MHz – 2170 MHz; FDD. n2; 1850 MHz – 1910 MHz; 1930 MHz – 1990 MHz; FDD. n3; 1710 MHz – 1785 MHz; 1805 MHz – 1880 MHz; FDD. n5; 824 MHz – 849 MHz; 869 MHz – 894 MHz; FDD. n7; 2500 MHz – 2570 MHz; 2620 MHz – 2690 MHz; FDD. n8; 880 MHz – 915 MHz; 925 MHz – 960 MHz; FDD. n12; 699 MHz – 716 MHz; 729 MHz – 746 MHz; FDD. n13; 777 MHz – 787 MHz; 746 MHz – 756 MHz; FDD. n14; 788 MHz – 798 MHz; 758 MHz – 768 MHz; FDD. n18; 815 MHz – 830 MHz; 860 MHz – 875 MHz; FDD. n20; 832 MHz – 862 MHz; 791 MHz – 821 MHz; FDD. n2416; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. Skip unrelated part…… Table 5.2.2-1: NTN satellite bands in FR1-NTN NTN satellite operating band; Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high; Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high; Duplex mode. n256; 1980 MHz – 2010 MHz; 2170 MHz – 2200 MHz; FDD. n255; 1626.5 MHz – 1660.5 MHz; 1525 MHz – 1559 MHz; FDD. n254; 1610 – 1626.5 MHz; 2483.5 – 2500 MHz; FDD. NOTE: NTN satellite bands are numbered in descending order from n256..

Observation 2: UE cannot distinguish whether the redirection frequency is an NR TN frequency or an NR NTN frequency.

However, NR TN measurements and NR NTN measurements are quite different from the UE perspective, i.e., there are different UE requirements, and also UE may do some pre-compensation either based on ephemeris information or the implementation.

Observation 3: There are different UE requirements or behaviours for TN measurements and NTN measurements.

Besides, since there is already the ephemeris information associated with the NR NTN frequency info in SIB24 after we finish the LTE TN to NR NTN topic. The simple solution is if UE finds the frequency in the redirection information is also present in SIB24 and also it is associated with the ephemeris information, UE can consider it as the NR NTN frequency. Otherwise, as we do for SIB3-NB, SIB5-NB and SIB24-NB, UE just considers the frequency as TN frequency.

Proposal 1: If UE finds the frequency in the redirection information is the same as the frequency linked with satellite assistance information in SIB24, UE considers it as an NTN frequency and reuses the corresponding satellite assistance information for measurements. Otherwise, UE considers the frequency in the redirection information as a TN frequency.

Besides, to enable the network to decide how to set the frequency and whether to support the redirection from LTE TN to NR NTN, a signalled UE capability for redirection can be supported.

Proposal 2: A signalled UE capability on whether to support the redirection from LTE TN frequency to NR NTN frequency is introduced.

- Conclusion

In this contribution, we have further discussed the redirection issue from LTE TN to NR NTN. And we make the following observations and proposals:

Observation 1: Up to implementation, NW configures the NTN frequency within the redirection information within the RRCConnectionRelease message.

Observation 2: UE cannot distinguish whether the redirection frequency is an NR TN frequency or an NR NTN frequency.

Proposal 1: If UE finds the frequency in the redirection information is the same as the frequency linked with satellite assistance information in SIB24, UE considers it as an NTN frequency and reuses the corresponding satellite assistance information for measurements. Otherwise, UE considers the frequency in the redirection information as a TN frequency.

Proposal 2: A signalled UE capability on whether to support the redirection from LTE TN frequency to NR NTN frequency is introduced.

- Reference

- R2-2409213, Report from Break-out session on NR-NTN and IoT-NTN.

- Draft_RAN2_128_Meeting_Report_v1.

- TS36.331, Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification.

- 38.101-1, NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone.

- 38.101-5, NR; User Equipment (UE) radio transmission and reception; Part 5: Satellite access Radio Frequency (RF) and performance requirements.