3GPP TSG-RAN WG2 Meeting #129	R2-2500092

Athens, Greece, February 17-21, 2025

Agenda item:	8.7.4.1

Source:	Qualcomm Incorporated

Title:	Discussion on LCP enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we follow up on the agreements made in RAN2#127-bis and discuss additional enhancements needed to support dynamic LCH priority.

# Discussion

In the evaluation of the priority adjustment criteria, it is self-evident that only data eligible for the UL grant, instead of all the data buffered for the LCH, should be considered. For example, if a PDCP SDU is already in an active HARQ process, then it is not considered in the LCP procedure and thus should not be included in the consideration of priority adjustment.

It would be good for RAN2 to confirm this aspect, since it has not been explicitly agreed.

Proposal 1. 	When a UE is scheduled for a new UL transmission, only data eligible for the UL grant are considered in the evaluation of the LCH priority adjustment criteria.

At RAN2#128, the following agreements on LCH prioritization were made:

- As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP

We think there are still potential fairness issues with the above agreements.

First, these agreements imply that there is no limit on how much data in an LCH may be given a higher priority. This lack of restriction can lead to reduced fairness for other LCHs.

For example, consider a configuration in which low bit rate flows (e.g. haptic and control) are mapped to LCH #1 and other high bit rate flows (e.g. video) are mapped to LCH #2. LCH #2 can adjust its priority to the same level as LCH #1 when it has priority-adjustable data.

Priority adjustment is most likely to occur when there is congestion, because data in low priority LCH can’t get enough UL grants to be scheduled in time. In addition, most UL grants during congestion may not be large enough to accommodate all data. In such cases, LCH #1 is penalized after LCH #2 adjusts its priority to the same level even in round 1. This happens because LCH #1 no longer has priority over LCH #2, even when its Bj is still positive (Otherwise, there would be no benefit in adjusting the priority of LCH #2).

Even when there is no congestion and sufficient UL resources are available, a high-priority LCH can still be affected, if a UE implementation does not support fallback to default priority during the second round. This happens because if a low-priority LCH continue to use the adjusted (higher) priority in the second round, the high-priority LCH has to share resources that it originally had strict priority over.

Observation 1.	Priority adjustment can penalize high-priority logical channels if applied without restriction.

The above analysis clearly indicates the need for restrictions on how often or how much priority adjustments an LCH may have. We think a simple yet effective approach is to limit the amount of data that can be adjusted to higher priority within a time window. For example, an LCH is allowed to adjust its priority at start of the LCP procedure only if the total amount of data sent with higher priority in the last T msec is less than a configured threshold. Of course this is only one of the possible options. Companies can discuss other possible approaches.

Proposal 2. 	At the start of LCP, an LCH with priority-adjustable data is allowed to apply its additional priority only if the total amount of data transmitted with the additional priority within the last T msec is less than a configured threshold.

In the LCH prioritization procedure, parameters such as Bj, PBR and BSD control how much data per unit time can be scheduled from an LCH. We do not think they should be adjusted together with the LCH priority, e.g. raise PBR to infinity. Otherwise, without the regulation by token bucket, extra data may be scheduled with higher priority unnecessarily, thus causing starvation for LCHs with lower priorities.

Observation 2.	Changing LCP parameters such as Bj, PBR or BSD in addition to priority adjustment can cause starvation for LCHs with lower priorities.

Proposal 3. 	When an LCH applies adjusted priority during LCP, other LCP parameters such as Bj, PBR and BSD are not adjusted.

There are also discussions on the scenario where two LCHs have equal priorities and one of them has adjusted priority (WLOG, let’s denote this LCH as LCH #2). In such cases, especially in round 2, we think the legacy rule still applies (i.e. the two LCHs should be served equally) and no additional differentiation between the two LCHs are needed. If one thinks that LCH #2 should be served first or more, then that means the additional priority of LCH #2 should have been set higher. This is an effect that can be achieved by proper network configuration instead. Similarly, if one thinks LCH #1 should be served first or more, then that means the additional priority of LCH #2 has been set too high. It can be fixed by proper network configuration too.

Proposal 4.	In the second round of LCP, if two LCHs apply the same priority, then they should be served equally as in legacy, regardless of whether the applied priority is the originally configured one or the additional priority.

Next, we discuss the impact of priority adjustment on other MAC procedures.

One of the issues is related to status reporting. If an LCH has priority-adjustable data, then the LCH applies its additional priority during LCP. To help gNB better align its UL grant allocation with the actual LCH priorities applied during the LCP procedure, gNB should be informed of the adjusted priority than the configured priority of an LCH. Therefore, when UE is sending a BSR or DSR MAC CE, an LCH should be included in the LCG based on its adjusted priority.

Proposal 5.	If an LCH has priority-adjustable data when a MAD PDU containing DSR or BSR MAC CE is assembled, the priority-adjustable data should be reported according to the adjusted LCH priority.

By the same argument, if an LCH has priority-adjustable data when it triggers an SR, UE should transmit the SR in the SR configuration corresponding to the adjusted priority of the LCH. This helps gNB be better informed in how UL grants should be scheduled.

Proposal 6.	If an LCH has priority-adjustable data when it triggers an SR, UE should transmit the SR in the SR configuration corresponding to the adjusted priority of the LCH.

There have been discussions on whether to use adjusted or configured priority during intra-UE prioritization. Given the significant efforts required for its implementation and testing (especially interoperability testing) and the low interest in its deployment, we believe it is better to leave this decision to UE implementation.

Proposal 7.	In intra-UE prioritization, it is up to UE implementation in determining the priority of a PUSCH transmission which contains data from an LCH that applied its additional priority during LCP.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1. 	When a UE is scheduled for a new UL transmission, only data eligible for the UL grant are considered in the evaluation of the LCH priority adjustment criteria.

Observation 1.	Priority adjustment can penalize high-priority logical channels if applied without restriction.

Proposal 2. 	At the start of LCP, an LCH with priority-adjustable data is allowed to apply its additional priority only if the total amount of data transmitted with the additional priority within the last T msec is less than a configured threshold.

Observation 2.	Changing LCP parameters such as Bj, PBR or BSD in addition to priority adjustment can cause starvation for LCHs with lower priorities.

Proposal 3. 	When an LCH applies adjusted priority during LCP, other LCP parameters such as Bj, PBR and BSD are not adjusted.

Proposal 4.	In the second round of LCP, if two LCHs apply the same priority, then they should be served equally as in legacy, regardless of whether the applied priority is the originally configured one or the additional priority.

Proposal 5.	If an LCH has priority-adjustable data when a MAD PDU containing DSR or BSR MAC CE is assembled, the priority-adjustable data should be reported according to the adjusted LCH priority.

Proposal 6.	If an LCH has priority-adjustable data when it triggers an SR, UE should transmit the SR in the SR configuration corresponding to the adjusted priority of the LCH.

Proposal 7.	In intra-UE prioritization, it is up to UE implementation in determining the priority of a PUSCH transmission which contains data from an LCH that applied its additional priority during LCP.