3GPP TSG-RAN WG2 Meeting #129	R2-2500093

Athens, Greece, February 17-21, 2025

Agenda item:	8.7.4.2

Source:	Qualcomm Incorporated

Title:	Discussion on DSR enhancements

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion and decision

# Introduction

In this paper, we discuss a few FFS issues on the enhanced DSR design. Namely, configuration aspects of reporting thresholds, format of the enhanced DSR MAC CE, and co-existence between Rel-18 and Rel-19 DSR MAC CEs.

# Discussion

At the RAN2#127-bis meeting, the following agreement on the coexistence between Rel-18 and Rel-19 DSR MAC CE was made:

- FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

If there is at least one LCG that is configured with multiple reporting thresholds, it implies that the network supports and prefers Rel-19 DSR. Therefore, a UE which is capable of Rel-19 DSR should prioritize the use of Rel-19 DSR MAC CE whenever possible.

Proposal 1.	If at least one LCG is configured with reporting threshold(s), the UE should use Rel-19 DSR MAC CE, as long as it can be accommodated by the available PUSCH resource.

However, if the UE is configured to use Rel-19 MAC CE but the available PUSCH resource is too small to include it completely, we think Rel-18 DSR MAC CE should be used instead, because it is still better to send some delay information than sending nothing. In that case, the delay information is calculated based on the triggering threshold, i.e. the reported buffer size equals the amount of delay critical data whose remaining time is below the triggering threshold of that LCG, same as in Rel-18.

Proposal 2.	If an available PUSCH resource is not large enough to accommodate a Rel-19 DSR MAC CE, a Rel-18 DSR MAC CE is sent instead, with the delay information calculated based on the trigger threshold.

As to the granularity of reporting thresholds, since different LCHs may be associated with QoS flows with different delay budgets, it makes sense to support different number of reporting thresholds for different LCGs.

Proposal 3.	Different LCGs may be configured with different number of reporting thresholds.

Next, we discuss the range of reporting thresholds that an LCG may have. First, if UE is configured to use R19 DSR, then technically speaking, an LCG may not need a reporting threshold, because the triggering threshold can be used to serve the purpose. However, this would complicate the spec in a number of places. To keep it simple, we propose that any LCG with a triggering threshold should have at least one reporting threshold.

In addition, we think the minimum number of reporting thresholds for an LCG can be one. For example, network can set the reporting threshold to match the triggering threshold. Since RAN2 already has made the following agreement at RAN2#128:

- Let the network configure the triggering and reporting thresholds without constraints

we think the configuration of reporting thresholds do not need to depend on the value of triggering threshold. It can be left to network to decide the number and value of reporting thresholds. When reporting, UE simply reports delay information based on whatever reporting thresholds configured by network. This approach decouples reporting thresholds from the triggering threshold and simplifies UE implementation.

Proposal 4.	If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

Proposal 5.	Triggering threshold is not used as a reporting threshold.

Since DSR MAC CE contains buffer size of an LCG, in some use cases it is useful to piggyback buffer size of selected LCGs in DSR MAC CEs. For example, traffic flows in such LCGs may not be delay critical and hence do not need to trigger DSR themselves. However, if they are associated with other delay-critical flows (e.g. they are associated with the same multi-modal session), then their buffer size information can be useful to gNB scheduler when sent together with delay status of their associated LCGs in the same DSR MAC CE.

Proposal 6.	An LCG without any triggering threshold may also be configured with DSR reporting threshold(s). Such an LCG never triggers a DSR itself but its delay status is included in a DSR MAC CE.

To simplify UE implementation, it is also desirable to define the maximum number of reporting thresholds, so that it is easier for UE to dimension the size of DSR MAC CE before its assembly. This maximum can be determined from the typical delay budget (e.g. 50 msec) and highest resolution for reporting (e.g. 4 msec). Hence 12 could be a good maximum number of reporting thresholds that can be configured.

Proposal 7.	Define a maximum number of reporting thresholds (e.g. 12).

At RAN2#127-bis, the following agreement was made:

- A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG

There can be different options in signaling this one bit. For example, this bit can be included in a bit map which indicates whether delay status information associated with each reporting threshold is included in the DSR MAC CE or not. An alternative is to reuse the reserved bit in the R18 DSR MAC CE to indicate whether there is another set of delay status information after the current one. This option has less overhead and also results in more commonality between R18 and R19 DSR MAC CE. Hence we think it is a more desirable option.

Proposal 8. 	Use an extension bit instead of a bitmap to indicate whether there is delay status information for another reporting threshold following the current one.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Proposal 1.	If at least one LCG is configured with reporting threshold(s), the UE should use Rel-19 DSR MAC CE, as long as it can be accommodated by the available PUSCH resource.

Proposal 2.	If an available PUSCH resource is not large enough to accommodate a Rel-19 DSR MAC CE, a Rel-18 DSR MAC CE is sent instead, with the delay information calculated based on the trigger threshold.

Proposal 3.	Different LCGs may be configured with different number of reporting thresholds.

Proposal 4.	If UE is configured to use R19 DSR, then any LCG with a triggering threshold shall be configured with at least one reporting threshold.

Proposal 5.	Triggering threshold is not used as a reporting threshold.

Proposal 6.	An LCG without any triggering threshold may also be configured with DSR reporting threshold(s). Such an LCG never triggers a DSR itself, but its delay status is included in a DSR MAC CE.

Proposal 7.	Define a maximum number of reporting thresholds (e.g. 12).

Proposal 8. 	Use an extension bit instead of a bitmap to indicate whether there is delay status information for another reporting threshold following the current one.