3GPP TSG-RAN2 Meeting#129	R2-2500111

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	8.18

Source:	OPPO

Title:	Discussion on ProSe in NPN

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of supporting ProSe in NPN.

- Discussion

Firstly, CT1 request to clarify the following restriction

The UE shall perform NR sidelink communication/positioning operation only if the conditions defined in this clause are met:

1>	if the UE's serving cell is suitable (RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED); and if either the selected cell on the frequency used for NR sidelink communication/discovery/positioning operation belongs to the registered or equivalent PLMN as specified in TS 24.587 [57] or TS 24.554 [72] or the UE is out of coverage on the frequency used for NR sidelink communication/discovery/positioning operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

It is CT1 understanding that the above statement needs to be updated to consider the NPN as well.

Considering this part originally comes from S2 spec, e.g., in TS 23.304

5.1.1a	General principles for applying policy/parameters

[…]

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN and authorization for the service to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources;

Based on S2 spec, the corresponding part has been captured as follows

4.2.9.1	Support for 5G ProSe in SNPN

[…]

-	For Authorization and Provisioning and for the subscription information of the ProSe service in NPNs, the procedures and functionality as specified in clauses 5.1 and 5.7 and 6.2 are used with the following adaptations:

-	The term PLMN is replaced by SNPN. The term HPLMN is replaced by subscribed SNPN. The term VPLMN is not applicable to SNPNs. Except the subscribed SNPN, no other SNPNs are authorized to perform the 5G ProSe service (i.e. 5G ProSe Direct Discovery, 5G ProSe Direct Communication, 5G ProSe UE-to-Network Relay discovery, 5G ProSe UE-to-Network Relay communication, 5G ProSe UE-to-UE Relay discovery and 5G ProSe UE-to-UE Relay communication) over PC5 reference point.

Since PNI-NPN can already be covered by PLMN, the left issue is only about S-NPN. It seems straightforward to do similar adaptation in R2 spec to align with S2/C1 intention, e.g.,

The UE shall perform NR sidelink communication/positioning operation only if the conditions defined in this clause are met:

1>	if the UE's serving cell is suitable (RRC_IDLE or RRC_INACTIVE or RRC_CONNECTED); and if either the selected cell on the frequency used for NR sidelink communication/discovery/positioning operation belongs to either the registered or equivalent PLMN as specified in TS 24.587 [57] or TS 24.554 [72] or subscribed SNPN as specified in TS 23.304 [xx] or the UE is out of coverage on the frequency used for NR sidelink communication/discovery/positioning operation as defined in TS 38.304 [20] and TS 36.304 [27]; or

Besides, we noted that there are some other terms being considered, which in our view are not rigorous:

- “registered SNPN”: it may be sometimes not equal to “subscribed SNPN”, e.g., UE subscribed SNPN1, but temporarily registered on SNPN2 due to no SNPN1 coverage;

- “selected SNPN”: it may be sometimes not equal to subscribed SNPN, e.g., UE subscribed SNPN1, but reselects to SNPN2 due to no SNPN1 coverage;

So in any case, reference to 23.304 would make the spec more rigorous.

- R2 update specification as requested by C1, by adding “or subscribed SNPN as specified in TS 23.304”, to align with S2 specification.

On the other hand, C1 asked R2 to

ACTION:  CT1 would like to ask RAN2 to check and consider performing any needed updates in their specifications to support TEI19_ProSe_NPN.

Based on our investigation, there is only one additional NPN-specific aspect required by S2/C1 design, on top of the existing PLMN-based procedure. I.e., based on the current S2, there is a difference between PLMN/PNI-NPN based ProSe and S-NPN based ProSe as follows,

4.2.9.1	Support for 5G ProSe in SNPN

The proximity based services defined in this specification are applicable to SNPN(s) with the following adaptations:

[…]

-	Inter-SNPN operation in clause 4.2.3 is not supported.

4.2.9.2	Support for 5G ProSe in conjunction with CAG in PNI-NPN

[…]

NOTE 2:	5G ProSe Layer-2 Remote UE and 5G ProSe Layer-2 UE-to-Network Relay can access different CAG IDs.

5.1.1a	General principles for applying policy/parameters

[…]

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for 5G ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell. The UE is allowed to perform the service with another UE not served by the same PLMN;

In our view, most can be enforced by upper layer mechanism, e.g.,

Table 1 Mechanism to enforce intra-SNPN ProSe operation

| ProSe functions | Mechanism to enforce intra-SNPN operation |
| --- | --- |
| 5G ProSe Direct Discovery 5G ProSe UE-to-Network Relay discovery 5G ProSe UE-to-UE Relay discovery | Upper layer can limit discovery related code match the filter within the same S-NPN only. |
| 5G ProSe Direct Communication 5G ProSe UE-to-UE Relay communication | Upper layer can ensure security mechanism is not compatible for different S-NPNs. |
| 5G ProSe UE-to-Network Relay communication | Since so far U2N relay has to rely on discovery procedure to establish connection between remote and relay UEs, same solution (as to be adopted by non-Relay discovery) can be adopted as for U2N Relay discovery |

- R2 reply S2 that no further change to R2 specification identified to support ProSe in NPN.

- Conclusion

Proposal 1	R2 update specification as requested by C1, by adding “or subscribed SNPN as specified in TS 23.304”, to align with S2 specification.

Proposal 2	R2 reply S2 that no further change to R2 specification identified to support ProSe in NPN.