3GPP TSG-RAN2 Meeting#129	R2-2500113

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	8.19

Source:	OPPO

Title:	Discussion on n5-n8 simultaneous operation

Document for:	Discussion, Decision

- Introduction

This is to discuss the issue of n5-n8 simultaneous operation, as indicated in R4 LS R4-2420410 as follows.

RAN4 respectively asks RAN2 to consider the above information. and to make necessary modifications into RAN2   specification and to define UE capability if necessary.

- Discussion

If we limit to CA, as indicated by R4, the following capability(ies) exist in the current spec

| simultaneousRxTxInterBandCA Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA. If this field is included in ca-ParametersNR-ForDC, it indicates the UE supports simultaneous transmission and reception between any UL/DL band pair within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. It is mandatory for certain TDD-FDD and TDD-TDD band combinations defined in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. This capability does not apply to the following components within TDD-TDD and TDD-FDD inter-band NR-CA or NR-DC combinations: - Intra-band NR-CA or NR-DC component - Inter-band NR-CA or NR-DC component where the frequency range of one TDD band is a subset of the frequency range of the other NR TDD band (as specified in TS 38.101-1 [2]). | BC | CY | N/A | N/A |
| --- | --- | --- | --- | --- |
| simultaneousRxTxInterBandCAPerBandPair Indicates whether the UE supports simultaneous transmission and reception in TDD-TDD and TDD-FDD inter-band NR CA for each band pair in the band combination. Encoded as a bitmap with size L * (L – 1) / 2, and bit N (leftmost bit is indexed as bit 0) is set to "1" if the UE supports simultaneous transmission and reception for band pair (x, y), where L is the number of band entries in the band combination, x and y are the indices of the band entry in the band combination (the first band entry is indexed as 0), x < y, and N = x*(2*L – x – 1)/2 + y – x – 1. If this field is included in ca-ParametersNR-ForDC, each bit of this field indicates whether the UE supports simultaneous transmission and reception between each band pair, within a cell group and across MCG and SCG in TDD-TDD and TDD-FDD inter-band NR-DC. The UE does not include this field if the UE supports simultaneous transmission and reception for all applicable band pairs in the band combination (in which case simultaneousRxTxInterBandCA is included) or does not support for any band pair in the band combination. It is mandatory for certain band pairs as specified in TS 38.101-1 [2], TS 38.101-2 [3] and TS 38.101-3 [4]. The UE shall consistently set the bits which correspond to the same band pair. | BC | CY | N/A | N/A |

Which is limited to TDD-TDD and TDD-FDD (except for intra-band, or inter-band with overlapping case). I.e., FDD-FDD case is missing. On the other hand, what R4 asked for is that for 2UL/2DL CA_n5-n8, for which so far the simultaneous operation is not supported.

Usually for FDD-FDD interband CA, a band pair nA+nB can support simultaneous (nAUL, nBDL) and (nBUL, nADL) by default. For the specific band pair n5+n8, the capabilities for simultaneous (n5UL, n8DL) and (n8UL, n5DL) can be different, currently simultaneous (n5UL, n8DL) is supported but simultaneous (n8UL, n5DL) is not.

For FDD-FDD CA case, so far usually UE support simultaneous Rx-Tx, but the exception is n8-UL and n5-DL, for which R4 asks R2 to take into account.

If the BC is only for FDD-FDD CA, then it seems no need for a new capability since

- If at least one band of the band pair is not n5 or n8, simultaneous Rx-Tx is assumed, or

- If the band pair is for n5 and n8, simultaneous Rx-Tx is not assumed.

Or if the BC is for a mixed FDD-FDD-TDD CA, then it seems no need for a new capability since

- For the FDD-TDD band pair, the legacy signaling applies

- For the FDD-FDD band pair, the logic above applies.

Based on the current R4 conclusion, there is no ambiguity for network configuration even without newly introduced capability signaling, since simultaneous (n8UL, n5DL) will anyway not supported.

If take a step back, the necessity of this signaling is for forwards compatibility, considering the possibility in the future that simultaneous (n8UL, n5DL) may be supported, then we can rely on the existing signaling, which was restricted to TDD-TDD and TDD-FDD case, but now extended the two to FDD-FDD case

- simultaneousRxTxInterBandCA: yet since legacy UE would save this signaling for FDD-FDD case, to avoid NBC concern, it should be used in a way that it is only present if later simultaneous (n8UL, n5DL) is supported. Otherwise, being absent as in legacy.

- simultaneousRxTxInterBandCAPerBandPair: for the concerned band pair (i.e., n5-n8), indicate 0 if the simultaneous (n8UL, n5DL) is not supported, or 1 if later it is supported (in both cases, assuming simultaneous (n8DL, n5UL) is supported).

But again, the capability bit is only meaningful when there is a possibility that simultaneous (n8UL, n5DL) is supported in the future.

- R2 discuss whether there is a need to define capability bits for 2UL/2DL CA_n5-n8 case, considering so far simultaneous (n8UL, n5DL) is anyway not supported.

- If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing simultaneousRxTxInterBandCA to cover FDD-FDD case, extend it in a way that it is present only if the concerned FDD-FDD BC includes (n5, n8) band pair, and when simultaneous (n8UL, n5DL) is supported.

- If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing -	simultaneousRxTxInterBandCAPerBandPair to cover FDD-FDD case, extend it in a way that the bit for the concerned band pair (n5, n8) indicates 0 if the simultaneous (n8UL, n5DL) is not supported, or 1 if it is supported (in both case, assuming simultaneous (n8DL, n5UL) is supported).

- Conclusion

Proposal 1	R2 discuss whether there is a need to define capability bits for 2UL/2DL CA_n5-n8 case, considering so far simultaneous (n8UL, n5DL) is anyway not supported.

Proposal 2	If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing simultaneousRxTxInterBandCA to cover FDD-FDD case, extend it in a way that it is present only if the concerned FDD-FDD BC includes (n5, n8) band pair, and when simultaneous (n8UL, n5DL) is supported.

Proposal 3	If R2 decides to define capability bit for 2UL/2DL CA_n5-n8 case, and if R2 decides to rely on existing - simultaneousRxTxInterBandCAPerBandPair to cover FDD-FDD case, extend it in a way that the bit for the concerned band pair (n5, n8) indicates 0 if the simultaneous (n8UL, n5DL) is not supported, or 1 if it is supported (in both case, assuming simultaneous (n8DL, n5UL) is supported).