3GPP TSG-RAN2 Meeting#129	R2-2500120

# Athens, Greece, Feburary 17th - Feburary 21st, 2025

Agenda Item:	6.1.3.2

Source:	OPPO

Title:	Left Issue on Maximum Number of UL Segments

Document for:	Discussion, Decision

- Introduction

This is to discuss the remaining issue of maximum UL segments.

- Discussion

In [POST128][601], one remaining issue for maximum UL segments was raised, i.e., whether the new rrc-MaxCapaSegAllowed flag can be treated in a similar way as for rrc-SegAllowed as follows.

5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon and rrc-SegAllowed) as defined in TS 36.331 [10], where applicable.

Looking back into the history where rrc-SegAllowed was added (i.e., CR4625 in R2-2401753), for which the reason for change was described as follows, i.e., 125 agreed on the change considering the feasibility of separate enquiry message scenario.

RAN2 understands that when the network queries UE capabilities for different RATs multiple times, rrc-SegAllowed-r16 field shall be consistent among multiple UECapabilityEnquiry messages.

Similar issue has been identified for rrc-SegAllowed for the scenario of separate enquiry messages.

When it comes to rrc-MaxCapaSegAllowed, the logic seems to be similar, i.e.,

- If we can limit to joint enquiry message, there is no remaining issue(s). Some voices have suggested this approach.

- However, some infra-vendor(s) still holds the concern for ENDC, where the network needs to first acquire LTE capability to determine if UE supports ENDC, and then acquire MR-DC/NR capability. As a result, there are also some voices supporting separate enquiry.

Based on offline, it seems there is a need to support separate enquiry case.

For rrc-MaxCapaSegAllowed, while there are supporter(s) to focus on joint enquiry only, there are still concern from some infra-vendor(s) on excluding separate enquiry.

Otherwise, the issue is similar to rrc-SegAllowed. I.e., if a first enquiry is with a large number of allowed segmentation but later a second enquiry is with a small number of allowed segmentation, it may lead to:

- Issue-1: the UE may report some FS:s which fail to have associated BC/FSC reported later due to message length restriction.

- Issue-2: the UE may report a high order BC/FSC reported in MR-DC container but only lower order BC/FSC reported in NR container and thus lead to network confusion.

Based on the offline, some solution candidates are listed

Solution-1: Allow separate enquiry for ENDC, and restrict the setting of rrc-MaxCapaSegAllowed in a way similarly to rrc-SegAllowed. Example in Annex. For this approach, chip-set vendor(s) expressed the concern on ‘look ahead’ implementation.

Solution-2: Allow separate enquiry for ENDC, and leave the setting of rrc-MaxCapaSegAllowed up to network implementation, meaning no restriction on rrc-MaxCapaSegAllowed but also relying on the network to handle if any consequences. For this approach, although support by some infra-vendor(s), some infra-vendor expressed the concern on additional effort required by network.

Solution-3: For ENDC, allow a separate enquiry for LTE first, and then

- Either a joint enquiry on LTE+MRDC+NR, with rrc-MaxCapaSegAllowed set. Based on this approach, there is no inconsistency at all, but some duplication/spamming due to double-reporting on LTE container

- Or a joint enquiry on MRDC+NR, with rrc-MaxCapaSegAllowed set. Based on this approach, there is no inconsistency between MR-DC and NR, and no double-reporting. Yet there could be a risk on inconsistency between LTE and MR-DC/NR (yet since LTE capability size is relatively smaller, hopefully the risk is not of major concern?),

Based on offline, it seems not preferred to impose a static ‘same value’ restriction on rrc-MaxCapaSegAllowed in specification. While for the negative impact (Issue-1/2 above), companies tend to leave it to inter-vendor coordination to solve it by implementation.

For the case of separate enquiry, based on offline, it is not preferred to impose a static ‘same value’ restriction on rrc-MaxCapaSegAllowed in specification, but can leave it to inter-vendor coordination to set proper value of rrc-MaxCapaSegAllowed by implementation.

- For the usage of rrc-MaxCapaSegAllowed, R2 understand it can rely on joint enquiry, or separate / multiple enquiry approach. In the latter case, it is left to network implementation to set proper value of rrc-MaxCapaSegAllowed, e.g., by inter-vendor coordination.

- Conclusion

Proposal 1	For the usage of rrc-MaxCapaSegAllowed, R2 understand it can rely on joint enquiry, or separate / multiple enquiry approach. In the latter case, it is left to network implementation to set proper value of rrc-MaxCapaSegAllowed, e.g., by inter-vendor coordination.

- Annex: TP for solution-1

Start change

5.6.1.4	Setting band combinations, feature set combinations and feature sets supported by the UE

The UE invokes the procedures in this clause if the NR or E-UTRA network requests UE capabilities for nr, eutra-nr or eutra. This procedure is invoked once per requested rat-Type (see clause 5.6.1.3 for capability enquiry by the NR network; see TS 36.331 [10], clause 5.6.3.3 for capability enquiry by the E-UTRA network). The UE shall ensure that the feature set IDs are consistent across feature sets, feature set combinations and band combinations in all three UE capability containers that the network queries with the same fields with the same values, i.e. UE-CapabilityRequestFilterNR, UE-CapabilityRequestFilterCommon, rrc-SegAllowed, rrc-MaxCapaSegAllowed and fields in UECapabilityEnquiry message (i.e. requestedFreqBandsNR-MRDC, requestedCapabilityNR, eutra-nr-only flag, requestedCapabilityCommon, rrc-SegAllowed and rrc-MaxCapaSegAllowed) as defined in TS 36.331 [10], where applicable.

Stop change