3GPP TSG-RAN WG2 Meeting #129	R2-2500139

Athens, Greece, 17 – 21 February 2025

Title:	[DRAFT] Reply LS on Update of Broadcast MCCH Information

Response to:	R3-247892/ R2-2500024

Release:	Release 18

Work Item:	NR_MBS_enh-Core

Source:	Nokia [TSG RAN WG2]

To:	TSG RAN WG3

Cc:

Contact Person:

#### Name:	Subin Narayanan

E-mail Address:	Subin.Narayanan@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN3 is discussing a recovery scenario related to MBS broadcast. In this scenario MCCH Information of the broadcast session is broadcasted in the cell. Some companies propose a solution where the gNB modifies one or more parameters in the broadcast MCCH information in order to force all UEs in the cell receiving this broadcast session to release/add MRB and re-establish PDCP with PDCP SN of the first next delivered packet.

This solution however assumes that all UEs will behave as above. Therefore, RAN3 would like a confirmation from RAN2 if the solution can work, i.e., whether a change in the MCCH information mandatorily leads to a PDCP re-establishment (and re-initialization of the PDCP SN based on the SN of the next received packet).

RAN3 asks RAN2 to confirm whether it is possible for the network to mandatorily force the re-establishment of PDCP of all UEs in a cell by an update of parameter broadcast in MCCH Information.

As per 38.331, Broadcast MRB configuration is initiated under the following condition:

### 5.9.3	Broadcast MRB configuration

#### 5.9.3.1	General

The broadcast MRB configuration procedure is used by the UE to configure PDCP, RLC, MAC and the physical layer upon starting and/or stopping to receive a broadcast MRB transmitted on MTCH, or upon modification of a configuration of a broadcast MRB received by the UE. The procedure applies to MBS capable UEs that are interested to receive or that are receiving an MBS broadcast service that are in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED with an active BWP with common search space configured by searchSpaceMTCH or searchSpaceMCCH.

NOTE:	How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation.

The important part related to RAN3 question is the NOTE in the above – “How to perform a modification of a broadcast MRB which is already configured in the UE is left to UE implementation”.  So, it is unclear from the specification that when a UE can modify an existing MRB and when the MRB should be released and a new established, e.g., if only a few parameters of an existing MRB change in PTM configuration within MCCH should a new MRB be established or should the already existing one be modified. In the given context, PDCP can be re-established only when MCCH update results in release of an old MRB and addition of a new MRB.  Given that the current specification left this to UE implementation, it is not possible for the network to mandatorily force the re-establishment of PDCP of all UEs in a cell by an update of parameter broadcast in MCCH Information.

2. Actions:

To RAN3

ACTION: 	RAN2 respectfully asks RAN3 to take the above responses into account when discussing a recovery scenario related to MBS broadcast.

3. Date of Next TSG-RAN WG2 Meetings:

RAN2#129-bis	from 2025-04-07	to 2025-04-11		China, CN

RAN2#130	from 2025-05-19	to 2025-05-23		Malta, MT

RAN2#131	from 2025-08-25	to 2025-08-29		India (TBC), IN