3GPP TSG-RAN WG2 Meeting #129	R2-2500158

Athens, Grace, 17th – 21st Feb, 2025

Agenda item:	8.4.2

Source:	Huawei, HiSilicon

Title:	Discussion on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the open issues related to LP-WUS considering the progress in RAN1/2/4.

# Discussion

## Issues for paging monitoring triggered by LP-WUS

### LP-WUS and PEI

The primary purpose of LP-WUS in RRC_IDLE/INACTIVE is to indicate paging monitoring. RAN1 made the following related agreements in RAN1-116:

RAN1#116 Agreements - For the case where a UE supports PEI and PEI is configured by the gNB, after the UE receives LP-WUS indicating wake-up, it is up to UE implementation whether to monitor PEI or not. - It is supported that the UE monitors the legacy PO after receiving LP-WUS indicating wake-up. - FFS: support of UE monitoring dynamic PO - Conclusion For idle/inactive mode, how to map a UE to a subgroup ID for LP-WUS is left to RAN2 to decide.

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

When LP-WUS and PEI are both configured and used, there are two cases of potential subgrouping types used for LP-WUS and PEI.

Case 1 - Same type of subgrouping method used for LP-WUS and PEI: In this case, both LP-WUS and PEI use either CN assigned subgrouping or UE_ID based subgrouping. For CN assigned subgrouping, CN can ensure proper subgrouping of the LP-WUS and PEI by implementation to reduce the power consumption. For UE_ID based subgrouping, it is straightforward to support it without CN control.

Case 2 - Different type of subgrouping method used for LP-WUS and PEI: In this case, CN assigned subgrouping is applied for LP-WUS and UE_ID based subgrouping is applied for PEI and vice-versa. We do not see any functional issues other than a slight impact on the power saving gain. Hence what type of subgrouping to apply for LP-WUS and PEI can be left to NW implementation.

Proposal 1: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

### LP-WUS subgrouping

As not all UEs in a subgroup are paged, the false paging rate for the UEs can be reduced if LP-WUS subgrouping is used with the existing subgrouping for PEI. This, referred to as 2-level subgrouping, is shown in Figure 1. UE1 and UE2 are placed in the same group for LP-WUS, and are placed in different groups for PEI. When UE1 is paged, UE2 does not need to monitor paging based on its PEI.

Figure 1: Illustration of 2-level subgrouping

Observation 2: The paging false alarm rate can be further reduced if UEs in one LP-WUS subgroup can be further divided into different PEI subgroups.

RAN2 made the following agreement in RAN2-128:

- FFS on the following options - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - Option 3: The formula for UE_ID based PEI subgrouping is reused.

The solution options are presented in Table 1 to facilitate subsequent discussion.

Table 1: Solution options of UE_ID based subgrouping formula for LP-WUS

| Solution | UE_ID based subgrouping formula |
| --- | --- |
| Option 1 | SubgroupID_LP-WUS = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS), where K = number of subgroups for UE_ID based PEI subgrouping |
| Option 2 | SubgroupID_LP-WUS = (floor(UE_ID/(N*Ns*K)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS), where K = configurable value or fixed |
| Option 3 | SubgroupID_LP-WUS = (floor(UE_ID/(N*Ns)) mod subgroupsNumForUEID_LP-WUS) + (subgroupsNumPerPO_LP-WUS - subgroupsNumForUEID_LP-WUS) Note: same formula as in PEI |

We analyse the solution options from paging false alarm rate and power saving gain point of view. Option 3 is re-using the PEI’s formula for LP-WUS. In RAN1-118bis, RAN1 made a working assumption for LP-WUS that the maximum number of subgroups per PO supported in Rel-19 is 32. Assuming the number of UE_ID based subgroups is set to 8 for PEI and 32 for LP-WUS, the UE_ID based subgrouping formula calculation for PEI and LP-WUS is shown in Figure 2.

Figure 2: Overlapping of bits for UE_ID in Option 3

As can be seen in Figure 2, UEs with the same least significant bits will end up in the same PEI subgroup. Thus Option 3 does not provide any additional benefit of reducing false paging rate.

Observation 3: Option 3 does not provide any additional benefit of reducing false paging rate.

For Option 1 and Option 2, the scenario of overlapping bits for UE_ID are shown in Figure 3 and Figure 4 respectively.

Figure 3: No overlapping of bits for UE_ID in Option 1

Figure 4: No overlapping of bits for UE_ID in Option 2

For both Option 1 and Option 2, there is no issue of “overlapping bits” that exist in Option 3. K ensures that the UEs from LP-WUS subgroup can be optimally divided into different PEI subgroups. It may happen that two UEs are assigned to the same subgroup if the subgroup number indicated by bits of one UE's ID matches with those​ of another UE's ID for LP-WUS and PEI. However, the possibility of this case is quite low as the generation of the UE_ID is entirely an independent and random event.

The evaluation of power saving gain when UE monitors both LP-WUS and PEI versus when UE monitors only PEI is presented in Figure 5. The UE power consumption model is based on TR 38.840 and TR 38.869, with some of the evaluation assumptions listed in Appendix A. Rel-17 PEI procedure is used as the baseline with an RRM relaxation allowing for one neighbor cell measurement per 3 I-DRX cycles. The number of subgroups for PEI and LP-WUS is configured to be 8 and 16 respectively, and the UE only needs to receive 1 SSB for finer re-synchronization after ramp-up, assuming that LP-WUR can help main receiver achieve coarse sync. The paging probability for each individual UE is set to 1%, and UEs are independently and randomly assigned to LP-WUS and PEI subgroups. To simplify the evaluation, we ignore the RACH procedure in the analysis.

Figure 5: Evaluation results of power saving gain for UE monitoring LP-WUS with/without PEI

The results indicate the UE can achieve an additional power saving gain of approximately 3%~6% by monitoring PEI after receiving LP-WUS. The additional power saving is due to the fact that the UE MR goes back to sleep if the PEI indicates that the UE is not paged.

In Table 2, we also provide the numerical analysis on paging false alarm rate for the above configuration. The paging false alarm rate for arbitrary UE comes from the probability that UE receives the invalid paging when the subgroup of this UE is indicated by LP-WUS to wake up to monitor legacy PO/PEI.

Table 2: Paging FAR for one UE monitoring LP-WUS & PEI with/without introducing offset K

| Number of UEs in one PO | Paging false alarm rate for one UE |  |
| --- | --- | --- |
|  | LP-WUS + PEI without offset K | LP-WUS + PEI with offset K = 8 |
| 128 | 87.1% | 13.9% |
| 160 | 89.6% | 17.2% |
| 192 | 91.2% | 20.5% |
| 224 | 92.4% | 23.6% |
| 256 | 93.3% | 26.5% |

As seen from the results in Table 2, the UE can achieve a great reduction of paging false alarm rate when an offset K is used in the formula to divide UEs into different subgroups of PEI and LP-WUS as much as possible.

Observation 4: An offset K can be introduced in the UE_ID based subgrouping formula to avoid the potential subgrouping duplication and obtain additional power saving gain when monitoring LP-WUS and PEI.

However, for Option 1, K depends on the number of subgroups configured for PEI. This implies that the network always needs to provide PEI configuration to configure/enable LP-WUS which is restricting the NW for LP-WUS feature. Also, if NW changes PEI configuration, UE needs to re-calculate the UE_UD for LP-WUS which makes the UE implementation complicated. Hence, we propose that Option 2 with fixed value of K equal to 8 as the solution for UE_ID based subgrouping. Table 3 summarizes the pros and cons of each solution option.

Observation 5: Option 1 restricts NW behavior and complicates NW implementation.

Table 3: Comparison of solutions

| Solution | Pros | Cons |
| --- | --- | --- |
| Option 1 |  | - NW always needs to provide PEI configuration to configure/enable LP-WUS (i.e., unnecessary restriction on the NW) - Change of PEI configuration requires re-calculation of LP-WUS subgroup ID (i.e., complicated UE implementation) |
| Option 2 | - With “K = fixed offset”, does not depend on PEI configuration | - With “K = configurable”, more complex than “K = fixed offset” |
| Option 3 | - Simple | - Does not provide any additional benefit of reducing false paging rate as UEs with the same LSBs end up duplicated subgroup. |

Proposal 2: Adopt Option 2 with fixed K = 8 as the solution for LP-WUS UE_ID based subgrouping formula.

### LP-WUS and emergency PDU session

RAN2 made the following agreement in RAN2-128:

- FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session.

Handling of subgrouping indication when UE has an emergency PDU session was discussed during R17 PEI and agreed that CN assigned subgrouping is not allowed for UE with emergency PDU session by UE not reporting relevant capability to the core network, as specified in TS 24.501. UE_ID based subgrouping is allowed since the additional latency introduced by PEI is considered tolerable. However, for LP-WUS, the latency is not negligible due to the ramp up time triggered by LP-WUS. Hence UE_ID based subgrouping of LP-WUS should also be not allowed for UE with emergency PDU session to further reduce the potential latency.

Proposal 3: For LP-WUS, both CN assigned and UE_ID based subgrouping are not allowed when UE has an emergency PDU session.

## Configuration Aspects of LP-WUS

### Dedicated configuration in RRC Signalling

RAN2 has reached the agreement that the LP-WUS related configuration for IDLE/INACTIVE state is provided at least via system information and whether dedicated signaling for configuration is still FFS.

The LP-WUS related configuration should be cell specific since it is for IDLE/INACTIVE state. The configuration includes both configuration for LP-SS and LP-WUS thus covering both OOK-based and OFDM-based receivers respectively. Providing dedicated configuration in RRC signalling for example in RRC Release limits the scenarios in which LP-WUS can be used. For example, when an LP-WUS UE is provided with dedicated configuration and if the UE selects a new cell due to mobility, the new cell is not aware of the dedicated configuration of the LP-WUS for the UE and thus the UE still needs to rely on the system information from the cell. Thus, there is no need for dedicated configuration.

Proposal 4: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

### Enabling of LP-WUS

If LP-WUS functionality is enabled for all the UEs supporting LP-WUS in a camped cell, for example, when a UE receives LP-WUS configuration via SIB from the camped cell, it can use LP-WUS functionality. It is simple but may lead to a lot of UEs using LP-WUS, which means the false paging probability increases and causes more transitions from LP-WUR to MR. This will reduce the power saving gain or even result in negative power saving gain considering the maximum number of subgroups for LP-WUS has been decided by RAN1 to be 32. Hence there can be UE/group specific control for enabling LP-WUS.

To reduce false paging probability, the network (CN or RAN) decides whether a LP-WUS capable UE is allowed to use the LP-WUS functionality based on the conditions in the network (e.g., radio resources) and/or the characteristics of the UEs (e.g., paging probability, shorter DRX cycle, etc.). For example, the CN uses NAS signaling to indicate whether a specific UE can use LP-WUS functionality, which is similar to Rel-17 CN-assigned subgrouping method. To minimize the impact for deployment, the UE by default can use LP-WUS functionality unless a specific indication to disable the LP-WUS functionality is received from CN, which implies that the absence of indication means UE is allowed to use LP-WUS functionality. In this case, if the CN does not implement this feature, LP-WUS can still be implemented as an AS solution; if CN prefers to have precise control, CN can implement the control indication in NAS signalling.

Proposal 5: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality.

### Configuration for different types of LP-WUR

RAN1 has been discussing to support both OOK-based and OFDM-based receivers for LP-WUS monitoring. Since the OFDM-based LP-WUR has stronger capability for LP-WUS signal reception and processing, it is possible for NW to configure a shorter LP-WUS to wake up only OFDM-based receiver to reduce the resource overhead. In this case, the UE with OFDM-based receiver can benefit from the low-complexity operation on LP-WUS, e.g., to cancel any OOK sequence/pattern detection or implement termination of LP-WUS procedure by early detection. The UE with OOK-based receiver will skip the current LP-WUS for obtaining more power saving gain. Thus, the NW should be able to support the scenario that only UEs with OFDM-based LP-WUR are paged, e.g., by sending a shorter LP-WUS that cannot be received by OOK-based receiver. This information can be implicitly indicated by the absence of LP-WUS configuration for OOK-based receiver.

Proposal 6: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

### LP-WUS entry/exit condition

In RAN2#127, RAN2 has agreed that the thresholds of LP-WUS entry/exit condition should be separately configured for different types of receiver. Considering the LP-WUS cell coverage cannot reach the full coverage of legacy cell as specified in the WID [1], the feasibility of LP-WUS feature may be greatly affected if the entry/exit condition is absent in the LP-WUS configuration, i.e., to leave when to enter/exit LP-WUS mode to UE implementation, since UE will spend plenty of time and resource overhead to decide whether LP-WUS can be reliably used. Therefore, it is beneficial that the thresholds of LP-WUS entry/exit condition are always carried in the LP-WUS related configuration as a reference for UE to determine the LP-WUS coverage with more precision. Thus, we propose that the thresholds of LP-WUS entry/exit conditions for a specific type of receiver should be always configured if supported by the network.

Proposal 7a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 7b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

RAN2 has also discussed whether the entry condition can be also based on the measurement result of LP-WUR in RAN2#127 and RAN2#127bis, and made the following working assumption and agreements.

Working Assumption in RAN2#127 - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. Agreement in RAN2#127bis - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

UE usually performs RRM measurements by MR as per legacy behavior. Thus, the serving cell measurements by MR are always available and good enough. As the hardware capability of MR is better than that of LP-WUR, the measurements by LP-WUR do not provide any additional benefit for LP-WUS entry decision. On the other hand, using both MR and LP-WUS measurements can make the LP-WUS entry procedure complicated, introducing much specification impact. Hence, the measurement by LP-WUR is not needed for LP-WUS entry condition and it can be left to NW to decide whether the threshold based on LP-WUR is needed for UE to enter LP-WUS monitoring.

Proposal 8: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

If the NW configures the LP-WUS entry thresholds for both MR and LP-WUR measurements, it should be up to UE implementation to obtain the measurement results on LP-WUR, i.e., the metrics and requirement of LR measurement should wait for RAN4’s discussion and there is no need to define in RAN2 spec the specific UE behavior on LP-WUR measurement for LP-WUS entry condition.

Proposal 9: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# Conclusion

In this contribution, we discuss procedure and configuration of LP-WUS for IDLE/INACTIVE modes, and have the following proposals:

Issues for paging monitoring triggered by LP-WUS

Observation 1: Both options of UE monitoring legacy paging DCI/paging and monitoring PEI after MR wakes up are supported in RRC_IDLE/INACTIVE.

Proposal 1: If LP-WUS and PEI are used together, it is up to NW to decide which types of subgrouping (i.e., CN assigned subgrouping or UE_ID based subgrouping) are applied.

Observation 2: The paging false alarm rate can be further reduced if UEs in one LP-WUS subgroup can be further divided into different PEI subgroups.

Observation 3: Option 3 does not provide any additional benefit of reducing false paging rate.

Observation 4: An offset K can be introduced in the UE_ID based subgrouping formula to avoid the potential subgrouping duplication and obtain additional power saving gain when monitoring LP-WUS and PEI.

Observation 5: Option 1 restricts NW behavior and complicates NW implementation.

Proposal 2: Adopt Option 2 with fixed K = 8 as the solution for LP-WUS UE_ID based subgrouping formula.

Proposal 3: For LP-WUS, both CN assigned and UE_ID based subgrouping are not allowed when UE has an emergency PDU session.

Configuration Aspects of LP-WUS

Proposal 4: Dedicated configuration in RRC signaling is not needed for providing LP-WUS related configuration in RRC_IDLE/INACTIVE modes.

Proposal 5: The CN indicates whether LP-WUS capable UE(s) is/are allowed to use the LP-WUS functionality by NAS signalling; the absence of indication means UE is allowed to use LP-WUS functionality.

Proposal 6: NW should configure whether only OFDM-based LP-WUR is supported in a cell.

Proposal 7a: Thresholds of LP-WUS entry/exit conditions for OOK-based receivers are always configured when OOK-based LP-WUS configuration is provided by the network.

Proposal 7b: Thresholds of LP-WUS entry/exit conditions for OFDM-based receivers are always configured when OFDM-based LP-WUS configuration is provided by the network.

Proposal 8: For LP-WUS entry condition, the threshold based on MR is mandatorily configured, while the threshold based on LP-WUR is optional.

Proposal 9: If the LP-WUS entry threshold based on LP-WUR is configured, it is up to UE implementation how to obtain the LP-WUR measurement results used for LP-WUS entry condition.

# References

RP-241824 Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

R1-2407559 LS on LP-WUS operation in IDLE/INACTIVE mode;

RP-242656 On LR and MR operating in different bands in release;

# Appendix A

Table 4: The evaluation assumptions for power saving gain for IDLE/INACTIVE mode

| Parameter | Default value used if not explicitly mentioned |
| --- | --- |
| I-DRX cycle length | 1.28s |
| Per UE paging rate | 1% |
| Number of subgroups indicated by PEI | 8 |
| Number of subgroups indicated by LP-WUS | 8 |
| Length of PO | 4ms |
| Length of SMTC window | 5ms |
| Number of frequency layers for inter-frequency measurement | 1 |
| Ratio of RRM relaxation for R17 baseline scheme | One measurement per 3 I-DRX cycles |
| Ratio of RRM relaxation for R19 LP-WIS scheme | One measurement per 16 I-DRX cycles |
| Length of re-sync/sync | 1 SSB (10ms) Assuming that LP-WUR can help main receiver to get coarse sync, and only 1 SSB is used for finer sync. |
| Relative power unit for LP-WUR ‘on’ state, i.e., the LP-WUR performs monitoring | 0.1 |
| Relative power unit for ultra-deep sleep of MR | 0.015 |
| Ramp-up and down transition energy (unit multiplied by ms) | 15000 |
| Ramp-up time | 400ms |
| How to monitor LP-WUS | Continuously |
| Paging information carried by LP-WUS | Per subgroup information |