3GPP TSG-RAN WG2 Meeting #129	R2-2500165

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.2.4

Source:	Spreadtrum, UNISOC

Title:	Discussion on A-IoT data transmission

Document for:	Discussion and Decision

1 Introduction

In RAN#106, a new WI was approved for A-IoT in order to standardize RAN aspects of A-IoT. To be specific, the WI will specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission. The objectives are illustrated below [1]:

RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

In this contribution, we provide our view on the A-IoT data transmission.

2 Discussion

## 2.1 MAC PDU format of A-IoT

Firstly, we should define MAC PDU format for A-IoT transmission. In legacy NR system, a MAC PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. A MAC PDU consists of one or more MAC subPDUs. Each MAC subPDU consists of one of the following:

- A MAC subheader only (including padding);

- A MAC subheader and a MAC SDU;

- A MAC subheader and a MAC CE;

- A MAC subheader and padding.

Each MAC subheader corresponds to either a MAC SDU, a MAC CE, or padding. A MAC subheader except for fixed sized MAC CE, padding, and a MAC SDU containing UL CCCH consists of the header fields R/F/LCID/(eLCID)/L. A MAC subheader for fixed sized MAC CE and padding consists of the header fields R/LCID/(eLCID). A MAC subheader for a MAC SDU containing UL CCCH consists of the header fields (LX)/R/LCID.

DL MAC subPDU(s) with MAC CE(s) is placed before any MAC subPDU with MAC SDU and MAC subPDU with padding as depicted in Figure 1.

Figure 1 example of a NR DL MAC CE

Generally, we prefer that the NR MAC PDU format can be reused for A-IoT with some adaptive design to simplify the protocol design.

Proposal 1: NR MAC PDU format is as the baseline for A-IoT.

In A-IoT, the upper layer data as the MAC SDU transmitted by MAC layer. If logical channel concept is reused in A-IoT, there are at least one logical channel ID for upper layer data transmission and multiple logical channel ID for MAC CEs (e.g. paging MCE CE, message size indication MAC CE, energy status report MAC CE and so on). The extended logical channel ID may not be needed.

The MAC subheader for variable sized MAC CE, padding, and a MAC SDU containing PRDCH consists of the header fields R/F/LCID/L. A MAC subheader for fixed sized MAC CE and padding consists of the header fields R/LCID. A MAC subheader for a MAC SDU containing PRDCH consists of the header fields R/LCID.

Since the LCIDs for A-IoT is less than NR, the size of the LCID field can be smaller, such as 4 bits. Or it can reserve more bits for expansion in subsequent versions, such as 6 bits same as NR.

Proposal 2: Support one logical channel ID for upper layer data transmission.

Proposal 3: The extended logical channel ID is not needed.

Proposal 4: The size of the LCID field is 4 or 6 bits.

## 2.2 AS ID

From higher layer perspective, it is assumed that "AS ID" is used at least for purpose of D2R scheduling and R2D reception. And this "AS ID" should be a short AS layer ID, rather than the full upper layer device ID.

SI phase discussed the "AS ID" with the following possible options:

- Option 1: a random ID (if used in first D2R message) can be reused;

- Option 2: the reader assigns this "AS ID" to the device. It needs to be further discussed via which R2D message.

- Option 3: it is up to the reader whether to reuse the random ID (if used in first D2R message) as the "AS ID" or to assign a new "AS ID". It needs to be further discussed via which R2D message.

In A-IoT TR [2], it noted that further down-selection for "AS ID" needs to consider the involved message and whether the reader needs to handle the collision.

We prefer option 3, it is up to reader implementation. If random ID is carried in A-IoT Msg1 (e.g., CBRA), then reader responds device Msg2 with the same random ID in Msg1. This means that competition is resolved, reader already identify the AIoT device. The random ID could be as AS ID once UE receives the Msg2. Reader and device can use the random ID as AS ID in the following D2R scheduling and R2D reception.

If random ID is not carried in Msg1 (e.g. CFRA), device only send the device ID and/or upper layer data to reader in Msg1. Reader can identify device by device ID. For subsequent D2R scheduling and R2D reception, reader can assign a new AS ID to device in Msg2. One simple solution is that reader can use the partial device ID as the AS ID. Using the partial device ID is not require device to maintain any additional ID assigned from reader and not require device to store any additional ID.

Proposal 5: Random ID in Msg1 can be reused as AS ID.

Proposal 6: If no random ID is carried in Msg1, reader assigns a AS ID to device in Msg2.

Proposal 7: AS ID assigned by reader can be based on partial device ID.

## 2.3 Command type information

In SI phase, we discussed that command type (e.g. read/write/disable) should be visible to reader for effective resource scheduling, but no agreement was reached, and the conclusion was that we need to wait for SA2 LS response on expected D2R message size.

Recently, SA2 responded that AIOTF can provides the reader with the approximate D2R message size if provided by AF, whether the CN can determine the approximate D2R message size if not provided by AF is under discussion in SA2 [3]. So we still need to wait for the final conclusion of SA2. If CN does not provide D2R message size to reader, the command type is useful information to reader for resource allocation.

Proposal 8: Waiting for the final conclusion of SA2 on expected D2R message size.

## 2.4 Message size information

In order for reader to schedule appropriate resource for device D2R transmission, the assistance information (e.g. D2R message size) to reader is studied. For message size information, two options are studied:

- Option 1: The estimate of expected following D2R message size from the CN to the reader;

- Option 2: The simple message size indication from the device to the reader.

For option 1, if this information is available from CN, it saves the overhead/latency caused by option 2 in AS layer. It can be also used by the reader as early as for the first D2R message, if needed.

In option 2, it is useful in case the CN is not able to provide or does not provide the expected D2R message size. Limited granularity is supported considering the AS layer overhead. It requires D2R message to carry this information in advance, before the reader can actually use this size information for scheduling.

We think both the option 1 and option 2 should be supported, if reader obtains estimate of expected following D2R message size from the CN, then device no longer needs to report. Otherwise, reader can indicate device to report following D2R message size.

Proposal 9: For D2R message size information indication to reader, support both the option 1 and option 2.

Proposal 10: Reader can indicate device to report following D2R message size, if CN does not provide the estimate of expected following D2R message size to the reader.

3 Conclusion

In this contribution we discuss the functionalities required for A-IoT, with the following proposals:

Proposal 1: NR MAC PDU format is as the baseline for A-IoT.

Proposal 2: Support one logical channel ID for upper layer data transmission.

Proposal 3: The extended logical channel ID is not needed.

Proposal 4: The size of the LCID field is 4 or 6 bits.

Proposal 5: Random ID in Msg1 can be reused as AS ID.

Proposal 6: If no random ID is carried in Msg1, reader assigns a AS ID to device in Msg2.

Proposal 7: AS ID assigned by reader can be based on partial device ID.

Proposal 8: Waiting for the final conclusion of SA2 on expected D2R message size.

Proposal 9: For D2R message size information indication to reader, support both the option 1 and option 2.

Proposal 10: Reader can indicate device to report following D2R message size, if CN does not provide the estimate of expected following D2R message size to the reader.

- References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Huawei.

- 3GPP TR 38.769, Study on solutions for ambient IoT (Internet of Things), Release 19, V2.0.1, 2024.

- S2-2413035 (R2-2409412), LS on A-IoT Conclusions in SA WG2, SA2, 3GPP TSG-WG SA2 Meeting #166.