3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500185

Athens, Greece, 17th – 21st February 2025

Source:	CATT

Title:	Consideration on XR-specific RLC Enhancement

Agenda Item:	8.7.5

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the issues on timely RLC retransmissions including enhanced polling and autonomous retransmission, and avoiding unnecessary retransmission for XR, our views on these issues are given in the following part.

Discussion

Timely RLC retransmissions for XR

In RAN2#128 meeting, the agreement on timely retransmissions on RLC layer is as follows [1]. Both autonomous retransmission and polling enhancement are supported. More details are discussed for these options in this section.

Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

On autonomous retransmission

By current discussion, two options as follows are mainly proposed for autonomous retransmission triggering:

- Option 1: the autonomous retransmission will be triggered if the remaining time is lower than a threshold.

- Option 2: the autonomous retransmission will be triggered based on the failed retransmissions indication from HARQ feedback.

For option 2, the cross-layer interaction at UE will be introduced, and there is no explicit HARQ feedback indication in DL, hence this triggering condition is not feasible. For the option 1 based on the remaining time, Tx side could perform retransmission when the remaining time is lower than a threshold, which seems reasonable and feasible at Tx side. As a result, it is preferable to select the option 1 for autonomous retransmission.

Proposal 1: The autonomous retransmission will be triggered if the remaining time is lower than a threshold.

For autonomous retransmission, the next issue is how to perform the autonomous retransmission. After autonomous retransmission is triggered, it can be seen that the UE could perform one or multiple of retransmissions in order to avoid the failed transmission. Obviously, if the UE receives the ACK indication in status report, the UE can cancel the autonomous retransmission.

Proposal 2: The UE could perform one or more autonomous retransmissions after triggering.

Proposal 3：The autonomous retransmission can be cancelled if UE receives the ACK indication in status report.

If multiple retransmissions are supported after autonomous retransmission is triggered, it is UE implementation to determine when to perform the next autonomous retransmission after the first autonomous retransmission is triggered, if necessary, the total number of autonomous retransmission will be configured by network to reduce the impact to system capacity.

Proposal 4: If multiple of retransmission is supported for autonomous retransmission, it is UE implementation to determine when to perform the next autonomous retransmission.

Proposal 5: The maximum number of autonomous retransmission could be restricted and configured by network.

The impact on RLF

In legacy RLC layer procedure, the following procedure is applied for RLF.  It can be seen that the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

5.3.2 Retransmission The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following: - STATUS PDU from its peer AM RLC entity. When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer: - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission. When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

When the autonomous retransmission is introduced, the number of autonomous retransmission(s) could not be used to deduce that channel quality is bad enough to indicate the RLF to high layer, it is not necessary to increase the RETX_COUNT when autonomous retransmission is performed, there is no impact to spec.

Proposal 6: It is unnecessary to increase the RETX_COUNT when autonomous retransmission is performed, and there is no specification impact foreseen.

Enhanced polling

In last meeting, RAN2 agreed to enhance polling for timely RLC retransmission. The legacy procedure on polling is as follows:

5.3.3.2 Transmission of a AMD PDU Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte: - include a poll in the AMD PDU as described below. Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); - include a poll in the AMD PDU as described below. NOTE: Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation. <--skip--> Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall: - set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer; - if t-PollRetransmit is not running: - start t-PollRetransmit. - else: - restart t-PollRetransmit. 5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Based on above procedure for polling, it shows that at least the number of PDU_WITHOUT_POLL, BYTE_WITHOUT_POLL，the expiry of t-PollRetransmit could be used to trigger a poll in an AMD PDU. For UL XR traffic, the value associated to above three parameters could be configured to be short one by network, the PDCP status report will be triggered more quickly. In another way, the condition to trigger polling could be at the moment when the data become delay critical, i.e., the Tx side will trigger the polling when the remaining time of the corresponding PDCP SDU is lower than a threshold, the threshold could be configured by network.

Proposal 7: For enhanced Polling, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: if the remaining time of the corresponding PDCP SDU is lower than a threshold, it can trigger polling.

Avoid unnecessary retransmission

In RAN2#128 meeting, the agreements on avoiding unnecessary retransmission on RLC layer are listed as below.

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

The Rx side behavior

In last meeting, a new RLC timer at the Rx is introduced to abandon the obsolete RLC SDU or segment. For the timer at RLC layer, one option is to introduce a timer per RLC SDU, the UE will abandon the RLC SDU based on the timer for each RLC SDU. Another option is introducing a timer per RLC entity, the timer is similarly to the t-Reordering timer. For the first option, Rx side needs to initiate and maintain so many timers that it will increase UE complexity and storage, the second option is aligned to the legacy t-Reordering timer to advance the Rx window, it is simple and feasible. Hence, the local timer to abandon the obsolete RLC SDU or segment is preferable to be a timer per RLC entity. This timer has been agreed to be started when a gap is generated at RLC layer. When the timer is expired, the UE will discard all RLC SDU or segment with SN less than the SN recorded for this timer and advance the window.

Proposal 8: A new timer per RLC entity is introduced for Rx side to avoid unnecessary RLC retransmission, it could be performed similar to the t-Reordering timer in PDCP layer.

Proposal 9: At Rx side, when the new timer expired, the Rx side will discard all segments with SN less than the SN recorded for this timer and advance the window.

Once the timer is expired, the Rx side may discard the corresponding RLC SDU or segment and advance its window, so it is better to transmit a status report to Tx side to ensure the windows at Tx side and Rx side not out of sync and unnecessary retransmission from Tx side to Rx side.

Proposal 10: The Rx side will trigger a status report if the Rx discards outdated RLC SDU or segment based on local timer and the window is advanced.

The Tx side behavior

For this combined Rx and Tx approach, it is agreed that Tx side stops to retransmit an outdated SDUs. It is still not clear how the Tx side discard the outdated SDU, one option is based on the Tx own indication from PDCP layer or a new timer at RLC layer, another option is based on the discard indication from Rx side, such as the fake ACK in status report, RAN2 needs to further discuss and clarify which option is selected.

Proposal 11: RAN2 discuss which option is selected for Tx side discarding outdated SDU:

- Option 1: Based on the Tx own indication from PDCP layer or a new timer at RLC layer;

- Option 2: Based on the discard indication from Rx side, such as the fake ACK in status report.

For the Tx window, RAN2 agreed that the RLC receiving window always advances to any given RLC SN before the transmitting window does. Hence, the Tx side will not advance its window if Tx side discard the outdated SDUs based on its own determination, anyway, it will update TX_Next_Ack and advance the Tx window with SNs in [TX_Next_Ack, TX_Next_Ack + AM_Window_Size) based on the status report from Rx side.

Proposal 12: The Tx side will follow the legacy procedure to update the TX_Next_Ack and advance the window, i.e., based on the received status report, no further spec impact.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: The autonomous retransmission will be triggered if the remaining time is lower than a threshold.

Proposal 2: The UE could perform one or more autonomous retransmissions after triggering.

Proposal 3：The autonomous retransmission can be cancelled if UE receives the ACK indication in status report.

Proposal 4: If multiple of retransmission is supported for autonomous retransmission, it is UE implementation to determine when to perform the next autonomous retransmission.

Proposal 5: The maximum number of autonomous retransmission could be restricted and configured by network.

Observation 1: In legacy procedure, the retransmission will be triggered by negative acknowledgement in the status report and max retransmission will be indicated to high layer if RETX_COUNT = maxRetxThreshold.

Proposal 6: It is unnecessary to increase the RETX_COUNT when autonomous retransmission is performed, and there is no specification impact foreseen.

Proposal 7: For enhanced Polling, the below two options could be used for down-selection:

- Option 1: the network can configure a new set of shorter value for pollPDU and pollByte to trigger the polling for timely RLC retransmission.

- Option 2: if the remaining time of the corresponding PDCP SDU is lower than a threshold, it can trigger polling.

Proposal 8: A new timer per RLC entity is introduced for Rx side to avoid unnecessary RLC retransmission, it could be performed similar to the t-Reordering timer in PDCP layer.

Proposal 9: At Rx side, when the new timer expired, the Rx side will discard all segments with SN less than the SN recorded for this timer and advance the window.

Proposal 10: The Rx side will trigger a status report if the Rx discards outdated RLC SDU or segment based on local timer and the window is advanced.

Proposal 11: RAN2 discuss which option is selected for Tx side discarding outdated SDU:

- Option 1: Based on the Tx own indication from PDCP layer or a new timer at RLC layer;

- Option 2: Based on the discard indication from Rx side, such as the fake ACK in status report.

Proposal 12: The Tx side will follow the legacy procedure to update the TX_Next_Ack and advance the window, i.e., based on the received status report, no further spec impact.

Reference

- RAN2#128 meeting, Report from session on R18 MBS, R18 QoE and R19 XR, Huawei.