3GPP TSG-RAN WG2 Meeting #129                                                        R2-2500187

Athens, GR, 17th – 21st February 2025

Source:	CATT

Title:	Discussion on Multi-hop Discovery and (Re)selection

Agenda Item:	8.13.2

Document for:	Discussion and Decision

Introduction

In RAN2#128 meeting, parts of issues on Rel-19 multi-hop SL U2N relay discovery and (re)selection had been discussed[1]. In the current contribution, we further discuss some key leftover issues:

- FFS on Uu upper bound for intermediate relay UE.

- PC5 AS conditions for discovery at the last relay UE for discovery model B.

- Any relay (re)selection triggering condition is needed for intermediate Relay UE.

Discussion

FFS on Uu upper bound for intermediate relay UE

In RAN2#128 meeting, whether an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE is needed or not was discussed and reached the below conclusion[2]:

Agreements:

A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE.

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible).  FFS if operation as an intermediate relay UE towards a different cell would be supported.

The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.  If the upper bound is not configured, there is no threshold, but this does not override the previous agreement.  FFS if there would be additional restrictions in the case of no upper bound.

For the first FFS, the related scenario is shown below according to the wording description:

Figure 1. Schematic Diagram of the First FFS

In the above figure, Remote UE goes through the U2N relay UE connecting with gNB-Y (the black line). While Remote UE1 goes through the U2N relay UE as intermediate relay UE1 and Last relay UE connecting with gNB-X (the red line). In case of supporting the current FFS, the U2N relay UE has another role(intermediate relay UE1) for the remote UE1, the U2N relay UE/intermediate relay UE has two different serving cells at a certain moment. As agreed, this cross “X” scenario is not supported in Rel-19 MH SL relay. Hence, for the first FFS, it is preferred not to support.

Proposal 1: If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards a different cell for any remote UE (i.e., there is only one serving cell for the corresponding UE at a certain moment).

For the second FFS, in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE, some additional restriction should be introduced to avoid the scenario in Figure1. RAN2 can leave this issue to SA2, or the UE role can be used as one AS condition to judge that whether a UE can be acted as an intermediate relay UE or not.

Proposal 2: The role of UE can be used as AS condition to judge that whether a UE can be acted as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 3: If RAN2 decides to leave the issue that whether a UE can be acted as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

PC5 AS conditions for discovery at the last relay UE for discovery model B

In the summary of post email discussion before RAN2#128 meeting, the below status was reached:

[DISCUSSION] Proposal 5b: In the case of discovery model B, the following two options can be further discussed:

(option 1) For the discovery response message transmission (in the case of model B), the last Relay UE needs to check the PC5 AS condition before sending discovery response message to the (first) intermediate Relay UE.

(option 2) As the legacy Rel-17 U2N relay, the U2N Relay UE doesn’t have any PC5 AS conditions for transmitting the announcement/response discovery message. The same principle can be applied to the last Relay UE.

We further discuss that whether any PC5 AS conditions for discovery model B at the last relay UE is needed or not. It seems reasonable to discuss the relay discovery for multi-hop U2N relay with an analogy to Rel-17 U2N relay design for the last relay UE. In Rel-17 U2N relay, for relay discovery related discussion, there is no PC5 AS condition captured yet for discovery message initiating/forwarding condition. Hence, we propose:

Proposal 4: For multi-hop relay discovery Model B, there is no PC5 AS condition needed for the last relay UE to forwards the response message (similar as Rel-17 U2N relay discovery).

Any relay (re)selection triggering condition is needed for intermediate Relay UE

As what we discussed in Rel-17 and Rel-18, the trigger condition for remote UE can be summarized as: upper layer, link quality with the current relay below the configured threshold, the link between the remote UE and the relay breaks, the link between the relay and other side breaks. For Rel-19 multi-hop U2N relay (re)selection, there is one leftover issue that any relay (re)selection triggering condition is needed for intermediate relay UE.

The proponent company raised that the intermediate relay UE also acts as remote UE for Paging/SIB and data forwarding, it performs relay reselection same as remote UE. Besides, there is no need to keep the link between the intermediate relay UE and its serving relay if the link is broken (RLF, link release, RRC failure), the intermediate relay can decide to release/switch to another link in such cases.

The opponent company raised that the intermediate relay UE shall not trigger relay reselection by itself, basically, it needs to send notification message towards the remote UE about RLF. Take one step back, when intermediate relay triggers relay reselection, all the downstream UEs are impacted. The reselected path by the intermediate relay may not be the good path for all the downstream UEs. It may be better for the remote UE to know the link status of the intermediate hops and to decide whether to trigger relay reselection by itself.

Besides, according to SA2’s conclusion, it is the remote UE’s responsibility to select the candidate intermediate relay UE and/or the path, hence we should adhere to this principle, any relay (re)selection triggering condition is not specified for intermediate relay UE in Rel-19 multi-hop U2N relay.

Proposal 5: Any relay (re)selection triggering condition is not specified for intermediate Relay UE.

Conclusion

According to the analysis in section 2, it is proposed:

Proposal 1: If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards a different cell for any remote UE (i.e., there is only one serving cell for the corresponding UE at a certain moment).

Proposal 2: The role of UE can be used as AS condition to judge that whether a UE can be acted as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE.

Proposal 3: If RAN2 decides to leave the issue that whether a UE can be acted as an intermediate relay UE or not in case the network doesn’t configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE to SA2, a LS is needed.

Proposal 4: For multi-hop relay discovery Model B, there is no PC5 AS condition needed for the last relay UE to forwards the response message (similar as Rel-17 U2N relay discovery).

Proposal 5: Any relay (re)selection triggering condition is not specified for intermediate Relay UE.

Reference

- R2-2410981 Report of [POST127][401][Relay] MH relay discovery and (re)selection	LG Electronics Inc.	discussion	Rel-19

- R2-2410914	Report from session on positioning and sidelink relay	Session chair (MediaTek)	report