3GPP TSG RAN WG2 #129                                                                                   R2-2500237

Athens, GR, Feb. 17th – 21st, 2025

Source:	CATT

Title:	MHI Enhancement for SCG Activation and Deactivation

Agenda Item:	8.10.5.3

Document for:	Discussion and Decision

Introduction

In RAN2#127 meeting [1], for MHI enhancement for SCG activation and deactivation, RAN2 sent an LS to RAN3 to ask if a NW solution is good enough to achieve the network to have information about time spent in the PSCell in activated/deactivated state, or if a UE based solution is needed.

RAN2#127 meeting: It is beneficial for the network to have information about time spent in the PSCell in activated state vs. deactivated state. Send an LS to RAN3 to ask if a NW solution is good enough to achieve the agreement above, or if a UE based solution is needed.

After RAN2#128 meeting, RAN2 receives the reply LS from RAN3 [2], it is stated that RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

- Overall description RAN3 thanks RAN2 for the LS on MHI enhancement solution for SCG deactivation/activation. RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

In this contribution, we will discuss whether the UE based solution is needed for MHI enhancement for SCG activation/deactivation, and what enhancement is needed if UE based solution is supported.

Discussion

For a Rel-17 version UE, if the UE supports both MHI and SCG activation/deactivation, the UE performs MHI logging based on the Rel-17 spec as follow.

5.7.9.2 Initiation If the UE supports storage of mobility history information, the UE shall: <omit part> 1> If the UE supports PSCell mobility history information and upon change, or release of a PSCell while being connected to the current PCell, or upon release of a PSCell while entering 'camped normally' state or 'any cell selection' state or 'camped on any cell' state: 2> include an entry in visitedPSCellInfoList of the variable VarMobilityHistoryReport possibly after performing the following, if necessary: 3> if visitedPSCellInfoListReport is available in the visitedCellInfoList in variable VarMobilityHistoryReport: 4> for the oldest PCell entry in visitedCellInfoList including visitedPSCellInfoListReport; 5> remove the oldest entry in the visitedPSCellInfoListReport; 3> else: 4> remove the oldest entry in visitedPSCellInfoList in variable VarMobilityHistoryReport; 2> for the included entry: 3> if the global cell identity of the previous PSCell is available: 4> include the global cell identity of that cell in the field visitedCellId of the entry; 3> else: 4> include the physical cell identity and carrier frequency of that cell in the field visitedCellId of the entry; 3> if the PSCell is changed or released while being connected to the current PCell, or if the PSCell is released while entering 'camped normally' state in the same serving cell which was previously the PCell: 4> set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the current PCell/serving cell; 3> if the PSCell is released while entering 'any cell selection' state or 'camped on any cell' state from a suitable cell in RRC_CONNECTED state in NR or LTE, or if the PSCell is released while entering 'camped normally' state while previously in RRC_CONNECTED state in a PCell different from the current serving cell: 4> set the field timeSpent of the entry as the time spent in the previous PSCell while being connected to the previous PCell; <omit part>

Based on the spec above, when the UE is configured with a activated SCG, the UE can log the PSCell MHI correctly, however, when the UE is configured with a deactivated SCG, if the UE logs the PSCell MHI based on current spec (without any enhancement), an issue can be caused that the UE logs the PSCell id and time spent even if the UE has already left the coverage area of this PSCell, e.g., when the UL TAT expiry and UE detects BF/RLF at the deactivated SCG, the network may not send the SCG reconfiguration with sync to UE due to no data needs to be transmitted at SCG, the UE logs the PSCell id and time spent until the UE receives the SCG reconfiguration with sync and SCG is activated when there is data needs to be transmitted at SCG. If the MHI is reported to network, the network will consider the UE is in the coverage area of the PSCell within the time spent, which can cause the network’s misunderstanding and impact the network optimization, e.g., can cause the error handover parameters optimization.

An example is listed in Figure 1 below, network configures UE to access PSCell 1 with activated state at T1, and deactivates the SCG at T2 because of SCG transmission is finished. When the UE moves to PSCell 1 boundary, i.e., at T3, the UL TAT expiry can occur and BF/RLF can be detected, the network may not send the SCG reconfiguration with sync to UE as no data needs to be transmitted at T3. The UE moves out of the coverage area of PSCell 1 and there is data to be transmitted at T4, the network sends SCG reconfiguration with sync to UE and indicates SCG state is activated state. When the UE receives the SCG reconfiguration with sync command at T4, based on the current spec, the UE will log PSCell MHI as {PSCell1, T4-T1}, however, the UE is out of coverage area of PSCell 1 during T3-T4. In this case, if the PSCell MHI is reported to network, the network will consider the UE is under PSCell 1 coverage area during T3-T4, which can cause error network optimization.

Figure 1 UE moving with SCG activation/deactivation

Observation 1: Based on current spec, the UE can log a PSCell id and the time spent in PSCell MHI even if the UE is out of coverage area of the PSCell, e.g., the PSCell is a PSCell of the deactivated SCG.

In order to address the issue above, the MHI enhancement for SCG activation/deactivation needs to be considered. If we use network based solution (i.e., enhance UHI for SCG activation/deactivation), the associated between MHI and UHI needs to be considered. Due to it is the issue regarding MHI (not UHI), it is better to consider the UE based solution.

Proposal 1: UE based solution is supported for MHI enhancement for SCG activation/deactivation.

If we agree to use UE based solution, the SCG activation/deactivation information needs to be included in MHI, e.g., time of SCG activation/deactivation. Considering that the SCG can be deactivated multiple times when connected to the SCG, the following two options can be considered to log the time of SCG activation/deactivation.

Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

For Option 1, it is a straight way to give all the details on SCG activation/deactivation procedure, but it will increase UE logging complexity. For Option 2, it is a simpler way, but the network cannot get the detailed information about SCG activation/deactivation. From our perspective, both the two options can be accepted.

Proposal 2: Enhance MHI to include time of SCG activation/deactivation; the following two logging methods can be considered:

- Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

- Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

Conclusion

According to the discussion in section 2, the observation and proposals are summarized as follows:

Observation 1: Based on current spec, the UE can log a PSCell id and the time spent in PSCell MHI even if the UE is out of coverage area of the PSCell, e.g., the PSCell is a PSCell of the deactivated SCG.

Proposal 1: UE based solution is supported for MHI enhancement for SCG activation/deactivation.

Proposal 2: Enhance MHI to include time of SCG activation/deactivation; the following two logging ways can be considered:

- Option 1: logging each SCG activation/deactivation time duration for a PSCell in chronological order;

- Option 2: logging the accumulated time for SCG activation/deactivation for a PSCell;

Reference

- R2-2409210 Report of 3GPP TSG RAN WG2 meeting #127, Maastricht, Netherlands

- R3-247906 Reply LS on MHI enhancement solution for SCG deactivation/activation