# 3GPP TSG RAN WG2 Meeting #129      	                           R2-2500259
Athens, Greece, 17th – 21th Feb., 2025

Agenda item:	8.1.2.2

Source:	Apple

Title:	Remaining issues on LCM procedure of UE-sided model for AI/ML based beam management

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

- AI/ML general framework for one-sided AI/ML models within the realm of what has been studied in the FS_NR_AIML_Air project [RAN2]:

  - Signalling and protocol aspects of Life Cycle Management (LCM) enabling functionality and model (if justified) selection, activation, deactivation, switching, fallback

    - Identification related signalling is part of the above objective

  - Necessary signalling/mechanism(s) for LCM to facilitate model training, inference, performance monitoring, data collection (except for the purpose of CN/OAM/OTT collection of UE-sided model training data) for both UE-sided and NW-sided models

  - Signalling mechanism of applicable functionalities/models

The revised Rel-19 WID of AI/ML for air interface (WID RP-243244) was agreed in RAN#106 [1]. The WI objective on general LCM framework, including applicable functionalities, is led by RAN2 which is copied below:

In RAN2#127 [2], baseline LCM procedure of UE-sided model of AI/ML based beam management was agreed, and some questions with potential RAN1 impacts were identified to be included in the LS to RAN1 (R2-2407848) [3]. After discussion of two meetings, RAN1 sent Reply LS to RAN2 (R1-2410898) for further discussion [4]. Meanwhile, we identify the following unaddressed FFSs of previous RAN2 agreements:

- Unaddressed FFS of RAN2#127b

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

- Unaddressed FFS of RAN2#128

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

- For data collection configuration UE-side model training, the UE can send a request for data collection.   FFS what the request contains.

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In this contribution, we discuss key remaining issues on LCM procedure, including above FFSs of RAN2 agreements and issues related to RAN1 response in Reply LS (R1-2410898). The discussion follows the following aspects:

- Signaling of data collection configuration and initiation

- Configuration of applicable functionality reporting in Step 3

- Message and format of applicable functionality reporting in Step 4

- Whether Step 5 is mandatory or optional

# 2 Discussion

According to latest RAN2 agreements and RAN1 Reply LS, the applicable functionality reporting framework can be illustrated in Figure. 1 with highlighted FFSs to be discussed in this contribution.

Figure.1: Illustration of basic LCM procedure of UE-sided model for beam management with FFS labelled

## 2.1 Signalling of data collection configuration and initiation

We discuss FFS1 and FFS2 labelled in Figure.1, which are unaddressed FFS in agreements of RAN2#128 [5].

- FFS1: What the request contains

- FFS2: Whether allow UE feedback on data collection configuration

### 2.1.1 FFS1: What the request contains

Before discussion of signaling of UE request, we think it is necessary to discuss what is the data collection configuration. According to RAN1 progress, we understand that the UE needs to collect data for UE-sided model training based on L1 measurements towards CSI-RS (e.g. L1-RSRP and/or beam ID). Thus, CSI-RS configuration needs to provide to the UE. Thus, it is straight forward to reuse L1 CSI framework to provide CSI-RS configuraiton. However, different from legacy L1 CSI framework, the UE is not required to report CSI via PUCCH/PUSCH because the UE-side dataset transfer is expected to be either 3GPP transparent (i.e. option 1a) or via higher layer signaling (i.e. CP or UP solution of option 2/3).

Observation 1: Upon reception of UE-side data collection configuration for AI/ML based beam management, the UE is not required to report CSI via PUCCH/PUSCH because the UE-side dataset transfer is expected to be either 3GPP transparent (i.e. option 1a) or via higher layer signaling (i.e. CP or UP solution of option 2/3).

Thus, we suggest RAN2 to agree the assumption of reusing L1 CSI framework to provide data collection configuration. Whether specfication impact is required to disable L1 CSI reporting via PUSCH / PUCCH can be further discussed.

Proposal 1: RAN2 assume L1 CSI framework is reused to provide data collection configuration for UE-sided model training in AI based BM. FFS whether spec change to disable L1 CSI reporting via PUSCH / PUCCH.

Then, we discuss the signaling of UE request. In our understanding, the request is usful at least from below three aspects:

- The UE request can indicate whether the data collection is needed based on its local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- The UE request can indicate when the data collection can start or stop based on its local / internal informaion on, e.g. available remaining battery and memory information, whether any on-going operations which can’t be interrputed by data collection.

  - The UE can’t share such information with the NW because it may be UE’s internal implementation.

- The UE request can indicate what assistance information and/or reference signal (RS) from NW are needed bsaed on its local / internal informaion, e.g. required CSI-RS resource for measurements.

  - The UE may provide its suggestion to the NW and it is up to NW to decide whether to acommendate the UE requrst as legacy.

Observation 2: The following information is useful to be included in UE request signaling on data collection:

- Whether the data collection is needed based on UE’s local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- When the data collection can start or stop based on UE’s local / internal information, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- What assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

The above 1) and 2) can be combined into one bit indication on whether initializing / stopping data collection, and 3) can be mapped to the suggested CSI-RS resource of set A and set B in AI/ML based beam management, whose details are up to RAN1. Thus, we propose:

Proposal 2: The UE request signaling may include the indication on whether initializing / stopping data collection and suggested CSI-RS resource of set A and set B. Details of UE suggested CSI-RS configuration are up to RAN1.

Finally, we discuss what signaling / message is used for UE request. As discussed in Observation 2, we think the intention to introduce UE request message is mainly to reflect UE’s local / internal information. Thus, it should at least support UE initiated signlaing. As UE-side data collection has no strict latency requirement, we think UAI is perfectly matched. Thus, we propose:

Proposal 3: Introduce UAI message for UE request of data collection configuration.

### 2.1.2 FFS2: Whether allow UE feedback on data collection configuration

This section discuss the following FFS of RAN2#128 [5]:

- FFS whether an indication from UE to network is needed when UE can’t perform data collection based on received configuration

In our understanding, this FFS is intended to discuss whether it is necessary to introduce NW initiated UE request signaling (e.g. via RRCReconfigurationComplete message as response to data collection configuration in RRCReconfiguartion).

If our Proposal 3 is agreed (i.e. UAI is introduced for UE request), it means the UE is allowed to send the request based on its implementation. Then, if the UE can’t perform data collection based on received configuration, it can send the request via UAI with indication of stopping data collection and suggestion on applicable CSI resource configuration. Thus, we think UAI should be sufficient and there is no strong need to introduce NW initiated UE request signaling on top of UE initiated UAI message. Thus, we propose:

Proposal 4: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Finally, we want to clarify one important point: the reception of data collection configuration doesn’t mean the UE has to immediately start model training. As captured in Section 1 of TR 38.843 [6], the specific AI/ML models are not expected to be specified and left to implementation.

From Section 1 of TR 38.843:

Note 1:	Specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved.

Note 2:	The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In our understanding, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training. Meanwhile, the UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Observation 3: After reception of data collection configuration, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training.

Observation 4: The UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Thus, we believe that it is up to UE implementation when to start / stop training of the UE-side model.

Proposal 5: After reception of data collection configuration, it is up to UE implementation when to start / stop training of the UE-side model.

## 2.2 Configuration of applicable functionality reporting in Step 3

### 2.2.1 FFS3: Configuration of applicability reporting

One major progress provided in RAN1 Reply LS [4] is the following agreed option A and option B for applicability reporting configuration in Step 3:

Agreement - In Step 3, following configurations are provided from NW to UE: - UE is allowed to do UAI reporting via OtherConfig, - The applicability report is based on A) and/or B) - It is up to RAN 2 to design the container - A) one or more of CSI-ReportConfig for inference configuration (wherein the associated ID may be configured in CSI framework as working assumption applied) - Note: CSI report configuration for UE-side model inference can’t be activated immediately upon receiving Step 3 - B) One set or multiple sets of inference related parameters for applicability report only (not for inference) - It is up to RAN2 to design the container. - The set of inference related parameters selected from the IEs in/or the IEs referred by CSI-ReportConfig as a starting point, e.g., - the associated ID - Note: this doesn’t imply the associated ID is mandatory - Set A related information - Set B related information - Report content related information - For BM-Case 2, - Time instances related information for measurements - Time instances related information for prediction

According to discussion with RAN1 colleague, we have the following understanding on above RAN1 agreement:

- RAN1 can’t down select between option A and option B, and finally decide to support both. Thus, RAN2 also don’t need to discuss their down-selection.

- Option A can be regarded as full inference configuration which allows the UE to immediately start inference operation for the applicable functionalities without need to wait Step 5 (e.g., periodic CSI reporting as RAN1 Reply LS mentioned).

- Option B can be regarded as partial inference configuration. The UE’s feedback on option B is used for the gNB to generate the full inference configuration (i.e. CSI-ReportConfig) in Step 5.

Observation 5: Option A can be regarded as full inference configuration which allows the UE to immediately start inference operation for the applicable functionalities after Step 4 (e.g., periodic CSI reporting as RAN1 Reply LS mentioned).

Observation 6: Option B can be regarded as partial inference configuration. The UE’s feedback on option B is used for the gNB to generate the full inference configuration (i.e. CSI-ReportConfig) in Step 5.

According to Observation 5 and 6, we identify the following RAN2 impacts:

- Option A and option B are not mutual exclusive. Thus, NW can configure both Option A and Option B in one single RRCReconfiguration message of Step 3 based on its implementation.

- As Option A allows the UE to immediately start inference before Step 5, it should be configured in legacy CSI framework (i.e. CSI-MeasConfig) rather than OtherConfig in RRCReconfiguration message of Step 3.

- As Option B can’t activate CSI inference alone, it should not be integrated into legacy CSI framework. It is more suitable to be included in OtherConfig in RRCReconfiguration message of Step 3.

One example of the RRC structure can be illustrated in Figure. 2.

Figure.2: Illustration of RRC structure of applicability report configuration in Step 3

Based on above analysis, we propose:

Proposal 6: It is up to NW implementation to configure either option A/B or both for applicability reporting configuration in one RRCReconfiguration message of Step 3:

- Option A (i.e. one or more of CSI-ReportConfig for inference configuration) is configured in CSI-MeasConfig of RRCReconfiguration.

- Option B (i.e. one or multiple sets of inference related parameters provided by RAN1 LS) is configured in OtherConfig of RRCReconfiguration.

### 2.2.2 FFS4: Whether associated ID is mandatory or optional

This issue was discussed in RAN1, but RAN1 can’t achieve consensus and will further discuss this issue:

Q4-1: In RAN2, it is FFS whether NW-side additional condition is mandatory or optional. In order to discuss further, RAN2 would like to understand whether it is feasible for UE to decide the applicable functionalities without NW-side additional condition?

Answer to Q4-1: There is no consensus yet on whether it is mandatory or optional. There is no conclusion yet on whether it is feasible or not for UE to decide the applicability without NW-side additional condition, and RAN 1 is discussing the related issues.

To make progress, we share our view from RAN2 perspective. We prefer the associated ID is mandatory provided in Step 3 because absence of associated ID may cause model ambiguity between UE and NW. As we know, one AI/ML functionality may include multiple NW-side additional conditions corresponding to multiple AI/ML models. Then, the UE may not know which model should be used for inference if the NW doesn’t indicate the current NW-side additional condition (i.e. associated ID) to the UE. We provide an example in Figure. 3, where the UE has model 1 (corresponding to associated ID 1) and model 2 (corresponding to associated ID 2) available for the same functionality A while the current NW condition is associated ID = 2. Then, after UE reports that the functionality is applicable under either associated ID=1 or ID=2 in step 4, the NW activates the functionality A in step 5, but the UE doesn’t know whether it should use model 1 or model 2 for inference. If the UE chooses to use model 1, it will lead to non-consistency with current NW condition.

Figure.3: Issue illustration of model ambiguity between UE and NW if associated ID is absent in Step 3

Thus, we propose that associated ID should be mandatory provided in Step 3 from RAN2 perspective. Otherwise, it may cause model ambiguity between UE and NW

Proposal 7: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

## 2.3 Message and format of applicability reporting in Step 4

### 2.3.1 FFS5: Which RRC message is used for reporting

When both UAI and RRCReconfigurationComplete were agreed for applicability reporting in RAN2#127b [7], there were concern raised on their duplicated reporting. That is why it was agreed to aim to align the design of their reporting.

- UAI is supported and RRCReconfigurationComplete message can be used to report applicable functionality.   We should aim to align the design on how the applicable functionality are signaled.   FFS on the applicability reporting content.

However, RAN1 has now introduced two options of configuration, i.e. both option A and option B can be configured in one single RRC message in Step 3. To better handle these two options, we think the simplest and cleanest UE behaviour is to use separate RRC messages for Option A and Option B, respectively.

Observation 7: Since RAN1 has introduced two applicability reporting configuration option A and B in step 3, the simplest and cleanest UE behavior is to use separate RRC messages (i.e. UAI and RRCReconfigurationComplete) for different options respectively.

Then, based on our Proposal 6, we propose to use UAI to report Option B and RRCReconfigurationComplete to report option A.

Proposal 8: RRCReconfigurationComplete is used to report applicability of CSI-ReportConfig(s) configured in Step 3 (i.e., Option A) while UAI is used to feedback configured inference parameter set(s) (i.e., Option B).

### 2.3.2 FFS6: Detailed reporting format

Given Proposal 8, we will discuss the reporting format for RRCReconfigurationComplete and UAI separately.

#### 2.3.2.1 Reporting format of RRCReconfigurationComplete

According to Observation 5, we think the benefit of Option A is that inference can start earlier because full inference configuration (i.e. CSI-ReportConfig) is provided in Step 3. Thus, the total number of CSI-ReportConfig is not expected to large and the UE needs to report all the configured CSI-ReportConfig in one RRC message upon reception of Step 3.

Observation 8: The benefit of Option A is inference can start earlier. Thus, the number of CSI-ReportConfig is not expected to large and the UE needs to report all the configured CSI-ReportConfig in one RRC message.

So, we think it is sufficient to report a bitmap on whether each CSI-ReportConfig is applicable or non-applicable.

Proposal 9: When option A is configured in Step 3, the UE reports a bitmap on whether each CSI-ReportConfig is applicable or non-applicable in RRCReconfigurationComplete.

According to Section 12 of TS 38.331 [8], there are two processing latency requirements (10ms vs 16ms) between reception of RRCReconfiguration and reporting RRCReconfigurationComplete.

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (Intra-NR mobility with LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration | DLDedicatedMessageSegment | RRCReconfigurationComplete | 16+( Nseg -1)*10 | Nseg is number of RRC segments |

As Rel-19 is the first release of AI/ML, we expect new / unpredicted challenges for the UE to handle AI/ML operation. Thus, we prefer to use relaxed RRC processing latency requirements (i.e. 16ms) which is a safer option.

Proposal 10: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

#### 2.3.2.2 Reporting format of UAI

Different from Option A, we understand that Option B is mainly used to handle the status change between “applicable” and “non-applicable”. Thus, it is not necessary to report applicability status for all configured sets in one single UAI message, i.e., the UE should have the flexibility to timely feedback only a subset of configured inference parameter sets.

Observation 9: Different from Option A, Option B is mainly used to handle the status change between “applicable” and “non-applicable”. Thus, the UE should have flexibility to timely feedback only a subset of configured inference parameter sets.

In RAN2#128 [5], explicit “non-applicable” functionality reporting was discussed because the change from “applicable” to “non-applicable” is expected to have more UE impacts than the change from “non-applicable” to “applicable”. However, it was not agreed because some companies prefer to have a unified reporting format.

- FFS whether the UE reports explicitly “non-applicable” functionality when there is a change of applicability.   Verify this aligns with RAN1 configuration design

Given that both Option A and Option B are supported, we think it is not necessary any more to keep the same reporting format for these two options. On the contrary, Observation 8 and 9 indicate there are indeed some differences on the required reporting between Option A and Option B. Thus, we propose to introduce explicit “non-applicable” reporting for the configured inference parameter sets in Option B.

Proposal 11: When option B is configured in Step 3, the UE reports the index / indices of non-applicable sets of inference related parameter in UAI upon change of applicability status.

Finally, we think it is useful to provide the NW the cause value on why one functionality becomes non-applicable because the potential NW reaction may depend on different cause values:

- “Non-applicable due to model non-availability”: this cause value means that the UE may complete model downloading shortly. Thus, the gNB can just wait the UE to complete the model downloading before reconfiguration.

- “Non-applicable due to NW-side additional condition”: this cause value means that the gNB can resolve the issue by reconfiguration of its inference configuration.

- “Non-applicable due to UE-side additional condition”: this cause value means that the UE may have battery or memory issue due to heavy inference operation. Correspondingly, the gNB may resolve the issue by reconfiguration of light inference operation (e.g., set A and/or set B with smaller size).

Observation 10: It is useful to provide the NW the cause value on why one functionality becomes non-applicable:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

Thus, to facilitate NW taking suitable reaction, we propose that the UE may include cause value in the reporting.

Proposal 12: Together with non-applicable functionalities, the UE can optionally report the following cause value of “non-applicable”:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

## 2.4 Whether Step 5 is mandatory or optional

On FFS7 (whether Step 5 is mandatory or optional), we think the following conclusion is straight forward based on Observation 5 and 6:

- If option A is configured in Step 3, the UE may autonomously activate the applicable functionalities (e.g. periodic CSI reporting as agreed in RAN1) upon successfully reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- If option B is configured in Step 3, the UE activates the applicable functionalities only after reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5.

Thus, we propose:

Proposal 13: If option A is configured in Step 3, the UE may autonomously activate the applicable functionalities (e.g. periodic CSI reporting as agreed in RAN1) upon successfully reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Proposal 14: If option B is configured in Step 3, the UE activates the applicable functionalities only after reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5.

# 3 Conclusion

In this contribution, we discuss key remaining issues on LCM procedure of UE-sided model for AI/ML based beam management. Our observations are:

Observation 1: Upon reception of UE-side data collection configuration for AI/ML based beam management, the UE is not required to report CSI via PUCCH/PUSCH because the UE-side dataset transfer is expected to be either 3GPP transparent (i.e. option 1a) or via higher layer signaling (i.e. CP or UP solution of option 2/3).

Observation 2: The following information is useful to be included in UE request signaling on data collection:

- Whether the data collection is needed based on UE’s local / internal information, e.g. whether the device local model is ready for interence or needs further training.

- When the data collection can start or stop based on UE’s local / internal information, e.g. remaining battery / memory status, whether any on-going operations which can’t be interrputed by data collection.

- What assistance information and/or reference signal (RS) from NW are needed for model training, e.g. required CSI-RS resource for measurements.

Observation 3: After reception of data collection configuration, the UE may not immediately start model training due to various UE internal reasons, e.g. the UE may need to perform pre-processing on the collected data before transferring the dataset to its server for training.

Observation 4: The UE may also stop performing model training due to various UE internal reasons, e.g. model is ready for inference or insufficient memory / battery.

Observation 5: Option A can be regarded as full inference configuration which allows the UE to immediately start inference operation for the applicable functionalities after Step 4 (e.g., periodic CSI reporting as RAN1 Reply LS mentioned).

Observation 6: Option B can be regarded as partial inference configuration. The UE’s feedback on option B is used for the gNB to generate the full inference configuration (i.e. CSI-ReportConfig) in Step 5.

Observation 7: Since RAN1 has introduced two applicability reporting configuration option A and B in step 3, the simplest and cleanest UE behavior is to use separate RRC messages (i.e. UAI and RRCReconfigurationComplete) for different options respectively.

Observation 8: The benefit of Option A is inference can start earlier. Thus, the number of CSI-ReportConfig is not expected to large and the UE needs to report all the configured CSI-ReportConfig in one RRC message.

Observation 9: Different from Option A, Option B is mainly used to handle the status change between “applicable” and “non-applicable”. Thus, the UE should have flexibility to timely feedback only a subset of configured inference parameter sets.

Observation 10: It is useful to provide the NW the cause value on why one functionality becomes non-applicable:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

Based on observations, our proposals can be found below:

Signaling of data collection configuration and initiation

Proposal 1: RAN2 assume L1 CSI framework is reused to provide data collection configuration for UE-sided model training in AI based BM. FFS whether spec change to disable L1 CSI reporting via PUSCH / PUCCH.

Proposal 2: The UE request signaling may include the indication on whether initializing / stopping data collection and suggested CSI-RS resource of set A and set B. Details of UE suggested CSI-RS configuration are up to RAN1.

Proposal 3: Introduce UAI message for UE request of data collection configuration.

Proposal 4: As the UE may send data collection request via UAI based on its implementation, no need to introduce a separate indication signaling when UE can’t perform data collection based on received configuration.

Proposal 5: After reception of data collection configuration, it is up to UE implementation when to start / stop training of the UE-side model.

Configuration of applicable functionality reporting in Step 3

Proposal 6: It is up to NW implementation to configure either option A/B or both for applicability reporting configuration in one RRCReconfiguration message of Step 3:

- Option A (i.e. one or more of CSI-ReportConfig for inference configuration) is configured in CSI-MeasConfig of RRCReconfiguration.

- Option B (i.e. one or multiple sets of inference related parameters provided by RAN1 LS) is configured in OtherConfig of RRCReconfiguration.

Proposal 7: RAN2 assume that associated ID is mandatory provided in Step 3. Otherwise, it may cause model ambiguity issue between UE and NW.

Format of applicable functionality reporting in Step 4

Proposal 8: RRCReconfigurationComplete is used to report applicability of CSI-ReportConfig(s) configured in Step 3 (i.e., Option A) while UAI is used to feedback configured inference parameter set(s) (i.e., Option B).

Proposal 9: When option A is configured in Step 3, the UE reports a bitmap on whether each CSI-ReportConfig is applicable or non-applicable in RRCReconfigurationComplete.

Proposal 10: As Rel-19 is the first release to introduce AI/ML, relaxed RRC processing timing requirement is applied for reporting applicability in RRCReconfigurationComplete (i.e. within 16ms after reception of Step 3).

Proposal 11: When option B is configured in Step 3, the UE reports the index / indices of non-applicable sets of inference related parameter in UAI upon change of applicability status.

Proposal 12: Together with non-applicable functionalities, the UE can optionally report the following cause value of “non-applicable”:

- “Due to model non-availability”: gNB can just wait the UE to complete the model downloading.

- “Due to NW-side additional condition”: gNB may reconfigure its inference configuration accordingly.

- “Due to UE-side additional condition”: gNB may reconfigure a light inference to reduce UE burden (e.g., set A and/or set B with smaller size).

Whether Step 5 is mandatory or optional

Proposal 13: If option A is configured in Step 3, the UE may autonomously activate the applicable functionalities (e.g. periodic CSI reporting as agreed in RAN1) upon successfully reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

Proposal 14: If option B is configured in Step 3, the UE activates the applicable functionalities only after reception of CSI-ReportConfig for inference configuration in RRCReconfiguration message of step 5.

# 4 References

[1] RP-243244, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm.

[2] RAN2#127, Chair Notes.

[3] R2-2407848, LS on applicable functionality reporting for beam management UE-sided model.

[4] R1-2410898, Reply LS on applicable functionality reporting for beam management UE-sided model.

[5] RAN2#128, Chair Notes.

[6] TR 38.843-v2.0.1, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), 2023-12.

[7] RAN2#127b, Chair Notes.

[8] TS 38.331-v18.0.0, Radio Resource Control (RRC) protocol specification, 2023-12.