3GPP TSG RAN WG2 Meeting #129              	                	                          R2-2500265

Athens, Greece, Feb 17th – 21st, 2025

Agenda Item:	8.1.5

Source:	Xiaomi Corporation

Title:	Feasibility analysis of model/dataset transfer solutions

Document for:	Discussion and Decision

# Introduction

During Rel-18, RAN2 studied potential solutions for AI/ML model transfer/delivery and provided potential RAN specification impact in TR 38.843 [2].

To support AI/ML model transfer/delivery, the following solutions are considered: - Solution 1a: gNB can transfer/deliver AI/ML model(s) to UE via RRC signalling. - Solution 2a: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via NAS signalling. - Solution 3a: LMF can transfer/deliver AI/ML model(s) to UE via LPP signalling. - Solution 1b: gNB can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 2b: Core Network (except LMF) can transfer/deliver AI/ML model(s) to UE via User Plane (UP) data. - Solution 3b: LMF can transfer/deliver AI/ML model(s) to UE via UP data. - Solution 4a: OTT server can transfer/deliver AI/ML model(s) to UE (e.g., transparent to 3GPP). - Solution 4b: OAM can transfer/deliver AI/ML model(s) to UE. The following areas are considered to evaluate the different model transfer/delivery solutions: - A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

Based on RAN1 further discussion, RAN2 receives RAN1 LS in R1-2410922 [1] asking the feasibility of standardized signaling for NW-side sharing model parameters and/or dataset to the UE.

In this contribution, based on RAN1 agreed information (i.e. dataset, encoder parameter, or both), we provide the feasibility analysis for each model transfer/delivery solution.

# Discussion

Recalling that RAN2 studied model transfer/delivery aspect during Rel-18 and analysed the feasibility of model transfer/delivery from the following four aspects [2]:

- A1: Large, no upper limit model/model parameter size, - A2: Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs), - A3: Network controllability on model transfer/delivery (e.g., management decision at gNB), - A4: Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB).

According to RAN1 LS in R1-2410922 [1], we try to summarize the key requirement from RAN1 on model transfer/delivery in below table:

| Method | Data/parameter size | Latency | Frequency |
| --- | --- | --- | --- |
| Option 4-1: sharing {target CSI, CSI feedback} dataset | 225MB | Relaxed latency (e.g. days/weeks) | Infrequent |
| Option 3a-1 without target CSI: sharing encoder parameter | 11.6MB (medium model size); 52MB (largest model size) |  |  |
| Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset | 236.6~277MB |  |  |

Observation 1: Based on RAN1 LS, model transfer/delivery for two-sided model has the following requirements:

- Data/parameter size may be as large as 277MB

- Relaxed latency (e.g. days/weeks)

- Infrequent updates

Before analyzing the feasibility of each solution, we first analyze how RAN1 requirements can be reflected to RAN2 studied aspects of model transfer/delivery feasibility analysis.

## Requirement for model transfer/delivery

A1: Data/model parameter size

For CP solutions, transferring the required data/parameter size over the air interface could be limited by the size of RRC/NAS signaling.

In Solution 1a (i.e. gNB transfer via RRC signaling), RRC signaling for model transfer (in Option 4-1, Option 3a-1 without parameter, Option 3a-1 with parameter) needs to be segmented into:

- Option 4-1: 225MB/9kB = 25,000 segments

- Option 3a-1 without parameter: 11.6MB/9kB = 1,288 segments (medium model size)

- Option 3a-1 with parameter: 236.6MB/9kB = 26,288 segments (medium model size)

As for Solution 2a, since segmentation is also not supported for NAS signaling, the same number of segments is need to be done in RRC layer.

On the other hand, from UP point of view, dataset/model parameter transfer over air interface can be considered same as packet transmission, where a packet size of 300MB.

A2: Model transfer/delivery continuity

As mentioned in RAN1 LS, dataset/model parameter can be transmitted with a relaxed latency (e.g. days/weeks). It is very possible that the UE will move to another cell during this relaxed period for model transfer/delivery. Therefore, continuity for model transfer/delivery across different cells/NG-RAN nodes needs to be supported.

A3: Network controllability at gNB

As quoted by RAN1 LS [1], the discussion of model transfer/delivery is mainly focused on CSI compression use case, where two-sided model is considered.

Study objectives: - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing necessary specification impact analysis, as well as, other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843.

RAN1 further made the following agreements [1]:

- NW-side shares model parameters and/or dataset to the UE or UE-side via standardized signaling, where the model parameters and/or dataset received at the UE or UE-side go through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., model (re-)training.

Based on the above agreement, it is clear that network will initiate dataset/model parameter sharing to UE if network configures two-sided model for CSI compression. Moreover, considering gNB and UE have two different components of the two-sided model, whenever the gNB updates the model, a corresponding update needs to be triggered to ensure the consistency between gNB and UE side model. Therefore, model transfer/delivery of two-sided model should be managed (i.e. initiated, maintained, controlled) at gNB side.

A4: Model transfer/delivery QoS

Last but not least, according to RAN1 LS, transferring dataset/model parameters over the air interface is infrequent with relaxed latency requirement for each transmission. Therefore, it does not require a high priority level and/or a short packet delay budget for data transmission.

Based on above analysis, we summarized the following requirement for dataset/model parameter transfer over the air interface.

Proposal 1: Based on RAN1 LS, the following requirements need to be considered when selecting a suitable solution for model transfer/delivery:

- Up to 30,000 RRC segments via CP, 300MB packet transmission via UP

- Support service continuity across multiple cells/NG-RAN nodes

- Model transfer/delivery of two-sided model managed at gNB side

- Low priority level, large packet delay budget and/or low data burst volume

## Feasibilty Analysis

Solution 1a/2a/4b

Note that current specification only supports up to 9 segments in DL RRC signaling. There’s a clear gap to support 30,000 segments over RRC signaling. Even though network can transmit such massive number of segments in a lower priority SRB (according to QoS requirement), it is still difficult for network to maintain this massive number of segments. Furthermore, service continuity is currently not supported over CP, where PDCP data forwarding needs to be considered over Xn interface for SRB.

As captured in TR 38.843 [2], OAM can transfer/deliver AI/ML models to UE via OAM→RAN→UE, where Control Plane (CP) signalling is used for RAN→UE. Hence, similar impact is also applicable to Solution 4b.

Observation 2: In Solution 1a/2a/4b, it is challenging to support over 30,000 segments PDCP data forwarding over SRB for service continuity, even considering a low priority SRB.

Solution 1b

The main challenge in Solution 1b is supporting establishment of a PDU terminating at gNB.

According to TS 38.413 [3], the AMF initiates the PDU session establishment procedure by sending a PDU SESSION RESOURCE SETUP REQUEST message to the NG-RAN node. If resources are available for the

requested configuration, the NG-RAN node shall execute the requested NG-RAN configuration and allocate associated resources over NG and over Uu for each PDU session. However, such PDU session setup cannot be self-initiated or requested by NG-RAN node.

During Rel-18 SI discussion, one way is to let gNB send an indication to CN and ask for assistance to establishment a PDU session. However, the feasibility of such approach needs to be further evaluated by SA2.

Observation 3: In Solution 1b, it is up to SA2 on whether PDU session setup can be requested by NG-RAN node.

Solution 2b

It is straightforward that Solution 2b as a user plane solution can naturally support large packet size (e.g. 300MB). As required by RAN1, dataset/model parameter transfer does not need a high priority level with urgent transmission over air interface. We noticed that there’s a new 5QI introduced for intelligent transport system in TS 23.501 [4]. However, the two-sided model transfer could be different from those intelligent transport systems, considering the model only needs to be updated daily or weekly.

However, recalling that this model transfer/delivery procedure is managed by gNB, some necessary enhancements between gNB and CN are needed to initiate/maintain model transfer/delivery procedure over NG interface. Furthermore, whether CN is suitable to store/transfer gNB-side model for air interface purpose needs to be further checked with SA2.

Observation 4: In Solution 2b, there’s less specification impact of defining a new 5QI value, but signalling overhead  may be introduced to support gNB controllability where model is transferred from CN.

Solution 4a

It was captured in TR38.843 [2] with the following RAN impacts:

| Discussion Area | Potential RAN specification impact |
| --- | --- |
| A1. Large, no upper limit model/model parameter size | No RAN impact |
| A2. Model transfer/delivery continuity (i.e., resume transmission of model (segments) across gNBs) | Note: supporting service continuity across LMF is out of RAN scope |
| A3. Network controllability on model transfer/delivery and management at gNB | - Requires management and model transfer interaction between OTT server and gNB when model management at gNB - Note: it is unclear whether this is within RAN scope - Requires interaction between UE and gNB for the network controllability of the model transfer/delivery (e.g., model identification, model transfer completion, etc.) if management is in gNB |
| A4. Model transfer/delivery QoS (for DRB) (including latency, etc.) and priority (for SRB) | Note: The detail QoS requirement for model transfer/delivery of solution 4a is out of RAN scope |

It is noticed that the only impact from RAN point of view is the management for model transfer/delivery. Based on our understanding, Solution 4a can be modeled as: gNB -> gNB server (two-sided model training and storage) -> UE OTT server -> UE, where management of model transfer/delivery is controlled by gNB towards gNB-side server by implementation. Moreover, gNB server and UE OTT server can also be connected based on IP tunnel, which can be purely transparent to 3GPP network. In the end, since UE OTT server is in the application layer, either transmitting dataset/model parameter over non-3GPP method or via application layer could be feasible.

In summary, below table summarizes the feasibility analysis of RAN2 agreed solutions.

Proposal 2: RAN2 captures below table in TR 38.843.

|  | Solution 1a (CP via RRC) | Solution 2a (CP via NAS) | Solution 1b (UP terminated at gNB) | Solution 2b (UP terminated at CN) | Solution 4a (OTT server - UE) | Solution 4b (OAM-gNB-UE) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | Challenging due to massive number of segments (25,000 segments for Option 4-1; 1,288 segments for Option 3a-1 without parameter; 26,288 segments for Option 3a-1 with parameter) |  | Supported |  | Transparent to 3GPP | Same as Solution 1a/2a |
| A2 | Can be supported by enhancing PDCP data forwarding in SRB |  | PDU terminated at gNB is not supported and depends on SA | Supported |  |  |
| A3 | Supported via RRC signaling | Can be supported by defining gNB and CN interface for model transfer/delivery management | Supported via RRC signaling | Can be supported by defining gNB and CN interface for model transfer/delivery management |  |  |
| A4 | Can be supported by low priority SRB |  | Can be supported by defining a new 5QI for relaxed latency and infrequent model transfer/delivery |  |  |  |
| Summary | Challenging | Challenging | Depends on SA | Depends on SA | Feasible | Challenging |

Proposal 3: RAN2 provides below feasibility analysis to RAN1:

It is feasible to use Solution 4a to support all options from RAN1. The feasibility of other solutions is provided as below:

|  | Option 4-1: sharing {target CSI, CSI feedback} dataset (model size 225MB) | Option 3a-1 without target CSI: sharing encoder parameter (model size 11.6MB) | Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset (model size 161.6MB) |
| --- | --- | --- | --- |
| Solution 1a (CP – gNB) Solution 2a (CP – CN) | Up to 25,000 segments | Up to 1288 segments | Up to 18,000 segments |
|  | Challenging for massive segmentation at RRC |  |  |
| Solution 1b (UP – gNB) | Depends on SA2 Currently PDU session terminated at RAN is not supported, need further evaluation in SA2 |  |  |
| Solution 2b (UP – CN) | Depends on SA2 Signaling overhead between gNB and CN will be introduced; Needs to further check with SA2 on whether CN is a suitable location for air interface use case model transfer/model storage. |  |  |
| Solution 4a (OTT – UE) | Feasible (NW-side server -> UE OTT server -> UE) |  |  |
| Solution 4b (OAM –UE) | CP: Challenging (same as CP solutions limited by RRC message size) UP: Challenging (no direct link between OAM and UE) |  |  |

# Conclusion

In this contribution, according to RAN1 LS and RAN2 studied outcome in TR 38.843, we further analyzed model transfer requirement and feasibility of RAN2 candidate solutions. We have below observations and proposals:

Observation 1: Based on RAN1 LS, model transfer/delivery for two-sided model has the following requirements:

- Data/parameter size may be as large as 277MB

- Relaxed latency (e.g. days/weeks)

- Infrequent updates

Proposal 1: Based on RAN1 LS, the following requirements need to be considered when selecting a suitable solution for model transfer/delivery:

- Up to 30,000 RRC segments via CP, 300MB packet transmission via UP

- Support service continuity across multiple cells/NG-RAN nodes

- Model transfer/delivery of two-sided model managed at gNB side

- Low priority level, large packet delay budget and/or low data burst volume

Observation 2: In Solution 1a/2a/4b, it is challenging to support over 30,000 segments PDCP data forwarding over SRB for service continuity, even considering a low priority SRB.

Observation 3: In Solution 1b, it is up to SA2 on whether PDU session setup can be requested by NG-RAN node.

Observation 4: In Solution 2b, there’s less specification impact of defining a new 5QI value, but signalling overhead may be introduced to support gNB controllability where model is transferred from CN.

Proposal 2: RAN2 captures below table in TR 38.843.

|  | Solution 1a (CP via RRC) | Solution 2a (CP via NAS) | Solution 1b (UP terminated at gNB) | Solution 2b (UP terminated at CN) | Solution 4a (OTT server - UE) | Solution 4b (OAM-gNB-UE) |
| --- | --- | --- | --- | --- | --- | --- |
| A1 | Challenging due to massive number of segments (25,000 segments for Option 4-1; 1,288 segments for Option 3a-1 without parameter; 26,288 segments for Option 3a-1 with parameter) |  | Supported |  | Transparent to 3GPP | Same as Solution 1a/2a |
| A2 | Can be supported by enhancing PDCP data forwarding in SRB |  | PDU terminated at gNB is not supported and depends on SA | Supported |  |  |
| A3 | Supported via RRC signaling | Can be supported by defining gNB and CN interface for model transfer/delivery management | Supported via RRC signaling | Can be supported by defining gNB and CN interface for model transfer/delivery management |  |  |
| A4 | Can be supported by low priority SRB |  | Can be supported by defining a new 5QI for relaxed latency and infrequent model transfer/delivery |  |  |  |
| Summary | Challenging | Challenging | Depends on SA | Depends on SA | Feasible | Challenging |

Proposal 3: RAN2 provides below feasibility analysis to RAN1:

It is feasible to use Solution 4a to support all options from RAN1. The feasibility of other solutions is provided as below:

|  | Option 4-1: sharing {target CSI, CSI feedback} dataset (model size 225MB) | Option 3a-1 without target CSI: sharing encoder parameter (model size 11.6MB) | Option 3a-1 with target CSI: sharing encoder parameters, along with {target CSI} dataset (model size 161.6MB) |
| --- | --- | --- | --- |
| Solution 1a (CP – gNB) Solution 2a (CP – CN) | Up to 25,000 segments | Up to 1288 segments | Up to 18,000 segments |
|  | Challenging for massive segmentation at RRC |  |  |
| Solution 1b (UP – gNB) | Depends on SA2 Currently PDU session terminated at RAN is not supported, need further evaluation in SA2 |  |  |
| Solution 2b (UP – CN) | Depends on SA2 Signaling overhead between gNB and CN will be introduced; Needs to further check with SA2 on whether CN is a suitable location for air interface use case model transfer/model storage. |  |  |
| Solution 4a (OTT – UE) | Feasible (NW-side server -> UE OTT server -> UE) |  |  |
| Solution 4b (OAM –UE) | CP: Challenging (same as CP solutions limited by RRC message size) UP: Challenging (no direct link between OAM and UE) |  |  |

# Reference

[1] R1-2410922 LS on signalling feasibility of dataset and parameter sharing

[2] TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[3] TS 38.413 NG Application Protocol (NGAP)

[4] TS 23.501 Procedures for the 5G System (5GS)