3GPP TSG-RAN WG2 Meeting #129                                                                    R2-2500275

Athens, Greece, Feb. 17th – 21st, 2025

Source:	CATT

Title:	Discussion on other aspects for SBFD

Agenda Item:	8.11.3

Document for:	Discussion

1. Introduction

The objectives of Sub-band full duplex (SBFD) which are related with RAN2 except random access can be found as below:

- For subband non-overlapping full duplex (SBFD) operation at gNB side within a TDD carrier:

  - Specify semi-static indication of time location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify semi-static indication of frequency domain location of SBFD subbands to UEs in RRC_CONNECTED mode [RAN1, RAN2]

  - Specify UE transmission, reception and measurement behavior and procedures in SBFD symbols and/or non-SBFD symbols for SBFD aware UE [RAN1, RAN2]

    - Transmission and reception behaviours on SBFD subbands configured in DL and/or flexible symbol indicated by TDD-UL-DL-ConfigCommon

    - Enhancement on resource allocation in frequency domain in SBFD symbols, including

    - Enhancements on physical channels/signals and procedure across SBFD symbols and non-SBFD symbols in different slots, where each transmission/reception within a slot has either all SBFD or all non-SBFD symbols,

    - Configurations for SRS, PUCCH and PUSCH on SBFD symbols and non-SBFD symbols, e.g., resources, frequency hopping parameters, UL power control parameters and/or beam/spatial relation

    - Collision handling between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol

  - Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4]

    - UL resource muting for PUSCH[RAN1, RAN2, RAN4]

    - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework,[RAN1, RAN2, RAN4]

We summarized the progress in RAN1 on SBFD configuration for UE transmission, reception and the new measurement in this contribution.

2. Discussion

RAN1 has discussed the signal and channel-specific procedures. The agreements aim to enhance the flexibility and efficiency by providing detailed configurations for SBFD, collision handling, frequency hopping, power control, and CLI mitigation. These configurations ensure optimal performance and resource management in various scenarios, including dynamic scheduling, multi-PUSCH/PDSCH scheduling, and UE-to-UE interference management.

Here is the agreement in RAN1 as below on the configurations for DL/UL BWP.

Agreement

For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE,

- Support of configuration 2 is subject to UE capability. Configuration 1 is the default capability.

- SBFD-aware UE can be configured with Configuration 2 on a per UL/DL BWP basis.

  - The configuration for DL BWP at least applies to PDSCH receptions within the DL BWP.

  - The configuration for UL BWP applies to PUCCH and PUSCH transmissions within the UL BWP.

- For SRS, only Configuration 1 is applicable.

We can conclude that the SBFD-aware UE can be configured with configurations for DL/UL BWP across SBFD symbols and non-SBFD symbols in different slots based on the UE capability.

Proposal 1: SBFD-aware UE can be configured with configurations on a per UL/DL BWP basis across SBFD symbols and non-SBFD symbols in different slots subject to the UE capability.

Agreement

For separate SRS configurations for SBFD and non-SBFD symbols, support Option 1.

Option 1: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage.

- SRS-ResourceSet configured for SBFD symbol is only applicable to SRS transmission occasions in SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in non-SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only SBFD symbols are available for SRS transmission.

  - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- SRS-ResourceSet configured for non-SBFD symbol is only applicable to SRS transmission occasions in non-SBFD symbols.

  - For periodic and semi-persistent SRS, the SRS transmissions in SBFD symbols are dropped.

  - For aperiodic SRS with available slot counting, only non-SBFD symbols are available for SRS transmission.

  - FFS impact on availableSlotOffsetList

  - For aperiodic SRS without available slot counting, it is expected that UE is indicated to transmit SRS in the non-SBFD symbols.

  - FFS: whether there is impact on frequency hopping counter

- The number of SRS resources in a SRS-ResourceSet for SBFD symbols is the same as the number of SRS resources in a  SRS-ResourceSet for non-SBFD symbols for the same usage.

The detailed SRS configuration is still under discussion in RAN1. We can conclude that separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage are supported.

Proposal 2: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage based on RAN1 further detailed progress. This includes distinct configurations for periodic, semi-persistent, and aperiodic SRS transmissions, with specific rules for SRS transmission occasions in SBFD and non-SBFD symbols.

Agreement

Define a new IE SRS-RSRP-MeasurementResourceSet containing a set of SRS-RSRP measurement resource(s) SRS-RSRP-MeasurementResource for L1 SRS-RSRP measurement

- Configuration of slot offset between the slot containing the DCI that triggers a set of aperiodic SRS-RSRP resources and the slot in which the SRS-RSRP resource set is measured (already agreed in RAN1#118bis)

- FFS: Other configurations

Define SRS-RSRP-MeasurementResource with the following parameters:

- Legacy SRS Resource IE

- FFS: Other parameters

Agreement

Define a new IE CLI-RSSI-MeasurementResourceSet containing a set of CLI-RSSI measurement resource CLI-RSSI-MeasurementResource for L1 CLI-RSSI measurement.

- Configuration of the slot offset between the slot containing the DCI that triggers a set of aperiodic CLI-RSSI resources and the slot in which the CLI-RSSI resource set is measured (already agreed in RAN1#118bis)

Define CLI-RSSI-MeasurementResource with the following parameters:

- CLI-RSSI measurement resource ID

- Starting PRB index

- Number of PRBs

- Starting symbol of the CLI-RSSI resource within a slot

- Number of symbols of the CLI-RSSI resource within a slot

- Periodicity and slot offset for the CLI-RSSI resource

- FFS: Other parameters

According to the agreement above, we can conclude that two new IE SRS-RSRP-MeasurementResourceSet for L1 SRS-RSRP measurement and CLI-RSSI-MeasurementResourceSet for L1 CLI-RSSI measurement are defined.

Proposal 3: For CLI handling, two new IEs are defined based on the RAN1 agreement:

- SRS-RSRP-MeasurementResourceSet: This IE contains a set of SRS-RSRP measurement resources for L1 SRS-RSRP measurement.

- CLI-RSSI-MeasurementResourceSet: This IE contains a set of CLI-RSSI measurement resources for L1 CLI-RSSI measurement.

Agreement

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of periodic CLI measurement resources, the TCI state(s) with QCL typeD for the CLI measurement resources in the periodic CLI measurement resource set are configured per CLI measurement resource (either for all resources in the set or none) by higher-layer parameter

- For each periodic CLI measurement resource, this higher-layer parameter provides a reference to one TCI-State in TCI-States for providing the QCL source and QCL typeD.

- If the TCI state is not configured, the UE QCL assumptions are the default rules agreed in RAN1#118bis

For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH based on a set of semi-persistent CLI measurement resources, the gNB may activate and deactivate the configured semi-persistent CLI measurement resource sets by sending a new SP CLI Measurement Resource Set Activation/Deactivation MAC CE

- TCI state IDi field is included in the new MAC CE, which is used as typeD QCL source for the CLI measurement resource within the Semi Persistent CLI measurement resource set. Either all TCI state ID fields are present or none.

- If the TCI state ID field(s) are absent, the UE QCL assumptions are the default rules agreed in RAN1#118bis

According to the agreement above, we can conclude that new MAC CE is required.

Proposal 4: A new MAC CE for SP CLI Measurement Resource Set Activation/Deactivation is required to activate and deactivate the configured semi-persistent CLI measurement resource sets from gNB. Detailed design of the MAC CE may follow RAN1 agreement.

3. Conclusion

Based on the analysis in section 2, the proposals are summarized as following:

Proposal 1: SBFD-aware UE can be configured with configurations on a per UL/DL BWP basis across SBFD symbols and non-SBFD symbols in different slots subject to the UE capability.

Proposal 2: Support separate SRS-ResourceSets configurations for SBFD and non-SBFD symbols for a given usage based on RAN1 further detailed progress. This includes distinct configurations for periodic, semi-persistent, and aperiodic SRS transmissions, with specific rules for SRS transmission occasions in SBFD and non-SBFD symbols.

Proposal 3: For CLI handling, two new IEs are defined based on the RAN1 agreement:

- SRS-RSRP-MeasurementResourceSet: This IE contains a set of SRS-RSRP measurement resources for L1 SRS-RSRP measurement.

- CLI-RSSI-MeasurementResourceSet: This IE contains a set of CLI-RSSI measurement resources for L1 CLI-RSSI measurement.

Proposal 4: A new MAC CE for SP CLI Measurement Resource Set Activation/Deactivation is required to activate and deactivate the configured semi-persistent CLI measurement resource sets from gNB. Detailed design of the MAC CE may follow RAN1 agreement.

4. References

- RP-241614 Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)  Huawei

- 3GPP TR 38.858 Study on Evolution of NR Duplex Operation (Release 18) V2.0.0 (2023-12)

- RAN1 Chair Notes WG1 #119