3GPP TSG-RAN WG2 Meeting #129	R2- 2500289

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.3.2.2

Source:	OPPO

Title:	Discussion on model generalization of RRM measurement prediction

Document for:	Discussion, Decision

Introduction

In RAN2#128, two generalization cases are agreed. We have agreed that

- Reuse the evaluation methodology in TR38.843 for generalization study, i.e., the generalization performance is evaluated with the following cases,

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B.

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B.

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B.

2	Companies can choose which case they compare with and should report it with simulation results.

Between GC#1 and GC#2, RAN2 focus on GC#2

Between the two cases, it was agreed that RAN2 would focus on GC#2. In the first part of this paper, we present the generalization results and associated observations for this case. In the second part, we prioritize certain simulations to reduce the workload.

Discussion

Generalization case #2

Generalization of FR1 temporal domain case B

In this section, we studied the generalization performance of FR1 temporal domain case B with sub case 3 with non-sliding filtering. The table below compares the performance of baseline and generalization models in terms of average L3 cell-level RSRP difference across different speeds and training epochs.

Table 1: performance of baseline and generalization models in FR1 temporal domain case B

|  |  | Baseline |  |  | Generalization |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE speed |  | 30km/h | 60km/h | 90km/h | 30km/h | 60km/h | 90km/h |
| Training data size |  | 50K |  |  | 150K |  |  |
| Testing data size |  | 5K |  |  | 5K |  |  |
| Average L3 cell-level RSRP difference (dB) | Epoch = 50 | 0.62 | 1.12 | 1.24 | 0.52 | 0.95 | 1.16 |
|  | Epoch = 100 | 0.59 | 1.00 | 1.10 | 0.68 | 1.03 | 1.16 |

For the baseline, it is observed that a model trained for 100 epochs generally exhibits better performance compared to one trained for only 50 epochs. This improvement is evident in lower average L3 cell-level RSRP differences at all tested speeds, suggesting that additional training can lead to more accurate predictions in the corresponding condition.

If the AI/ML model is trained extensively on a specific UE speed (e.g., 100 epochs), the generalization results indicate a degradation in performance ranging from 3% to 15%. However, if training is not enough (e.g., 50 epochs), the generalization model may outperform the baseline, highlighting the importance of balancing training depth and breadth.

Observation 1: If the AI/ML GC#2 model is trained extensively on a specific UE speed (e.g., 100 epochs), the generalization results indicate a degradation in performance ranging from 3% to 15% in FR1 temporal domain case B. However, if training is not enough (e.g., 50 epochs), the generalization model may outperform the baseline.

This highlights the risk of focusing too narrowly on one set of conditions, as the model may not adapt well to variations. Conversely, when the model is well-generalized, it can outperform the baseline model due to more training data, suggesting that a balance between specificity and adaptability is essential. This underscores the importance of not only training the model on a wide range of data but also ensuring that it does not become too specialized to the point of losing its ability to generalize.

Table 1 also reveals a clear trend: when the speed of the UE increases, there is a corresponding rise in the prediction error when using the generalized model. This observation aligns with our previous agreement in non-generalized scenarios, where we noted that predictions become more difficult at higher UE speeds.

Observation 2: For the model in FR1 temporal domain case B GC#2, there is a clear trend indicating that an increase in UE speed correlates with a rise in prediction error.

Generalization of FR2 temporal domain case A

Following our previous agreement, we studied the generalization of FR2 temporal domain case A with sub case 2 with non-sliding filtering. The performance is listed in Table 2.

Table 2: performance of baseline and generalization models in FR2 temporal domain case A

|  |  | Baseline |  |  | Generalization |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| UE speed |  | 60km/h | 90km/h | 120km/h | 60km/h | 90km/h | 120km/h |
| Training data size |  | 3.5K |  |  | 10.5K |  |  |
| Testing data size |  | 1.2K |  |  | 1.2K |  |  |
| Average L3 cell-level RSRP difference (dB) | Epoch = 50 | 1.61 | 1.63 | 1.75 | 1.32 | 1.38 | 1.41 |
|  | Epoch = 100 | 1.24 | 1.37 | 1.52 | 1.42 | 1.62 | 1.80 |

Similar to FR1 temporal domain case B, it is observed that for the baseline, a longer training period (100 epochs) yields improved performance over a shorter one (50 epochs). This improvement is characterized by lower average L3 cell-level RSRP differences at all tested speeds, indicating that extended training can refine prediction accuracy.

For the generalization model, increasing UE speed generally leads to higher prediction errors. This trend suggests that as speed increases, maintaining accurate signal strength predictions becomes more challenging. Notably, this effect is also seen in FR1 temporal domain case B, reinforcing the notion that higher speeds introduce additional complexity into prediction tasks.

Observation 3: For generalization model of FR2 temporal domain case A GC#2, the increase of UE speed leads to the increase of prediction error.

For generalization performance, an observation similar to FR1 temporal domain case B can be found:

Observation 4: If the AI/ML GC#2 model is trained extensively on a specific UE speed (e.g., 100 epochs), the generalization results indicate a degradation in performance ranging from 15% to 18% in FR2 temporal domain case A. However, if training is not enough (e.g., 50 epochs), the generalization model may outperform the baseline.

Another interesting observation to both FR1 temporal domain case B and FR2 temporal domain case A is that the impact of speed on model generalization appears relatively small. Despite the percentage increase in prediction errors with 100 epochs, the absolute differences of prediction accuracy between the baseline and the generalized model are relatively small. This could imply that the models have developed some level of resilience to speed variations.

Observation 5: For both FR1 temporal domain case B and FR2 temporal domain case A, the absolute difference of prediction accuracy between the baseline and the generalized model is relatively small, i.e., the impact of UE speed on model generalization is minimal.

Prioritization for generalization simulation

In GC#1, we consider different UE speeds. For models trained under more challenging conditions, such as high-speed scenarios, they are likely to perform adequately in less challenging, low-speed environments due to the inherent nature of how UE speed impacts performance as discussed above. Conversely, models trained in low-speed conditions are typically harder to generalize well to high-speed scenarios. Therefore, it is more prudent to evaluate models by training them in low-speed conditions and testing their performance in high-speed environments. For example, if a model is trained at 30 km/h, it would be essential to verify its effectiveness at higher speeds, such as 60 or 90 km/h. Should these points gain consensus, we could leverage this approach to optimize simulation efforts by prioritizing certain training and testing combinations over others, thereby marking some less critical evaluations as lower priority.

Proposal 1: For updated GC#1, prioritize the cases where models are trained at lower speed and subsequently tested at higher speeds, while relegate the converse cases (i.e., training at high speeds and testing at low speeds) to a lower priority.

The basic principle of proposal 1 is to test the model in more challenging scenarios instead of all scenarios to reduce the simulation workload. A similar prioritization method can be applied for generalization across cell configuration study. For generalization study on different cell configuration, it was recommended in POST128 email discussion that configuration #A uses 200m ISD and configuration #B uses 500m ISD. Given that 200m ISD typically presents a more challenging environment for AI/ML prediction compared to 500m ISD, it is proposed that:

Proposal 2: For both GC#1 and GC#2 with different cell configurations, testing the model in a more challenging environments can be prioritized to reduce the simulation workload, e.g., testing the generalized model under ISD=200m instead of under both ISD=200m and ISD=500m.

Conclusion

In this contribution, we have shared our simulation results on model generalization of RRM measurement prediction and have the following observations and proposals:

GC#2 of FR1 temporal domain case B:

Observation 1: If the AI/ML GC#2 model is trained extensively on a specific UE speed (e.g., 100 epochs), the generalization results indicate a degradation in performance ranging from 3% to 15% in FR1 temporal domain case B. However, if training is not enough (e.g., 50 epochs), the generalization model may outperform the baseline.

Observation 2: For the model in FR1 temporal domain case B GC#2, there is a clear trend indicating that an increase in UE speed correlates with a rise in prediction error.

GC#2 of FR2 temporal domain case A:

Observation 3: For generalization model of FR2 temporal domain case A GC#2, the increase of UE speed leads to the increase of prediction error.

Observation 4: If the AI/ML GC#2 model is trained extensively on a specific UE speed (e.g., 100 epochs), the generalization results indicate a degradation in performance ranging from 15% to 18% in FR2 temporal domain case A. However, if training is not enough (e.g., 50 epochs), the generalization model may outperform the baseline.

GC#2 of both FR1 temporal domain case A and FR2 temporal domain case B:

Observation 5: For both FR1 temporal domain case B and FR2 temporal domain case A, the absolute difference of prediction accuracy between the baseline and the generalized model is relatively small, i.e., the impact of UE speed on model generalization is minimal.

Prioritization for generalization simulation:

Proposal 1: For GC#1, prioritize the cases where models are trained at lower speed and subsequently tested at higher speeds, while relegate the converse cases (i.e., training at high speeds and testing at low speeds) to a lower priority.

Proposal 2: For both GC#1 and GC#2 with different cell configurations, testing the model in a more challenging environments can be prioritized to reduce the simulation workload, e.g., testing the generalized model under ISD=200m instead of under both ISD=200m and ISD=500m.