3GPP TSG RAN WG2#129	R2-2500312

Athens, GR, February 17th – 21st, 2025

Agenda Item:	8.17

Source:	OPPO

Title:	Discussion on IoT NTN TDD mode

Document for:	Discussion, Agreement

Introduction

In RAN#105 meeting, a new WID on IoT NTN TDD mode was approved, which was further revised to [1] in RAN#106 meeting. This work item aims to enable Iridium’s TDD band to provide NB-IoT service leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures. The objectives include the following.

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands

In this contribution, we will discuss TDD support for NB IoT NTN from RAN2’s perspective and present our view.

Discussion

According to the WID, the new NB-IoT NTN TDD mode is expected to allow the operator to use the radio resources in a periodic subset of the UL and DL subframes in N radio frames, targeting for coexistence with TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band. Obviously, this new TDD mode is not backward compatible, i.e. legacy NB-IoT UEs cannot work in this TDD mode. Therefore, NB-IoT UEs not supporting this new TDD mode should not be allowed to access a cell in the TDD mode. So the question is whether we need to introduce a new barred bit to control whether a R19 UE supporting TDD mode is allowed to access a cell in the TDD mode. According to the WID, the TDD mode is only intended to be applicable to the 1616-1626.5 MHz MSS band. This band is not expected for TN deployment, and only partially overlaps with UL band of NB-IoT NTN FDD band 254, which means legacy NB-IoT UEs in either TN or NTN anyway would not camp on a cell in 1616-1626.5 MHz MSS band. So we think legacy cellBarred-NTN can be reused to control whether a R19 UE supporting TDD mode is allowed to access a cell in the TDD mode.

- The new TDD mode is not backward compatible, i.e. legacy NB-IoT UEs cannot work in this TDD mode.

- Legacy NB-IoT UEs in either TN or NTN anyway would not camp on a cell in 1616-1626.5 MHz MSS band.

Reuse the legacy barred bit cellBarred-NTN in SIB1 to control whether a R19 UE supporting TDD mode is allowed to access a cell in the 1616-1626.5 MHz MSS allocated band.

- IoT NTN TDD mode has been discussed in the last RAN1 meetings, and the following agreements have been made in RAN1#119 meeting.

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints. Observation RAN1 concludes that at least D=8 and U=8 is feasible with N=9 from point of view of the design constraints (from earlier agreement) imposed by the TDD frame structure of the legacy system in the 1616-1626.5 MHz MSS band.

- Based on above RAN1 agreements, the TDD frame structure constraints that all the downlink and uplink transmission can only use one downlink slot and one uplink slot, respectively, with the same index across 90 ms periods. And it has been approved in RAN#106 meeting that D=U=8 and N=9.

- RAN1 also discussed the TDD mode impact on scheduling, with the following agreements made.

Agreement RAN1 to further discuss how to handle the overlap of NPUSCH with non-U NB-IoT subframes and the overlap of NPRACH (including NPRACH occasions) with non-U NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-U NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of U NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-U NB-IoT subframes. - Impact on segmented pre-compensation. Agreement RAN1 to further discuss how to handle the overlap of NPDCCH/NPDSCH (other than the one carrying SIB1-NB) (including e.g. starting point, windows for SI or RAR and other window sizes for DL channels/signals, PO, etc.) with non-D NB-IoT subframes, including studying at least the following: - New periodicities to align with the TDD structure. - Postponement of a channel when it overlaps with non-D NB-IoT subframes. - Restriction of a channel to be fully confined within a single set of D NB-IoT subframes. - Dropping (full or partial) of channels when they overlap with non-D NB-IoT subframes. - The non-D NB-IoT subframes are not “NB-IoT DL subframes” from the specifications

- Regarding handling of overlap of NPUSCH/NPRACH with non-U NB-IoT subframes or overlap of NPDCCH/NPDSCH with non-D NB-IoT subframes, since the new TDD mode is not back compatible, for the periodic channels/signals or transmission, e.g. i-DRX cycle, periodicity for SI window, RACH periodicity and UL SPS periodicity, it would be most simple and straightforward to introduce new periodicities to align with the TDD structure, i.e. with a periodicity value of multiple of 90ms.

Introduce new periodicities for the following to align with the TDD structure, i.e. with a periodicity value of multiple of 90ms

- i-DRX cycle

- SI window periodicity

- RACH periodicity

- UL SPS periodicity

- Regarding i-DRX, for NB-IoT, UE derives T as follows:

- For NB-IoT: If UE specific DRX value is allocated by upper layers and minimum UE specific DRX value is broadcast in system information, T = min (default DRX value, max (UE specific DRX value, minimum UE specific DRX value broadcast in system information)). If UE specific DRX is not configured by upper layers or if the minimum UE specific DRX value is not broadcast in system information, the default DRX value is applied.

- As highlighted above, if UE is configured by CN with UE specific DRX cycle and minimum UE specific DRX value is broadcasted, T is derived based on default DRX value, UE specific DRX value, minimum UE specific DRX value. Based on proposal 1, for the TDD mode, we need to introduce new values for default DRX value and minimum UE specific DRX value. For UE-specific DRX cycle, it is configured by CN, so it should be up to SA2 to decide how to deal with it. We need to send a LS to check with SA2.

Introduce new values for defaultPagingCycle and ue-SpecificDRX-CycleMin.

Send a LS to check with SA2 how to handle UE-specific DRX cycle for IoT NTN TDD mode.

- In TS36.321, a note on scheduling restriction is given as follow.

NOTE 4: For NB-IoT, for operation in FDD mode, and for operation in TDD mode with a single HARQ process when the associated HARQ RTT timer has been started, DL and UL transmissions will not be scheduled in parallel, i.e. if a DL transmission has been scheduled an UL transmission will not be scheduled until HARQ RTT Timer of the DL HARQ process has expired (and vice versa).

- According to the above note, for a NB-IoT UE configured with a single HARQ process, the UE is not required to monitor PDCCH since a DL or UL transmission has been scheduled. This has also been reflected in the start condition of onDurationTimer as highlighted below.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

- In NTN TDD mode, for UL or DL transmission with repetition, if transmission duration is longer than 8ms, the transmission may be segmented. For UL segmented transmission, there will be DL NB-IoT subframes located between successive UL segmented transmissions. Following the principle in the above note, UE should not monitor PDCCH during these DL NB-IoT subframes, and this should be considered as an additional start condition restriction for onDurationTimer

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Conclusion

Based on the discussion we propose the following:

- Reuse the legacy barred bit cellBarred-NTN in SIB1 to control whether a R19 UE supporting TDD mode is allowed to access a cell in the 1616-1626.5 MHz MSS allocated band.

Introduce new periodicities for the following to align with the TDD structure, i.e. with a periodicity value of multiple of 90ms

- i-DRX cycle

- SI window periodicity

- RACH periodicity

- UL SPS periodicity

Introduce new values for defaultPagingCycle and ue-SpecificDRX-CycleMin.

Send a LS to check with SA2 how to handle UE-specific DRX cycle for IoT NTN TDD mode.

In NTN TDD mode, UE doesn’t start onDurationTimer within the UL segmented transmission gap.

- Reference

[1] RP-243293, Revised WID on introduction of IoT-NTN TDD mode