3GPP TSG-RAN WG2 Meeting #129	R2-2500314

Athens, Greece, Feb. 17th – 21st, 2025

Source:	NTT DOCOMO, INC.

Title:	Discussions on generalization of RRM measurement prediction

Agenda Item:	8.3.2.2

Document for: 	Discussion and Decisions

- Introduction

During the RAN#102 meeting, the SID on “Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR”[1] was approved. The SID describes the study scope of the AI/ML-aided mobility enhancements, as below,

Study and evaluate potential benefits and gains of AI/ML aided mobility for network triggered L3-based handover, considering the following aspects:

- AI/ML based RRM measurement and event prediction,

  - Cell-level measurement prediction including intra and inter-frequency (UE sided and NW sided model) [RAN2]

    - Inter-cell Beam-level measurement prediction for L3 Mobility (UE sided and NW sided model) [RAN2]

  - HO failure/RLF prediction (UE sided model) [RAN2]

  - Measurement events prediction (UE sided model) [RAN2]

- Study the need/benefits of any other UE assistance information for the network side model [RAN2]

- The evaluation of the AI/ML aided mobility benefits should consider HO performance KPIs (e.g., Ping-pong HO, HOF/RLF, Time of stay, Handover interruption, prediction accuracy, and measurement reduction) etc.) and complexity tradeoffs [RAN2]

  - NOTE: Simulation assumption and methodology can leverage TR 38.901, 38.843 and 36.839. And leave the detail discussion to RAN2

- Potential AI mobility specific enhancement should be based on the Rel19 AI/ML-air interface WID general framework (e.g. LCM, performance monitoring etc) [RAN2]

  - NOTE: This would only be treated after sufficient progress is made in the Rel-19 AI/ML air interface WID

- Potential specification impacts of AI/ML aided mobility [RAN2]

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4]

This paper discusses the evaluation results of the generalization performance of AI/ML models for RRM measurement predictions.

- Simulation setup and assumptions

Following the agreements in RAN2 #128[2], we evaluated the generalization performance of AI/ML models for RRM measurement predictions with respect to the UE speed (w/ temporal domain prediction Case A and Case B) and frequency (w/ inter-frequency). Following the agreements at RAN2 #126 and RAN2 #127, the common simulation assumptions are listed below in Appendix.

As suggested in RAN2 #128 [2], the following cases for generalization over UE speed are evaluated,  which are shown in Table 1 and Table 2, and the cases for generalization over frequency are shown in Table 3.

Table 1 Evaluated Cases for Generalization over UE Speed for Case B

| Training data | Inference: 30 kmph | Inference: 60 kmph | Inference: 90 kmph |
| --- | --- | --- | --- |
| 30 kmph | GC baseline | GC #1 | GC #1 |
| 60 kmph | GC #1 | GC baseline | GC #1 |
| 90 kmph | GC #1 | GC #1 | GC baseline |
| Mixed data | GC #2 | GC #2 | GC #2 |

Table 2 Evaluated Cases for Generalization over UE Speed for Case A

| Training data | Inference: 60 kmph | Inference: 90 kmph | Inference: 120 kmph |
| --- | --- | --- | --- |
| 60 kmph | GC baseline | GC #1 | GC #1 |
| 90 kmph | GC #1 | GC baseline | GC #1 |
| 120 kmph | GC #1 | GC #1 | GC baseline |
| Mixed data | GC #2 | GC #2 | GC #2 |

Table 3 Evaluated Cases for Generalization over Frequency

| Training data | Inference: 2GHz -> 4 GHz | Inference: 4GHz -> 2GHz |
| --- | --- | --- |
| 2GHz -> 4GHz | GC baseline | GC #1 |
| 4GHz -> 2GHz | GC #1 | GC baseline |
| Mixed data | GC #2 | GC #2 |

- Evaluation results

3.1 Generalization performance over UE speed for temporal domain Case B

The evaluation results of generalization performance over UE speed are plotted on Figure 1, and the detailed values are submitted in the sheet RRM_Scen2_Gen_129_DCM_2500314_V1.xlsx. In the generalization evaluations, we consider the sliding window option for the Layer 1 filtering, and the MRRT is 0.5.

Figure 1 Generalization performance over UE speed for temporal domain Case B

From the results, we have the following observations.

Observation 1

On the generalization performance of FR1-to-FR1 temporal domain prediction Case B,

- For GC #1, the performance degradation from generalization can be observed, but the average RSRP gap increases by less than 0.03 dB, which does not significantly impact the performance.

- For GC #2, when the AI/ML model is trained with multiple UE speeds, the performance degradation is reduced to <0.01 dB. The generalization over UE speed can be achieved.

3.2 Generalization performance over UE speed for temporal domain Case A

The evaluation results of generalization performance are plotted on Figure 2, and the detailed values are submitted in the sheet RRM_Scen4_Gen_129_DCM_2500314_V1.xlsx. For this study, we consider the non-sliding option for Layer 1 filtering, and the prediction window is 400ms.

Figure 2 Generalization performance over UE speed

Observation 2

On the generalization performance of FR2-to-FR2 temporal domain prediction Case A,

- For GC#1, significant generalization loss is observed from the simulation results.

- For GC #2, when the AI/ML model is trained with multiple UE speeds, the generalized model achieves better performance, comparable to GC#1.

3.3 Generalization performance over frequency

Figure 3 plots the evaluation results of generalization performance over frequency, and the detailed values are submitted in the sheet RRM_Scen3_Gen_129_DCM_2500314_V1.xlsx.

Figure 3 Generalization performance over frequency

From the results, we have the following observations.

Observation 3

On the generalization performance of FR1-to-FR1 inter-frequency prediction,

- For all generalization cases, the AI/ML inter-frequency prediction outperforms the legacy RSRP offset scheme.

- For GC#1, significant generalization loss is observed from the simulation results.

- For GC #2, the generalized model performs better than GC#1 when the AI/ML model is trained with mixed cases.

3.4 Summary

Summarizing the generalization evaluation results obtained so far, we have the following proposals for further study.

Proposal 1

- Further study the GC#2 to improve the generalization of prediction models for the following cases,

  - FR1-to-FR1 inter-frequency prediction over frequency.

  - FR2-to-FR2 temporal domain prediction Case A over UE speed.

- Conclusion

In this contribution, we have the following observations and proposals,

Observation 1

On the generalization performance of FR1-to-FR1 temporal domain prediction Case B,

- For GC #1, the performance degradation from generalization can be observed, but the average RSRP gap increases by less than 0.03 dB, which does not significantly impact the performance.

- For GC #2, when the AI/ML model is trained with multiple UE speeds, the performance degradation is reduced to <0.01 dB. The generalization over UE speed can be achieved.

Observation 2

On the generalization performance of FR2-to-FR2 temporal domain prediction Case A,

- For GC#1, significant generalization loss is observed from the simulation results.

- For GC #2, when the AI/ML model is trained with multiple UE speeds, the generalized model achieves better performance, comparable to GC#1.

Observation 3

On the generalization performance of FR1-to-FR1 inter-frequency prediction,

- For all generalization cases, the AI/ML inter-frequency prediction outperforms the legacy RSRP offset scheme.

- For GC#1, significant generalization loss is observed from the simulation results.

- For GC #2, the generalized model performs better than GC#1 when the AI/ML model is trained with mixed cases.

Proposal 1

- Further study the GC#2 to improve the generalization of prediction models for the following cases,

  - FR1-to-FR1 inter-frequency prediction over frequency.

  - FR2-to-FR2 temporal domain prediction Case A over UE speed.

Appendix: Simulation assumptions

Table  A1 Common simulation assumptions

| Parameter | FR1 Band #1 (common) | FR1 Band #2 (inter-frequency) | FR2 |
| --- | --- | --- | --- |
| Carrier frequency | 4 GHz (TDD) | 2 GHz (FDD) | 30 GHz |
| SCS | 30 kHz | 15 kHz | 120 kHz |
| Deployment | 2-tier with wrap around (7 sites, 3 sectors per site) |  | 2-tier with wrap around (7 sites, 3 sectors per site) |
| ISD | 500 m |  | 200 m |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Inter-frequency correlation model: Section 7.6.5 in TR38.901. |  | UMi w/ distance-dependent LoS probability function defined in Table 7.4.2-1 of TR38.901. |
| System bandwidth | 20 MHz |  | 80 MHz |
| Sample period | 40 ms |  | 80 ms |
| UE speed | 90 kmph |  | 60, 90, 120 kmph |
| UE distribution/dropping | 100% outdoor |  | 100% outdoor |
| BS antenna configuration | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ Number of BS beams: 4 |  | (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Number of BS beams: 32 |
| BS antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |  | TR 38.802 Table A.2.1-6 |
| UE antenna configuration | 2RX: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) |  | (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 |
| UE antenna radiation pattern | Omni-direction |  | TR 38.802 Table A.2.1-8 |
| BS Tx power | 44 dBm for 20 MHz bandwidth |  | 40 dBm |
| Maximum UE Tx power | 23 dBm |  | 23 dBm |
| BS receiver noise figure | 5 dB |  | 7 dB |
| UE receiver noise figure | 9 dB |  | 10 dB |
| BS antenna height | 25 m |  | 10 m |
| UE antenna height | 1.5 m |  | 1.5 m |
| Spatial consistency | Procedure A in TR38.901 |  | Procedure A in TR38.901 |
| UE trajectory model | Option 3 |  | Option 3 |
| UE boundary model | Option 3 |  | Option 3 |

The RRC parameters are configured following the post-RAN2 #126 email agreements in the following tables.

Table  A2 RRC parameters for the Layer 3 filtering

| L3 filtering parameter | value |
| --- | --- |
| FR1 Filter Coefficient | 4 |
| FR2 Filter Coefficient | 4 |

Table  A3 RRC parameters for the measurement period

| Measurement period | value |
| --- | --- |
| FR1 to FR1 intra-frequency w.o. gap | 200 ms |
| FR1 to FR1 inter-frequency with gap | 200 ms |
| FR2 to FR2 intra-frequency w.o. gap | 400 ms |

Table  A4 RRC parameters for beam consolidation/selection

| Consolidation parameter | value |
| --- | --- |
| nrofSS-BlocksToAverage for FR1 | 1 |
| nrofSS-BlocksToAverage for FR2 | 3 |
| absThreshSS-BlocksConsolidation for FR1 | -110 dBm |
| absThreshSS-BlocksConsolidation for FR2 | -110 dBm |

References

- Moderator (CMCC), RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023.

- RAN2 Chair (InterDigital), “RAN2 #128 Chair Notes”, Nov., 2024.

- 3GPP TR 38.843 v2.0.0, Study on Artificial Intelligence (AI)/Machine Learning(ML) for NR air interface (Release 18), Dec., 2023.