3GPP TSG-RAN WG2 Meeting #129	R2-2500331

Athens, Greece, 17th – 21st of February 2025

Agenda item:	8.8.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Further Details on MBS in Rel-19 NR NTN

WID/SID:	NR_NTN_Ph3 - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In December 2023, RAN approved a Rel-19 work item (WI) aimed at defining Phase 3 enhancements to Non-Terrestrial Networks (NTN). There was a minor revision of the WID at RAN#103. Among the objectives captured in the WID [1] the following can be found:

- Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3]

In RAN2#127bis meeting, the following decision has been made [2]:

Agreements: 1. For each MBS service we include one or more intended service area IDs into MCCH. FFS whether the list of the intended service areas (and related IDs) is also included in MCCH or if it is provided in a new or existing SIB. We will consider possible enhancements (including enhancements left up to UE implementation) to allow UE skipping MCCH re-acquisition when UE is not within intended service area of any interested broadcast service.

Few other topics have been marked as “Come back in the next meeting”. At RAN2#128 meeting (November 2024) the following agreements have been made:

Agreements: 1. The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. 2. The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. 3. The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) 4. A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). 5. RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. 6. Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) 7. The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In the meantime, SA1 has responded to our LS on ETWS primary notification. In their response LS [4] the following is stated:

SA1 thanks RAN2 for the LS on requirements for ETWS primary notification (R2-2407823). SA1 would like to inform RAN2 of the resulting alignment CR which is attached.

In one of the following subsections we provide the details of SA1 CR and discuss the implications to RAN2. Besides, this paper presents our further view on what remains to be addressed for MBS Rel-19 objective in RAN2.

# 2	Discussion

## 2.1	Service area signalling – MCCH and new SIB

So far, the design of service area signalling for MBS in NTN has been made in baby steps.

At RAN2#126 (May 2024) the following has been agreed [6]:

For MBS broadcast RAN2 considers the following possibilities for including the service area information: SIB20/ SIB21/ MBSBroadcastConfiguration. FFS for ETWS

At RAN2#127 (August 2024) the topic was discussed via offline thread [8] and the following was agreed [7]:

2. RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

Eventually a solid, even if not in the right direction, step was taken at RAN2#128, and the following decision can be found in the meeting notes [3]:

- RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. - Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

Observation 1: RAN2 has decided to introduce a new SIB which will include the list of service areas. The list of intended service area identifiers is also to be carried in MCCH for each MBS service (as per earlier RAN2 agreements).

So the service definition is given in MCCH and associated ID/index points to a new SIB where service area details are provided. There is an FFS in the agreement made at RAN2#128 regarding the way to identify the service areas in SIB: either via an ID or index or in “another way” (e.g. via a specific position in a bitmap). Our understanding on the reason behind this FFS is the complex relationship between the services and corresponding service area(s). As per legacy MBS, one service can be mapped to a single service area. However, multiple services can be present within a single service area (i.e. many-to-one relationship). Nevertheless, RAN2 should consider if many-to-many relationship should be supported for NTN MBS.

Observation 2: As per legacy specification One service can be available in a single service area. One service area can comprise multiple services.

Proposal 1: RAN2 to consider if many-to-many relationship should be supported for NTN MBS service and service area.

When inspecting the NR RRC [5], it can be found that MCCH comprises MBSMulticastConfiguration which can have a list of MBS sessions (i.e. MBS-SessionInfoListMulticast IE). Thus, under each of such sessions (i.e. MBS-SessionInfo) there needs to be an identifier pointing to a specific service area or areas defined in the new SIB.

Proposal 2: Insert a list of service area IDs/indices under each MBS-SessionInfo provided in MCCH.

## 2.2	ETWS Primary Notification

SA1 has attempted to address the issue described in our LS [7] and agreed a corresponding CR [5]. In [7] RAN2 has asked the following:

RAN2 kindly request SA1 to take the above information into account and provide feedback whether the capability to geofence emergency messages to an area should be supported for ETWS in a NTN cell, considering the delay requirements for ETWS primary notification.

whereas SA1 CR provides the following new requirement [5]:

## 5.6 Additional PWS Requirements Specific to ETWS Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN. Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

According to the additional PWS requirement applicable to ETWS, defined recently by SA1, the UE capable of ETWS shall be able to present the ETWS notification if the UE is able to determine it is within the Warning Notification Area. If the UE is unable to determine its location (i.e. within or outside the Warning Notification Area) it shall also present the ETWS notification.

Observation 3: According to SA1, ETWS-capable UE shall present the ETWS notification if the UE is able to determine it is within the Warning Notification Area or when the UE is unable to determine its location.

The latter in Observation 3 may result in lots of false alerts if the UE, for any reason, does not have its location information available at the time Warning Notification Area is broadcast along with ETWS. E.g. imagine a large NTN cell covering hundreds of kilometres in southern USA and Gulf of Mexico (if that is still the valid name). The UE is located in Orlando (FL), while the ETWS concerns Dallas (TX). The UE fails to assess its geo-position, so the ETWS notification is displayed to the user in Orlando even though it should not be alerted.

Observation 4: The SA1 requirement to display ETWS warning when UE has not assessed its location may lead to false alerts.

On the other hand, one can ask whether the UE in NTN cell may not have a valid location information as without such awareness the UE cannot perform any UL transmissions (i.e. the UE needs to pre-compensate time and frequency shift using its location and satellite ephemeris).

Observation 5: NTN-capable UE performing UL transmission needs to anyway have a valid location information.

As it is expected the ETWS area could be smaller than NTN cell footprint (i.e. see the description above, preceding Observation 4), it may be worth finding means to avoid alerting UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information).

Observation 6: ETWS area is likely to cover a smaller area than NTN cell’s footprint.

Observation 7: It is beneficial if UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information) are not alerted.

To achieve such goal, the UE should be told where the intended ETWS area actually is. In the simplest approach the reference location + radius, known from Rel-18 terrestrial coverage (TN) area signalling, can be reused. Such information can be delivered to the UE in SIB.

Proposal 3: RAN2 considers a solution wherein the UE is given the geographical coordinates associated with the ETWS notification area.

Furthermore, RAN2 in the original LS [7] has asked about “delay requirements for ETWS primary notification” which SA1 has not taken into account (or at least nothing is explicitly stated in the agreed CR’s text).

Observation 8: RAN2 has asked SA1 to consider ETWS Primary Notification delay requirements in their work on the geofencing for ETWS. This apparently has not happened in SA1 as per agreed CR in S1-244753.

## 2.3	MBS Interest Indication (MII)

At RAN2#127bis the following has been discussed and not resolved :

Proposal 4: For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the MBS Interest Indication procedure when UE is entering or leaving the intended area. - QC thinks this is up to UE and there is nothing to reflect in the spec. Xiaomi agrees - HW thinks that when entering the area this makes sense, but wonder about the case when the UE leaves the area. - Apple thinks the UE could initiate the MII procedure in any case and then it’s up to NW implementation how to behave. LG agrees with Apple. Oppo also agrees. CMCC thinks in this case the NW has no idea of the UE location in this case. Come back in the next meeting

In section 5.9.4.2 of [6] the following text can be found:

An MBS capable UE in RRC_CONNECTED may initiate the procedure in several cases including upon successful connection establishment/resume, upon entering or leaving the broadcast service area, upon MBS broadcast session start or stop, upon change of interest, upon change of priority between MBS broadcast reception and unicast/multicast reception, upon change to a PCell providing SIB21 (i.e. where the SIB1 scheduling information contains SIB21), upon receiving SIB20 of an SCell via dedicated signalling, upon handover, and upon RRC connection re-establishment, upon change to a PCell providing nonServingCellMII in SIB1, upon starting or stopping reception of MBS broadcast service on a non-serving cell, upon change of CFR information or subcarrier spacing for MBS broadcast reception on a non-serving cell.

As can be seen, the MII might be sent when UE enters or leaves the broadcast service area and this is already supported by NR RRC [6].

Observation 9: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Thus, it that nothing new has to be agreed for MBS II in NTN. Furthermore, the behaviour described in [6] is not binding (the term used in the procedure is “UE may initiate”), so in our view this issue is not worth spending additional time. Note that the MII feature can be used by the UE in case UE cannot find the MBS broadcast service that it is expecting in a cell. That would make, e.g., a gNB decides whether to handover the UE to a possible neighbor gNB/cell providing MBS broadcast service UE has interest in. Thus, as has been indicated at RAN2#127bis, it is a UE implementation issue and a UE can decide when to send the MII (which could be somehow related to the intended service area).

Proposal 4: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

## 2.4 	Configuration aspects of MBSBroadcastConfiguration and service continuity in SIB21

At RAN2#127bis the following has been discussed and not decided :

In MBSBroadcastConfiguration, the intended service area within each neighbor cell is provided for each MBS broadcast service session. In SIB21, the intended service area is provided for each MBS broadcast service session, or for each FSAI (depending on common understanding selected). - Ericsson thinks we should focus on the scenario where the quasi-Earth-fixed cell is replaced due to satellite movement - ZTE thinks we currently support both and then supports p4 - QC also thinks both scenarios should be supported. - HW thinks this covers the case of UE movement. For the satellite movement there would be no difference wrt previous releases. HW is ok to reflect the second part of p4 - Ericsson disagrees with the proposal as there is no need for service continuity for this. Apple thinks this is needed for cell reselection. - CATT wonders whether we really need to provide the service area for service continuity, in a per neighbour cell way. - Ericsson thinks we need to clarify the UE behavior when receiving this information Come back in the next meeting

As can be seen in [6], MBS-SessionInfo (a part of MBSBroadcastConfiguration) comprises mtch-neighbourCell. This IE indicates neighbour cells which provide this service on MTCH. Thus, achieving the first goal stated above seems to be possible using the existing means (i.e. it is feasible to tell the UE in which neighbouring cells the same service is provided). There is no exact indication about the intended service area, but that is perhaps not critically needed (the UE will know the service area details from its current serving cell).

Proposal 5: RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be known thanks to mtch-neighbourCell.

Regarding the second aspect highlighted above, whether intended service area is provided in SIB21 for each session or per FSAI, this would somewhat change/extend the purpose of SIB21. SIB21 is defined as follows [6]:

SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). SIB21 information element -- ASN1START -- TAG-SIB21-START SIB21-r17 ::= SEQUENCE { mbs-FSAI-IntraFreq-r17 MBS-FSAI-List-r17 OPTIONAL, -- Need R mbs-FSAI-InterFreqList-r17 MBS-FSAI-InterFreqList-r17 OPTIONAL, -- Need R lateNonCriticalExtension OCTET STRING OPTIONAL, ... } MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17 MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17 MBS-FSAI-InterFreq-r17 ::= SEQUENCE { dl-CarrierFreq-r17 ARFCN-ValueNR, mbs-FSAI-List-r17 MBS-FSAI-List-r17 } MBS-FSAI-r17 ::= OCTET STRING (SIZE (3)) -- TAG-SIB21-STOP -- ASN1STOP

Thus, introducing the service area information to SIB21 does not match its main purpose (i.e. frequency mapping to FSAI). FSAIs are configured per gNB or per cell. If these are to be configured per intended service area, this requires lots of changes to core and service announcements. Overall, the benefits are limited, if any, while it might have a large specification impact, not only in RAN2.

Proposal 6: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

## 2.5 	Entering and leaving the service area versus MRBs

At RAN2#127 it was agreed that [9]:

For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature should not establish MRB(s) for the MBS session associated to the intended area when it is outside the intended area (capture this in Stage 2) For an MBS broadcast service intended for a certain area, a R19 UE supporting the feature may initiate the broadcast MRB establishment procedure when UE is inside the intended area; the UE may initiate the broadcast MRB release procedure when UE leaves the intended area (capture this in stage 3)

As can be noticed, no firm requirements have been defined for the UE regarding MRBs handling when the UE is outside of intended service area.

Observation 10: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

In our view at least the action when the UE can establish the MRBs should be clear, i.e. the UE should be allowed to establish the MRBs based on its location versus MBS service area, not just whenever it can receive the MBS. Thus, we suggest the following:

Proposal 7: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

At RAN2#127 it was also decided the UE may release the MRBs when it leaves the intended area. That is acceptable, but we shall not let the UE keep the MRBs for excessively long time. Thus, we suggest the following:

Proposal 8: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

It was debatable how the UE determines it has left the service area. We acknowledge that it requires new means when compared to MBS in terrestrial networks. Nevertheless, it is expected the NTN-capable UE will monitor its geo-position (as we have also argued in section 2.2), so it should be able to realize it has left or entered the MBS service area.

Proposal 9: NTN-capable UE can determine whether it is inside or outside of MBS service area based on the GNSS location it is using for other NTN purposes, i.e. there is no need to update UE’s GNSS location specifically for this purpose.

# 3	Conclusion

This document has made the following observations:

Observation 1: RAN2 has decided to introduce a new SIB which will include the list of service areas. The list of intended service area identifiers is also to be carried in MCCH for each MBS service (as per earlier RAN2 agreements).

Observation 2: As per legacy specification One service can be available in a single service area. One service area can comprise multiple services.

Observation 3: According to SA1, ETWS-capable UE shall present the ETWS notification if the UE is able to determine it is within the Warning Notification Area or when the UE is unable to determine its location.

Observation 4: The SA1 requirement to display ETWS warning when UE has not assessed its location may lead to false alerts.

Observation 5: NTN-capable UE performing UL transmission needs to anyway have a valid location information.

Observation 6: ETWS area is likely to cover a smaller area than NTN cell’s footprint.

Observation 7: It is beneficial if UEs outside of the ETWS area but located within the same NTN cell (i.e. receiving the same System Information) are not alerted.

Observation 8: RAN2 has asked SA1 to consider ETWS Primary Notification delay requirements in their work on the geofencing for ETWS. This apparently has not happened in SA1 as per agreed CR in S1-244753.

Observation 9: As per legacy NR RRC the MBS capable UE may send the MBS Interest Indication upon entering or leaving the broadcast service area.

Observation 10: UE’s behaviour concerning MRBs handling when the UE is outside of intended service area is unspecified (i.e. the UE may or may not establish or release the MRBs).

And proposed the following:

Proposal 1: RAN2 to consider if many-to-many relationship should be supported for NTN MBS service and service area.

Proposal 2: Insert a list of service area IDs/indices under each MBS-SessionInfo provided in MCCH.

Proposal 3: RAN2 considers a solution wherein the UE is given the geographical coordinates associated with the ETWS notification area.

Proposal 4: RAN2 does not specify any new behaviour with respect to MBS Interest Indication when Rel-19 NTN capable UE enters or leaves the MBS broadcast service area.

Proposal 5: RAN2 does not work on providing the intended service area within each neighbor cell for each MBS broadcast service session, as this can be known thanks to mtch-neighbourCell.

Proposal 6: As FSAIs are configured per gNB or per cell, RAN2 does not pursue FSAI to intended service area mapping, which might have impact on core and service announcements (not limited to RAN2) for no major benefits.

Proposal 7: Make the RAN2#127 agreement UE-binding and capture that the NTN-capable UE is not allowed to/shall not establish MRBs if it can receive broadcast service, but its location is not within the MBS service area.

Proposal 8: UE may release MRBs when leaving the intended area. However, releasing MRBs is eventually subject to additional conditions, such as the time or distance that has elapsed since leaving the MBS area, as the UE cannot keep the MRBs eternally.

Proposal 9: NTN-capable UE can determine whether it is inside or outside of MBS service area based on the GNSS location it is using for other NTN purposes, i.e. there is no need to update UE’s GNSS location specifically for this purpose.

# References

- RP-240775 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3 3GPP TSG RAN Meeting #103 Maastricht, The Netherlands, March 18-22, 2024

- R2-2409213 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127bis Changsha, China, Apr 15th – 19th, 2024

- R2-2410913	Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #128 Orlando, USA, Nov 18th – 22nd, 2024

- S1-244857 Reply LS on requirements for ETWS primary notification 3GPP TSG SA WG WG1 Meeting #108 Orlando, FL, USA, 18th – 22nd November 2024

- S1-244753 Warning notification with geofencing for ETWS over satellite access (CR to TS 22.268) 3GPP TSG-SA1 Meeting #108	 Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

- 3GPP TS 38.331 NR RRC, v18.4.0, December 2024

- R2-2407823 LS on requirements for ETWS primary notification 3GPP TSG-RAN WG2 Meeting #127 Maastricht, Netherlands, August 19th – 23rd, 2024

- R2-2405703 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #126 Fukuoka, Japan, May 20th – 24th, 2024

- R2-2407573 Report from Break-out session on NR-NTN and IoT-NTN 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2407619 [Offline 307] [R19 NR NTN] service area information (Apple) 3GPP TSG-RAN WG2 Meeting #127 Maastricht, The Netherlands, Aug 19th – 23rd, 2024

- R2-2402355, Discussion on support of broadcast service via NR NTN, CATT, China Broadnet, 3GPP TSG RAN2#125bis meeting, Rel-19, April 2024.

- R3-245844 LS on Supporting MBS broadcast service for NR NTN 3GPP TSG RAN WG3 125-bis Hefei, China, 14 - 18 October, 2024