3GPP TSG-RAN WG2 Meeting #129		R2-2500348

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:		Discussion on conditional LTM

Agenda Item:		8.6.4

Document for:	Discussion and Decision

- Introduction

In RAN2 #127bis and #128 meeting, conditional intra-CU LTM was discussed and some agreements have been reached. This contribution would further discuss some left issues, including:

- Early TA related issues, e.g. MAC CE content, TA timer start/expiry conditions and corresponding UE behaviours;

- CLTM execution phase issues, e.g. beam selection, CG/DG-based RACH-LESS procedure, when to use RACH-based procedure and CLTM failure recovery.

- Discussion

  - Content of the MAC CE for CLTM early TA acquisition

In RAN2 #128 meeting[1], it was agreed that:

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

Then we will discuss the contents and format of the new MAC CE in this section. Firstly, the TA value corresponding to the early TA acquisition should be included in the MAC CE. We think the field TAC in Rel-18 intra-CU LTM Cell Switch MAC CE could be reused directly, i.e., the field with 12 bits length indicates the index value TA used to control the amount of timing adjustment that the MAC entity has to apply as specified in TS 38.213. Besides, the following issues should be considered for the TA value within the new MAC CE:

- Issue#1: Whether the TA value could be FFF similar to the TAC field in Rel-18 intra-CU LTM, which means that no valid timing adjustment is available.

- Issue#2: Whether two TAGs could be configured for the CLTM candidate cell.

- Issue#3: Whether there is a need to indicate the CLTM candidate cell ID associated with the TA in the new MAC CE.

Regarding the issue#1, since the new MAC CE is only sent to UE after UE performed early RACH, the valid TA could be acquired by the source gNB in this case, thus there is no need to consider the case that the TAC field of the new MAC CE is FFF.

Regarding the issue#2, since Rel-18 intra-CU LTM could be configured together with the MIMO 2 TA, the same principle should be reused for CLTM. Thus, the scenario in which the CLTM candidate cell is configured with two TAGs may occur, in this case, the new MAC CE should indicate the related TAG information in TAC field. In Rel-18 LTM, it is indicated by the TCI state ID within the LTM Cell Switch MAC CE and the tag-Id-ptr configured by RRC signalling for the TCI stated ID, however, there is no need to include the TCI state ID in the new MAC CE, and it is straightforward using 1 bit to indicate the TAG information in this case.

Regarding the issue#3, before we discuss this issue, we should first clarify whether UE could perform early TA acquisition procedure to more than 1 cell concurrently, e.g. UE receives the PDCCH order to candidate Cell#2 before UE receives the new MAC CE for candidate Cell#1 shown as the following figure. In our view, same as the Rel-18 early TA acquisition, since UE only maintain 1 power ramping counter for both serving cells and candidate cells, it is impossible to perform more than 1 early TA acquisition procedure concurrently. Thus, the new MAC CE corresponds to the latest PDCCH order and there is no ambiguity, and considering the PDCCH order will include the candidate cell information, there is no need to indicate the CLTM candidate cell ID in the new MAC CE.

Proposal 1: TAC field is included in the MAC CE for CLTM early TA acquisition, and the value couldn’t be FFF.

Proposal 2: TAG information field is included in the MAC CE for CLTM early TA acquisition.

Proposal 3: CLTM candidate cell ID is not included in the MAC CE for CLTM early TA acquisition, and the TA within the MAC CE is for the CLTM candidate cell related to the latest PDCCH order.

Figure 1. Early TA acquisition procedure for CLTM

Based on the above proposals, an example of the MAC CE format is given in the MAC running CR, in section 6.1.3.4x[2].

Furthermore, in Rel-18 LTM, the candidate cell may reserve the dedicated CG resource for a UE for the first UL data transmission during the RACH-less LTM. Since the LTM is subsequent, the CG resource will be reserved for a long time. To reduce the resource wastage, we made the following agreements in Rel-18:

- Observation: In cases for which it is desired that CG used for LTM is not used further once the UE has made the cell its new serving cell, it is assumed that the network could release Type1 CG resource on LTM completion (existing functionality) - P4: RAN2 to define the UE behaviour on the R18 CG for RACH-less LTM, if it is not released by NW after LTM completion: Option 1: UE stops using those CG (FFS on the spec impact/wording details);

However, only when the candidate cell becomes current serving cell, its CG resources could be released or re-assigned to be used, and the CG resources in other candidate cells will always be reserved.

Observation 1: The CG handling method could only save the resources in the candidate cell which becomes serving cell.

It could be observed there is CG resource wastage issue in Rel-19 for CLTM. Considering the new MAC CE will be introduced for early TA acquisition, we could introduce the CG resource information in the new MAC CE. In this case, the candidate cell will only assign the CG resource to UEs when it receives the preamble for early TA and release the CG resource after UE completes the LTM. In this way the candidate cell won’t reserve the dedicated CG resource for a UE forever and the resources could be saved.

Proposal 4: CG resources related information is included in the MAC CE for CLTM early TA acquisition, detail is FFS.

  - UE behaviors to the new timer

After UE receives the new MAC CE for CLTM early TA acquisition, we agreed that a new timer is needed in RAN2 #128 meeting[1]:

- Candidate cell TA is maintained by a new timer.

In legacy LTM, the UE does not maintain the TA timer for the candidate cell and relies on network implementation to guarantee the TA validity. Here, as we agreed on the new timer, it is straightforward that the timer indicates whether the UL is synchronized or not: when the timer is running, the UL is considered synchronized to the associated candidate cell; otherwise, the UL is considered non- synchronized.

Therefore, the UE’s behavior is simple, when the new MAC CE is sent to the UE, the UE should (re)start this TA timer. Upon CLTM execution when triggering event is satisfied, if the timer expires, then it means the TA is not valid anymore, and the UE should perform RACH-based procedure instead of RACH-less C-LTM.

Proposal 5: UE (re)starts new TA timer upon receiving TA in new MAC CE.

Proposal 6: Upon C-LTM execution, UE performs RACH-less C-LTM if the new TA timer of the target cell is still running.

Furthermore, it is not clear about the granularity of TA maintenance, as there are two options:

- Option1: per TAG

- Option2: per candidate cell

In our understanding, the configuration of this new TA timer is supposed to be included in the candidate cell configuration, which means the TA timer should be per-candidate cell granularity. Then the following question would be that how many TA can be maintained by UE at the same time. We think RAN2 should discuss this issue.

Proposal 7: TA of CLTM candidate cell(s) is maintained in the granularity of per candidate cell.

Proposal 8: RAN2 to discuss how many TA can be maintained by UE at the same time.

  - Beam selection in RACH-less C-LTM with CG

In RAN2 #128 meeting, it is agreed to support CG-based RACH-less LTM. Upon LTM execution, the UE should select the beam which fulfils the conditions of triggering event. However, in case multiple beams fulfil the condition of triggering event, it is not clear how to select the beam for CLTM execution, thus we will discuss this issue in this section.

Firstly, it is better to select the beam which corresponds to the CG resource. Secondly, it is also better to select the beam which has been activated since it could reduce the latency.

However, we think there is no need to restrict the UE to only select a beam from the beams which are associated with activated TCI state or with the CG, because those beams may not be the best ones and it is also not clear how the network can know the final beam selected by the UE. Therefore, the simplest way is left to UE implementation.

After the beam is selected by the UE, if the UE has valid TA of the cell corresponding to that beam and the beam corresponds to a CG resource, then CG based RACH-less procedure can be performed by the UE. Therefore, we propose:

Proposal 9: If multiple beams fulfill the conditions of triggering event of CLTM, it is up to UE implementation to select one beam.
Proposal 10: If the TA of the cell corresponding to selected beam has been acquired by the UE and is valid, the UE performs RACH-less procedure.

  - RACH-less C-LTM with DG

It is agreed in RAN2 #128 meeting[1] that:

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

In our understanding, it means the TCI state activation/deactivation is performed by network and it is left to network implementation to determine which TCI state(s) should be activated/deactivated.

However, if it is left to network implementation, the beam with activated TCI state may not correspond to the beam finally selected/used by the UE based on its LTM related measurement results. In our understanding, it is not a big issue even if the beam with activated TCI state is not the final beam used upon LTM execution, because in Rel-18 the network can also activate some TCI state before the LTM cell switch, but use the LTM cell switch command to provide another indicated TCI state. The consequence is that as the network does not know which beam is finally selected by the UE, it is better that the DG-based approach (i.e. network can schedule the UE using DG after LTM execution) is not supported, which is related to another FFS:

For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

If there is no DG-based approach for LTM, in our understanding, the UE does not need to inform NW of which beam is selected. A UE can send UL message after switching to candidate cell in CG, so the NW knows the DL beam for subsequent transmission (which of course requires NW monitor provided CG on all beams) based on the relation between SSB index to the CG resource.

Observation 2: If the UE does not inform NW which beam is selected upon conditional LTM execution, the NW could still know the beam because the UE can send UL message after switching to candidate cell in CG (which requires NW monitor provided CG on all beams).

Proposal 11: DG-based first UL transmission approach for RACH-less conditional LTM is not supported.

  - Other issues for C-LTM completion/failure

For completion phase, there are some other questions to be answered, e.g.

- Whether/how to define the supervisor timer for conditional LTM?

- For RACH-based procedure, how can UE acquire the CFRA resource for the initial access in target cell?

- What should be the UE behavior upon conditional LTM failure?

For second question, which is about the CFRA resources, it is quite different to legacy LTM as legacy LTM switch command MAC CE can include the CFRA information which could be shared by multiple UEs such as Random Access Preamble index, which UL carrier to transmit the PRACH, SS/PBCH index, PRACH Mask index, and Repetition number[3]. For conditional LTM, if we would like to support CFRA, the CFRA resource may only be included in the candidate cell configurations of conditional LTM, which is dedicated for a UE. However, the resources waste issue should be considered, considering the conditional LTM may never be executed for a certain cell while this cell has to reserve the CFRA resource for this UE during the whole subsequent LTM procedure. Anyway, we understand this may be left to candidate cell implementation, and if CFRA resource is included, CFRA can be performed by the UE.

Proposal 12: For RACH-based conditional LTM procedure, CFRA is performed when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

For other questions, we understand we should reuse the legacy LTM as much as possible. LTM supervisor timer was discussed in Rel-18 and T304 is finally extended to cover the LTM case, as follows[4]:

| Timer | Start | Stop | At expiry |
| --- | --- | --- | --- |
| T304 | <omitted...> Also, for the MCG and SCG upon an indication from lower layer that an LTM cell switch procedure is triggered and, for the MCG, upon performing an LTM cell switch procedure following cell selection performed while timer T311 is running. | Upon successful completion of random access on the corresponding SpCell. Upon receiving an indication from lower layers of successful completion of Rach-less handover, or upon receiving an indication from lower layers of successful completion of an LTM RACH-less cell switch. <omitted...> | For T304 of MCG, in case of the handover from NR or intra-NR handover, or path switch from a L2 U2N Relay UE to a NR cell, or an LTM cell switch procedure, initiate the RRC re-establishment procedure; In case of handover to NR, perform the actions defined in the specifications applicable for the source RAT. If any DAPS bearer is configured and if there is no RLF in source PCell, initiate the failure information procedure. <omitted...> |

In our understanding, T304 can be simply reused for conditional LTM, i.e. same UE behavior as R18 LTM.

For the stop condition of T304, which is how to determine the conditional LTM is successfully complete, it is agreed in RAN2 #127bis meeting that

The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

which means that the stop condition of legacy LTM can also be reused, as follows[5]:

8. The UE completes the LTM cell switch procedure by sending RRCReconfigurationComplete message to target cell. If the UE has performed a RA procedure in step 7 the UE considers that LTM cell switch execution is successfully completed when the random access procedure is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the network has successfully received its first UL data.

And for UE behavior upon conditional LTM failure, we think it is also reasonable to support fast recovery to a candidate cell which is a conditional LTM candidate cell. Generally speaking, almost all the UE behaviors for conditional LTM execution and completion are quite similar, because the main difference for conditional LTM and legacy LTM is on the configuration and evaluation phase before LTM cell switch is triggered.

Therefore, we propose:

Proposal 13: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

Proposal 14: For conditional LTM failure, fast recovery to a candidate cell of conditional LTM is supported. Details are FFS.

- Conclusion

In this contribution, we discussed early TA related issue, e.g. MAC CE content, TA timer start/expiry conditions and corresponding UE behaviours. We also discussed beam selection, CG/DG-based RACH-LESS procedure, when to use RACH-based procedure and CLTM failure recovery.

We have the following Observations and Proposals:

New TA MAC CE content

Observation 1: The CG handling method could only save the resources in the candidate cell which becomes serving cell.

Proposal 1: TAC field is included in the MAC CE for CLTM early TA acquisition, and the value couldn’t be FFF.

Proposal 2: TAG information field is included in the MAC CE for CLTM early TA acquisition.

Proposal 3: CLTM candidate cell ID is not included in the MAC CE for CLTM early TA acquisition, and the TA within the MAC CE is for the CLTM candidate cell related to the latest PDCCH order.

Proposal 4: CG resources related information is included in the MAC CE for CLTM early TA acquisition, detail is FFS.

UE behaviors to the new timer

Proposal 5: UE (re)starts new TA timer upon receiving TA in new MAC CE.

Proposal 6: Upon C-LTM execution, UE performs RACH-less C-LTM if the new TA timer of the target cell is still running.

Proposal 7: TA of CLTM candidate cell(s) is maintained in the granularity of per candidate cell.

Proposal 8: RAN2 to discuss how many TA can be maintained by UE at the same time.

Beam selection in RACH-less C-LTM with CG

Proposal 9: If multiple beams fulfill the conditions of triggering event of CLTM, it is up to UE implementation to select one beam.

Proposal 10: If the TA of the cell corresponding to selected beam has been acquired by the UE and is valid, the UE performs RACH-less procedure.

RACH-less C-LTM with DG

Observation 2: If the UE does not inform NW which beam is selected upon conditional LTM execution, the NW could still know the beam because the UE can send UL message after switching to candidate cell in CG (which requires NW monitor provided CG on all beams).

Proposal 11: DG-based first UL transmission approach for RACH-less conditional LTM is not supported.

Other issues for C-LTM completion/failure

Proposal 12: For RACH-based conditional LTM procedure, CFRA is performed when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

Proposal 13: T304 is reused as the supervisor timer for conditional LTM, i.e. the timer starts upon conditional LTM execution, stops upon successful conditional LTM procedure completion, and expiry with RRC re-establishment procedure by the UE.

Proposal 14: For conditional LTM failure, fast recovery to a candidate cell of conditional LTM is supported. Details are FFS.

- References

- RAN2 #128 meeting chair notes;

- R2-2500xxx, Running MAC CR for Mob Ph4 (vivo);

- 3GPP TS 38.321 V18.4.0 (2024-12);

- 3GPP TS 38.331 V18.4.0 (2024-12);

- 3GPP TS 38.300 V18.4.0 (2024-12).