3GPP TSG-RAN WG2 Meeting #129	R2-2500350

Athens, Greece, 17th – 21st Feb. 2025

Source:	vivo

Title:	Remaining issues on LCP enhancements for XR

Agenda Item:	8.7.4.1

Document for:	Discussion and Decision

- Introduction

According to the progress in the previous meetings, the following issues in relation to LCP enhancements are still open and discussed in this contribution,

- the signaling design of LCP enhancement;

- the relaxation of rate limit for LCH priority-adjusted data;

- whether/how additional priority impacts intra-UE prioritization.

- Discussion

  - Signaling design of LCP enhancement

In RAN2#128, there are the following agreements regarding LCP enhancement[1]:

Agreements on LCP prioritization - As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

In RAN2#127 bis meeting, it was agreed that LCP configuration is provided per LCH basis as below:

3. Introduce an independent per-LCH remaining time threshold for applying delay-critical priority.

Then one remaining issue is whether to introduce a new indicator to indicate that LCP enhancement is enabled for an LCH or not. As usual, to avoid the overhead due to the new indicator to enable LCP enhancement for an LCH, the independent per-LCH remaining time threshold can be used to indicate whether the LCP enhancement is configured for an LCH or not.

- When the independent per-LCH remaining time threshold is configured for an LCH, the LCP enhancement is enabled for the LCH.

  - Rate limit for LCH priority-adjusted data

For a first LCH with LCH priority-adjusted data, the gNB may issue UL grants to speed up the LCH priority-adjusted data transmissions. After receiving the UL grants, the UE may prioritize the resource allocation for the first LCH according to the additional priority of this LCH. However, when the UE has a large amount of LCH priority-adjusted data for the first LCH, after the UE has allocated radio resource up to Bj for the first LCH, there is still remaining LCH priority-adjusted data for the first LCH. According to the current LCP procedure, the UE will allocate the remaining resource for the second LCH of a lower priority. After radio resource allocation for the second LCH, there may be no or no enough remaining resources to transmit all the remaining LCH priority-adjusted data of the first LCH.

In another instance, when there can be certain scheduling restriction, e.g. due to measurement gap (MG) overlapping with the PDB/PSDB window of a frame burst, the actual available time period(s) for transmission of the frame burst after DSR triggering could be further reduced due to the overlapping between the MG and the remaining time period. In this case, even when the UE gets enough UL grants providing enough capacity for the LCH priority-adjusted data of a LCH, the UE may not be able to include all LCH priority-adjusted data of this LCH into the corresponding MAC PDUs due to the restriction of the Bj of this LCH and some capacity of the UL grants intending for the LCH priority-adjusted data of this LCH are allocated to transmit the non-LCH priority-adjusted data of other LCHs instead according to the existing multiplexing mechanism.

- Upon MAC PDU construction, the LCH with LCH priority-adjusted data may not be allocated enough radio resource due to its buckets (Bj) exhausted.

To speed up the LCH priority-adjusted data transmission, the UE should be temporarily allowed to allocate radio resources exceeding the rate limit (Bj limit).

- In addition to LCH priority adaptation, RAN2 should also consider the option to allow UE to temporarily raise the rate limit (e.g. Bj) of the LCH with LCH priority-adjusted data.

  - Impact on intra-UE prioritization

In RAN2#127[3], there were the following agreements regarding LCP enhancements:

RAN2 to no longer consider the enhancement of the LCP restriction, as one of the candidate solutions for LCP enhancements in Rel-19 XR. FFS whether/how additional priority impacts intra-UE prioritization (can be discussed in stage-3)

Yet there is another issue on whether or how to handle the additional priority impact on the intra-UE prioritization. It is well known that intra-UE prioritization was specified to prioritize the transmission of URLLC service which is specific for industry wireless. It is doubtful whether it is a reasonable assumption that one UE could have concurrent URLLC service and XR service. Furthermore, we need also to think about whether the gNB can ensure desired intra-UE prioritization procedure without specification impact even if one UE “could” have the concurrent XR service and the URLLC service. When one UE “occasionally” has concurrent URLLC service and XR service, the gNB should be able to ensure that both the additional LCH priority and the original LCH priority for delay-critical LCHs carrying XR service data should be lower than that of the URLLC service data. With this assumption, the URLLC service data transmission would not be blocked by the additional LCH priority of XR data due to the URLLC service data always has higher priority than the XR service data no matter whether the original LCH priority or additional priority is applied for the LCH carrying the XR service data.

- Intra-UE prioritization was initially for ensuring URLLC data transmission while it is less likely that one UE has concurrent URLLC service and XR service.

- Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above assumption, one concluded that no special enhancement is required for the intra-UE prioritization procedure due to introducing the additional LCH priority:

- No enhancement is needed for intra-UE prioritization procedure due to introducing the additional LCH priority.

- Conclusion

In this contribution, we have discussed the LCP enhancements and potential solutions. Based on the discussions, there are the following observations:

Observation 1	Upon MAC PDU construction, the LCH with LCH priority-adjusted data may not be allocated enough radio resource due to its buckets (Bj) exhausted.

Observation 2	Intra-UE prioritization was initially for ensuring URLLC data transmission while it is less likely that one UE has concurrent URLLC service and XR service.

Observation 3	Even if one XR UE has concurrent URLLC service and XR service, by proper additional LCH priority configuration, gNB can ensure that URLLC service data always has higher priority than the XR service data, regardless of whether the additional LCH priority or the original LCH priority is applied for the XR data.

Based on the above discussions and the observations, we have the following proposals:

Proposal 1	When the independent per-LCH remaining time threshold is configured for an LCH, the LCP enhancement is enabled for the LCH.

Proposal 2	In addition to LCH priority adaptation, RAN2 should also consider the option to allow UE to temporarily raise the rate limit (e.g. Bj) of the LCH with LCH priority-adjusted data.

Proposal 3	No enhancement is needed for intra-UE prioritization procedure due to introducing the additional LCH priority.

- References

- RAN2#128 Chair notes

- RAN2-127bis, Chair notes

- RAN2-127, Chair notes