3GPP TSG-RAN WG2 Meeting #129	R3-2500365

Athens, Greece, 17 – 21 February 2025

Agenda Item:	8.10.5

Source:	Nokia

Title:	MHI/UHI Enhancement for SCG Deactivation/Activation

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1 Introduction

RAN2 sent an LS [R2-2407664/R3-245009] to RAN3 asking RAN3 to analyse whether Network based solution is sufficient enough for network optimization, or whether a UE based solution is needed.

This topic was discussed during RAN3#126 meeting and the following LS reply was provided to RAN2 [R2-2500025/R3-247906]:

RAN3 has agreed to support a network-based solution for enhancing SCG activation/deactivation. RAN3 understands that UE based solution may also be beneficial in order to monitor SCG deactivation/activation across RRC connected->idle->connected transitions.

In this Tdoc with discuss some aspects of MHI/UHI enhancements with SCG Activation/de-activation time.

# 2 Discussion

### 2.1 SCG activation and deactivation

The mechanism for SCG activation and deactivation was introduced in Rel-17 to enable reasonable UE battery consumption while having fast usage of SCG when (NG)EN-DC or NR-DC is configured. While the SCG is deactivated, there is no transmission via SCG RLC bearers. Only the NR SCG can be deactivated, and all SCG SCell(s) are in deactivated state while the SCG is deactivated.

SCG activation/deactivation can be triggered by UL or DL traffic on Data Radio Bearers (DRBs) with an SCG Radio Link Control (RLC) bearer (split or SCG bearer) by MN, SN or UE based on some activation/deactivation condition that is not clearly specified in 37.340. The following figure shows an example of how this could work assuming the activation/deactivation condition is based on a traffic threshold.

The concept of SCG activation and deactivation is most efficient for traffic where bursts of data large enough to benefit from SCG transmission are separated by periods of lower activity, during which the SCG can be deactivated. As the energy consumption in SCG active state is high, the criterion deciding upon SCG activation and deactivation shall be carefully selected. The simplest way is a UE traffic threshold where periods with data traffic above the threshold correspond to SCG activated state, while periods with data traffic below the threshold correspond to SCG deactivated state which may be configured for each UE or group of UEs with similar traffic profile.

In case the threshold is set as too low, the SCG activated period may contains also bursts of data which are not large enough to benefit from SCG transmission, hence leading to unnecessary power consumption. In case the threshold is set as too aggressively high, it could happen that not all bursts of data which are large enough benefit from SCG transmission are detected, which may lead to QoS degradation from end user perspective, though energy consumption may be reduced in this case.

Currently, the UHI and MHI for SCG include no information about SCG activation and deactivation. Despite majority of companies consider the information about the time of SCG activation and/or deactivation could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity in our understanding, due to possible different setting of traffic thresholds between the PSCells of different nodes this information may be insufficient. The detailed information about relative time in SCG activated state may therefore need to be extended with some more information. For instance, if the relative time in SCG activated state is quite low, it means that the Dual Connectivity is possibly not so necessary for the UE, and the network might not need configure DC for the UE, to reduce the UE’s battery consumption and the signaling overhead of DC configuration at network side. However, such decision may not be optimal if the traffic threshold for the PSCells(s) reported within the MHI is set as too aggressively high. On the other hand, if the relative time in SCG activated state is quite high, it possibly means that the Dual Connectivity is highly demanded, and the MN could configure and/or activate the SCG more efficiently, which again may not be optimal if traffic threshold is set as too low.

Observation 1: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

### 2.2 Network-based solution and UE-based solution

The main argument to have a UE-based solution based on MHI is that the NW-based solution (the UHI stored) is lost upon RRC state transitions, i.e. the NW cannot maintain the PSCell information because it removes all the UE context while UE leaves RRC CONNECTED state. Therefore, some of the companies think, the network-based solution is insufficient for the optimization of dual connectivity configuration, and the UE-based solution is needed.

We tend to agree with this conclusion. However, in relation to MHI/UHI enhancement with the information about the time of SCG activation and/or deactivation to help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity, this information is insufficient as described in previous chapter.

Proposal 1: It is proposed to include the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, in MHI.

# 3 Conclusion

In this contribution the following observation and proposal are made:

Observation 1: The information of SCG activation and/or deactivation, e.g., the relative time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, could help the network understand the actual utilization of Dual Connectivity and further optimize the configuration of Dual Connectivity.

Proposal 1: It is proposed to include the information of SCG activation/deactivation, e.g., the ratio of time in SCG activated state together with some more assistance information related to the condition(s) for SCG activation/deactivation, in MHI.