3GPP TSG-RAN2 #129	R2-2500373

Athens, Greece, Feb.  17th – 21st, 2025

Agenda item:	8.6.3 (NR_Mob_Ph4-Core)

Source:	Ofinno

Title: 	Discussion on event triggered LTM report

Document for:	Discussion and Decision

# Introduction

An objective of the WID [1] for R19 mobility enhancements is to specify necessary components to support event triggered L1 measurement reporting. RAN2 made the following agreements:

RAN2#128 agreed the following [2]: - For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE. - For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE. - A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals. - Support the truncated measurement report MAC CE. RAN2#127b agreed the following [3]: - MR can be sent when the leaving condition is met, based on NW configuration. - Event triggered periodic MR can be supported, based on NW configuration. - Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5). - Basic information included in MR MAC CE: - Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id) - Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams - Triggered event information (e.g., ReportConfigID) - MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW. - The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE. - NW can configure a dedicated SR configuration for MR MAC CE transmission. RAN2#127 agreed the following [4]: - Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell. - Any beam in candidate RS configuration can be used for LTM event evaluation. - Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case. RAN2#126 agreed the following [5]: - Event triggered L1 measurement should be designed for the following LTM purposes: - Select the candidate beam/cell to trigger early synchronization. - Select the target beam/cell and trigger LTM cell switch procedure.

In this contribution, we discuss issues related to event triggered LTM measurement report.

# 2. Discussion

## 2.1 LTM event evaluation

RAN2#127 agreed the beams used for LTM event evaluation. Based on the agreement [4], current beam is used for event evaluation of a serving cell, and any beam in the candidate RS configuration can be used for LTM event evaluation. Further, RAN#127b confirmed the working assumption that the same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

A UE may evaluate an LTM event using multiple candidate beams. The UE may also be configured with multiple candidate cells. This implies that the UE may evaluate each LTM event using multiple candidate beams for each of the multiple candidate cells, according to the measurement resource configuration and the measurement reporting configuration. This leads to a case where there is frequent triggering of event triggered LTM MRs by the UE. Some of these may be redundant or unnecessary. Several scenarios may occur where multiple candidate beams satisfy the entry condition of an LTM event:

Case 1: More than one candidate beams of a candidate cell satisfies the entry condition of the LTM event at the same time.

UE may evaluate the LTM event using multiple candidate beams of a candidate cell. A first candidate beam may satisfy the entry condition for the LTM Event. Before triggering the MR, for the first candidate beam satisfying the entry condition for the LTM event, a second candidate beam may also satisfy the entry condition of the LTM event.

Observation 1: For a candidate cell, before triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

In this case, there are two possible behaviors. In a first option, the UE may trigger separate MRs for each of the candidate beams that satisfied the entry condition of the LTM event. In a second option, the UE may trigger a single MR for the multiple candidate beams that satisfied the entry condition of the LTM event. RAN2#127b agreed that MAC CE for MR can include measurement results of multiple beams. The first option seems redundant, and it is unnecessary to trigger separate MRs.

Proposal 1:  If multiple candidate beams of a candidate cell satisfy an entry condition of an LTM Event and an MR for the LTM event has not been triggered, trigger a new MR.

Case 2: A candidate beam of a candidate cell satisfies the entry condition of the LTM event, soon after triggering an MR for the LTM event for a different candidate beam of the candidate cell.

The UE may evaluate the LTM event using multiple candidate beams of a candidate cell. The UE triggers an MR after a first candidate beam of the candidate cell satisfies the entry condition for the LTM Event. After triggering the MR, a second candidate beam of the same candidate cell may satisfy the entry condition for the same LTM event.

Observation 2: For a candidate cell, after triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

In this case, there are two possible behaviors. In a first option, the UE may trigger a new MR for the second candidate beams that satisfied the entry condition of the LTM event, irrespective of how much time has elapsed since triggering the MR for the first candidate beam. This may lead to triggering many MRs. In a second option, the UE may trigger a new MR for the second candidate beam, if a significant amount of time has elapsed since triggering the MR for the first candidate beam. For example, to the UE doesn’t trigger a new MR if the time elapsed since triggering the MR for the first candidate beam is small or an insignificant amount. On the other hand, if the time elapsed is a significant one, the UE triggers a new MR. The network may configure the time value based on which the UE may trigger the new MR. This is similar to operation of a prohibit timer.

Proposal 2: Introduce prohibit timer for event trigger LTM MR, with the following behavior:

- The prohibit timer is stared after triggering a new MR for a candidate cell satisfying an entry condition of an LTM event.

- While the prohibit timer is running, a new MR is not triggered for the same candidate cell for the same LTM event.

- While the prohibit timer is running, a new MR is triggered for the same candidate cell satisfying an entry condition of a different LTM event.

## 2.2 LTM MR validity

UE may trigger an MR if a candidate beam satisfies the entry condition of an LTM event. The UE may transmit the MR if an uplink resource is available. If an uplink resource is not available, the UE may wait to receive an uplink resource for transmitting the MR. RAN#127b agreed that the UE triggers SR for resource allocation to send the event triggered MR. If an uplink resource is received after sending SR, the UE may transmit the MR. If an uplink resource is not received after transmitting the SR, the UE may initiate a RACH procedure and transmit the MR after receiving the uplink resource. The information in the triggered MR may be outdated by the time the uplink resource is available.

Observation 3: The measurement results in a triggered MR may be outdated if an uplink resource for transmitting the triggered MR is available only after a significant time has elapsed since triggering the MR.

Proposal 3: RAN2 to discuss handling of outdated information in event triggered LTM MR.

### 2.3 LTM MR MAC CE

In the case of layer 3 measurements, RRC layer may trigger multiple layer 3 measurement reports. The RRC layer may submit to the MAC entity, the triggered layer 3 measurement reports for transmission. The MAC entity, based on the available uplink resource and logical channel prioritization procedure, may multiplex the triggered layer 3 measurement reports into a single MAC PDU and transmit to the network.

As discussed above, there may be frequent triggering of event triggered LTM MRs. Some of these MRs may be triggered concurrently, e.g., for different LTM events by different beams of the candidate cell. There may be multiple MRs triggered and pending for transmission. RAN2 has already agreed that the MRs is sent via a MAC CE. However, the current MAC specification only allows multiplexing a single instance of a MAC CE into an MAC PDU.

Observation 4: A MAC PDU can accommodate multiple layer 3 measurement reports while a MAC PDU can only accommodate one event triggered LTM MR MAC CE.

When multiple MRs are triggered and if the MAC entity can accommodate only one LTM MR MAC CE in a MAC PDU, and if an LTM MR MAC CE can accommodate only one triggered MR, this may cause frequent triggering of LTM MR MAC CEs. This leads to higher signalling due to different MRs being transmitted via different MAC PDUs and each LTM MR MAC CE having its subheader. This may even lead to frequent transmission of scheduling request if uplink resource for transmitting the MR is not available. There are multiple approaches to solve this problem. In a first option, LTM MR MAC CE may consist of multiple triggered MRs. In a second option, LTM MR MAC CE may consist of only one MR and a MAC PDU may accommodate multiple LTM MR MAC CEs.

Proposal 4: RAN2 to discuss the following:

- The number of event triggered LTM MRs that can be accommodated in an LTM MR MAC CE.

- The number of LTM MR MAC CEs that can be accommodated in a MAC PDU.

# 3. Conclusion

The contribution discussed aspects related to event triggered LTM measurement report. The following observations and proposals are made:

LTM Event evaluation:

Observation 1: For a candidate cell, before triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

Proposal 1:  If multiple candidate beams of a candidate cell satisfy an entry condition of an LTM Event and an MR for the LTM event has not been triggered, trigger a new MR.

Observation 2: For a candidate cell, after triggering MR for a first candidate beam satisfying the LTM event, a second candidate beam may satisfy the LTM event.

Proposal 2: Introduce prohibit timer for event trigger LTM MR, with the following behavior:

- The prohibit timer is stared after triggering a new MR for a candidate cell satisfying an entry condition of an LTM event.

- While the prohibit timer is running, a new MR is not triggered for the same candidate cell for the same LTM event.

- While the prohibit timer is running, a new MR is triggered for the same candidate cell satisfying an entry condition of a different LTM event.

LTM MR validity:

Observation 3: The measurement results in a triggered MR may be outdated if an uplink resource for transmitting the triggered MR is available only after a significant time has elapsed since triggering the MR.

Proposal 3: RAN2 to discuss handling of outdated information in event triggered LTM MR.

LTM MR MAC CE:

Observation 4: A MAC PDU can accommodate multiple layer 3 measurement reports while a MAC PDU can only accommodate one event triggered LTM MR MAC CE.

Proposal 4: RAN2 to discuss the following:

- The number of event triggered LTM MRs that can be accommodated in an LTM MR MAC CE.

- The number of LTM MR MAC CEs that can be accommodated in a MAC PDU.

# References

- RP-240299, Revised Work Item: NR mobility enhancements Phase 4

- Chairman Notes: TSG RAN WG2 Meeting #128

- Chairman Notes: TSG RAN WG2 Meeting #127bis

- Chairman Notes: TSG RAN WG2 Meeting #127

- Chairman Notes: TSG RAN WG2 Meeting #126