3GPP TSG-RAN WG2 Meeting #129	R2-2500374

Athens, Greece, Feb. 17th – 21st, 2025

Source:               Ofinno

Agenda item:      7.2 Further NR mobility enhancements

Title:	Reverting variable for LTM

Document for:	Discussion and decision

- Introduction

In this contribution, we address some remaining RRC issue for LTM.

- Discussion

According to section 5.3.5.18.6 of TS 38.331, when the UE execute an LTM cell switch procedure (e.g., a first LTM cell switch procedure), the UE may replace:

- a value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with a value of ltm-NoResetID if the value of ltm-ServingCellNoResetID is equal to the value of ltm-NoResetID; or

- a value of ltm-ServingCellUE-MeasuredTA-ID in ltm-ServingCellUE-MeasuredTA-ID with a value of ltm-UE-MeasuredTA-ID if the value of ltm-ServingCellUE-MeasuredTA-ID is equal to the value of ltm-UE-MeasuredTA-ID.

---------------------------------------------- section 5.3.5.18.6 of TS 38.331 -------------------------------------------------------

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

…

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

…

2>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

3>	replace the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID in the LTM-Candidate in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3;

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

NOTE 0:	The UE is not expected to perform UE-based TA measurements for an SpCell.

--------------------------------------------------------------------------------------------------------------------------------

Observation 1    when the UE executes an LTM cell switch procedure (e.g., first LTM cell switch procedure), the UE may replace:

- the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID; or

- the value of ltm-ServingCellUE-MeasuredTA-ID in ltm-ServingCellUE-MeasuredTA-ID with the value of ltm-UE-MeasuredTA-ID.

The UE executing the LTM cell switch may start T304 timer. The UE determines T304 expiry. According to section 5.3.5.8.3 of TS 38.331, the UE revert the UE configuration used in the source PCell when T304 is expired:

---------------------------------------------- section 5.3.5.18.6 of TS 38.331 -------------------------------------------------------

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

…

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to clause 5.3.10.3:

…

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB or multicast MRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

…

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6:

4>	revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else:

4>	revert back to the UE configuration used in the source PCell;

3>	if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3:

4>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

--------------------------------------------------------------------------------------------------------------------------------

According to the NOTE 1, the UE configuration includes state variables and parameters of each radio bearer.

Observation 2     the UE revert the UE configuration used in the source PCell when T304 is expired:

Observation 3     the UE configuration includes state variables (e.g., PDCP, RLC state variables) and parameters of each radio bearer.

The UE configuration doesn’t include LTM variables such as VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID. The UE doesn’t revert the LTM variables once T304 is expired.

Observation 4     the UE configuration doesn’t include LTM variables such as VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID.

Observation 5     the UE doesn’t revert the LTM variables once T304 is expired.

According to the grey highlighted text, after reverting the UE configuration not comprising the LTM variables, the UE initiates the connection re-establishment procedure.

---------------------------------------------- section 5.3.7.3 of TS 38.331 -------------------------------------------------------

#### 5.3.7.3	Actions following cell selection while T311 is running

Upon selecting a suitable NR cell, the UE shall:

…

1>	if attemptLTM-Switch is configured; and

1>	if the selected cell is one of the LTM candidate cells in the LTM-Candidate IE within ltm-Config associated with the MCG:

2>	perform the LTM cell switch procedure for the selected LTM candidate cell according to the actions specified in 5.3.5.18.6;

NOTE 2:	In case both attemptCondReconfig and attemptLTM-Switch are configured, it is left to the UE implementation which procedure to execute.

--------------------------------------------------------------------------------------------------------------------------------

According to section 5.3.7.3 of TS 38.331, during the connection re-establishment procedure, the UE may execute the LTM cell switch procedure (e.g., second LTM switch procedure or LTM recovery procedure) as specified in 5.3.5.18.6.

When the UE executes the LTM recovery procedure, the UE determines whether to perform a L2 reset (e.g., RLC re-establishment, PDCP data recovery) based on the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID. The value is a value the UE replaced with the value of ltm-NoResetID when the UE executed the first LTM cell switch procedure.

Observation 6     when the UE performs LTM recovery procedure, the UE determines whether to perform L2 reset based on a value replaced during the first LTM cell switch procedure.

The replaced value (i.e., the current value) of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID may be different from a value of ltm-ServingCellNoResetID used in the source PCell. This may cause wrong L2 reset operations.

Observation 7     Not reverting the value of ltm-ServingCellNoResetID on T304 expiry cause the UE to perform wrong L2 reset actions.

To avoid the this issue, a possible solution would be revert VarLTM-ServingCellNoResetID when T304 is expired.

Proposal 1     RAN2 to confirm that the UE reverts VarLTM-ServingCellNoResetID used in the source PCell when T304 is expired.

---------------------------------------------- section 5.3.5.18.6 of TS 38.331 -------------------------------------------------------

##### 5.3.5.18.6	LTM cell switch execution

Upon the indication by lower layers that an LTM cell switch procedure is triggered, or upon performing LTM cell switch following cell selection performed while timer T311 was running, as specified in 5.3.7.3, the UE shall:

…

1>	if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 does not contain the field ltm-NoResetID and if the UE does not have any value stored of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID; or

1>	if the value of field ltm-NoResetID contained within the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 is not equal to the value of ltm-ServingCellNoResetID within VarLTM-ServingCellNoResetID:

2>	for each logicalChannelIdentity and logicalChannelIdentityExt that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	if servedRadioBearer is set to drb-Identity:

4>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each bh-LogicalChannelIdentity that is part of the current UE configuration for the cell group for which the LTM cell switch procedure is triggered:

3>	after the end of this procedure, re-establish the corresponding RLC entity as specified in TS 38.322 [4], after applying the LTM configuration in ltm-CandidateConfig within the LTM-Candidate IE in ltm-Config;

2>	for each drb-Identity value that is part of the current UE configuration:

3>	if this DRB is an AM DRB:

4>	after the end of this procedure, trigger the PDCP entity of this DRB to perform data recovery as specified in TS 38.323 [5], after applying the LTM configuration in ltm-CandidateConfig within LTM-Candidate IE in ltm-Config;

--------------------------------------------------------------------------------------------------------------------------------

When the UE executes the LTM recovery procedure, based on the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID, the UE determines whether to replace the value of ltm-ServingCellUE-MeasuredTA-ID and determine whether to configure the UE-based TA measurement. The value is a value the UE replaced with the value of ltm-UE-MeasuredTA-ID when the UE executed the first LTM cell switch procedure.

The replaced value of ltm-ServingCellUE-MeasuredTA-ID may be different a value of ltm-ServingCellUE-MeasuredTA-ID used in the source PCell. It causes wrong UE behavior about UE based TA measurements. For example, the UE performs the UE-based TA measurements on a cell not configured for the UE based TA measurement or not perform the UE-based TA measurements on a cell configured for the UE based TA measurement.

Observation 8    Not reverting the value of ltm-ServingCellNoResetID on T304 expiry may cause wrong UE behavior for UE based TA measurement.

To avoid the this issue, a possible solution would be revert ltm-ServingCellUE-MeasuredTA-ID when T304 is expired.

Proposal 2     RAN2 to confirm that the UE reverts ltm-ServingCellUE-MeasuredTA-ID used in the source PCell when T304 is expired.

---------------------------------------------- section 5.3.5.18.6 of TS 38.331 -------------------------------------------------------

…

1> if the LTM-Candidate IE in ltm-Config indicated by lower layers or for the selected cell in accordance with 5.3.7.3 contains the field ltm-UE-MeasuredTA-ID:

2>	if the value of ltm-UE-MeasuredTA-ID is not equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

3>	replace the value of ltm-ServingCellUE-MeasuredTA-ID in VarLTM-ServingCellUE-MeasuredTA-ID with the value received within ltm-UE-MeasuredTA-ID;

3>	for each LTM-Candidate IE in ltm-Config:

4>	if the value of ltm-UE-MeasuredTA-ID within LTM-Candidate IE is equal to the value of ltm-ServingCellUE-MeasuredTA-ID within VarLTM-ServingCellUE-MeasuredTA-ID:

5>	inform lower layers that the UE is configured with UE-based TA measurements for the LTM-Candidate;

4>	else:

5>	inform lower layers that the UE is not configured with UE-based TA measurements for the LTM-Candidate;

--------------------------------------------------------------------------------------------------------------------------------

Proposal 3    RAN2 to agree to the TP for proposal 1 and proposal 2 in Annex.

- Conclusion

The following observation are made according to the discussion:

Observation 1    when the UE executes an LTM cell switch procedure (e.g., first LTM cell switch procedure), the UE may replace:

- the value of ltm-ServingCellNoResetID in VarLTM-ServingCellNoResetID with the value of ltm-NoResetID; or

- the value of ltm-ServingCellUE-MeasuredTA-ID in ltm-ServingCellUE-MeasuredTA-ID with the value of ltm-UE-MeasuredTA-ID.

Observation 2     the UE revert the UE configuration used in the source PCell when T304 is expired:

Observation 3     the UE configuration includes state variables (e.g., PDCP, RLC state variables) and parameters of each radio bearer.

Observation 4     the UE configuration doesn’t include LTM variables such as VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID.

Observation 5     the UE doesn’t revert the LTM variables once T304 is expired.

Observation 6     when the UE performs LTM recovery procedure, the UE determines whether to perform L2 reset based on a value replaced when the UE performs the first LTM cell switch procedure.

Observation 7     Not reverting the value of ltm-ServingCellNoResetID on T304 expiry cause the UE to perform wrong L2 reset actions.

Observation 8    Not reverting the value of ltm-ServingCellNoResetID on T304 expiry may cause wrong UE behavior for UE based TA measurement.

Based on the discussion in the previous sections we propose the following:

Proposal 1     RAN2 to confirm that the UE reverts VarLTM-ServingCellNoResetID used in the source PCell when T304 is expired.

Proposal 2     RAN2 to confirm that the UE reverts ltm-ServingCellUE-MeasuredTA-ID used in the source PCell when T304 is expired.

Proposal 3    RAN2 to agree to the TP for proposal 1 and proposal 2 in Annex.

- Annex:

  - Alt 1 TP for proposals

Start of Change

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to clause 5.3.10.3:

3>	reset MAC for the target PCell and release the MAC configuration for the target PCell;

3>	for each DAPS bearer:

4>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

3>	for each SRB:

4>	if the masterKeyUpdate was not received:

5>	configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5];

4>	release the PDCP entity for the target PCell;

4>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity for the source PCell;

3>	release the physical channel configuration for the target PCell;

3>	discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

3>	resume suspended SRBs in the source PCell;

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB or multicast MRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the failure information procedure as specified in clause 5.7.5 to report DAPS handover failure.

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6:

4>	revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else:

4>	revert back to the UE configuration used in the source PCell;

3> revert back to VarLTM-ServingCellNoResetID used in the source PCell;

3> revert back to VarLTM-ServingCellUE-MeasuredTA-ID used in the source PCell;

3>	if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3:

4>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables and parameters of each radio bearer.

1>	else if T304 of a secondary cell group expires:

2>	if MCG transmission is not suspended:

3>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated, if configured;

3>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

3>	else (the UE is in (NG) EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

End of Change

  - Alt 2 TP for proposals

Start of Change

##### 5.3.5.8.3	T304 expiry (Reconfiguration with sync Failure) or T420 expiry (Path switch failure)

The UE shall:

1>	if T304 of the MCG expires; or

1> if T420 expires; or,

1> if the target L2 U2N Relay UE (i.e., the UE indicated by targetRelayUE-Identity in the received RRCReconfiguration message containing reconfigurationWithSync indicating path switch as specified in 5.3.5.5.2) changes its serving PCell before path switch:

2>	release dedicated preambles provided in rach-ConfigDedicated if configured;

2>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated if configured;

2>	if any DAPS bearer is configured, and radio link failure is not detected in the source PCell, according to clause 5.3.10.3:

3>	reset MAC for the target PCell and release the MAC configuration for the target PCell;

3>	for each DAPS bearer:

4>	release the RLC entity or entities as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	reconfigure the PDCP entity to release DAPS as specified in TS 38.323 [5];

3>	for each SRB:

4>	if the masterKeyUpdate was not received:

5>	configure the PDCP entity for the source PCell with state variables continuation as specified in TS 38.323 [5];

4>	release the PDCP entity for the target PCell;

4>	release the RLC entity as specified in TS 38.322 [4], clause 5.1.3, and the associated logical channel for the target PCell;

4>	trigger the PDCP entity for the source PCell to perform SDU discard as specified in TS 38.323 [5];

4>	re-establish the RLC entity for the source PCell;

3>	release the physical channel configuration for the target PCell;

3>	discard the keys used in target PCell (the KgNB key, the KRRCenc key, the KRRCint key, the KUPint key and the KUPenc key), if any;

3>	resume suspended SRBs in the source PCell;

3>	for each non-DAPS bearer:

4>	revert back to the UE configuration used for the DRB or multicast MRB in the source PCell, includes PDCP, RLC states variables, the security configuration and the data stored in transmission and reception buffers in PDCP and RLC entities ;

3>	revert back to the UE measurement configuration used in the source PCell;

3>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the failure information procedure as specified in clause 5.7.5 to report DAPS handover failure.

2>	else:

3>	if attemptLTM-Switch is configured and the T304 of the MCG expiry is caused by an LTM cell switch execution triggered by an indication from lower layers as described in 5.3.5.18.6:

4>	revert back to the UE configuration used in the source PCell except for the PDCP state variables for SRB(s) associated to the MCG;

3>	else:

4>	revert back to the UE configuration used in the source PCell;

3>	if the associated T304 was not initiated upon cell selection performed while timer T311 was running, as defined in clause 5.3.7.3:

4>	store the handover failure information in VarRLF-Report as described in the clause 5.3.10.5;

3>	initiate the connection re-establishment procedure as specified in clause 5.3.7.

NOTE 1:	In the context above, "the UE configuration" includes state variables, parameters of each radio bearer, VarLTM-ServingCellNoResetID and VarLTM-ServingCellUE-MeasuredTA-ID.

1>	else if T304 of a secondary cell group expires:

2>	if MCG transmission is not suspended:

3>	release dedicated preambles provided in rach-ConfigDedicated, if configured;

3>	release dedicated msgA PUSCH resources provided in rach-ConfigDedicated, if configured;

3>	initiate the SCG failure information procedure as specified in clause 5.7.3 to report SCG reconfiguration with sync failure, upon which the RRC reconfiguration procedure ends;

2>	else:

3>	if the UE is in NR-DC:

4>	initiate the connection re-establishment procedure as specified in clause 5.3.7;

3>	else (the UE is in (NG) EN-DC):

4>	initiate the connection re-establishment procedure as specified in TS 36.331 [10], clause 5.3.7;

1>	else if T304 expires when RRCReconfiguration is received via other RAT (HO to NR failure):

2>	reset MAC;

2>	perform the actions defined for this failure case as defined in the specifications applicable for the other RAT.

NOTE 2:	In this clause, the term 'handover failure' has been used to refer to 'reconfiguration with sync failure'.

End of Change