3GPP TSG-RAN WG2 Meeting #129												R2-2500376

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.11.2

Source: 	Sharp

Title:  	Random Access Issues for SBFD

Document for: 	Discussion

# Introduction

In RAN2#128 meeting, RAN2 made the following agreements:

CFRA The RO type is indicated by NW for CFRA. FFS on signaling (can FFS for the SI request case if needed). CBRA FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one. RACH configuration Only one RACH configuration option (i.e., either RACH configuration Option 1 with Alt 1-1 or RACH configuration Option 2) is supported in a cell.

This document discusses remaining opens issues on SBFD random access.

# Discussion

## Early Indication

In RAN2#128 meeting, whether an early indication of SBFD capability during the random access using legacy RO was discussed but no conclusion has been reached, as follows:

Early indication via msg3 R2-2410385 Random access for SBFD Operation Sony discussion Rel-19 NR_duplex_evo-Core Noted Proposal 2: Early identification of SBFD-aware UEs in message 3 is not needed during RA procedure. R2-2410088 RA Aspects for SBFD Sharp discussion Rel-19 NR_duplex_evo-Core Noted Proposal 1 Msg3-based early indication in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability. DISCUSSION - Sony do not think this is critical and think we should not spend too much time on this. - Apple think today we have many features that use msg3 based EI and if we do so here it requires more time to discuss.

We see that an early indication of SBFD capability during the random access is beneficial. One of the most important benefits of SBFD is latency reduction compared to legacy TDD. An early indication enables the benefit of latency reduction during the random access.

Regarding how to support the early indication, RAN2 concluded that there is no need to introduce SBFD as a feature combination. It means that Msg1-based early indication is not supported. Instead, we can consider Msg3-based early indication which still has a benefit to start transmission and reception early on SBFD symbols. It can be realized by using LCID field.

Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

## RO Selection Condition for Initial Transmission

The next issue is RO selection condition for initial transmission. In RAN2#128 meeting, RAN2 discussed two options as follows:

FFS on the following options Option 1 - Upon initiation of RACH procedure for a SBFD-aware UE, network provides the indication on the prioritization of the additional ROs over legacy RO. If there is no such indication from the NW, FFS on the following mechanism - UE select legacy RO or SBFD RO based on SSB RSRP, or - UE select the legacy RO, or - UE select the SBFD RO, or - Other metrics than SSB RSRP. Option 2 UE select legacy RO or SBFD RO based on SSB RSRP if such condition is configured, and if not configured, then UE can prioritize one type of the ROs, FFS which one.

Both Option 1 and Option2 seem to support RSRP-based RO selection. Also, both options support a strict prioritization that one RO type is always selected. Thus, we see that there is no actual difference between options. Instead, it is more important to decide which RO selection rule can be possible.

For RO selection among additional RO in SBFD symbols and legacy RO, an issue is not to cause too strong cross link interference. If the UE selects an additional RO without consideration of the interference or coverage, it will lead unnecessarily many RA attempts with power ramp-up. We think that potential cross link interference aggressor UEs should not utilize additional ROs in SBFD symbols. To achieve this, we can reuse well-known RSRP-based criteria for RO selection. More specifically, if the measured RSRP is higher than a new RSRP threshold for RO selection, which is provided upon PRACH configuration, the UE can select additional ROs. Otherwise, the UE selects legacy ROs. With this approach, SBFD-aware UE will not select additional ROs if the UE is expected to use high transmission power for PRACH by checking the RSRP threshold for RO selection.

Regardless of Option 1 or Option 2, RSRP-based RO selection rule can be configured for initial RO selection.

If the network wants that an SBFD-aware UE always uses SBFD RO, the configuration should allow this case. It can be realized by very low value (or RSRP of minus infinity) or an explicit indication. Conversely, if the network does not want to allow any UE to use SBFD RO, it can be possible that SBRD RO is not configured or the RSRP threshold is set to a very high value (or RSRP of infinity). The detail can be discussed after we agree the principle.

It is possible to configure that UE always selects SBFD RO without RSRP comparison, based on RSRP of minus infinity or an explicit indication.

It is possible to configure that UE always selects legacy RO without RSRP comparison, based on RSRP of infinity or an explicit indication or no configuration of SBFD RO.

## RO Selection Condition for Retransmission

Due to the mobility or other obstacles, UE’s RSRP at the preamble retransmission could be different from that at the initiation of random access. So, if the UE keeps using the same RO-type, e.g. additional RO, it may cause significant cross-link interference or unnecessary retransmissions of PRACH preamble. We think the same RO-type selection rule based on RSRP should be also applied for retransmission. In other words, the UE should always check the RO-type selection condition for both initial transmission and retransmissions.

An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of RA re-attempts.

## RO-Type Change from Legacy RO to SBFD RO

In RAN2#127bis meeting, RAN2 agreed to support RO fallback from SBFD RO to legacy RO. The other direction of RO-type change was discussed in RAN2#128 meeting, but no consensus was reached, as follows:

Fallback R2-2409794 Random Access for SBFD Operation NEC discussion Noted Proposal-1: the UE should be allowed to switch to additional SBFD RACH occasions after certain (configured) number of times of RACH attempt in legacy RACH occasions, if the UE selects legacy ROs first. R2-2409913 Discussion on Random Access in SBFD LG Electronics Inc. discussion Rel-19 NR_duplex_evo-Core Noted Proposal 5. Do not support switching from legacy RACH occasions to SBFD RACH occasions for SBFD-aware UE. DISCUSSION - Nokia, Sharp, QC, Ericsson agree with NEC proposal. - Samsung support LG E proposal.

We see that, regardless of the direction, the RO-type change procedure is beneficial when one RO is experiencing congestion that many UEs are attempting the preamble transmissions. For example, additional ROs in SBFD symbols can be less crowded as legacy UEs are not aware of them. In this case, RO-type change procedure from legacy RO to additional RO can be also useful from offloading perspective.

We do not see the fallback procedure requires a big complexity. The procedure will be almost symmetric for both directions. It is desirable that the network tries to maximize the system performance. Hence, we see that RO-type changes for both directions can be supported.

RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

## Working Assumption on RA Trigger Events

In RAN2#127 meeting, RAN2 made a working assumption on RA trigger events in SBFD symbols, as follows:

- Working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

The issue is whether there is a big reason to exclude some events for random access in SBFD symbols. One could argue that some events may not be so essential for SBFD. However, none of these events will bring a big technical issue in any functionality related to SBFD. It means that relevant enhancements may be considered as an optimization but not considered as an essential feature.

RAN2 to confirm the working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

- Msg3-based early indication with using LCID field in legacy ROs during RA procedure is supported to indicate UE’s SBFD capability.

- Regardless of Option 1 or Option 2, RSRP-based RO selection rule can be configured for initial RO selection.

- It is possible to configure that UE always selects SBFD RO without RSRP comparison, based on RSRP of minus infinity or an explicit indication.

- It is possible to configure that UE always selects legacy RO without RSRP comparison, based on RSRP of infinity or an explicit indication or no configuration of SBFD RO.

- An SBFD-aware UE selects either additional RO or legacy RO based on comparing measured RSRP with a RSRP threshold for RO selection in case of RA re-attempts.

- RO-Type change procedure on RO type selection from legacy RO to additional RO in SBFD symbols is supported when the number of PRACH transmission attempts exceed a threshold.

- RAN2 to confirm the working assumption: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events.