3GPP TSG-RAN WG2 Meeting #129												R2-2500379

Athens, Greece, February 17th – 21st, 2025

Agenda Item:	8.7.4.2

Source: 	Sharp

Title:  	Issues on New DSR MAC CE

Document for: 	Discussion

# Introduction

In RAN2#128 meeting, RAN2 made the following agreements on DSR enhancements:

Let the network configure the triggering and reporting thresholds without constraints. RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

This document discusses leftover issues on new DSR MAC CE.

# Discussion

## New DSR MAC CE Format

The first issue on the MAC CE format is how to change the DSR format to report multiple portions. Figure 1 depicts a possible DSR format for the enhancements. We see that R field in Rel-18 DSR format can be simply used to indicate the next pair of remaining time and buffer size is for the different portion of the same LCG (shown in E field in the figure). In this case, minimum change can be achieved without increasing MAC CE size unnecessarily. Remaining time field can be associated with one of portion reported in the DSR, so additional indication of the portion (e.g. bitmap for each portion) is not necessary. To identify the Rel-19 DSR MAC CE, a separate LCID value in eLCID field can be used.

R field in Rel-18 DSR MAC CE is changed to E field to indicate whether the next pair of remaining time and buffer size is for the different portion of the same LCG.

A new LCID value in eLCID field is used to identify Rel-19 DSR MAC CE.

Figure 1. New DSR MAC CE Format with Multiple Portions

## New DSR MAC CE Configuration

The next issue is how many portions can be reported. The equivalent issue is the maximum number of configured reporting thresholds. Considering a trade-off between granularity and MAC CE overhead, we see that at most four portions seem to be sufficient.

At most 4 reporting thresholds for an LCG can be configured.

If the DSR MAC CE format shown in Figure 2 is agreeable, there is no size difference between old DSR and new DSR in case that only one portion is reported. Thus, it does not have any additional overhead even if the new DSR MAC CE is always used. Then, if a reporting threshold is separately configured, the UE can always use the new DSR MAC CE.

If a reporting threshold is separately configured from the triggering threshold, the UE can always use the new DSR MAC CE.

## Buffer Size of Non-Delay-Critical Data

In RAN2 #128 meeting, RAN2 agreed to include non-delay critical data ahead of delay critical data in buffer size calculation for DSR, depending on UE capability. It means that Buffer Size field in Rel-19 DSR includes non-delay-critical data as well as delay-critical data. The main motivation of the consideration of non-delay-critical data is that non-delay-critical data ahead of the delay-critical data is essential information for timely transmission of the delay-critical data reported in the DSR. Thus, all non-delay-critical data which can impact scheduling of delay-critical data reported in the DSR should be reported together.

BS field in Rel-19 DSR MAC CE includes 1) amount of delay-critical data for the portion and 2) amount of non-delay-critical data ahead of the delay-critical data in the portion and not included in any previous portion.

## PDCP Data Volume Calculation for DSR MAC CE

Rel-19 DSR MAC CE may include multiple pairs of remaining time and buffer sizes for an LCG. The UE should determine the data volume for each pair of remaining time and buffer sizes for an LCG.

Currently, the following data in PDCP entity is considered as data volume for Rel-18 DSR [1]:

- the delay-critical PDCP SDUs for which no PDCP Data PDUs have been constructed; - the PDCP Data PDUs that contain the delay-critical PDCP SDUs and have not been submitted to lower layers; - the PDCP Control PDUs; - for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13; - for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

It is clear that the first two types of data can be sorted and included in the DSR according to their associated discardTimer. We would like to confirm it.

RAN2 to confirm that the following data in PDCP entity is considered as data volume for the pair of remaining time and buffer sizes associated with reporting threshold:

- PDCP SDUs with discardTimer <= reporting threshold and no PDCP Data PDUs have been constructed for the PDCP SDUs

- PDCP data PDUs that contain the PDCP SDUs with discardTimer < = reporting threshold and have not been submitted to lower layer.

In Rel-18, the last three types are included in the DSR regardless of remaining time. They should be transmitted before other PDCP SDUs or PDCP PDUs. We see that the same principle should be kept in Rel-19. It is reasonable that those are included in the first portion of the DSR report.

UE sorts the following data to the first reported portion with smaller remaining time value range:

- the PDCP Control PDUs

- for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13

- for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

## RLC Data Volume Calculation for DSR MAC CE

In addition to PDCP, RLC data volume should consider Rel-19 DSR with multiple portions. In Rel-18, the following data in RLC entity is considered as data volume [2]:

For the purpose of MAC delay status reporting, the UE shall consider the following as delay-critical RLC data volume: - delay-critical RLC SDUs and delay-critical RLC SDU segments that have not yet been included in an RLC data PDU; - RLC data PDUs pending for initial transmission, and containing a delay-critical RLC SDU or a delay-critical RLC SDU segment; - RLC data PDUs that are pending for retransmission (RLC AM). In addition, if a STATUS PDU has been triggered and t-StatusProhibit is not running or has expired, the UE shall estimate the size of the STATUS PDU that will be transmitted in the next transmission opportunity, and consider this as part of RLC data volume for MAC buffer status reporting and as part of delay-critical RLC data volume for MAC delay status reporting.

Similar to PDCP, the first two types of data, which are determined by remaining time, can be sorted to the corresponding pair of remaining time and buffer sizes based on the indication from PDCP.

RAN2 to confirm that UE can sort the following data to the pair of remaining time and buffer sizes associated with each reporting threshold:

- RLC SDUs and RLC SDUs segments that have not yet been included in an RLC data PDU and their corresponding discardTimer <= reporting threshold

- RLC data PDUs pending for initial transmission, and containing a RLC SDU or a RLC SDU segment whose corresponding discardTimer <= reporting threshold.

However, RLC data PDUs that are pending for retransmission and the RLC STATUS PDUs in RLC AM should be transmitted before other RLC PDUs or their segment. It is reasonable that those are included in the first portion of the DSR report.

UE sorts RLC data PDUs that are pending for retransmission and RLC control PDU in RLC AM to the first reported portion with smaller remaining time value range.

# Conclusions

RAN2 is requested to discuss and agree the following proposals:

- R field in Rel-18 DSR MAC CE is changed to E field to indicate whether the next pair of remaining time and buffer size is for the different portion of the same LCG.

A new LCID value in eLCID field is used to identify Rel-19 DSR MAC CE.

At most 4 reporting thresholds for an LCG can be configured.

If a reporting threshold is separately configured from the triggering threshold, the UE can always use the new DSR MAC CE.

BS field in Rel-19 DSR MAC CE includes 1) amount of delay-critical data for the portion and 2) amount of non-delay-critical data ahead of the delay-critical data in the portion and not included in any previous portion.

RAN2 to confirm that the following data in PDCP entity is considered as data volume for the pair of remaining time and buffer sizes associated with reporting threshold:

- PDCP SDUs with discardTimer <= reporting threshold and no PDCP Data PDUs have been constructed for the PDCP SDUs

- PDCP data PDUs that contain the PDCP SDUs with discardTimer < = reporting threshold and have not been submitted to lower layer.

UE sorts the following data to the first reported portion with smaller remaining time value range:

- the PDCP Control PDUs

- for AM DRBs, the PDCP SDUs to be retransmitted according to clause 5.1.2 and clause 5.13

- for AM DRBs, the PDCP Data PDUs to be retransmitted according to clause 5.5.

RAN2 to confirm that UE can sort the following data to the pair of remaining time and buffer sizes associated with each reporting threshold:

- RLC SDUs and RLC SDUs segments that have not yet been included in an RLC data PDU and their corresponding discardTimer <= reporting threshold

- RLC data PDUs pending for initial transmission, and containing a RLC SDU or a RLC SDU segment whose corresponding discardTimer <= reporting threshold.

UE sorts RLC data PDUs that are pending for retransmission and RLC control PDU in RLC AM to the first reported portion with smaller remaining time value range.

# References

[1] 3GPP TS 38.323, NR PDCP specification, v18.3.0

[2] 3GPP TS 38.322, NR RLC specification, v18.2.0