3GPP TSG-RAN WG2 Meeting #129	R2-2500399

# Athens, Greece, 17 – 21 February 2025

Agenda item:		8.3.3

Source:	Xiaomi

Title:	Simulation results for measurement event prediction

Document for:		Discussion and Decision

Introduction

In RAN2#127bis meeting, following was agreed on measurement event prediction:

Agreements

Measurement event prediction simulations will at least focus on intra-frequency FR2, case A, and second study goal (i.e. HO KPI improvement).   FFS what is KPI.

Companies can bring simulation results for intra-frequency measurement reduction for FR1 and report what they are doing.  Focus on temporal case B.

Companies will prioritize simulations on indirect method.   Companies can bring simulations on direct method and should report what method is being used.

Measurement event prediction results are expected in RAN2#129

Agreements on simulation assumptions

The Simulation assumption of RRM measurement prediction can be reused unless otherwise specified.

Companies can pick and report what they are using for filtering options (similar to RRM prediction)

Companies will focus on sub-case 2 for measurement event prediction.   Companies can simulate other sub-cases if they wish and report what they are using.

Leave the simulation parameter discussion for email discussion.  Pick only one value for A3.

It is up to company implementation how to model UE behavior after A3 event is trigger.  Focus on intermediate KPIs for this exercise.  System level KPI is FFFs

Agreements on inputs/outputs and KPIs - For indirect measurement event prediction, the intermediate output (i.e., the output of RRM prediction model) is RSRP of serving/neighboring cells. The final output is the expected occurrence time of a certain measurement event (ex. event A3). - For direct measurement event prediction, the model output is the probability of event occurrence within a time window. - A3 event prediction should follow legacy rules (i.e. the “predicted” conditions have to persist for the duration of TTT). - As baseline, we will use RLF event prediction KPI: - for indirect: F1 score. the following can be reported: RSRP difference, missed event detection, false event detection. FFS how to define F1 score. - time difference of true time event reporting triggered and predicted time event reporting triggered, true event prediction. - for direct: F1 score. The following can be reported: missed event detection, false event detection, - Continue discussion over email discussion to see if there is a difference.

In this contribution, we provide simulation results for measurement event prediction.

Evaluation for measurement event prediction

Evaluation results are provided for	intra-frequency measurement reduction for FR1, Case B. Simulation assumptions follow TR 38.744 [1] (v0.0.5 in R2-2410186), with measurement event related simulation assumptions shown in Table 1 below.

Table 1: Simulation assumptions for temporal domain case B

| Parameters | Value |
| --- | --- |
| Direct / indirect | Indirect prediction |
| A3 event offset (dB) | 2 |
| TTT (ms) | 320 |
| UE speed (km/h) | 30, 60, 90 |
| OW length (ms) | 80 |
| PW length (ms) | 40 ms (sliding), 80 ms (sliding) |
| Max ETD (ms) | 80 |
| MRRT | 50% |

Simulation results are shown in Table 2 and Figure 1.

Table 2: Simulation results

| PW length (ms) | 40 |  |  | 80 |
| --- | --- | --- | --- | --- |
| UE speed (km / h) | 30 | 60 | 90 | 30 |
| Precision | 0.9335 | 0.9101 | 0.9052 | 0.8898 |
| Recall | 0.8947 | 0.8151 | 0.7938 | 0.8801 |
| F1 score | 0.9137 | 0.8600 | 0.8458 | 0.8849 |

Figure 1: Simulation results

From the simulation results, we have following observations, which are aligned with the observations for RRM prediction.

Observation 1: Longer PW length results in decreased prediction accuracy.

Observation 2: Higher UE speed results in decreased prediction accuracy.

Proposal 1: Capture the above observations into TR 38.744 for measurement event prediction temporal domain case B.

Conclusion

In this contribution, we provide simulation results for measurement event prediction, and have the following observations and proposal:

Observation 1: Longer PW length results in decreased prediction accuracy.

Observation 2: Higher UE speed results in decreased prediction accuracy.

Proposal 1: Capture the above observations into TR 38.744 for measurement event prediction temporal domain case B.

- References

[1] TR 38.744, "Study on Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR"