3GPP TSG RAN WG2 #129				R2-2500415

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.2.3

Source:		ASUSTeK

Title:				Unified solution for Ambient IoT random access type

Document for: Discussion and Decision

- Introduction

In the WID agreed in RAN#106 meeting [1], the following objectives on Ambient IoT RAN2 scope were set:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

And in current report on Ambient IoT study TR 38.769 [2], the description of Ambient IoT random access is provided:

- There are three candidate solutions being studied for the contention resolution, as below (it needs to be further discussed on the down selection and/or the unified design): - Solution 1: A-IoT Msg1 without data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. - Solution 2: A-IoT Msg1 with data - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends the A-IoT Msg1 including the upper layer data, which can be the device ID and/or any other upper layer data, in addition to one 16-bit random ID generated by the A-IoT device to the reader. - A-IoT Msg2: The reader may respond with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful. If the A-IoT Msg2 is not received by the device, the re-access is not autonomously performed i.e., the re-access is always controlled by the reader. - Solution 3: A-IoT Msg1 optionally includes data (unified solution supporting both Solution 1 and 2) - A-IoT Msg1: When the A-IoT device identifies the start of its own access occasion, it sends one 16-bit random ID generated by the A-IoT device to the reader. In addition, it is controlled by the reader on whether to also include the upper layer data, which can be the device ID and/or any other upper layer data. - A-IoT Msg2: The reader responds with the successfully received random ID. If the A-IoT device receives the A-IoT Msg2 including a random ID, which is the same as the previously transmitted one in A-IoT Msg1, it considers the contention resolution as successful.

In this contribution, we further discuss the unified solution for A-IoT random access type.

- Discussion

In TR 38.769 [2], there are two types of Ambient IoT random access, which are 2-step and 3-step. For 3-step RA, Msg1 includes a random ID and Msg2 echoes the random ID. The data (e.g. device ID) is contained in Msg3, if contention resolution succeeded in Msg2. And for 2-step RA, Msg1 includes a random ID and the data, and Msg2 echoes the random ID to complete the RA procedure.

Due to the limited device multiplexing capability, it is complicated for the device to transmit different formats of Msg1 specifically for different RA types. There has been discussion on whether to down-select between the two RA types in previous meetings. And a unified solution for 2-step RA and 3-step RA was agreed in the last meeting, captured as the solution 3 in the TR [2]. In WID, the 3-step RA is considered as the baseline, and support of the unified solution would be discussed in this meeting.

Since the only difference between the two types of RA in the message is whether to include the data in Msg1, we believe such unified solution is possible to achieve. For example, there are cases where there are fixed number of devices. The devices could perform 2-step RA to reduce signalling on Msg3 transmission and potential monitoring for the response. Therefore, the energy saving for the device is attained. On the other hand, the unified solution for supporting both 2-step and 3-step RA could bring flexibility on resources scheduling on the network. The network could schedule the different RA types based on different scenario. Moreover, under low congestion circumstances, lower signalling overhead and lower latency could be achieved with 2-step RA. As a result, we support to include Solution 3 in the Rel-19 work item.

Proposal 1:  A unified solution for 2-step RA and 3-step RA (i.e., Solution 3) is supported.

- Conclusion

Proposal 1:  A unified solution for 2-step RA and 3-step RA (i.e., Solution 3) is supported.

- Reference

[1] RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”

[2] TR 38.769, “Study on solutions for ambient IoT (Internet of Things)”