3GPP TSG-RAN WG2 Meeting #129	R2-2500417

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.3

Source: 	ASUSTeK

Title:  	Discussion on MR MAC CE for LTM

Document for:	Discussion and Decision

- Introduction

In the previous meeting RAN2#128, L1 event triggered report for LTM was discussed and has made several agreements including details of Measurement report (MR) MAC CE:

Agreements on L1 event-driven MR:

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

In this contribution, we discuss further details for L1 measurement report for LTM.

- Discussion

Contents of LTM MR MAC CE

Currently, it is agreed that SSBRI/CRI will be used to indicate candidate beams reported in LTM MR MAC CE, and a maximum N of total number of all beams included into LTM MR MAC CE. In LTM measurement reporting, since there can be more than one candidate configuration fulfilling LTM measurement report triggering event(s) at a time, it is proposed to confirm that a single LTM MR MAC CE can include measurement information of multiple candidate cells (e.g., event ID, SSBRI/CRI, or RSRP value).

Proposal 1:	A LTM MR MAC CE can include measurement information of multiple candidate cells fulfilling LTM measurement report triggering event(s).

Information prioritization of LTM MR MAC CE

During the previous meeting, it was agreed to support truncated LTM MR MAC CE. A main reason is that available UL grants may not always be able to include all information generated by the UE for all triggered measurement. To support truncated LTM MR MAC CE, when UL grant cannot accommodate all measurement report information (e.g., cannot accommodate all N beams or cannot accommodate information of all candidate cells), the UE will need to decide which information to be included first. In our view, since the LTM MR MAC CE indicates information for mobility execution (e.g., for the NW to decide whether to perform Cell switch or early UL synchronization), it would be beneficial for the UE to prioritize including information of a candidate cell with higher quality beam so that the network can select the best candidate cell to perform LTM Cell switch/early sync for the UE. Also, information of the best beam could assist the NW to select the beam for RACH-less LTM.

Proposal 2:	When a UL grant cannot accommodate a LTM MR MAC CE plus its subheader, the UE decides which information to be included in a truncated LTM MR MAC CE by:

- prioritizing including information of candidate cells with higher quality beam, and

- prioritizing including information of beams with higher quality.

- Conclusion

We have the following proposals for LTM-related RA reports:

Proposal 1:	A LTM MR MAC CE can include measurement information of multiple candidate cells fulfilling LTM measurement report triggering event(s).

Proposal 2:	When a UL grant cannot accommodate a LTM MR MAC CE plus its subheader, the UE decides which information to be included in a truncated LTM MR MAC CE by:

- prioritizing including information of candidate cells with higher quality beam, and

- prioritizing including information of beams with higher quality.

- Reference