3GPP TSG-RAN WG2 Meeting #129	R2-2500477

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.6.4

Source: 	Fujitsu

Title:	Discussion on conditional Intra-CU LTM

Document for:	Discussion and Decision

- Introduction

At previous RAN2 meetings, some agreements on conditional Intra-CU LTM are reached as follows [1][2]:

Agreements on C-LTM

- Source cell sends the conditional LTM configuration via RRCReconfiguration to UE, which includes the LTM candidate configurations, and the corresponding execution conditions.

- Event LTM3-like and LTM5-like are used as the conditional LTM execution condition. FFS on reuse of CHO conditions.

- Source cell and each candidate cell provides its own execution condition for conditional LTM.

- It is DU to generate the L1 execution condition. FFS on a case that L3 measurement is used.

- RACH-less Conditional intra-CU LTM is supported.

- RACH based conditional intra-CU LTM is supported.

- UE based TA measurement mechanism is supported for conditional intra-CU LTM.

- PDCCH ordered early TA acquisition is supported for conditional LTM.

- Rel-18 Early candidate TCI State activation/deactivation is supported for conditional intra-CU LTM.

- For RACH-less conditional LTM, CG-based first UL transmission on target cell is supported. FFS on DG-based approach.

- The LTM completion defined for Rel-18 intra-CU LTM is reused for conditional LTM.

Agreements on C-LTM:

- The triggering condition of conditional LTM can be based on L3 measurement.

- CondEventA3 and CondEventA5 conditions can be baseline for the conditional LTM execution.

- The L1 execution condition of a candidate cell is associated to only one triggering event.

- For L3 execution condition, it may consist of one or two triggering condition(s). If there are two triggering conditions associated with the same candidate cell, the UE shall consider the execution condition is fulfilled only when both triggering conditions are met. Only single RS type is supported and at most two different trigger quantities can be configured simultaneously for the evaluation of execution condition of a single candidate cell.

- To support initial and subsequent conditional LTM, the following items can be considered for the configuration of execution condition:

- The CLTM configuration of each candidate cell shall include the execution condition for initial conditional LTM, which is generated by the initial source cell to trigger the CLTM for the candidate cell.

- The CLTM configuration of each candidate cell may include execution conditions for subsequent conditional LTM, which is generated by the candidate cell to trigger the CLTM for other candidate cells when the candidate cell becomes a serving cell.

- The network can configure measurement reports e.g., L1 periodic, semi-persistent, aperiodic and event triggered report, or L3 measurement reports for conditional LTM, e.g., to trigger PDCCH ordered early RACH.

- For CLTM, the Candidate Cell TCI States Activation/Deactivation MAC CE is re-used for the early activation/deactivation of TCI state(s) of a CLTM candidate configuration.

- The Early TA is signalled to the UE from the source cell (i.e., not from the candidate cell directly to the UE). This agreement will be included in the LS to RAN1/3/4.

- The network can inform the candidate cell’s TA information to UE via new MAC CE, which is the TA value when UE switches to that candidate cell during CLTM.

- Candidate cell TA is maintained by a new timer.

- For L1-based conditional LTM the condition evaluation is at MAC level and for L3-based conditional LTM the condition evaluation is at RRC level.

This document further discusses basic function of conditional LTM for non-DC case, considering the following aspects:

- beam(s) to be used for C-LTM event evaluation

- early TA MAC CE

- UE behaviours to the new timer

- beam selection in RACH-less C-LTM with CG

- whether to support RACH-less C-LTM with DG

- whether LTM and C-LTM co-exist

- Discussions

C-LTM event evaluation

Regarding L1 measurement events, RAN2 has agreed that LTM event 3/5-like conditions can be used for triggering the LTM cell switch execution. While for L1 measurement reporting, beam level measurement is used for the event evaluation of the LTM event 3/5. To avoid too frequent measurement reporting, TTT is applied based on network configuration for both entering and leaving condition of event-triggered L1 measurement reporting. For conditional Intra-CU LTM, a similar mechanism can be considered to avoid Ping-Pong for LTM cell switch. The network can configure TTT for LTM event 3/5-like conditions. Alternatively, multiple beams or cell level measurement will be used for the event evaluation to trigger the LTM cell switch execution. For example, the number, e.g. X of beams can be configured by the network and the best X beams will be used to evaluate the LTM event 3/5-like conditions. Whether multiple beams or cell level measurement is used will be controlled by the network.

Proposal 1: For conditional Intra-CU LTM, the network controls whether TTT, multiple beams or cell level measurement is used for LTM event 3/5-like conditions to trigger LTM execution.

Early synchronization of conditional Intra-CU LTM

RAN2 has agreed that early RACH towards candidate cell for TA acquisition can be supported. The network can configure L1 or L3 measurement reports for the UE and then trigger the PDCCH ordered early RACH based on the measurement reports. Based on Rel-18 intra-CU LTM mechanism, the source DU can know the TA information and the candidate cell to which the TA information belongs. With this, the UE can inform the candidate cell’s TA information to UE via new MAC CE. Therefore, it is required and the source DU is able to include the TA information and candidate cell information in the early TAC MAC CE. The candidate cell can be identified by the Target Configuration ID. Considering two TAs in MIMO, TAG ID is optionally included.

One issue for the content of the early TAC MAC CE is the TA information. In legacy, TA information is provided by Timing Advance Command field in (Absolute) TAC MAC CE, RAR, MSGB or LTM cell switch command MAC CE. Usually, the TA command field indicates the index value TA used to control the amount of timing adjustment with size of 12 bits or 6 bits. While in the LTM CSC MAC CE, the TA command field indicates whether the TA is valid or not for the LTM target cell. If the TA is valid, then the UE applies the provided TA value. Otherwise, the UE will apply the UE measured TA or perform RACH based LTM cell switch procedure. For conditional Intra-CU LTM, both RACH based and RACH-less LTM cell switch procedure are supported. Also, PDCCH ordered RACH procedure or UE based TA measurement mechanism can be used for early TA acquisition. However, whether the TA is valid can be determined by a new timer for the UE. So, it seems that the TA information, i.e. the amount of timing adjustment is provided by the early TAC MAC CE and there is no need to determine whether the TA is valid or not by the early TAC MAC CE.

In summary, the Target Configuration ID and TAC indicating the amount of timing adjustment will included in the early TAC MAC CE. The early TAC MAC CE contains TAG ID associated to the TAC in case of multiple DCI multiple TRP operation with two TAs.

Proposal 2: Early TAC MAC CE contains Timing Advance Command indicating the amount of timing adjustment as in legacy TAC MAC CE, Target Configuration ID and optional TAG ID.

In Rel-18, TA = 0 or same TA value with current TAG can be supported by including corresponding TA value in the LTM cell switch command. For conditional Intra-CU LTM, no LTM cell switch MAC CE will be used and the early TAC MAC CE provides the TA information for candidate cell. So, TA = 0 or same TA value with current TAG can be supported by including corresponding TA value in the early TAC MAC CE.

Proposal 2bis: For conditional intra-CU LTM, TA = 0 or same TA with current TAG will be supported by indicating corresponding TA value in the early TAC MAC CE.

As agreed, candidate cell TA is maintained by a new timer. The timer is maintained per LTM candidate cell, i.e. the timer associated to the LTM candidate cell should be started or restarted when the early TAC MAC CE indicating TA information and corresponding LTM candidate ID is received.

Proposal 3: A timer for an LTM candidate is (re)started when an early TAC MAC CE is received.

Execution and completion of conditional Intra-CU LTM

Conditional Intra-CU LTM cell switch execution can be performed with or without RACH. In Rel-18, intra-CU LTM cell switch execution without RACH is performed if the UE has valid TA. We think that the same principle can be reused for the determination in C-LTM. To determine if the UE has valid TA, a timer is used for the LTM candidate. If Proposal 3 is agreeable, the UE considers that it has valid TA if the timer is running. In other words, the UE considers that it has no valid TA if the timer is not running, e.g. not configured, not started or expired. In this case, based on the network configuration, the UE will perform UE based TA measurement or RACH-based LTM cell switch, as legacy.

Proposal 4: Conditional RACH-less LTM cell switch is performed if TA timer for the LTM candidate is running; Conditional RACH-based LTM cell switch or UE based TA measurement is performed if TA timer for the LTM candidate is not running.

For RACH-based conditional LTM, as legacy, TX/RX beam can be selected by the UE during RACH according to corresponding L1 RSRP even if there is no LTM cell switch command indicating the TCI state(s) from the network. The selected beams can be used during the LTM cell switch and after the cell switch until a new TCI state is activated/indicated by the network.

In case of conditional RACH-less LTM, how to determine TX/RX beam needs more discussions. RAN2 has agreed that Rel-18 TCI state activation/deactivation for candidate cells, i.e. the Candidate Cell TCI States Activation/Deactivation MAC CE can be used for conditional Intra-CU LTM to activate/deactivate TCI state(s) of a CLTM candidate configuration. Based on this, one question is whether and how to determine the beam to be used during and after the LTM execution. At least, the beam should satisfy the execution condition. If there are multiple beams satisfying the condition, it depends on the UE implementation to select one from these beams, e.g. best activated beam for the target cell. Besides, considering the first UL transmission in case of RACH-less LTM, the beam should be associated to configured grant configuration if dynamic grant cannot be used. Beam selection may have impacts to the lower layer behaviours and thus RAN1 needs to be involved.

Proposal 5: For conditional Intra-CU RACH-less LTM, it is up to UE to select a beam from beams satisfying the execution condition and associated with CG, if DG-based approach is not supported. RAN1 confirmation is needed.

In Rel-18, in case of RACH-less LTM, the gNB considers the LTM completed when the first UL transmission is received, and the UE considers the LTM completed when the UE confirms that the first UL transmission is received at the gNB. RAN2 has agreed that the same mechanism is reused for conditional Intra-CU LTM.

The first UL transmission can be dynamically scheduled by the gNB in Rel-18 LTM. In case of intra-DU C-LTM, DG-based approach can be used. However, there are some concerns for conditional Intra-CU inter-DU LTM. Because the target cell or the network cannot know when the UE will perform the LTM cell switch and will not send PDCCH to schedule UL resources for the first UL data. Some enhancements are needed to support DG-based approach. For example, candidate cell needs to determine whether or when to schedule the UE and which beams should be used for the UL transmission. We don’t see significant benefit to support dynamic scheduling of first UL transmission considering CG-based approach can be used for the transmission. So, no DG-based approach is preferred.

Proposal 6: For conditional Intra-CU RACH-less LTM, we will not support DG-based approach for the first UL transmission at target cell.

Co-existence between LTM and C-LTM

For L3 handover, conditional handover is introduced to reduce the RLF during traditional HO. The coexistence between network triggered L3 HO and CHO is supported. With same motivation, the coexistence between LTM and conditional LTM should be supported. It means there is no restriction for network to send the LTM cell switch command to the UE even if the C-LTM is configured.

Proposal 7: The network can send LTM cell switch command to the UE even if conditional Intra-CU LTM is configured.

- Conclusion

This document discussed basic function of conditional LTM for non-DC case and made the following proposals:

C-LTM event evaluation

Proposal 1: For conditional Intra-CU LTM, the network controls whether TTT, multiple beams or cell level measurement is used for LTM event 3/5-like conditions to trigger LTM execution.

Early synchronization of conditional Intra-CU LTM

Proposal 2: Early TAC MAC CE contains Timing Advance Command indicating the amount of timing adjustment as in legacy TAC MAC CE, Target Configuration ID and optional TAG ID.

Proposal 2bis: For conditional intra-CU LTM, TA = 0 or same TA with current TAG will be supported by indicating corresponding TA value in the early TAC MAC CE.

Proposal 3: A timer for an LTM candidate is (re)started when an early TAC MAC CE is received.

Execution and completion of conditional Intra-CU LTM

Proposal 4: Conditional RACH-less LTM cell switch is performed if TA timer for the LTM candidate is running; Conditional RACH-based LTM cell switch or UE based TA measurement is performed if TA timer for the LTM candidate is not running.

Proposal 5: For conditional Intra-CU RACH-less LTM, it is up to UE to select a beam from beams satisfying the execution condition and associated with CG, if DG-based approach is not supported. RAN1 confirmation is needed.

Proposal 6: For conditional Intra-CU RACH-less LTM, we will not support DG-based approach for the first UL transmission at target cell.

Co-existence between LTM and C-LTM

Proposal 7: The network can send LTM cell switch command to the UE even if conditional Intra-CU LTM is configured.

# Reference

- R2-2407571, “Report from session on V2X/SL, R18/19 MOB, and R19 NES”, Vice Chairman (Samsung), RAN2#127bis

- R2-2410911, “Report from session on R16/17/18 V2X/SL, R18/19 MOB and R19 NES”, Vice Chairman (Samsung), RAN2#128