3GPP TSG-RAN WG2 #129 	R2-2500499

Athens, Greece, 17th – 21st February 2025

Agenda Item:	8.2.4

Source:	KT Corp.

Title:	Discussion on A-IoT data segmentation and transmission

Document for:	Discussion/Decision

WID/SID:	Ambient_IoT_solutions

1	Introduction

Ambient IoT SI (Study Item) was concluded at the last meeting, and a new WID was approved [1]. And the new RAN2 objectives for Ambient IoT are as follows:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs. Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes. - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129). - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R. - Only MAC layer is included

As in the scope of the new WID, it was agreed to support functionality of A-IoT data transmission and D2R segmentation. In this contribution, we will first look at the agreements regarding the segmentation which was made at the previous meeting. Next, we would like to discuss how to segment the A-IoT data and how to transmit each segment.

2	Discussion

2.1	Agreements on segmentation

In RAN2#127bis, RAN2#128 meeting, the following agreements were made related to segmentation in functionality aspects AI 8.2.2. As below, RAN2 assumes that A-IoT only supports up to MAC layer including PHY layer. If the segmentation is supported, an indication of whether the segment is the last segment or not can be included in each segment by using one- or two-bit indicator. However, it was agreed that the segmentation indicator would be covered in detail in the WI phase.

RAN2#127bis Agreements on segmentations 1 RAN2 will study segmentation and RAN2 will need to give some guidance once there is more clarity on solution, on the minimum TB size. 2 Study the functionality of segmentation in the MAC. We focus on D2R and can consider the support for R2D later depending on SA2 and RAN1. The solution should aim to be applicable to both direction (i.e. unified solution), if we decide to support on both directions. 3 Do not support sequence number for A-IoT segmentation functionalities. 4 Do not support segment number and the number of segments for A-IoT segmentation functionalities. 5 Capture the following option in TR: An indication is used to indicate to reader whether the data is segmented and whether it is last segment. FFS the details on how this is designed and number of bits (one or two). 6 RAN2 assumes that for the device we will not support AS layer buffering for A-IoT segmentation functionalities, i.e., all buffered segment(s) are stored in upper layer(s). RAN2#128 Agreements 1 RAN2 assumes that there is no other L2 AS layer (i.e. A-IoT MAC layer only). There is no CP/UP protocol stack differentiation on AIoT interfaces. 2 Capture the option that it is up to the reader whether to use the random ID as AS ID or assigns a new AS ID. FFS what message is used 3 Capture in the TR that to down-select we need to consider which message need to consider whether we need the reader to handle the collision. 4 Segmentation indication will be discussed in WI phase. 5 If segmentation is supported, it is beneficial for the reader to be able to trigger a re-transmission of a segment. FFS how this is done.

Below are the current agreements regarding the expected D2R message size. RAN2 assumes that it will be beneficial for the reader to know an expected D2R message size. How the reader will get the expected D2R message size was not yet decided but will be determined through further discussion. In our view, the expected D2R message size information is very useful to reader when allocating resource(s) for D2R segmentation and transmission.

RAN2#127bis Agreements From RAN2 perspective, it is beneficial for the reader to know an estimate of expected D2R message size. Two options can be captured: 1) from the CN and 2) from the device (simple message size indication). Capture the advantages/disadvantages. The D2R message size would be beneficial but it is not essential. Ask SA2 if it is possible to provide the expected (e.g. approximate, estimate, exact, max) future response D2R message size. Is it always available, sometimes, or never. RAN2#128 Agreements on visibility 1. The reader needs to know whether a response is expected and expected D2R message size. FFS how the reader gets this information. Wait for SA2 LS response on expected D2R message size to resolve the FFS. FFS if command type is explicit, if it can be inferred from expected D2R message size if available. 2. Ask SA2 if they can provide information that RAN2 agreed on some information that are useful for the reader and ask whether they can be provided (e.g. service type). Ask SA2 if there are requirements from SA2 perspective (e.g. on latency of completion time of procedure)

2.2	Segmentation based transmission

The segmentation is required when the A-IoT data size is larger than the amount of D2R resources (e.g., TBS) allocated from reader. That is, segmentation is necessary when the device cannot transmit whole A-IoT data at once through the D2R resources allocated from the reader. Considering the discussion so far and the segmentation specifications in NR, we can consider the D2R segments transmission procedure as shown in the figure 1 below.

The figure 1 shows the procedure in which the reader allocates dedicated D2R resources to the device via UL grant when transmitting each D2R segment. Since the reader does not know the whole A-IoT data (D2R) size, it can allocate dedicated D2R resources of arbitrary size or configured size to the device. In other words, the reader cannot allocate the exact size of the required D2R resources. When the device transmits D2R segment via the dedicated D2R resource, if there is an additional A-IoT data to be transmitted, device can set the segment indication (segment field) to 0 bits. Segment field is a segment indication that indicates whether the segment is the last or not. The reader can allocate additional D2R resources to the device via segment indication.

Figure 1. Simple segmentation procedure

However, the above procedure requires that the device should receive dedicated D2R resources for every D2R segment. If the A-IoT data size is large and the number of segments increases, the number of PRDCH transmissions containing dedicated D2R resources increases. It can eventually cause latency in A-IoT data transmission.

If the A-IoT data size is large and the number of segments increases, the number of PRDCH transmissions containing dedicated D2R resources increases, and it can cause latency.

To efficiently transmit A-IoT data while reducing the above latency issue, the expected D2R message size mentioned above can be considered. According to the above agreements, the reader can receive the expected D2R message size information from the core network or the device. Based on the expected D2R message sized, reader can allocate dedicated D2R resources to the device at once via PRDCH.

Figure 2 shows the procedure in which the reader provides multiple D2R resources corresponding to multiple segments based on the expected D2R message size information. Then, device can transmit each of segments via received D2R resource information. The procedure in Figure 2, compared to the procedure in Figure 1, allows the reader to provide the device with D2R resources to transmit the entire A-IoT data through one PRDCH. Therefore, the device can transmit multiple segments at once, thereby reducing the latency of A-IoT data transmission. So, we would like to discuss the procedure in which the reader provides multiple D2R resources corresponding to the multiple segments based on the expected D2R message size.

RAN2 to discuss the procedure in which the reader provides multiple D2R resources corresponding to the multiple segments based on the expected D2R message size.

Figure 2. Segmentation procedure based on resource allocation considering expected D2R msg size

For the procedure in Figure 2, when one or more segments fail to be transmitted, we need to additionally discuss the retransmission method to resolve the problem. In addition, we also need to discuss how to resolve the case where the last segment transmission fails.

RAN2 to discuss the solution for the case where the last segment transmission fails.

3	Conclusion

In the previous sections we made the following observations:

Observation 1	If the A-IoT data size is large and the number of segments increases, the number of PRDCH transmissions containing dedicated D2R resources increases, and it can cause latency.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the procedure in which the reader provides multiple D2R resources corresponding to the multiple segments based on the expected D2R message size.

Proposal 2	RAN2 to discuss the solution for the case where the last segment transmission fails.

4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, Madrid, Spain, December 9-12, 2024.

RP-242965, “Status report for Study on solutions for Ambient IoT (Internet of Things) in NR”, CMCC, RAN #106, Madrid, Spain, December 9-12, 2024.

RAN2#127-bis chairman’s notes.

RAN2#128 chairman’s notes.

TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR” V19.0.0 (2024-12).