3GPP TSG-RAN WG2 Meeting #129	R2-2500525

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.9.2

Source: 	CATT

Title: 	Discussion on RAN2 impacts for the support of S&F operation

Document for: 	Discussion and Decision

Introduction

This contribution discusses possible impacts on RAN2 due to support of S&F operation, including the transition time, new release cause and MME configured satellite list.

Discussion

2.1	 On transition time

In RAN2#128 [1], RAN2 agreed to introduce time information to indicate mode transition from S&F mode to normal mode. There are several FFSs on how to define and use this time information.

Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start)

In RAN2#127bis [2], it was agreed that the an NTN cell operating in S&F mode should broadcast the S&F operation indication and the UEs not supporting S&F are not allowed to access the cell.

4.	If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

5.	When present, the “S&F operation” indication has two possible settings:

‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

As a result, when a cell indicates that its current satellite operation mode will transit from "S&F operation" to "normal mode" and the corresponding transition time, only the UE supporting S&F and operating in S&F is able to receive this transition time information. Also, per the agreement above, an NTN cell currently operating in normal mode will not broadcast the transition time from "S&F operation" to normal mode. As a result, the Rel-19 UE not supporting S&F mode and camping on the normal cell will not receive this transition time information. In other words, the mode transition time only applied for UE supporting S&F mode and we do not need to discuss the behaviour of UE not supporting S&F mode on this time info.

Proposal 1: RAN2 confirms that the agreed transition time from current "S&F operation mode" to "normal mode" only applies to UE supporting and operating S&F mode, but does not to a UE not supporting S&F mode.

In the agreement, it says that this transition time information may be applied to delay some NAS procedure for UE operating in S&F mode. There were also some contributions proposing to use this transition time information to determine whether to initiate the service immediately or postpone the initiation a bit. Irrespective of how it is used, i.e. impacting NAS procedure or service initiation, this transition time information is used to impact the NAS layer/upper layer behaviour of the UE. The only impact on AS layer is to forward the time information to upper layer when received. This is proposed as follows.

Proposal 2: UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (when received) to the upper layers. Whether/how it is used by upper layers is up to CT1/SA2. No other AS impact is expected.

For simplicity, the transition time can be in the format of absolute time. As to which SIB is used to contain the time information, we can postpone the discussion a bit in stage 3 stage, with consideration on the signalling impacts due to other possible enhancements as proposed in other sections.

Proposal 3: The transition time is in the format of absolute time.

There were also proposals to introduce time information to indicate when the current satellite operation mode will transit from normal operation to S&F operation. At present, we do not identify the need for the transition time in this direction. The RRC_CONNECTED UE is under NW control, and can be handed over to another cell or be released to RRC_IDLE, in case this transition happens. The RRC_IDLE UE which is camping on the cell operating in normal mode (no matter whether it support S&F mode or not) can start perform measurements for cell reselection before t-Service, and in this case it seems better for the UE to reselect to a cell in normal mode (if any). So we fail to find any benefit/necessity to introduce such transition time information from normal mode to S&F mode, and tend to think an additional transition time is not needed.

Proposal 4: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

2.2 On new release cause

Another FFS is whether a new release cause is needed when the NW releases UE to RRC_IDLE.

Proposal 8 When the feeder-link is not available, eNB can suspend UE’s RRC and NAS by transmitting an RRC Connection Release message to the UE with full S&F support. FFS whether a new release cause is needed. - ZTE is fine with this and thinks we could have a new case - Ericsson wonders what is meant by suspend RRC - vivo thinks there is no need to suspend RRC - CATT agrees on this under the assumption that we are not introducing a new RRC state. - Samsung thinks this is not needed It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable

When the feeder link becomes unavailable, the satellite transits from normal mode to S&F mode. Under this situation, it is up to the NW to hand over the RRC_CONNECTED UE to another cell or release the UE to RRC_IDLE. If the NW chooses to release the UE to RRC_IDLE, the cell (re-)selection procedure can be triggered as in legacy for the UE to choose one cell to camp on. Since the UE is aware of the current operating mode of the selected cell, its NAS can determine how to trigger the following NAS procedure and behaves properly. Hence the UE does not need to know the specific release cause that feeder-link becomes unavailable.

Proposal 5: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC IDLE because the feeder link becomes unavailable.

For the case that service link of an S&F mode cell becomes unavailable, when the NW releases the UE to RRC_IDLE, the UE operating in S&F may need to hold the NAS procedure and context. For example, under the architecture of split MME, the UE needs to hold the process status during the attach procedure, e.g., wait timer and assistance info of satellite ID list. However, such UE NAS behaviour has already been designed by SA2 with necessary indication from MME (as shown below), so a new RRC release cause is not needed. The NW can release the RRC connection of UE with RRC release cause ‘other’. After the UE AS forwards the RRC release cause ‘other’ to NAS layer, it is up to NAS layer to keep the NAS procedure and context.

2) When UE initiates Attach or TAU procedure, it indicates support for S&F mode to the MME following existing NAS capability, the MME sends Attach or TAU Reject message to the UE if these procedures cannot be completed due to S&F operation. The Attach or TAU Reject message includes: a) A new information indicating the UE that attach or TAU procedure cannot be completed because of the S&F operation and that the UE can re-attempt the attach or TAU in this PLMN in a next satellite pass. This indicates to the UE that the information contained in the Attach or TAU Request message is stored by the MME and the network will be available to the UE after interaction with ground network. b) Wait timer: Indicates to the UE the time it should wait before re-attempting the Attach/TAU procedure in the current or another satellite of the same PLMN. c) Optionally, The list of Satellite IDs over which the UE may re-attempt the Attach/TAU procedure, after wait timer expires. The Satellite IDs are based on the SIB information broadcasted by eNB.

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

2.3	 On MME configured satellite List

SA2 discussed the LS in [3] RAN2 sent last meeting during the AH meeting in January, and agreed the CR in [4]. However, no LS reply from SA2 was received till now, because SA2 has not completed all the necessary discussions with especially the following editor's notes left. This postpones the reply LS.

Editor’s Note: It is for further consideration whether specific UE behaviour or clarification needs to be added to prevent the UE from accessing satellites where the NW is not expected to keep a UE context.

Considering that SA2 is going to discuss and conclude the remaining part in February meeting and the LS response is also expected to be received at that time, we propose to wait for SA2 LS response before the resumption on further RAN2 discussion on this MME configured satellite List aspect.

Proposal 7: Wait for SA2 LS response before further RAN2 discussion related to MME configured satellite List.

Our perspective on the needed AS aspects due to this MME configured satellite list can be found from [5].

Conclusion

In this contribution, some leftover issues on support of S&F operation are discussed with corresponding proposals listed as follows:

On transition time

Proposal 1: RAN2 confirms that the agreed transition time from current "S&F operation mode" to "normal mode" only applies to UE supporting and operating S&F mode, but does not to a UE not supporting S&F mode.

Proposal 2: UE AS indicates the information on transition time from current "S&F operation mode" to "normal mode" (when received) to the upper layers. Whether/how it is used by upper layers is up to CT1/SA2. No other AS impact is expected.

Proposal 3: The transition time is in the format of absolute time.

Proposal 4: Do not introduce transition time to indicate when the current satellite operation mode will transit from normal operation to S&F operation.

On new release cause

Proposal 5: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC IDLE because the feeder link becomes unavailable.

Proposal 6: Do not introduce a new release cause when the NW wants to release a R19 UE to RRC IDLE because the service link becomes unavailable when the cell is operating in S&F mode.

On MME configured satellite List

Proposal 7: Wait for SA2 LS response before further RAN2 discussion related to MME configured satellite List.

Reference

- RAN2#128 ChairNotes.

- RAN2#127bis ChairNotes.

- R2-2411197 LS on Satellite IDs for store-and-forward operation.

- S2-2501339 Support of S&F Monitoring List in S&F Mode.

- R2-2409674	Discussion on RAN2 impacts due to the satellite ID list from MME in S&F operation	CATT