3GPP TSG-RAN WG2 Meeting #129	 R2-2500527

Athens, Greece, Feb. 17th – 21st, 2025

Agenda Item:	8.17

Source: 	CATT

Title: 	Discussion on support of NB-IoT NTN TDD

Document for: 	Discussion and Decision

Introduction

In WID [1], RAN2 objectives are shown as follows:

This work item includes the following objectives: - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

This contribution analyzes potential RAN2 impacts to support NB-IoT TDD NTN on “other necessary impacts on higher layer” from CP and UP perspectives.

Discussion

  - Preliminaries

Based on the latest WID [1], the following assumption should be taken into consideration:

The work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes (U) and set of usable contiguous DL subframes (D), and guard periods, which is periodic every N radio frames, with N=9 for the target MSS allocated band.

In RAN1#119 meeting, the following agreements were made for IoT NTN TDD mode. It can be seen that only one of the downlink/uplink slots in TDD frame structure across the 90 ms period can be used, and the uplink slot and downlink slot in the TDD frame structure have the same index.

Agreement Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraints.

Besides, according to the WID, the periodic pattern which consists of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods is provided every N radio frames, with N=9 as baseline where D=U=8 with fixed guard period [1].

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

Per agreements in RAN1 and RAN, it can be seen that the timing of downlink and uplink in IoT NTN TDD is quite different from legacy frame structure. Hence, the UE has to wait for the “right” downlink or uplink occasion based on the new TDD frame structure to perform corresponding reception and transmission.

Observation 1: The periodicity for downlink and uplink transition of IoT-NTN TDD mode is 90 ms which may require new frame structure and result in new timing design in uplink transmission and downlink reception.

In the following sections, the discussion is based on the periodic pattern with the period of N=9 radio frame (i.e. 90 ms).

Potential impacts on CP

The periodicity of the TDD frame is 90 ms; however the periodicities of MIB, SIB1, other system information and paging are now times of 2 radio frames in general (e.g. 16 RF, 32 RF, etc.), so the mismatch between the IoT NTN TDD configuration and the periodicity of MIB/SI/SIB1/Paging may bring some issues.

- MIB

Currently, the periodicity of MIB-TDD-NB is 640 ms, and repetition is made within 640 ms in the fixed subframe, as follows [2]:

The MasterInformationBlock-TDD-NB (MIB-TDD-NB) uses a fixed schedule with a periodicity of 640 ms and repetitions made within 640 ms. The first transmission of the MIB-TDD-NB is scheduled in subframe #9 of radio frames for which the SFN mod 64 = 0 and repetitions are scheduled in subframe #9 of all other radio frames. The transmissions are arranged in 8 independently decodable blocks of 80 ms duration.

Obviously, within one period, the MIB should be broadcast in subframe #9 every radio frame within 8 radio frames (i.e. 80 ms). However, in the 90 ms IoT NTN TDD frame structure, the DL channel is only kept for totally 8 ms as per above RAN1 agreement. As a result, the subframes, which are determined by the current Spec to broadcast MIB with above periodicity and repetition pattern, are not ensured to be the DL channel with this 90 ms TDD configuration. That is, it is unclear how the 90 ms TDD configuration can support the MIB-TDD-NB transmission mechanism in the current Spec.

- SIB1

For TDD, SIB1-NB transmission on the anchor carrier occurs in either subframe #0 or subframe #4 of every other frame in 16 continuous frames and SIB1-NB transmission on a non-anchor carrier occurs in subframe #0 and next in subframe #5 of every other frame in 16 continuous frames. The starting frame for the first transmission of the SIB1-NB is derived from the cell PCID and the number of repetitions within the 2560 ms period and repetitions are made, equally spaced, within the 2560 ms period (see TS 36.213 [23]). TBS for SystemInformationBlockType1-NB, the repetitions made within the 2560 ms, and the subframe index (#0 or #4) are indicated by schedulingInfoSIB1 field in the MIB-TDD-NB

The SIB1 is broadcast with fixed period (2560 ms) and repetition with fixed subframe (e.g. either subframe #0 or subframe #4 of every other frame in 16 continuous frames). Similar as MIB, it cannot be ensured that the subframes needed for SIB1-NB transmission as per the current Spec are really the DL channel with this 90 ms TDD configuration. Thus, it is also unclear how the 90 ms TDD configuration can support the SIB1-NB transmission mechanism in the current Spec.

- Other SI

SI is broadcast and repeated within SI window with the configured periodic and repetition pattern. In the current Spec, the length of SI window can be configured with the following values [2]:

si-WindowLength-r13					ENUMERATED {ms160, ms320, ms480, ms640,

ms960, ms1280, ms1600, spare1},

And the periodicity and the repetition pattern can be configured with following values:

si-Periodicity-r13				ENUMERATED {rf64, rf128, rf256, rf512,

rf1024, rf2048, rf4096, spare},

si-RepetitionPattern-r13		ENUMERATED {every2ndRF, every4thRF, every8thRF, every16thRF},

Within the SI-window, the corresponding SI message can be transmitted several times over 2 or 8 consecutive NB-IoT downlink subframes depending on TBS.

For example, the SI is broadcast within 8 consecutive NB-IoT downlink subframes every 2 frame. Based on the 90 ms TDD configuration, it is also difficult to ensure that the potential subframe and frame used to transmit SI as determined by the current Spec are really DL channel.

- Paging

The UE monitoring paging message is based on the configuration of paging cycle, nB-r13 and repetition [2]:

PCCH-Config-NB-r13 ::=					SEQUENCE {

defaultPagingCycle-r13					ENUMERATED {rf128, rf256, rf512, rf1024},

nB-r13									ENUMERATED {

fourT, twoT, oneT, halfT, quarterT, one8thT,

one16thT, one32ndT, one64thT,

one128thT, one256thT, one512thT, one1024thT,

spare3, spare2, spare1},

npdcch-NumRepetitionPaging-r13			ENUMERATED {

r1, r2, r4, r8, r16, r32, r64, r128,

r256, r512, r1024, r2048,

spare4, spare3, spare2, spare1}

}

Based on the current spec, the density of the PO ranges from every frame to every 1024 radio frames. Similar as above discussion, the 90 ms TDD configuration may not be able to support the POs determined by the current Spec.

Observation 2: IoT NTN TDD frame structure with 90 ms periodicity will bring the issue that it cannot ensure the frames/subframes, which should be used to transmit SIB1/MIB/SI/PO per the current Spec, are really DL channel, and thus cannot support the MIB/SIB1/SI/paging transmission mechanism in the current Spec.

One possible solution is that current time pattern on the periodicity and repetition for above functionalities are maintained, but specify that if a subframe which should be used to transmit MIB/SIB1/SI/Paging is not DL channel, the UE will ignore this subframe for corresponding MIB/SIB1/SI/Paging reception. However, considering that RAN1also has discussion to introduce new time domain pattern for MIB/SIB1/SI/paging to match the 90 ms TDD configuration, RAN2 can wait for further RAN1 progress. If RAN1 does not introduce new time domain pattern for these functionalities, RAN2 can discuss potential solutions.

Proposal 1: RAN2 waits for further RAN1 progress before carrying out the discussion on how to adapt the MIB/SIB1/SI/paging procedure to the 90 ms IoT NTN TDD frame structure.

Potential impacts on UP

### 2.2.1	Potential issues on related timer/window handling in MAC

A potential issue we found is that the MAC windows/timers related to the reception of NW response may not adapt to the 90 ms IoT NTN TDD frame structure as agreed by RAN1. In particular, considering the agreements by RAN1, the related timer/window length may cover too limited DL occasions for the UE to receive NW response, so that the UE may end up with inefficient/unexpected handling upon the timer/window expiry. As some examples: for RAR window and contention resolution timer, if the NW has not enough DL transmission occasions to send the responses (with only one DL slot having to send all DL signals in the 90ms periodicity), it is likely that some UEs end up with RACH failure per the current Spec and enter exceptional handling; for DRX, the UE may inefficiently start the on duration timer and wake up again and again, but end up with receiving nothing in most of the DRX cycles; for retxBSR-Timer, if the UE cannot receive DL scheduling timely, it may end up with the timer expiry and unexpected BSR triggered again, even if the NW has already planned to schedule the UE in future 90 ms frame based on the previous BSR already received from the UE.

Although in the current Spec some timers are already with a configurable values that can cover several 90 ms periodicities of the IoT NTN TDD radio frame, it is still uncertain whether these existing values are already sufficient to overcome such unexpected/efficient handling due to the limited availability of DL occasions (esp. considering that those existing values were absolutely not introduced due to the reasons of this 90 ms periodicity). Therefore, RAN2 may need to discuss whether/how to enhance the related MAC timers/windows to adapt to this 90 ms IoT NTN radio frame periodicity, e.g. introducing longer timer/window length, consider the length in the number of DL slot numbers, etc..

Proposal 2: RAN2 discusses whether/how to enhance related MAC timers/windows to adapt to the 90 ms IoT NTN TDD radio frame structure (e.g. RAR window, CR timer, retx BSR timer, DRX timer, etc.)

### 2.2.2	Potential issues on periodical operations in MAC

For NB-IoT, UL Semi-Persistent Scheduling configuration is supported for BSR transmission per Serving Cell. Per the current specification, the values configured for SPS periodicity for NB-IoT are not multiples of 90 ms. Further considering RAN1 agreements that only one UL slot is usable within a 90 ms period, it may happen that there is no uplink subframe on the UL SPS occasion.

SR-SPS-BSR-Config-NB-r15 ::= CHOICE {

release								NULL,

setup								SEQUENCE {

semiPersistSchedC-RNTI-r15			C-RNTI,

semiPersistSchedIntervalUL-r15		ENUMERATED {sf128, sf256, sf512, sf1024,

sf1280, sf2048, sf2560, sf5120}

}

}

Similar situation is faced by PUR that allows the UE performs uplink transmission from RRC_IDLE using a preconfigured uplink resource without performing RACH procedure. We can see that in the current Spec, preconfigured uplink resource can be configured by the network with specific periodicity and offset., but the configurable periodicity values for PUR resources are not multiple of 90 ms either.

PUR-PeriodicityAndOffset-NB-r16 ::=	CHOICE {

periodicity8		INTEGER (1..7),

periodicity16		INTEGER (1..15),

periodicity32		INTEGER (1..31),

periodicity64		INTEGER (1..63),

periodicity128		INTEGER (1..127),

periodicity256		INTEGER (1..257),

periodicity512		INTEGER (1..511),

periodicity1024		INTEGER (1..1023),

periodicity2048		INTEGER (1..2047),

periodicity4096		INTEGER (1..4095),

periodicity8192		INTEGER (1..8191)

}

Proposal 3: RAN2 discusses whether/how to adapt the SPS/PUR to the 90 ms IoT NTN TDD frame structure (e.g. to resolve the case that there is no available UL subframe on an SPS/PUR transmission occasion).

### 2.2.3	Potential issues on SFN wrap around

(H-)SFN plays a key role in MAC. For example, SFN_id and H-SFN are required for RA-RNTI calculation for NB-IoT in TDD mode.

For NB-IoT UEs operating in TDD mode, the RA-RNTI associated with the PRACH in which the Random Access Preamble is transmitted, is computed as: RA-RNTI = 1 + floor(SFN_id/4) + 256*(H-SFN mod 2)

Apart from RA-RNTI calculation, SFN is also needed to determine when to start onDurationTimer.

- if the Long DRX Cycle is used and [(SFN * 10) + subframe number] modulo (longDRX-Cycle) = drxStartOffset: - if NB-IoT: - if there is at least one HARQ process for which neither HARQ RTT Timer nor UL HARQ RTT Timer is running, start onDurationTimer. - else: - start onDurationTimer.

Considering that the SFN is of 10-bit length with 1024 as maximum value (0-1023), the TDD pattern needs to be repeated every 10240 msec. If the H-SFN is enabled, the TDD pattern may need to be repeated every 10240*1024 msec. Considering 90 ms periodicity frame structure is adopted for TDD in IoT-NTN, the SFN wrap around periodicity is not a multiple of 90 ms (e.g. with only a 70 ms left at the very end as shown in Figure 1), which means that subset of UL/DL subframes may be different in different hyper frames.

Observation 3: The SFN wrap around periodicity is not a multiple of the periodicity of IoT-NTN TDD frame, which may bring impacts on SFN wrap around.

Figure 1: SFN and periodicity issue in TDD for IoT-NTN

In RAN1#119 meeting, RAN1 made the following agreements:

Agreement The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Hence, RAN2 can wait for further inputs from to address SFN wrap around issue.

Conclusion

In this contribution, the issue of RAN2 impact of TDD is discussed, with the following proposals:

Observation 1: The periodicity for downlink and uplink transition of IoT-NTN TDD mode is 90 ms which may require new frame structure and result in new timing design in uplink transmission and downlink reception.

Observation 2: IoT NTN TDD frame structure with 90 ms periodicity will bring the issue that it cannot ensure the frames/subframes, which should be used to transmit SIB1/MIB/SI/PO per the current Spec, are really DL channel, and thus cannot support the MIB/SIB1/SI/paging transmission mechanism in the current Spec.

Observation 3: The SFN wrap around periodicity is not a multiple of the periodicity of IoT-NTN TDD frame, which may bring impacts on SFN wrap around.

Proposal 1: RAN2 waits for further RAN1 progress before carrying out the discussion on how to adapt the MIB/SIB1/SI/paging procedure to the 90 ms IoT NTN TDD frame structure.

Proposal 2: RAN2 discusses whether/how to enhance related MAC timers/windows to adapt to the 90 ms IoT NTN TDD radio frame structure (e.g. RAR window, CR timer, retx BSR timer, DRX timer, etc.)

Proposal 3: RAN2 discusses whether/how to adapt the SPS/PUR to the 90 ms IoT NTN TDD frame structure (e.g. to resolve the case that there is no available UL subframe on an SPS/PUR transmission occasion).

Reference

- RP-243264 Revised WID on introduction of IoT-NTN TDD_cl

- 3GPP TS 36.331: "Universal Terrestrial Radio Access (UTRAN); Radio Resource Control (RRC)"