3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500530

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.8.4

Source:	Qualcomm Incorporated

Title:	Signaling of MBS broadcast service area information

Document for:	Discussion and Decision

# Introduction

In RAN#128, following agreements were made on MBS broadcast service.

Agreements:

1.	The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle.

2.	The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon.

3.	The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed)

4.	A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s).

5.	RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates.

6.	Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way)

7.	The legacy SIB modification procedure is applied to update the intended service area information in the new SIB.

In this document, we provide signaling details for MBS broadcast service area information.

# Discussion

Signaling of MBS broadcast service area

It was agreed in RAN2#128 that a new SIB is introduced to broadcast the list of coverage area information. In our view, the UEs not supporting intended service area do not have to acquire the unnecessary part, i.e., coordinates, from MCCH channel or SIB20. Each area information in the list can be identified by An Area ID. It is possible to use the Area ID in MCCH for serving cell MBS broadcast information.

One solution for mapping the serving cell MBS broadcast service with the geographical area information is that the mbs-SessionInfoList-r17 is extended to include list of geographic service area IDs for each session.

One potential issue with this approach is that when the UE is not in the intended area, it might have to check MCCH if it is interested in MBS broadcast to know if any change in geographical area info has changed. As RAN2 has made following agreements, we could also leave this to UE.

- RAN2 understands that the expected UE behavior is that when the UE is not in any intended service area of its interested broadcast services, the UE may not need to (re)acquire up-to-date MCCH. FFS on solutions

The other solution is that all the mapping information such as mbs-SessionInfo-17 and g-RNTI-r17 are included within each geographical area information in SIB21 or SIB25. In this case, the UE will only need to track the changes of SIB and does not need to monitor MCCH to determine whether intended service area has changed.

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, OPTIONAL,  -- Need R

TMGI-r19					TMGI-r19,   OPTIONAL,  -- Need R

coverageAreaInfo-r19 ::=    CHOICE {

circularAreaInfo-r19		circularAreaInfo-r19,

polygonAreaInfo-r19			polygonAreaInfo-r19

}

In our view, this could also add complexity on SIB change when there is change in MBS session information. Therefore, it could be ok to add the service area information index or Area ID in the mbs-SessionInfoList-r17 as what has already been agreed.

- For each MBS service we include one or more intended service area IDs into MCCH.

Introduce service Area ID for each geographical area information in the list in the new SIB.

To include one or more intended service area IDs into MCCH, mbs-SessionInfoList-r17 is extended.

Service continuity

In our view, UE can already map the MBS broadcast frequency to the associated MBS service through USD (User Service Description) or SIB21. It is to note that the UE needs to be aware of the frequency of the service it is interested in before it can receive the MBS broadcast service. The UE knows the interested frequency either from SIB21 or from USD. Therefore, it may be simple just to map the provided geographical area to one or more MBS broadcast service frequency.

The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

-- ASN1START

-- TAG-SIB21-START

SIB21-r17 ::= SEQUENCE {

mbs-FSAI-IntraFreq-r17                   MBS-FSAI-List-r17                OPTIONAL,  -- Need R

mbs-FSAI-InterFreqList-r17               MBS-FSAI-InterFreqList-r17       OPTIONAL,  -- Need R

lateNonCriticalExtension                 OCTET STRING                     OPTIONAL,

... ,

[[

mbs-CoverageAreaInfoList-r19             MBS-CoverageAreaInfoList-r19       OPTIONAL,  -- Need R

]]

}

MBS-FSAI-List-r17 ::= SEQUENCE (SIZE (1..maxFSAI-MBS-r17)) OF MBS-FSAI-r17

MBS-FSAI-InterFreqList-r17 ::= SEQUENCE (SIZE (1..maxFreq)) OF MBS-FSAI-InterFreq-r17

MBS-FSAI-InterFreq-r17 ::= SEQUENCE {

dl-CarrierFreq-r17         ARFCN-ValueNR,

mbs-FSAI-List-r17          MBS-FSAI-List-r17

}

MBS-FSAI-r17 ::= OCTET STRING (SIZE (3))

MBS-CoverageAreaInfoList-r19 ::=   SEQUENCE (SIZE (1..maxMBS-AreaInfo-r19)) OF MBS-CoverageAreaInfo-r19

MBS-CoverageAreaInfo-r19 ::=       SEQUENCE {

mbs-SessionAreaId-r19		MBS-SessionAreaId-r19,

mbs-FSAI-List-r19           MBS-FSAI-List-r17, OPTIONAL,  -- Need R

}

-- TAG-SIB21-STOP

-- ASN1STOP

MBS FSAI (Frequency Selection Area Identity) is another way how the intended service area can be associated with MBS session. SIB21 contains the mapping between the current and/or neighbouring carrier frequencies and MBS FSAI. The UE can obtain the FSAI of the MBS service from USD. A frequency can cover multiple MBS service Areas and the MBS FSAI could be different for the different service area. In this case, the service area can be associated with the MBS session simply by FSAI.

For service continuity, extend SIB21 to include the list of applicable geographical service area IDs and corresponding FSAI list.

# Conclusion

Following proposals are made.

Proposal 1	Introduce service Area ID for each geographical area information in the list in the new SIB.

Proposal 2	To include one or more intended service area IDs into MCCH, mbs-SessionInfoList-r17 is extended.

Observation 1.	The existing procedure (e.g., via SIB21 or USD) is already sufficient for UE to map Frequency and MBS broadcast service(s).

Proposal 3	For service continuity, extend SIB21 to include the list of applicable geographical service area IDs and corresponding FSAI list.