3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500535

Athens, Greece, February 17th – 21st, 2025

Agenda item:	8.9.4

Source:	Qualcomm Incorporated

Title:	Discussion on PWS in NB-IoT NTN

Document for:	Discussion and Decision

- Introduction

Following agreements were made in RAN2#128 meeting.

Agreements:

1.	We will clarify in 36.300 section 4.10 that in general a NB-IoT UE may not support all PWS requirements.

2.	We will extend the existing ETWS/CMAS notification RRC procedures for eMTC to NB-IoT. FFS if SIB1-NB acquisition is needed

3.	Add the following PWS indication in Paging-NB:

- etws-Indication;

- cmas-Indication.

4.   Send a LS to RAN1 asking whether it’s possible to specify support for PWS reception in RRC_CONNECTED for NB-IoT in Rel-19.

In this document, we provide details on remaining aspects of PWS in NB-IoT.

- Discussion

Currently NB-IoT UEs do not support “acceptable cell”, “Camped on Any cell state” and “emergency call” functionalities. NB-IoT UEs cannot also read broadcast notification message in RRC_CONNECTED. When PWS is supported in a cell (i.e., in NB-IoT RAT), the UE and its user need to be aware of the network capability.

## 4.4 NB-IoT functionality in Idle Mode This specification is applicable to NB-IoT, except for the following functionality which is not applicable to NB-IoT: - Acceptable cell - Accessibility measurements - Access Control based on ACDC categories - Camped on Any cell state - CSG, including support for manual CSG selection and CSG or Hybrid cell related functionality in PLMN selection, or HNB name (SIB9), Cell selection and Cell reselection. - Emergency call - E-UTRAN Inter-frequency Redistribution procedure - Inter-RAT Cell Selection and Reselection including measurements in other RATs - Logged measurements - Mobility History Information - Mobility states of a UE - Priority based reselection - Public warning system including CMAS, ETWS, PWS.

In NB-IoT, cell reselection is based on ranking regardless of frequency priorities. If a UE is PWS capable and camps in a cell that does not support PWS while there are neighbor cells supporting PWS, then UE implementing such important feature will not be able to take benefit and is going to miss the warning message. Therefore, we think UE needs to be aware of the PWS capability of the cell to prioritize cell search and PLMN search.

Introduce a new indication in SIB that a NB-IoT cell supports PWS feature.

In TS 36.306, currently it is specified in section 6.2 that PWS reception is optional for UE without UE capability signaling. Similarly, the support of geofencing is also optional without UE capability signaling.

However, the support of PWS will be a new feature for NB-IoT and there will be mix of UEs supporting and not supporting the feature. Whether the UE can support PWS in RRC_CONNECTED depends on RAN1 as an LS is sent to check the feasibility of PWS in RRC_CONNECTED. We can wait for RAN1 progress for RRC_CONNECTED.

Nevertheless, there will be UEs supporting PWS in IDLE mode but not in RRC_CONNECTED mode. For such UEs, the network should release the UEs to RRC_IDLE so they can receive the PWS message. However, there will be legacy UEs which will not support PWS in any RRC state. The network should not release these UEs to RRC_IDLE as legacy they can continue UL/DL transmission. Therefore, we think a new UE capability should be introduced to indicate whether the UE supports PWS in RRC_IDLE and help network to decide whether to release to IDLE mode so that the NB-IoT UE can receive the PWS message.

Introduce a new UE capability signaling to indicate whether the NB-IoT UEs support PWS feature in RRC_IDLE.

Geofencing for ETWS

RAN2 has received LS response from SA1 in [2]. SA1 has agreed that geofencing for ETWS can be supported and added following requirements in TS 22.268.

## 5.6	Additional PWS Requirements Specific to ETWS

Subject to local/regional regulatory requirements, in addition to ETWS Primary and Secondary Notifications, ETWS shall be able to support broadcasting Warning Notification Area(s). This requirement applies to Satellite E-UTRAN and Satellite NG-RAN.

Subject to local/regional regulatory requirements, an ETWS-capable UE shall present the received ETWS Notification(s) (Primary or Secondary) if there is a corresponding Warning Notification Area(s) and if the ETWS-capable UE is able to determine that its location is within the corresponding Warning Notification Area(s). Otherwise, if an ETWS-capable UE is unable to or fails to determine its location, the ETWS-capable UE shall present the ETWS Notification(s) (Primary or Secondary) to the user.

Therefore, the coordinates information can also be added in SIB10 and SIB11 for primary and secondary ETWS messages. For this purpose, we do not need to introduce new geofencing encoding. We can reuse the same encoding of coordinated used in SIB12.

[[	warningAreaCoordinatesSegment-r15		OCTET STRING	OPTIONAL	-- Need OR

]]

Same geocoordinates format (warningAreaCoordinatesSegment-r15) of SIB12 is added in SIB10 and SIB11 for NB-IoT.

PWS message size

The maximum PWS size is determined based on the maximum SI size limit and maximum number of segments of SI messages supported. As shown below, in LTE, the maximum number of segments for PWS is 64. This means if the SIB size becomes very large to transmit the PWS message size, the SI message (e.g., SIB11) is segmented with the limit of maximum of 64 segments.

SystemInformationBlockType11 ::=	SEQUENCE {

messageIdentifier					BIT STRING (SIZE (16)),

serialNumber						BIT STRING (SIZE (16)),

warningMessageSegmentType			ENUMERATED {notLastSegment, lastSegment},

warningMessageSegmentNumber			INTEGER (0..63),

warningMessageSegment				OCTET STRING,

dataCodingScheme					OCTET STRING (SIZE (1))		OPTIONAL,	-- Cond Segment1

...,

lateNonCriticalExtension				OCTET STRING				OPTIONAL

}

Given very limited resources in NB-IoT compared to LTE and NR, transmitting many segments can take forever. Just to transmit 64 segments considering minimum SI periodicity of 640ms, it would take minimum of 40.96 seconds given all 64 SI segments are successfully decoded by UE. Assuming max SI TBS size of 680 bits, maximum size of PWS message is 5440 octets. As clarified in reference [1], in general the number of characters of warning messages is ≤ 360. Therefore, 5540 octets limit should be sufficient for the warning message including the geographical coordinates. Considering Geofencing information, whether primary ETWS notification needs segmentation can be further discussed.

Same as in NR and LTE, the maximum number of segments for SIB11/12 is 64 for NB-IoT. Discuss whether primary ETWS notification in SIB10 needs segmentation.

We think the maximum PWS size for NB-IoT should also be added in CT1 and RAN3 specifications as it is being introduced as new feature in NB-IoT. Currently maximum size of 9600 octets considering 1200bits SI size and 64 segments is defined for only LTE UE.

For example, in CT1 stage 2 TS 23.041, following specified for E-UTRAN.

9.3.35     Warning Message Content E-UTRAN

This parameter is applicable for E-UTRAN only.

The Warning Message Content E-UTRAN IE contains user information, e.g., the message with warning contents, and will be broadcast over the radio interface. This element is a string of maximum 9600 octets; see 3GPP TS 36.413 [36].

In RAN3 36.413, following is specified.

9.2.1.53 Warning Message Contents

The Warning Message Content IE contains user information, e.g., the message with warning contents, and will be broadcast over the radio interface.

| IE/Group Name | Presence | Range | IE Type and Reference | Semantics Description |
| --- | --- | --- | --- | --- |
| Warning Message Contents | M |  | OCTET STRING (SIZE(1..9600)) | The length of this IE varies between 1 to 9600 bytes. |

Therefore, an LS can be sent to CT and RAN3 for alignment work.

Send LS to CT1 and RAN3 informing the maximum warning message size that can be broadcast via SIB.

- Conclusion

Following proposals are made:

Proposal 1	Introduce a new indication in SIB that a NB-IoT cell supports PWS feature.

Proposal 2	Introduce a new UE capability signaling to indicate whether the NB-IoT UEs support PWS feature in RRC_IDLE.

Proposal 3	Same geocoordinates format (warningAreaCoordinatesSegment-r15) of SIB12 is added in SIB10 and SIB11 for NB-IoT.

Proposal 4	Same as in NR and LTE, the maximum number of segments for SIB11/12 is 64 for NB-IoT. Discuss whether primary ETWS notification in SIB10 needs segmentation.

Proposal 5	Send LS to CT1 and RAN3 informing the maximum warning message size that can be broadcast via SIB.

# Reference:

# [1] https://www.fema.gov/emergency-managers/practitioners/integrated-public-alert-warning-system/public/wireless-emergency-alerts

# [2] R2-2500045, “Reply LS on requirements for ETWS primary notification”, Source SA1, To RAN2