3GPP TSG-RAN WG2 Meeting #129                                    	R2-2500537

Athens, Greece, February 17th – 21st, 2025

Agenda item:	7.4.2

Source:	Qualcomm Incorporated

Title:	Discussion on RAN4 LS on IDC issue in NTN

Document for:	Discussion and Decision

# Introduction

RAN2 has received LS from RAN4 in [1] with the following action.

- RAN4 respectfully ask to introduce improvements, if necessary, to existing IDC solution(s) and related IDC capabilities to address this in-device coexistence issue between GNSS reception and NR NTN transmission.

In this document, we discuss the issue and potential way forward.

# Discussion

In NTN, a UE needs to read GNSS periodically to maintain UL synchronization and this is different from TN scenario. Unfortunately, as shown in Figure 1, the NTN band is very close to the GNSS band. The NTN band n254 (1610 – 1626.5 MHz) is even closer to the GNSS G1 band. Then an UL transmission would cause leakage to UE’s GNSS receiver resulting in erroneous GNSS measurement and hence location.

Figure 1 Illustration of leakage to GNSS receiver.

RAN4 discussed the potential solutions such as very long (40s-90s) IDC cycles, and few seconds of Tx muting between cycles. But no decision could be made and RAN4 is asking RAN2 in [1] to introduce improvements, if necessary, to existing IDC solution(s) and related IDC capabilities to address this in-device coexistence issue between GNSS reception and NR NTN transmission.

Figure 2 An example of GNSS fix pattern in NR NTN.

In NTN, a UE would need to maintain the GNSS position accuracy for any UL transmission and would require reading GNSS signal periodically as shown in Figure 1. Since priority is to first fix the GNSS periodically, it can create issue and interruption to 3GPP radio. The main reasons for this being shortcoming of the existing IDC solutions:

- FDM solution:

  - NTN does not have much option to take benefit from the existing FDM-based IDC solution as there are limited bands. For example, an NTN UE may only support n255, then network wouldn’t be able to solve the issue by changing the frequency band.

- TDM solution:

  - Existing TDM-based IDC solution is also not optimized for NTN. The TDM solution is based on existing C-DRX cycle which may not match the required GNSS fix cycle. Moreover, this solution may not be useful when there is a PDCCH scheduling a new UL or DL transmission (which means UE is in active time and UE is mandated with active tasks such CSI reporting).

  - This would be mainly configured by network considering trade-off between power consumption and QoS for DL traffic, i.e., not considering IDC issue.

- Autonomous denials

  - This has more flexibility for UE but the current values of number of denial slots and validity are not sufficient for UE to complete the GNSS reading and fix GNSS.

Current IDC solutions are not optimized for NTN where a UE needs to perform GNSS measurements for x seconds without IDC problem in every y seconds.

In our view, the FDM solution would not be useful because of limited NTN bands. As explained above, the TDM solution can work but it is intended for the balancing the power saving vs QoS and adding another layer of criteria for NTN IDC solution would be challenging to solve the issue. Frequent RRC signalling overhead to configure and reconfigure C-DRX configuration would also not be desirable.

The current autonomous denial solution can be useful, however, as shown below the current parameters are not sufficient for the number of denials needed periodically to complete the GNSS reading without IDC problem it is because the duty cycle is too small for UE to be able to read GNSS without IDC problem.

AutonomousDenialParameters-r18 ::=  SEQUENCE {

autonomousDenialSlots-r18           ENUMERATED {n2, n5, n10, n15, n20, n30, spare2, spare1},

autonomousDenialValidity-r18        ENUMERATED {n200, n500, n1000, n2000}

}

The duty cycle in current autonomous denial configuration is too small for a UE to be able to complete GNSS reading without IDC problem in NTN.

In our view, a simple and quick fix for this could be to introduce new values of autonomous denials parameters such as more aggressive values with 50% duty cycles, for example, extending autonomousDenialSlots-r18 to n500 or n1000. Two spare values are available in autonomousDenialSlots-r18. As clarified in the LS [1], this issue is for both redcap UE and non-redcap UE in NTN. The IDC solution was introduced in Rel-18, therefore, the issue can be quickly fixed by using spare values to define new denials parameters from Rel-18.

Introduce a new aggressive denials parameter (i.e., with larger duty cycle) to solve the IDC issue in NTN.

If proposal 1 is agreed, consider fixing it from Rel-18 and introduce a UE capability.

Network awareness

Another drawback is the large signaling overhead to activate/deactivate the IDC configuration. The network would need to receive the IDC problem report from UE first before it can enable/configure the appropriate parameters for the IDC solution. However, if the IDC is detected by UE, then there could already be problem in meeting UL pre-compensation accuracy requirements for UE to be able to transmit the report successfully.

As shown in figure 2, a UE would need to use the IDC solution mainly during x seconds of GNSS reading. If network becomes aware about the periodicity when the UE would need to fix GNSS, the IDC solution can be activated and deactivated quickly by network. We think RAN2 should further study on the signaling overhead reduction mechanism by making network aware of UE’s GNSS reading pattern.

Consider a mechanism to reduce signaling overhead for enabling/disabling IDC solutions such as C-DRX based on IDC issue report or aggressive denials parameters.

# Conclusion

Following proposals are made.

Observation 1.	Current IDC solutions are not optimized for NTN where a UE needs to perform GNSS measurements for x seconds without IDC problem in every y seconds.

Observation 2.	The duty cycle in current autonomous denial configuration is too small for a UE to be able to complete GNSS reading without IDC problem in NTN.

Proposal 1	Introduce a new aggressive denials parameter (i.e., with larger duty cycle) to solve the IDC issue in NTN.

Proposal 2	If proposal 1 is agreed, consider fixing it from Rel-18 and introduce a UE capability.

Proposal 3	Consider a mechanism to reduce signaling overhead for enabling/disabling IDC solutions such as C-DRX based on IDC issue report or aggressive denials parameters.

# Reference

[1] R4-2419898, “LS on simultaneous operation between GNSS and NR NTN”, Source RAN4, To RAN2