3GPP TSG-RAN WG2 Meeting #129			     R2-2500538

Athens, Greece, February 17 – 21, 2025

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Further details of RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses further details of unnecessary retransmission and timely RLC retransmissions based on the RAN2 agreements so far.

Discussion

Unnecessary retransmissions avoidance

- Agreements on unnecessary RLC retransmissions avoidance in RAN2#128 - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced - Special handling to avoid PDCP control PDU discard is not needed. - A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Based on the RAN2 agreements above in RAN2#128, RAN2 can discuss further details of combined approach for unnecessary retransmission avoidance, especially, for Rx side operation. The first discussion point is that whether a new RLC timer to determine obsolete RLC SDUs is per RLC entity or per RLC SDU.

- Option 1. Per RLC SDU timer

- Option 2. Per RLC entity timer

As shown in the above figure, when a Rx entity receives the RLC SDU with SN5, the Rx entity detects a gap comprising multiple consecutive RLC SDUs, e.g., SN 2/3/4 are missed. In this situation, the option 1 should start three timers for each RLC SDU at the same time and multiple timers can be expired simultaneously, but the option 2 starts one timer associated with this gap. If the missed RLC SDU is received, the option 1 stops the timer associated with the corresponding RLC SDU, but the option 2 can stop the timer only when the related gap which starts this new timer is filled in the receiving window. Considering that each timer should have same length in the option 1, starting multiple timers having same length simultaneously would be redundant and does not bring benefits. In addition, according to the RLC specification, multiple consecutive RLC SDUs from one RLC entity would be included in one MAC PDU after receiving an UL grant and the above example would be a typical scenario. Thus, the option 2 should be considered to determine obsolete RLC SDUs.

Proposal 1. A new RLC timer to determine obsolete RLC SDUs is maintained per RLC entity.

With the proposal 1, when to start the new timer is already clear from the RAN2 agreement, i.e., “The new timer starts when the gap is detected at RLC layer.”, and RAN2 need to determine when to stop the new timer and we think that the new timer should be stopped when the gap which starts the new timer is filled in the receiving window.

Proposal 2. A new RLC timer is stopped when the gap is filled in the receiving window.

After expiry of the new RLC timer, the Rx entity discards obsolete RLC SDU(s) then sends the legacy STATUS report with fake ACK for the abandoned RLS SDU(s) based on the RAN2 agreement below.

Agreements in RAN2#127bis - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified Agreements in RAN2#128 - The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

According to the RLC specification below, when the STATUS report is constructed, ACK_SN and only NACK information of RLC SDUs between RX_Next and RX_Highest_Status, i.e., RX_Next <= SN < RX_Highest_Status, are included in the STATUS report. This means that if a RLC SDU lower than ACK_SN is not indicated by NACK in the STATUS report, the Tx entity considers this RLC SDU is positively acknowledged. Thus, if RX_Next is set to the SN which triggers the new RLC timer after expiry of the new RLC timer, the abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report like RAN2 agreement since the abandoned RLC SDU(s) is not indicated by NACK in the STATUS report.

When constructing a STATUS PDU, the AM RLC entity shall: - for the RLC SDUs with SN such that RX_Next <= SN < RX_Highest_Status that has not been completely received yet, in increasing SN order of RLC SDUs and increasing byte segment order within RLC SDUs, starting with SN = RX_Next up to the point where the resulting STATUS PDU still fits to the total size of RLC PDU(s) indicated by lower layer: - for an RLC SDU for which no byte segments have been received yet: - include in the STATUS PDU a NACK_SN which is set to the SN of the RLC SDU. - for a continuous sequence of byte segments of a partly received RLC SDU that have not been received yet: - include in the STATUS PDU a set of NACK_SN, SOstart and SOend. - for a continuous sequence of RLC SDUs that have not been received yet: - include in the STATUS PDU a set of NACK_SN and NACK range; - include in the STATUS PDU, if required, a pair of SOstart and SOend. - set the ACK_SN to the SN of the next not received RLC SDU which is not indicated as missing in the resulting STATUS PDU.

Observation 1. According to the RLC specification, if a RLC SDU lower than ACK_SN is not indicated by NACK in the STATUS report, the Tx entity considers that this RLC SDU is positively acknowledged.

Proposal 3. When the new RLC timer expires, RX_Next is set to the SN of the first RLC SDU with SN >= SN which triggers the new RLC timer for which not all bytes have been received.

The next detail is about the Tx entity behaviour after receiving a discard indication for a RLC SDU from upper layer. RAN2 already agreed that Tx entity stops transmissions of an outdated SDU, but there is no clear agreement that the outdated SDU is discarded or not at the RLC entity.

5.4 SDU discard procedures When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

The current RLC specification as shown above does not allow the Tx entity to discard a RLC SDU which is already submitted to the lower layer and introduce an RLC SN gap when discarding an RLC SDU. We think that this basic principle for discarding RLC SDU can be retained for the combined approach and the important point in the Tx entity is to stop transmission of outdated RLC SDU, not to discard the RLC SDU. If the RLC SDU indicated by the discard indication from upper layer is discarded in the transmitting window, an RLC SN gap is introduced unnecessarily in the transmitting window. In addition, considering that anyway the RLC SDU indicated by the discard indication from upper layer will be positively acknowledged by the STATUS report with fake ACK from the Rx entity, it would be better not to discard the RLC SDU indicated by the discard indication from upper layer and stopping transmission of the outdated RLC SDU would be sufficient.

For the last point, if window stalling occurs or both the transmission buffer and the retransmission buffer are empty after expiry of t-PollRetransmit timer, the current RLC specification as shown the below box selects a RLC SDU for retransmission regardless whether the RLC SDU is indicated by the discard indication from upper layer. However, the RLC SDU indicated by the discard indication from upper layer should not be considered for retransmission since if the RLC SDU indicated by the discard indication is selected for retransmission, this RLC SDU cannot be retransmitted by the Tx entity and the window stalling problem would not be resolved.

5.3.3.4 Expiry of t-PollRetransmit Upon expiry of t-PollRetransmit, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling): - consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or - consider any RLC SDU which has not been positively acknowledged for retransmission. - include a poll in an AMD PDU as described in clause 5.3.3.2.

Proposal 4. Even though the unnecessary retransmissions avoidance is configured, if the RLC SDU or RLC SDU segment indicated by the discard indication from upper layer has been submitted to the lower layers, the RLC entity does not discard this RLC SDU or RLC SDU segment as in legacy, i.e., no RLC specification change required.

Proposal 5. When the unnecessary retransmissions avoidance is configured, the RLC SDU indicated by the discard indication from upper layer is not selected for retransmission after expiry of t-PollRetransmit timer.

Timely RLC retransmissions

- Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them.

In the last RAN2 meeting, the below two options are discussed as compromise solutions for timely RLC retransmission and the above agreement is made.

- Option 1. If the remaining time drops below a threshold, poll is triggered. If SR is not received within a time, then Tx can autonomously retransmit the delay critical packet.

- Option 2. Have both autonomous retransmission and polling enhancement.

The big difference between two options is that the option 2 has two independent enhancements for timely RLC retransmission, but, in the option 1, autonomous retransmission has dependent on the polling mechanism since the option 1 can perform autonomous retransmission after expiry of a poll retransmission timer. This means that the option 1 needs more latency to perform autonomous retransmission of the delay critical data. For this reason, companies have concerns on the option 1 and want to avoid this dependency to provide full flexibility to the network for timely RLC enhancement.

As already indicated in the last RAN2 meeting discussion as shown below box, another point is that complexity of both solutions is same and the option 2 can cover the option 1 without any bad impact, i.e., when both auto reTx and polling are configured, but the option 1 may not cover the option 2.

DISCUSSION on autonomous reTx vs polling in RAN2#128: <skipped> - CMCC also prefers option 2 as different network implementations may use different options. - ZTE is OK with option 2 if UE will always support both together. - In Apple’s view option 1 covers option 2 if we set polling timer to 0, but it will have much smaller impact to specs. Ericsson agrees. - LGE agrees with ZTE it should be a single UE capability. - MTK thinks that option 2 means the UE needs to implement more, so prefers option 1. - Futurewei thinks we can harmonize both options by having two remaining thresholds. - Sharp thinks from UE point of view complexity is the same for both options. Option 2 is simply more general. - Huawei thinks option 1 is a special case of option 2, i.e. when both auto reTx and polling are configured.

The proponent of the option 1 said that if the poll retransmission timer is set to 0, the option 1 covers the option 2 as green highlight above. However, we have doubt whether the option 1 can cover the option 2 without any bad impact if poll retransmission timer is set to 0. For instance, when the UE receives an UL grant, the RLC entity submits a delay-critical RLC SDU with a poll to lower layer for transmission. Then the poll retransmission timer expires upon submission of the delay-critical RLC SDU and the RLC entity determine to retransmit this delay-critical RLC SDU again in the same MAC PDU since the retransmitted RLC SDU is prioritized over new transmitted RLC SDU.

More specifically, in the below figure, the RLC SDU with SN1 is a delay-critical RLC SDU in the transmission buffer. Assuming that UL resource is allocated to transmit three RLC SDUs in this RLC entity. When the RLC entity submits the RLC SDU with SN1, the RLC entity includes a poll in the RLC PDU containing the RLC SDU with SN1 and starts the poll retransmission timer, but it expires right away due to zero timer value. Then the RLC entity determine to retransmit the RLC SDU with SN1 since the RLC SDU with SN1 is the delay-critical RLC SDU as addressed in the option 1 above and the RLC entity submits the RLC SDU with SN1 again to lower layer since the retransmitted RLC SDU is prioritized over new transmitted RLC SDU. Finally, one MAC PDU may contain multiple MAC SDU having same RLC SDU and it wastes radio resources unnecessarily. To avoid this problem in the option 1, another mechanism should be introduced and it will make the procedure more complex.

The option 1’s proponent may argue that the option 2 has same problem. The option 2 does not have any dependency between autonomous retransmission and polling enhancement and the network can configure one of enhancements and this can avoid above problem. In addition, there is a proposal for autonomous retransmission that when remaining time of a RLC SDU becomes lower than a threshold in the transmitting window, this RLC SDU is considered for retransmission without receiving NACK for this RLC SDU. Thus, even though both autonomous retransmission and polling enhancement are configured together, if this proposal is agreed, the above problem can be avoided since autonomous retransmission is allowed when remaining time of a RLC SDU becomes lower than a threshold in the transmitting window. Thus, the option 2 should be selected for timely RLC enhancement.

Proposal 6. Autonomous retransmission and polling enhancement are introduced independently in Rel-19 XR, i.e., the option 2 of compromise solutions discussed in the RAN2#128.

Now, if the proposal 7 is agreeable, we can discuss which solution should be selected for each enhancement.

For how to get a STATUS report for timely retransmission, the current STATUS report can be triggered by polling at the transmitting entity or a SN gap detection at the receiving entity. As per the current RLC specification, the transmitting entity can include a poll bit to request a STATUS report when the number of transmitted RLC PDUs or the amount of transmitted bytes reaches a certain threshold, but there is no condition to check delay status of a RLC SDU. In this situation, even though the delay-critical RLC SDU is transmitted, the transmitting entity cannot include a poll bit unless the current condition for polling is satisfied. The transmitting entity may not receive a STATUS report before expiry of discard timer for the delay-critical RLC SDU since there is no poll included in the last transmission and the receiving entity may not detect any missing RLC SDUs. As a result, the delay-critical RLC SDU may not have another retransmission opportunity before exceeding PDB and this becomes an out-dated RLC SDU unnecessarily. We think that it would be important to receive the STATUS report after the delay-critical RLC SDU transmission to receive another retransmission opportunity and satisfy the PDB of the delay-critical RLC SDU. Thus, the transmitting entity should include a poll when the delay-critical RLC SDU is submitted to the lower layer for transmission. This enhancement does not have capacity issue and packet delay can be reduced because more retransmission opportunities can be given before exceeding PDB.

Proposal 7. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer.

Even if polling enhancements to receive a STATUS report is adopted, it should be also considered how to retransmit the RLC SDU without the STATUS PDU since RLC PDUs can be missed anytime and there is still possibility not to receive the STATUS report timely. For instance, if the PDU including a poll is missed, another poll bit can be included again after expiry of a poll retransmission timer or satisfying another triggering condition which would take an unacceptable latency for delay-critical RLC SDUs. In addition, even the RLC PDU including a poll is successfully transmitted, the receiving entity cannot know exactly which RLC SDU is a delay-critical RLC SDU at the transmitting entity. The receiving entity cannot selectively include necessary NACK for the delay-critical RLC SDU and NACK in the STATUS report is determined only based on the identified missing SN from the receiving window. It would be hard to receive the STATUS report at the time point when the transmitting entity wants for retransmission of delay-critical RLC SDU. To overcome this limitation of RLC AM retransmission mechanism, we think that if remaining time of a RLC SDU in the transmitting window becomes lower than a threshold, e.g., the RLC SDU is indicated as a delay-critical RLC SDU by the PDCP, this delay-critical RLC SDU should be considered for retransmission without receiving NACK for this delay-critical RLC SDU.

For concerns on capacity loss due to blind autonomous retransmission, when remaining time of a RLC SDU becomes lower than a threshold, a delay status report (DSR) at the MAC layer would be already triggered and transmitted. The network would give the UE both an uplink grant and the STATUS report together after receiving a DSR. In this situation, the transmitting entity can cancel the retransmission of the RLC SDU having lower than the threshold based on the positive acknowledgment in the STATUS report. This means that capacity loss would be marginal since only RLC SDUs which are not yet positively acknowledged can be retransmitted although autonomous retransmission is determined based on remaining time of a RLC SDU. From packet delay perspective, autonomous retransmission can save much time for retransmission because this does not need to wait a RTT (round trip time) to receive a STATUS report for determining retransmission of a delay-critical RLC SDU.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes lower than a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. According to the RLC specification, if a RLC SDU lower than ACK_SN is not indicated by NACK in the STATUS report, the Tx entity considers that this RLC SDU is positively acknowledged.

Proposal 1. A new RLC timer to determine obsolete RLC SDUs is maintained per RLC entity.

Proposal 2. A new RLC timer is stopped when the gap is filled in the receiving window.

Proposal 3. When the new RLC timer expires, RX_Next is set to the SN of the first RLC SDU with SN >= SN which triggers the new RLC timer for which not all bytes have been received.

Proposal 4. Even though the unnecessary retransmissions avoidance is configured, if the RLC SDU or RLC SDU segment indicated by the discard indication from upper layer has been submitted to the lower layers, the RLC entity does not discard this RLC SDU or RLC SDU segment as in legacy, i.e., no RLC specification change required.

Proposal 5. When the unnecessary retransmissions avoidance is configured, the RLC SDU indicated by the discard indication from upper layer is not selected for retransmission after expiry of t-PollRetransmit timer.

Proposal 6. Autonomous retransmission and polling enhancement are introduced independently in Rel-19 XR, i.e., the option 2 of compromise solutions discussed in the RAN2#128.

Proposal 7. The transmitting entity include a poll when the RLC SDU having remaining time lower than a threshold is submitted to the lower layer.

Proposal 8. If remaining time of a RLC SDU in the transmitting window becomes lower than a threshold, this RLC SDU should be considered for retransmission without receiving NACK for this RLC SDU.