3GPP TSG-RAN WG2 Meeting #129			     R2-2500539

Athens, Greece, February 17 – 21, 2025

Agenda Item:	8.7.6 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Rate control signaling for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses XR UL rate control signalling based on RAN2 agreements so far.

Agreements in RAN2#127bis - FFS if the indication is per DRB or per QoS flow. Companies should analyse the impact on QoS enforcement, interworking with L4S etc. - RAN2 to consider the following approaches to provide recommended bit rate values better fitting XR applications: - Extend the Bit Rate field - Define a new bit rate table to provide sufficient granularity for XR traffic - Introduce new values for the bitRateMultiplier - Send LS to SA4 asking about range/granularity which is required Agreements in RAN2#128 - RAN2 confirms it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. - gNB can be indicated which QoS flows can be throttled. FFS whether this is indicated from UE/CN - Rate indication from gNB to the UE on a per QoS flow level is supported. FFS the details, e.g. if: 1) flows are indicated by MAC CE or 2) by RRC while MAC CE is per DRB. - RAN2 will not discuss/support rate indication for DL unless WID is updated to include it by RANP. - RAN2 assumes that the congestion situation can be known at the gNB without any indication from the UE - FFS whether UL MAC CE rate query/preference is supported as UE recommendation to the NW or whether legacy MAC CE can serve this already. FFS in which scenarios this is useful.

Discussion

For UL XR rate control, RAN2 finally agreed to support per QoS flow indication in the last RAN2 meeting and FFSes are remained for further details as shown above.

The first one is whether a UE indicates to a gNB which QoS flows can be throttled or the CN indicates it to a gNB. The gNB is responsible for QoS enforcement to the UE and the required QoS profile already comes from the CN. QoS flow throttling is sort of QoS management and it would be reasonable that this information should come from the XR server. Thus, the CN indicates which QoS flows can be throttled by the gNB for UL XR rate control.

Proposal 1. The CN indicates which QoS flows can be throttled by the gNB.

For per QoS flow indication, two options can be considered based on the FFS above.

- Option 1. QoS flows are indicated by RRC while MAC CE is per DRB

- Option 2. QoS flows are indicated by MAC CE

In the option 1, multiple QoS flows can be mapped to one DRB and the gNB configures which QoS flow can be throttled in one DRB by RRC. Then if the UE receives a MAC CE with per DRB indication for XR UL rate control, the UE can throttle only QoS flows allowed for throttling by RRC configuration among all QoS flows in one DRB. We think that this option may have limitation to support finer QoS flow throttling since the MAC CE for XR UL rate control only has per DRB information with one bit rate filed applied to multiple QoS flows which may not throttle each QoS flow differently. In addition, if QoS flow which can be throttled in one DRB is changed, RRC signalling is always needed to change this configuration in one DRB. On the other hand, the option 2 has full flexibility to indicate which QoS flow should be throttled and RRC signalling is not needed even when QoS flow can be throttled in one DRB is changed. Note that even though the option 1 use a MAC CE with per DRB, the legacy RBR MAC CE cannot be reused and separate MAC CE with new LCID should be defined to make the UE recognize that this MAC CE with per DRB information needs QoS flow throttling based on RRC configuration. Considering aforementioned reason, it would be better to have new MAC CE with per QoS flow indication for XR UL rate control signalling.

Proposal 2. Per QoS flow indication is indicated by MAC CE, i.e., option 2.

For whether to support rate query MAC CE, we don’t think rate query MAC CE for XR UL rate control is beneficial since the target scenario is different. The RBR MAC CE is targeted for VoIP and both a caller and receiver UE are under 3GPP network control. In this scenario, rate query MAC CE is useful to change UL or DL transmission rate for the receiver UE based on the caller UE’s rate query because everything is under 3GPP network control. However, XR service is different. The XR UE is under 3GPP network control, but the XR server would be out-side-of 3GPP network which cannot be controlled by the 3GPP network. In this situation, even though the UE sends rate query to request UL rate change, if out-side-of 3GPP network is congested in the middle of the path to the XR server, the gNB cannot do anything and just ignore the rate query from the UE. As explained in the proposal 1 above, whole QoS related information for QoS enforcement should be provided from the CN and RAN2 already confirmed it is feasible for RAN to estimate the congestion information at both per-DRB and per-QoS flow level. Thus, the gNB can control XR UL rate control well without rate query MAC CE from the UE and we think that unless there is a clear useful scenario identified, rate query MAC CE from the UE is not needed.

Proposal 3. Unless a clear useful scenario is identified, rate query MAC CE from the UE is not supported.

As SA4 replied to the RAN2 question about range/granularity required for XR UL rate control, RAN2 can discuss details about MAC CE format now.

SA4 reply LS in R2-2500056 The SA4 reply is as follows: The range varies from 100kbps to 40Mbps. The numbers are based on the SA4 traffic characteristics study for XR services documented in 3GPP TR 26.926. Specifically, for XR split rendering and cloud gaming, a typical uplink data rate is 200 kbps. For AR conversational, a typical uplink data rate is 20Mbps. The granularity varies among existing data rate adaptation algorithms, and it can be as small as 1% for some algorithms (e.g., transport-wide congestion control as documented in 3GPP TR 26.822), yet also coarser for other algorithms (e.g., video codecs that can adapt to a given target bit rate constraint). From an application perspective, finer granularity, e.g., 1%, is generally preferred, if possible, but coarser granularity may also be sufficient, e.g., 10%.

For the current RBR MAC CE, following values are defined for bit rate field and bit rate multiplier.

- Bit rate field is defined from 0 kbps to 8000 kbps, i.e., 8 Mbps.

- bitRateMultiplier-r16               ENUMERATED {x40, x70, x100, x200}

The reply SA4 LS said that the required bit range is from 100kbps to 40Mbps and they prefer to have 1% granularity. Based on the above information, if bit rate multiple is used with the bit rate field, the current RBR MAC CE can cover the required bit range requested by the SA4, but the 1% granularity may not be supported. For this, we think that the bit rate field should be extended at least up to 7bits for a new bit rate table and the smaller bit rate multiplier such as x5, x10, and x20 should be also introduced.

Proposal 4. The bit rate field in the new MAC CE for XR UL rate control is extended at least up to 7bits and smaller new value is introduced for the bitRateMultiplier, e.g., x5, x10, and x20.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Proposal 1. The CN indicates which QoS flows can be throttled by the gNB.

Proposal 2. Per QoS flow indication is indicated by MAC CE, i.e., option 2.

Proposal 3. Unless a clear useful scenario is identified, rate query MAC CE from the UE is not supported.

Proposal 4. The bit rate field in the new MAC CE for XR UL rate control is extended at least up to 7bits and smaller new value is introduced for the bitRateMultiplier, e.g., x5, x10, and x20.