3GPP TSG-RAN WG2 Meeting #129	R2-2500547

Athens, Greece, 17 – 21 February 2025

Agenda item:	7.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	On SA2 LS for coarse UE location reporting in NB-IoT NTN

WID/SID:	IoT_NTN_enh-Core - Release 18

Document for:	Discussion and Decision

# 1	Introduction

The SA2 LS [1] confirms that, if requested by eNB, the MME can send the Coarse Location Information received from the UE (via NAS) to the eNB.  Furthermore, SA2 asked whether RAN2/RAN3 had analysed the coarse UE location report for NB-IoT NTN cases with AS security activated, where the UE can provide Coarse Location Information to the eNB via RRC.

Excerpt from S2-2412665[1]: SA2 nevertheless observed that the CR approved by RAN3 (TS 36.413 CR1957) did not include the “Coarse UE Location Requested” IE in UL NAS Transport, Resume Request and the “Coarse UE Location IE” in Resume Response. Even though Coarse Location Information can be requested once in the Initial UE message also in these cases (and provided to the eNB in a downlink S1-AP message), this prevents the Coarse Location Information of the UE to be requested by eNB and provided from MME to eNB at a later stage during a RRC connection when user plane bearers are established. The relevant procedures in the SA2 specification are “Establishment of S1-U bearer during Data Transport in Control Plane CIoT EPS Optimisation” (see clause 5.3.4B.4 of TS 23.401), “Connection Resume procedure” (see clause 5.3.5A) and “MT-EDT procedure for User Plane CIoT EPS Optimisation” (see clause 5.3.5B). SA2 would like to understand if RAN2 and RAN3 has performed any analysis on whether Coarse Location Information is needed by the eNB in any of the above procedures. In case RAN2 and RAN3 conclude that Coarse Location Information is needed in these cases, SA2 would like to highlight that AS security is activated during these procedures which would also allow the UE to provide the Coarse Location Information to the eNB via the RRC protocol.

In this contribution, we would like to provide our analysis to the SA2 question.

# 2	Discussion

RAN2 had discussed the coarse UE location reporting for NB-IoT NTN in both Rel-17 and Rel-18. Based on the input from [2], it was recognized that the majority of NB-IoT NTN devices will support the CP CIoT optimization solution, in which AS security cannot be assumed to be used.

Excerpt from R2-2310192 [2]: Observation 3: In practice, we have to recognize that majority of NB-IoT NTN devices will focus on CP CIoT Optimizations with Data over NAS, thus AS security cannot be assumed to be in place.

Therefore, RAN2 focused on the coarse UE location reporting requirements for NB-IoT CP solution without AS security activation and asked SA2/CT1 and RAN3 to design the UE location reporting via NAS layer in Rel-18.

Excerpt from LS R2-2311326 [3]: No consensus could be reached in RAN2 as to whether an AS-based solution of reporting UE location, even coarse-grained, for NB-IoT UEs using control plane optimization could be supported, due to privacy and security concerns raised by some companies. RAN2 would thus like to ask SA2 and CT1 if UE location reporting via NAS layer can be considered in Rel-18, to satisfy this operator requirement.

Accordingly, the NAS-based coarse UE location reporting has been specified in Rel-18 after extensive discussions in CT1, SA2, RAN3 and RAN2, with multiple LS [1][3][4][5][6][7] exchange between the WGs. According to the current specification, the coarse UE location Information request and report can be sent in NAS signalling. The location information can be delivered not only to the MME but also can be sent back from the MME to the eNB, if requested.

Observation 1: For NB-IoT NTN, it is recognized that the majority of NB-IoT NTN devices support the CP CIoT solution, thus AS security cannot be assumed to be used.

Observation 2: For NB-IoT NTN, according to the current specification, the coarse UE location information request and report can be sent in NAS signalling even when the AS security is not activated. This information can be delivered not only to the MME but also from the MME back to the eNB, if requested.

For the cases raised by SA2 LS [1], it is mainly for the UE supporting User Plane CIoT EPS optimization, in which the AS security can be activated between the UE and the network. However, as indicated in observation 1, it seems the number of NB-IoT NTN device only supporting UP solution is quite limited. Therefore, first of all, we think RAN2 should discuss whether the requirements to support the coarse UE location reporting for UP solution is really needed.

Observation 3: The requirement for supporting coarse UE location reporting in cases where the UE only supports UP CIoT solution is unclear.

Even if the requirement for UP solution is confirmed, there may be two ways forward to fulfil the requirement.

In one way, the coarse UE location information can be reported to eNB via RRC protocol after the AS security is activated. As agreed in Rel-17 NR NTN and eMTC over NTN, after AS security is established, coarse UE location information can be requested by gNB/eNB and UE report the coarse UE location accordingly. In details, below two approaches are captured in the RRC specifications.

-	Approach 1: gNB/eNB requests coarse UE location report in UEInformationRequest and the UE provides the information to the gNB/eNB in UEInformationResponse.

-	Approach 2: gNB/eNB configures coarse UE location report in reportConfigEUTRA(NR) and the UE provides the information to the gNB/eNB in MeasResults.

Following the similar approach for NR NTN and eMTC over NTN, for reporting approach 1, the NB-IoT eNB should request the UE to report coarse location information via the UEInformationRequest-NB message, using the similar IE as coarseLocationReq-r17 (copied in Annex for eMTC NTN messages). Correspondingly, the NB-IoT UE should report the coarse location information via the UEInformationResponse-NB message, using the similar IE as coarseLocationInfo-r17. This implies that ASN.1 release 18 changes are inevitable for both the UEInformationRequest-NB and UEInformationResponse-NB messages. Similarly, for reporting approach 2, supporting coarse location information via a measurement result could be standardized. The NB-IoT does not support connected mode mobility, but via SIB2-NB the eNB can configure to UE to report MeasResultServCell-NB in the RRC resume/reestablishment/setup complete messages. The MeasResultServCell-NB information element could be enhanced to include the coarse location information, but it will then require ASN.1 release 18 modifications of SIB2-NB and the MeasResultServCell-NB.

Observation 4: For NB-IoT UP solution, if the coarse UE location reporting is reported to eNB via RRC, the release18 ASN.1 and RRC procedure impact is inevitable.

The other way is that the coarse UE location information can be reported to eNB via NAS-based signalling. In our understanding, the NAS-based coarse UE location reporting can also be applied to UEs that supports the UP solution only. In other words, NAS-based coarse UE location reporting is a general solution that can be applied to NB-IoT NTN devices supporting either the CP solution, the UP solution, or both. The NAS-based UE coarse locations for any regulatory requirement are sufficient. The necessity to introduce the RRC-based coarse UE location reporting for the UP solution is quite low.

Observation 5: The NAS-based coarse UE location reporting can be applied to NB-IoT NTN devices supporting either the CP solution, the UP solution or both. The necessity to introduce the RRC-based coarse UE location reporting for the UP solution is quite low.

Considering that the ASN.1 for Release 18 has already been frozen, the bar to change the specification should be high. Given the limited scenarios applicable for RRC-based coarse UE location reporting and low necessity to introduce the enhancement, it seems too late to support coarse UE location reporting via RRC for NB-IoT NTN in Release 18.

Proposal 1: For NB-IoT NTN, reporting Coarse Location Information to eNB via the RRC protocol is not supported in Rel-18.

Proposal 2: RAN2 reply SA2 that there is no need to introduce additional enhancements to report Coarse Location Information to the NB-IoT eNB via the RRC protocol.

# 3	Conclusion

This document has made the following observations:

Observation 1: For NB-IoT NTN, it is recognized that the majority of NB-IoT NTN devices support the CP CIoT solution, thus AS security cannot be assumed to be in place.

Observation 2: For NB-IoT NTN, according to the current specification, the coarse UE location information request and report can be sent in NAS signalling even the AS security is not activated. This information can be delivered not only to the MME but also from the MME back to the eNB, if requested.

Observation 3: The requirement for supporting coarse UE location reporting in cases where the UE only supports UP CIoT solution is unclear.

Observation 4: For NB-IoT UP solution, if the coarse UE location reporting is reported to eNB via RRC, the ASN.1 and RRC procedure impact is inevitable.

Observation 5: The NAS-based coarse UE location reporting can be applied to NB-IoT NTN devices supporting either the CP solution, the UP solution or both. The necessity to introduce the RRC-based coarse UE location reporting for the UP solution is quite low.

And proposed the following:

Proposal 1: For NB-IoT NTN, reporting Coarse Location Information to eNB via the RRC protocol is not supported in Rel-18.

Proposal 2: RAN2 reply SA2 that there is no need to introduce additional enhancements to report Coarse Location Information to the NB-IoT eNB via the RRC protocol.

# Reference

[1] S2-2412665 Reply LS on UE Location Information for NB-IoT NTN

[2] R2-2310192 NB-IoT NTN Coarse UE location reporting, Inmarsat, Viasat, Sateliot, Novamint, ESA, Thales

[3] R2-2311326 - LS on UE Location Information for NB-IoT

[4] C1-239363 LS on UE Location Information for NB-IoT NTN

[5] S2-2403851 Reply LS on UE Location Information for NB-IoT NTN

[6] R2-2403770 LS on UE Location Information for NB-IoT NTN (R2-2402143/ S2-2403851)

[7] R3-245819 Reply LS on UE Location Information for NB-IoT NTN

# Annex

#### –	UEInformationRequest

The UEInformationRequest is the command used by E-UTRAN to retrieve information from the UE.

Signalling radio bearer: SRB1

RLC-SAP: AM

Logical channel: DCCH

Direction: E-UTRAN to UE

UEInformationRequest message

-- ASN1START

UEInformationRequest-r9		::=				SEQUENCE {

rrc-TransactionIdentifier		RRC-TransactionIdentifier,

criticalExtensions				CHOICE {

c1								CHOICE {

ueInformationRequest-r9				UEInformationRequest-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

…

UEInformationRequest-v1710-IEs ::= SEQUENCE {

coarseLocationReq-r17				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				UEInformationRequest-v1800-IEs	OPTIONAL

}

UEInformationRequest-v1800-IEs ::= SEQUENCE {

rach-ReportReqNR-r18				ENUMERATED {true}					OPTIONAL,	-- Need ON

nonCriticalExtension				SEQUENCE {}							OPTIONAL

}

-- ASN1STOP

UEInformationRequest field descriptions

coarseLocationReq This field is used to request UE to report coarse location information.

#### –	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer the information requested by the E-UTRAN.

Signalling radio bearer: SRB1 or SRB2 (when logged measurement information is included)

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to E-UTRAN

UEInformationResponse message

-- ASN1START

UEInformationResponse-r9	::=		SEQUENCE {

rrc-TransactionIdentifier			RRC-TransactionIdentifier,

criticalExtensions					CHOICE {

c1									CHOICE {

ueInformationResponse-r9			UEInformationResponse-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture			SEQUENCE {}

}

}

…

UEInformationResponse-v1710-IEs ::= SEQUENCE {

coarseLocationInfo-r17				OCTET STRING					OPTIONAL,	nonCriticalExtension				UEInformationResponse-v1800-IEs	OPTIONAL

}

UEInformationResponse-v1800-IEs ::= SEQUENCE {

rach-ReportNR-r18					RACH-ReportNR-r18				OPTIONAL,

nonCriticalExtension				SEQUENCE {}						OPTIONAL

}

RACH-Report-r16 ::=					SEQUENCE {

numberOfPreamblesSent-r16			NumberOfPreamblesSent-r11,

contentionDetected-r16				BOOLEAN

}

…

-- ASN1STOP

UEInformationResponse field descriptions

coarseLocationInfo This field indicates the coarse location information reported by the UE. This field is coded as the Ellipsoid-Point IE defined in TS 37.355 [109]. The first/leftmost bit of the first octet contains the most significant bit. The least significant bits of degreesLatitude and degreesLongitude are set to 0 to meet the accuracy requirement which corresponds to a granularity of approximately 2 km. It is up to UE implementation as to how many LSBs are set to 0 to meet the accuracy requirement.