3GPP TSG-RAN WG2 Meeting #129	R2-2500549

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.17

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on support of TDD mode for IoT-NTN

WID/SID:	IoT_NTN_TDD - Release 19

Document for:	Discussion and Decision

# 1	Introduction

At RAN #106 the work item on NB-IoT TDD NTN mode [1] was agreed with the following objective:

- Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern, necessary adaptation (if needed) and associated UE procedures [RAN1, RAN2] - Support a pattern with a period of 9 radio frames for the target MSS allocated band, where D=U=8 with a fixed guard period.RAN1 to consider whether there is a need for a mechanism to achieve an adjustable guard period for the purpose of allowing deployment with the TDD frame structure of the legacy system operating in the target MSS allocated band. - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4]

In this contribution we discuss aspects of TDD NTN mode, which impacts RAN2.

# 2	Discussion

According to the WID and RAN1 discussions the NB-IoT TDD NTN frame structure is as illustrated in Figure 1.

Figure 1 NB-IoT TDD NTN frame structure.

Furthermore, RAN1 has agreed that the UL and DL occasions are paired, i.e. if UE has received scheduling information in DL1 the corresponding UL occasion is UL1.

Agreement (RAN1 #119) Regarding operation within the same band as the TDD frame structure of legacy system in the 1.6GHz MSS band (shown below), RAN1 work in Rel-19 considers the following design constraints: - At the satellite, all downlink NB-IoT channels/signals in a cell can only use one of the downlink slots in the TDD frame structure (DL1, DL2, DL3 or DL4) across 90ms periods. - The same downlink slot is used in all the 90ms periods. - At the satellite, all uplink NB-IoT channels in a cell can only use one of the uplink slots in the TDD frame structure (UL1, UL2, UL3 or UL4) across 90ms periods. - The same uplink slot is used in all the 90ms periods. - The one uplink slot and one downlink slot in the TDD frame structure have the same index (DL1 & UL1, DL2 & UL2, DL3 & UL3, or DL4 & UL4). - NOTE: this does not imply that the only configuration(s) to be specified are according to these constraint

In general, the sparsity of DL and UL subframes impacts the network capacity, especially when considering the mandatory signalling (NPSS, NSSS, NPBCH, SIB1, SIB31). According to RAN1 conclusion the overhead of transmitting the aforementioned, excluding SIB31, is about 40 % of the anchor carrier.

Conclusion (RAN1 #119) For N=9, D=8, RAN1 concludes that SIB1-NB reception following current specifications is feasible for some SIB1-NB TBS values, at least when the configured number of repetitions is 16 and when SIB1-NB TBS value <=X bits. - When considering only NPSS/NSSS/NPBCH/SIB1-NB, and if the number of SIB1-NB repetitions is 16: - If additionalTransmissionSIB1 is set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 43.75% in the anchor carrier. - If additionalTransmissionSIB1 is not set to TRUE, the overhead of NPSS/NSSS/NPBCH/SIB1-NB is 37.5% in the anchor carrier. - NOTE: In the above calculations, overhead due to transmission of other SIBs has not been included. - The value of X may be further studied during the normative phase. Note: X is a TBS value supported by the specifications for SIB1-NB

Thus the number of subframes, which can carry control- and user-plane data is limited. For example, the NB-IoT PRACH preamble has a high capacity due to the use of 3.75 kHz subcarrier. However, the RAR/Msg2 will require a larger bandwidth and thus it may be challenging for the eNB to respond timely to all UEs if many preambles were transmitted in the preceding UL occasion.

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

RAN1 has initiated a discussion to define how SFN and H-SFN is counted given that the 90 ms TDD frame structure does not match the 10240 ms legacy structure. In our view, RAN2 can wait for RAN1 to conclude on this before discussing aspects related to SFN.

Agreement (RAN1 #119) The offset between the 90ms TDD structure and the 10240ms H-SFN is to be studied considering at least the following options: - Option 1: Always the same across H-SFNs. - Option 1-1: H-SFN duration is changed to X radio frames, with X a multiple of 9. - Option 1-2: H-SFN duration is unchanged - Option 2: Different across some H-SFNs. - Option 2-1: UE is indicated (implicitly or explicitly) the offset between H-SFN and 90ms TDD structure based on some downlink signal (e.g. one of NPSS/NSSS/NPBCH/SIB1-NB) - Option 2-2: Total number of H-SFN is changed to be a multiple of 9.

Furthermore, RAN1 is also discussing what the starting DL subframe will be in the DL occasion (with options starting in DL subframe 3, 4, 8 or 9). The definition of the SFN and subframe number may impact the calculation of the resource allocation such as SPS. RAN2 can also wait for RAN1 to progress with this discussion.

Agreement (RAN1 #119) The downlink subframes within D=8 are to be down-selected among the following: - Option 1: [3 4 5 6 7 8 9 0] (across two consecutive radio frames) - Option 2: [4 5 6 7 8 9 0 1] (across two consecutive radio frames) - Option 3: [8 9 0 1 2 3 4 5] (across two consecutive radio frames) - Option 4: [9 0 1 2 3 4 5 6] (across two consecutive radio frames)

Proposal 1: RAN2 to wait for RAN1 progress regarding SFN and starting DL subframe for the NB-IoT NTN TDD frame structure.

Based on the RAN1 agreements, if the UE has transmitted in UL1 the UE will have to monitor for a response in DL1. According to the frame structure definition, the time-separation is 3*8.28 ms + 3 guard periods which approximates 25 ms (see also Figure 1).

The WID assumptions define LEO 600 km and 1200 km orbits and thus the RTT is in the range 4-12.88 ms for a regenerative LEO600 deployment and 8-25.76 ms for a transparent LEO600 deployment depending on the UE’s elevation angle to the satellite. This means that the UL-DL occasion gap in some scenarios is significantly longer than the RTT and in other scenarios the RTT and the gap is quite comparable.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

The large UL-DL time separation impacts the RAN2 specification, because some MAC procedures are based on the UE-eNB RTT. For example, the 4-step Random Access procedure utilize the UE-eNB RTT when defining the start of the RAR window after Msg1 transmission and for start of the Contention Resolution timer after Msg3 transmission.

An example is given for the RAR window (TS 36.321), which starts based on X subframes + UE-eNB RTT.

### 5.1.4 Random Access Response reception … If the UE is an NB-IoT UE: - if the random access preamble was transmitted in a non-terrestrial network: - RA Response window starts at the subframe that contains the end of the last preamble repetition plus X subframes plus UE-eNB RTT and has length ra-ResponseWindowSize for the corresponding enhanced coverage level, where value X is determined from Table 5.1.4-1 based on the used preamble format and the number of NPRACH repetitions; … Table 5.1.4-1: Subframes between preamble transmission and RA Response Window in NB-IoT TDD/FDD mode; Preamble format; Number of NPRACH repetitions; X. FDD; 0 or 1; >= 64; 41. FDD; 0 or 1; < 64; 4. FDD; 2; >= 16; 41. FDD; 2; < 16; 4. TDD; Any; Any; 4.

Starting the RAR window based on X (of table 5.1.4-1) and the UE-eNB RTT may result in most of the window occurring during the UL-DL occasions gap. This is problematic from UE energy consumption point of view and also limits the eNB scheduling flexibility, because it may reduce the overlap between the RAR window and the DL subframes in the DL occasion.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

A similar issue also exists for the Contention Resolution timer, because it is started based on the UE-eNB RTT as well.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

Based on this issue we propose that RAN2 discuss how to better align the UE’s PDCCH monitoring, based on the RAR window and Contention Resolution timer, with the DL occasions.

Proposal 2: RAN2 to discuss how to better align the RAR window and Contention Resolution timer with the DL occasions.

# 3	Conclusion

This document has made the following observations:

Observation 1: The 8 DL subframes of a DL occasion may have insufficient capacity to carry RAR for all UEs transmitting a preamble in the preceding UL occasion.

Observation 2: The time separation between an UL and a DL occasion in the NB-IoT TDD NTN frame structure is about 25 ms.

Observation 3: The RAR window may occur primarily during the UL-DL occasion gap if the start of the window is based on UE-eNB RTT.

Observation 4: The Contention Resolution timer may run primarily during the UL-DL occasion gap if the start of the timer is based on UE-eNB RTT.

And proposed the following:

Proposal 1: RAN2 to wait for RAN1 progress regarding SFN and starting DL subframe for the NB-IoT NTN TDD frame structure.

Proposal 2: RAN2 to discuss how to better align the RAR window and Contention Resolution timer with the DL occasions.

# Reference

- RP-243293, Iridum, “Revised WID on introduction of IoT-NTN TDD mode”, December 2024