3GPP TSG-RAN WG2 Meeting #129	R2-2500552

Athens, Greece, 17 – 21 February 2025

Agenda item:	8.5.2

Source:	Nokia, Nokia Shanghai Bell

Title:	On-demand SIB1 for Idle/Inactive mode UEs

WID/SID:	Netw_Energy_NR_enh

Document for:	Discussion and Decision

# 1	Introduction

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In this contribution, we will discuss the issues related to the on-demand SIB1 for idle/inactive mode UEs. And in our companion contribution, we will discuss the issues on-demand SSB SCell operation and adaptation of common signal/channel transmissions in [1] and [2], respectively.

The following agreements were made:

Agreements on OD-SIB1: RAN2#128

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

- New NES-specific reselection priority parameters for NES UEs are defined for the purpose of prioritizing/deprioritizing a NES frequency.

- Introduce new IntraFreqExcludedCellList-NES / InterFreqExcludedCellList-NES IEs enable proper reselection behaviour of legacy and NES UEs.

- Reuse legacy cell reselection criterion as trigger condition of OD-SIB1 acquisition. No need to specify other conditions (e.g. a new RSRP threshold).

- The existing 1-second rule in the cell reselection criteria is still applied to the triggering condition of UL WUS transmission.

- The UE considers the cell as barred after MAC indicates max number of preamble transmission for the OD-SIB1 request.

- If MSG2 (ACK) is received, but UE fails to receive SIB1 then the UE may consider this cell as barred, no spec impact.

- A UE bars the NES/SIB1 less cell and/or excludes it as a candidate for reselection since the UE had no corresponding UL WUS configuration, the UE would treat this cell as if cell status is “not barred” and consider it as candidate for cell reselection once it has received a UL-WUS configuration to request SIB1 for this cell.

- UE considers WUS configuration only valid in directly succeeding cell reselection from cell where UE acquired WUS configuration. FFS on SI validity area rather than cell level.

- Working assumption: UL-WUS configuration in RRC release is not supported.

In this paper we discuss further details of the OD-SIB1 for case 2 agreed in the RAN#105 plenary meeting.

# 2	Baseline operation for case 2 – For information only

- UE obtains the WUS configuration from cell A,

  - PCI+Frequency (at least) used to associate configuration to a cell.

  - Cell A may provide configuration for multiple cells.

  - Multiple Cell A(s) may provide WUS configuration for same cell.

  - At least configuration is provided by cell A SIB(s). in addition to cell A SIB (s), RRCRelease may provide WUS configuration. This makes sense to us as there is likely scenarios that cell starts to use OD-SIB1 and releases UES to IDLE/INACTIVE – It seems quite good idea not to mandate reselection to Cell A and then back to NES Cell

- Once UE (re)selects to cell for which UE has WUS configuration and NW indicates it is utilizing OD-SIB1 procedure - UE may request OD-SIB1. Legacy UE should bar this kind of cell but OD-SIB1 supporting UE should not.

  - Request is assumed to be preamble transmission

    - We don’t combine this with direct request for moving to RRC_CONNECTED i.e. UE will first acquire SIB1 and after that resume/request to be moved to RRC_CONNECTED

  - RAN2 assumes UE will receive RAR

  - Assuming NW detects OD-SIB1 request it will provide SIB1 to UE. It has not been discussed in RAN2 whether SIB1 provision is:

    - via OD-SIB1 specific transmission AND/OR

    - NW resumes regular SIB1 transmission

- If SIB1 reception is successful

  - UE may camp on the cell will apply regular operations – paging reception, SI change modification monitoring etc…

  - It was left for further consideration how UE determines validity of SIB1

  - RAN2 also agreed that UE needs reacquire SIB1 in case of SIB change notification (legacy behaviour).

- If not successful

  - As baseline upon random access procedure failure of OD-SIB1 request, UE regards OD-SIB1 can’t be acquired in the NES cell and considers it as barred. It doesn’t exclude the option to leave the determination to the UE implementation. Without modification to specification if UE has not been able to acquire SIB1 it will bar the cell for up to 300 seconds

As we can see RAN2 has established baseline operation for case 2 but of course there are still various aspects that need to be handled.

# 3	Access restriction for legacy UEs and NES UE behavior

RAN2#127bis agreed about the following:

1. Legacy UEs bar the OD-SIB1 cell based on no SIB1 indication in MIB e.g. via ssb-SubcarrierOffset. Detailed solution is up to RAN1. If this works, no separate barring bit for R19 NES UEs is introduced. 2. NES UEs should be allowed to reselect to cells that are prevented from legacy UEs (e.g. by excluded cell list, reselection priorities).

Based on R2-2410911, R16/17/18 V2X/SL, R18/19 MOB and R19 NES, RAN2 is waiting RAN1 progress about access restriction for legacy UEs when SIB1 is not transmitted. The following is agreed:

Let’s wait for more RAN1 progress on Kssb discussion.

RAN1 #118bis does not have any agreement about the barring legacy UEs. However, based on R1-2410681 FL summary, many companies are waiting for RAN1 to send an LS to RAN1 before triggering this kind of discussion. However, the following was agreed in RAN1 #118bis regarding K_ssb:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

- 	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

One will set ssb-SubcarrierOffset (K_SSB) value for NES cell with OD-SIB1 operation considering RAN1 #118bis agreement:  K_SSB > 23 for FR1 and K_SSB > 11 for FR2.

With these values the SSB carrying K_ssb with this value is referred as non-Cell defining SSB, NCD-SSB. However, 38.300 specifies the following for cell selection and cell reselection for NCD-SSB:

Cell selection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4):

Cell reselection is always based on CD-SSBs located on the synchronization raster (see clause 5.2.4).

This mean legacy UE will not select or re-select a NES with OD-SIB1 operation if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.  The common understanding from RAN1 and RAN2, that legacy UE will not select/re-select a cell associated with NCD-SSB. For NCD-SSB on sync raster, the legacy UEs can’t camp in the cell.

Observation 1: For a cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell

For a cell associated to a NCD-SSB, legacy UEs treats the cell as if the cell status is "barred" due to being unable to acquire the SIB1. Legacy UEs may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds based on 38.304.

In summary, legacy UEs behaves as following:

- Do not perform NES cell selection.

- Do not perform NES cell re-selection

- and/or 2) imply that legacy UEs do not camp on NES cell.

- Treat NES cell as if the cell status is “barred”.

For NES UE without SIB1 request configuration of a NES cell, they behave similarly as legacy UEs.

Observation 2: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell.

Observation 3: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, RAN2 to consider the following for legacy UEs:

- Legacy UEs treat the cell as if the cell status is "barred" due to being unable to acquire the SIB1. Legacy UEs may exclude the barred cell as a candidate for cell selection/reselection for up to 300 seconds.

Observation 4: If NES UE does not acquire SIB1 request configuration of a cell, it behaves similarly with respect to NES cell for cell selection/re-selection and barring.

Observation 5: If NES UE has SIB1 request configuration of a cell, NES UE may re-selects on a cell with NCD-SSB located on the synchronization raster and acquire SIB1 with on-demand fashion even if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.

Proposal 1: There is no need for additional barring mechanisms (in addition to the k_ssb signaling in MIB) to handle legacy to be able to bar cell using OD-SIB1

| UE operation when trying to camp on Cell indicating K_SSB > 23 for FR1 and K_SSB > 11 for FR2 |  |  |  |
| --- | --- | --- | --- |
| UE type | Cell selection | Cell re-selection | Barring |
| Legacy UE and NES UE without WUS configuration | N/A | N/A | May treat the cell as barred as no way to acquire SIB1. For K_SSB <30 for FR1 and K_SSB <14 for FR2, UE determine the nearest SS/PBCH based on a GSCN offset (see 38.213) associated to CD-SSB For K_SSB = 30¦31 for FR1 and K_SSB = 14¦15 for FR2, UE determines that the SS/PBCH associated to the cell is not carrying information about SIB1. |
| NES UE with WUS configuration | N/A | YES | NO |

# 4	Switching between legacy SIB1 transmission and on-demand SIB1 transmission

First, we provide a summary about related agreements and discussion for this aspect. These agreements are used as starting point and assumptions for our proposals and observations for this aspect.

RAN2 agreed about the following:

RAN2 #127bis agreements:

- A cell for which SIB1 request configuration is available, can periodically broadcast SIB1.

- If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

RAN2 #127bis agreements:

- NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing K_SSB < 24 for FR1 and K_SSB < 12 for FR2, will acquire SIB1 as in legacy.

RAN1#118bis agreed about the following:

For further work of on-demand SIB1 in idle/inactive mode, it is assumed that:

-	SSB transmitted in the NES cell supporting OD-SIB1 is with K_SSB > 23 for FR1 and K_SSB >11 for FR2 if it is transmitted on sync raster

-	UE determines a cell supporting OD-SIB1 based at least on UL WUS configuration from Cell A.

From the above agreements and RAN1 discussion in FL summary R1-241068, switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission should be supported in release 19 for OD-SIB1 operation. In addition, how the UE check, detect or being informed about the broadcast status for OD-SIB1 needs to be discussed.

Observation 6: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

For NES SIB1 transmission mode switch, the following points are discussed in this contribution:

- UE expectation to the value of K_SSB in NES, if SIB1 is currently being broadcasted or provided on-demand.

- How UE can check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

- MIB TTI is 80ms. This means that MIB payload does not change every 80ms. However, with repetition MIB can be transmitted every 20ms.

For point 1) and 2) we summarize in the state diagram below, K_SSB value with respect to NES cell SIB1 transmission mode based on RAN1 and RAN2 agreements.

Observation 7: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

- K_SSB < 24 for FR1 and K_SSB < 12 for FR2, is broadcasting SIB1 as legacy and NES UE will acquire SIB1 as in legacy.

- K_SSB > 23 for FR1 and K_SSB > 11 for FR2, is providing SIB1 on-demand. UE needs to monitor SIB1 to check if it is being broadcasted within a limited window before requesting SIB1 of that cell. If UE does not detect a SIB1, UE to request OD-SIB1 for that cell.

For 3), 38.212 specifies that:

“Data arrives to the coding unit in the form of a maximum of one transport block every 80ms.”

In 38.331, it specifies that SSB periodicity is at least 80 ms, with repetition within 80 ms. Basically from 38.331 and 38.212, UE is expecting to no change in PBCH payload. Based on this, many UEs apply combining for SSB detection/decoding, mainly cell-edge UEs.

Observation 8: Legacy and NES UEs assume MIB TTI is 80ms.

To avoid impact to legacy UE, NW should ensure MIB TTI of 80ms is respected. As K_SSB is a part of MIB, hence NW cannot switch between SIB1 broadcasting state or SIB1 provided on demand with a periodicity less than 80ms. This implies that when NES cell is operating with OD-SIB1 and K_SSB > 23 for FR1 and K_SSB > 11 for FR2, UE assumes that the same value of K_SSB within 80ms.

To avoid impact to legacy UEs and to guarantee MIB TTI of 80ms (8 radio frames), NW shall consider only specific timing for the switching from broadcasting SIB1 to on-demand SIB1. RAN2 can discuss to define this specific timing as function of SFN, e.g. SFN mod 8 = 0. Hence, Rel. 19 NES UE with SIB1 request configuration intended to transmit a request for OD-SIB1, should check at least every 80ms the SIB1 transmission status based on K_SSB carried by the MIB.

Proposal 2: RAN2 to discuss the timing that NW is allowed to switch SIB1 transmission mode to avoid impact to legacy UEs while ensuring a MIB TTI of 80ms or to leave this up to NW implementation.

Proposal 3: For switching of SIB1 transmission mode, RAN2 to consider If NES UE has SIB1 request configuration of a cell, NES UE is expected to re-acquire MIB at least one every 80ms before SIB1 acquisition is completed

- About NES UE checking if SIB1 is currently being broadcasted or provided on demand before requesting SIB1

Based on above discussion, it is clear to avoid any misalignment between NES UE and NW, if NES UE has SIB1 request configuration of a cell, NES UE needs to acquire/re-acquire MIB of that cell at least once per MIB TTI.

Within MIB TTI, NES UE with SIB1 request configuration, identifies the SIB1 cell transmission mode. If NES UE is intended to request SIB1, NES UEs can determine based on WUS configuration the potential SIB1 monitoring occasions and NES UEs can monitor type0 PDCCH for SIB1 within those monitoring occasions. Having the type0 PDCCH configurations from WUS configuration, the effort for NES UE to blindly check if SIB1 is broadcasted or not, is small as legacy implementation can be re-used. In addition, RAN2 may restrict the Type0-PDCCH monitoring occasions for OD-SIB1 to be aligned with Type0-PDCCH monitoring occasions for broadcasting legacy/regular SIB1.

Proposal 4: RAN2 can discuss further the mechanism to allow NES UEs with OD-SIB1 request configuration to check if SIB1 being broadcasted or provided on-demand before requesting SIB1.

# 5	RAR response handling

RAN2 shall discuss the issue whether or not the common PRACH preamble is used for all UEs requesting on-demand SIB1 operation, and this could allow a RAR response to be interpreted/decoded by all UEs. Practically, when a UE intends to transmit the UL WUS for requesting SIB1, it can actually monitor if any RAR response is sent by network, and to check if on-demand SIB1 is already requested by other UEs. RAN1 agreed that dedicated ROs, time/frequency resources can be used for requesting OD-SIB1. To allow other UEs intended to request OD-SIB1 to monitor the RAR response and OD-SIB1, a common preamble index per SSB should be used.

Observation 9: For SIB1 requesting by assuming a common preamble used by all UEs, it allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 10: Configuration of a common preamble may be required in order to enable UEs monitoring on-demand SIB1 without transmitting UL WUS.

Based on TS38.321, a MAC PDU subheader with RAPID only is used for acknowledgment for on-demand SI request. For OD-SIB1, same clause can be used. No need for designing a new RAR response as the WID recommend low specification impact.

Proposal 5: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

# 6	RRC connected mode UEs

The release 19 work item on network energy saving (RP-242354) includes the following objective:

- Specify support for on-demand SIB1 for UEs in idle/inactive mode [RAN1/2/3] - Specify procedures and signaling method(s) for Case 2 [RAN1/2] - Case 2: UE obtains UL WUS configuration from Cell A, UE transmits UL WUS on NES Cell, UE receives on-demand SIB1 from NES Cell - Triggering method by UL WUS using PRACH - Specify inter NG-RAN node signalling at least for the configuration of UL WUS [RAN3] - Note 1: No modification of SSB will be discussed under this objective - Note 2: - UL WUS: Uplink wake-up signal - Cell A: A cell that is periodically transmitting at least its own SIB1 - NES Cell: A cell that may transmit SIB1 transmission in response to UL WUS from a UE - Note 3: - RAN1 strives to minimize impact to legacy UE - RAN1 specification impact to support this feature should be minimized

In RAN#106 following was endorsed in RP-243297:

In RAN2#128 there was discussion also on RRC_CONNECTED mode UE handling – especially concerning RLF scenarios where from RAN2 perspective UE is in RRC_CONNECTED although it requires to perform cell selection and acquire SIB1 when re-establishing connection.

Observation 11: In order to reestablish connection UE stays technically in RRC_CONNECTED and needs to acquire SIB1

If UE cannot acquire SIB1 from cells utilizing OD-SIB1 it would potentially delay reestablishment procedures before UE can find a cell providing SIB1 – leading to increased amount of RLFs.

Observation 12: If UE cannot reestablish connection in OD-SIB1 cell it would increase amount of failed re-stablishments

In our understanding motivation to limit the OD-SIB1 operation in WI discussions was to limit new mechanisms for OD-SIB1 operation in dedicated signaling and/or when UE is actually in RRC_CONNECTED and reachable by network as then NW can by implementation update SIB1 (if needed) via existing RRC ASN.1 procedures, namely dedicated dedicatedSIB1-Delivery.

In RAN2#128 the decision concerning new delivery mechanism was:

Wait one cycle of RAN discussion (i.e. to see whether WID is updated or not to handle RRC connected UEs)

In RAN2 we have agreed to provide information for OD-SIB1 operation in system information, namely SIBX. UE in RRC_CONNECTED in RAN2 discussion has been seen able to obtain such a SIBX as well. One reason for not agreeing adding the information in RRCRelease was that UE will have this from SIBX in RRC_CONNECTED as well.

Observation 13: RAN2 understanding is that UE can acquire SIBX in RRC_CONNECTED (being one reason not to include this information in RRCRelease message)

Now question is that is the understanding that UE can utilize this SIBX in the RRC_CONNECTED when RLF occurs. In this scenario UE performs cell selection and it may select cell that uses OD-SIB1 procedure. In order to reestablish connection in a cell using OD-SIB1 operation one should allow/require UE to receive OD-SIB1 in RRC_CONNECTED. It should be noted that also UE performs cell selection in case of RRCRelease and RAN2 implicitly already agreed that in this scenario UE can use SIBX (And agreed not to include WUS configuration in RRCRelease).  Thus it would seem logical to capture this also for RLF and for RRCRelease explicitly in the specification.

So technically what in stage-3 one would need is:

    - SIBX is/may be acquired in RRC_CONNECTED by UE

    - UE can use SIBX information in case of RLF and RRCRelease and it selects to OD-SIB1 utilizing cell for which nformationincluded in the acquired SIBX)

If SIBX cannot be received by RRC_CONNECTED mode ( we don’t see why that would be the case) we would need to reconsider decision of having  the WUS configuration in the RRCRelease to at least allow then UE to utilize SIBX in case of RRCRelease to IDLE/INACTIVE camp on cell utilizing OD-SIB1.

Proposal 6: If SIBX reception is not possible in RRC_CONNECTED then we should revisit decision on not having WUS configuration in RRCRelease

But we assume it is possible to receive SIBX then we only need to agree following:

Proposal 7: Agree to add in stage-3 CR UE to be allowed to use SIBX in case of cell selection (e.g. due to RLF/RRCRelease)  in order to reestabslish connection and/or camp in a cell using OD-SIB1 feature.

# 8	Conclusions:

Observation 1: For a cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell

Observation 2: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, legacy UEs does not camp on the cell.

Observation 3: For a NES cell with K_SSB > 23 for FR1 and K_SSB > 11 for FR2, RAN2 to consider the following for legacy UEs:

Observation 4: If NES UE does not acquire SIB1 request configuration of a cell, it behaves similarly with respect to NES cell for cell selection/re-selection and barring.

Observation 5: If NES UE has SIB1 request configuration of a cell, NES UE may re-selects on a cell with NCD-SSB located on the synchronization raster and acquire SIB1 with on-demand fashion even if K_SSB > 23 for FR1 and K_SSB > 11 for FR2.

Proposal 1: There is no need for additional barring mechanisms (in addition to the k_ssb signaling in MIB) to handle legacy to be able to bar cell using OD-SIB1

Observation 6: Switching between legacy periodic SIB1 transmission and on-demand SIB1 transmission is a valid issue to be considered by RAN2.

Observation 7: NES UE with SIB1 request configuration of a NES cell assumes that a NES cell, with SSB containing:

Observation 8: Legacy and NES UEs assume MIB TTI is 80ms.

Proposal 2: RAN2 to discuss the timing that NW is allowed to switch SIB1 transmission mode to avoid impact to legacy UEs while ensuring a MIB TTI of 80ms or to leave this up to NW implementation.

Proposal 3: For switching of SIB1 transmission mode, RAN2 to consider If NES UE has SIB1 request configuration of a cell, NES UE is expected to re-acquire MIB at least one every 80ms before SIB1 acquisition is completed

Proposal 4: RAN2 can discuss further the mechanism to allow NES UEs with OD-SIB1 request configuration to check if SIB1 being broadcasted or provided on-demand before requesting SIB1.

Observation 9: For SIB1 requesting by assuming a common preamble used by all UEs, it allows UEs intended to request SIB1, to monitor on-demand SIB1 transmission that is requested by other UEs, without transmitting UL WUS.

Observation 10: Configuration of a common preamble may be required in order to enable UEs monitoring on-demand SIB1 without transmitting UL WUS.

Proposal 5: RAN2 to align with legacy RAR for OSI for OD-SIB1 operation. Legacy RAR MAC PDU subheader with RAPID only to be used as NW acknowledgement for OD-SIB1 request.

Observation 11: In order to reestablish connection UE stays technically in RRC_CONNECTED and needs to acquire SIB1

Observation 12: If UE cannot reestablish connection in OD-SIB1 cell it would increase amount of failed re-stablishments

Observation 13: RAN2 understanding is that UE can acquire SIBX in RRC_CONNECTED (being one reason not to include this information in RRCRelease message)

Proposal 6: If SIBX reception is not possible in RRC_CONNECTED then we should revisit decision on not having WUS configuration in RRCRelease

Proposal 7: Agree to add in stage-3 CR UE to be allowed to use SIBX in case of cell selection (e.g. due to RLF/RRCRelease)  in order to reestabslish connection and/or camp in a cell using OD-SIB1 feature.