3GPP TSG-RAN WG2 Meeting #129	R2-2500561

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.13.3

Source: 	China Telecom

Title:	Discussion on control plane aspects for NR sidelink multi-hop relay

WID/SID:	NR_SL_relay_multihop

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the following agreements on control plane procedures and SRAP impact in multi-hop relay were reached.

Agreements in RAN2#128 - In one approach (“approach 1”) of U2N relays, each of the Intermediate Relay UEs must be in RRC_CONNECTED when the U2N remote UE is in RRC_CONNECTED. Connection establishment in the U2N remote UE first requires that each Intermediate Relay UE which is in RRC_IDLE/RRC_INACTIVE first enters RRC_CONNECTED. FFS whether connection establishment of an Intermediate Relay UE (other than the Last Relay UE) is captured in specification as connection establishment of a remote UE or a relay UE. FFS if SRB0 would be forwarded before connection establishment of each intermediate relay UE. - The figure and description under P1 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 1 (all relay UEs must be in RRC_CONNECTED when the remote UE is in RRC_CONNECTED) is adopted. - In one approach (“approach 2”) of U2N relays, Intermediate Relay UEs (other than the Last Relay UE) can be in any RRC state when the U2N remote UE is in RRC_CONNECTED. - In approach 2, any intermediate relay UE which happens to be in RRC_CONNECTED towards the last relay UE’s serving gNB and is operating as a remote UE is assumed to obtain its relaying configuration (RLC channel configuration, SRAP configuration, etc.) in dedicated signalling from the last relay UE’s serving gNB via the U2N connection. - The figure and description under P4 of R2-2410006 serves as a baseline connection establishment procedure for discussion for multi-hop U2N Relays if Approach 2 (relays other than the Last Relay may/may not remain in RRC_IDLE/RRC_INACTIVE when the remote UE is in RRC_CONNECTED) is adopted. - For the baseline procedure, the PDB for each hop is provided by the network according to gNB implementation. This does not preclude the possibility of assistance information from the UE(s). - If enhancements allow some intermediate relay UEs to remain in idle/inactive, the PDB split between the Uu hop and all remaining hops is performed by the network. FFS how to split the PDB over each of the individual remaining hops. - In multi-hop, the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s). FFS how to perform the forwarding and whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE). - Support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. 1. Support this case based on existing U2N framework, with all the UEs RRC_CONNECTED to the last relay UE’s serving cell. 2. Continue to discuss whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. - All agreements in this WI apply to both these cases unless otherwise specified. - Maintain the principle that the SRAP header enables mapping from the e2e bearer ID to the egress PC5 RLC channel. This does not preclude changes/rewrites/addition of the SRAP header by the intermediate UEs. - Remote UE ID and BEARER ID are included in Rel-19 SRAP header for multi-hop L2 U2N relay.

In this contribution, we would like to share our opinions on control plane remaining issues for multi-hop U2N relay.

- Discussion

General aspects on supporting multi-hop U2N relay

In last RAN2 meeting, it was agreed to support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. Whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment remain FFS.

For single hop U2N relay, the following aspects on RRC state of U2N relay UE are specified in TS 38.300[2].

Copied from TS 38.300 A U2N Relay UE shall be in RRC_CONNECTED to perform relaying of unicast data. For L2 U2N Relay operation, the following RRC state combinations are supported: - Both L2 U2N Relay UE and L2 U2N Remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data; and - The L2 U2N Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Regarding multi-hop U2N relay, the related U2N relays on the relay path shall follow the same principle as single hop U2N relay scenario. All the U2N Relay UEs on the relay path shall be in RRC_CONNECTED to perform relaying of unicast data. All the related multi-hop U2N relays and U2N remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data. The multi-hop U2N relay can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) or L2 U2N relay(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

The agreed baseline procedure in last RAN2 meeting follows the same principle as single hop U2N relay quite well. On top of the baseline procedure, we see no strong motivation to support additional case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. Furthermore, if we support such optimisation for multi-hop U2N relay, it will bring huge spec impact on almost all the related control plane aspects, including SI, paging, SRAP design, etc.. Moreover, it may also bring extra impacts on the support of service continuity with multi-hop indirect path switch involving intermediate relay UEs in RRC_INACTIVE or in RRC_IDLE. Otherwise, service continuity with multi-hop indirect path switch involving intermediate relay UEs in RRC_INACTIVE or in RRC_IDLE would not be supported without additional enhancement. If the optimisation case is only supported for connection establishment for simplicity, the user experience will be really bad since the service continuity is not guaranteed. Considering the limited TU for Rel-19 multi-hop U2N relay, we suggest RAN2 not support the optimisation case.

Proposal 1: The case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment is not supported.

Control plane aspects on supporting multi-hop U2N relay

Radio link failure

For radio link failure in multi-hop U2N relay scenario, the Uu RLM and RLF handling for remote UE and relay UEs can follow the same behaviour as single hop U2N relay scenario. For the intermediate relay UE, upon detecting PC5 RLF, some enhancement might be needed to indicate the RLF to the related nodes in the other direction that are not suffering the RLF. PC5-RRC message could be used to carry the indication of PC5 RLF between the relay UEs and the remote UE. The NotificationMessageSidelink message could be reused in this case. Whether to use the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3 CR drafting.

Proposal 2: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hop relay UE in other direction to indicate the PC5 RLF.

Proposal 3: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

System information

For single hop U2N relay, the in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

For multi-hop U2N relay, the same principle can be followed and it was agreed that the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s) in last RAN2 meeting. Whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE) remains FFS. In our opinion, allowing intermediate relay UE to forward available SI directly is a kind of optimisation with limited benefits. Assuming the remote UE requires SIBx and SIBy while the intermediate relay UE only has available SIBx. In such case, the intermediate relay UE still need to forward the SI request to the parent relay or last relay. Besides, it will also bring additional complicity on intermediate relay UE. Thus, we suggest not support it.

Proposal 4: RAN2 to confirm that an intermediate relay UE cannot forward available SI directly to the remote UE. The available SI is forwarded hop by hop from the last relay UE.

In [POST128][401], the details on SI handling in multi-hop relay was discussed. There are a few remaining issues highlighted by the rapporteur for further discussion. One issue is whether the intermediate relay UE can receive SI directly from SIB broadcast by the cell on Uu. From intermediate relay UE’s perspective, it is actually a remote UE as well. Thus, it shall follow the remote UE behaviour on SI handling as legacy. There is no strong motivation to enhance the intermediate relay UE to keep on tracking the broadcasted SI in Uu for multi-hop U2N relay, which may also bring extra complexity to the intermediate relay UE. Besides, based on the agreement made in last RAN2 meeting, an upper bound of Uu RSRP is configured to determine the operation as an intermediate relay UE. Thus, the intermediate relay UE is very likely to be at the edge of the cell or even out of coverage. The case raised in the post meeting discussion is more a corner case.

Proposal 5: Regarding the SI receiving of an intermediate relay UE, RAN2 confirms that the legacy SI handling as a remote UE shall be followed. Intermediate relay UE receiving SI directly from SIB broadcast by the cell on Uu is not supported.

- Conclusion

In this contribution, we discuss control plane related issues for multi-hop U2N relay. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: The case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment is not supported.

Proposal 2: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hop relay UE in other direction to indicate the PC5 RLF.

Proposal 3: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

Proposal 4: RAN2 to confirm that an intermediate relay UE cannot forward available SI directly to the remote UE. The available SI is forwarded hop by hop from the last relay UE.

Proposal 5: Regarding the SI receiving of an intermediate relay UE, RAN2 confirms that the legacy SI handling as a remote UE shall be followed. Intermediate relay UE receiving SI directly from SIB broadcast by the cell on Uu is not supported.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".