3GPP TSG-RAN WG2 Meeting #129	R2-2500564

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.6.2

Source: 	China Telecom

Title:	Discussion on Inter-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding inter-CU LTM, the following agreements were reached in last RAN2 meeting.

Agreements on inter-CU LTM:

- Rel-19 Set ID is configured for a candidate configuration. For DRB:

- Inter-CU MCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> UE performs PDCP re-establishment to all radio bearers.

- Inter-CU SCG LTM: When Rel-19 Set ID of candidate cell is different from serving cell,

> For MN terminated bearer without change of termination point, UE does not perform PDCP re-establishment. PDCP recovery is needed if it is split bearer.

> For SN terminated bearer without change of termination point, UE performs PDCP re-establishment.

> If there is change of termination point for a radio bearer (the keyToUse in the RadioBearerConfig is different from the keyToUse in the current UE configuration), UE performs PDCP re-establishment.

- For SRBs in inter-CU SCG LTM, Rel-19 ID is used to determine whether PDCP re-establishment or PDCP SDU discard is performed for LTM execution for SRB3.

- For SRB1/2 in inter-CU SCG LTM, PDCP re-established is not performed based on NW configuration (PDCP re-establishment flag and SDU discard).

- RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

- RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

- From RAN2 perspective, the following coexistence cases in NR-DC can be supported:

- Case 1: Intra-CU MCG LTM + Inter-CU MCG LTM

- Case 2: Intra-CU SCG LTM + Inter-CU SCG LTM

- In coexistence cases of inter-CU MCG/SCG LTM and intra-CU MCG/SCG LTM, when inter-CU MCG or SCG LTM is executed, it’s up to the NW to ensure that maintained LTM candidate configurations are valid, e.g. reconfigure or release invalid intra-CU MCG/SCG LTM candidate configurations. UE does not autonomously release invalid intra-CU candidate configurations.

- RAN2 to support intra-CU SCG LTM in MN RRC message (i.e. MN RRCReconfiguration message), in addition to SN RRC message.

- RAN2 to support intra-CU MCG LTM with SCG configuration.

- It’s up to NW to ensure that the complete configuration includes the MCG part and SCG part configuration when UE combines the reference and candidate configuration for inter-CU SCG LTM.

- RAN2 assumes that how to indicate the list of candidate PSCells from source SN to MN is up to RAN3. From RAN2 perspective, in INM, source SN may send measurement results of candidate PSCells to the MN. The MN then forwards the measurement results to the candidate SN(s), and then the candidate SN(s) determines the LTM candidate cells based on the measurement results and the upper limit for the number of PSCells that can be prepared by each candidate SN. The existing IEs defined in INM can be reused as a baseline.

However, there are still some open issues on the details of inter-CU LTM in both non-DC and DC scenarios. In this contribution, we would like to share our opinions on the following remaining issues for inter-CU LTM.

- Coexistence of one-shot LTM and subsequent LTM;

- Inter-CU MCG LTM.

- Discussion

Coexistence of one-shot LTM and subsequent LTM

In Rel-16/17, the feature of CPAC is introduced. To reduce signalling overhead, subsequent CPAC is introduced in Rel-18. Since the procedure of CPAC and subsequent CPAC are introduced separately in different release, the coexistence issue was discussed in Rel-18. It is agreed that coexistence of CPAC and subsequent CPAC for the same candidate SN is not supported.

For LTM, one-shot LTM and subsequent LTM may influence the signalling interaction among the network nodes. It is better to clarify the support of subsequent LTM is mandatory or optional for a certain candidate gNB or SN.

Taking inter-SN SCG LTM as an example, Fig.1 illustrates a potential stage-2 signalling flow for SN-initiated inter-SN SCG LTM based on the agreements made in previous RAN2 meetings. In step 5~6, the MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. Then the source SN generates the common CSI resource configuration for L1 measurement on candidate PSCells. The source SN sends the generated common CSI resource configuration and the updated source SCG configuration to the MN. If the support of subsequent LTM is mandatory, step 7~8 shall be mandatory to transfer the common CSI resource configuration and the collected information of candidate cells to each candidate SN. The candidate SN(s) responds with the updated candidate SCG configuration to the MN. If the support of subsequent LTM is optional, step 7~8 could be optional as well.

Fig.1 Stage-2 signalling flow for SN-initiated inter-SN LTM without MN changed

Proposal 1: RAN2 clarifies whether the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Inter-CU MCG LTM

In last RAN2 meeting, RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel-19. In RAN2#127bis, the coexistence of MCG LTM and SCG LTM was discussed with the following agreement reached.

Agreements in RAN2#127bis

RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

During the post meeting discussion of RAN2#128 on drafting TS 37.340, companies seemed to have different understandings on the above agreements on whether LTM for simultaneous PCell and intra-SN PSCell change had been supported in Rel-19. In our opinion, the support of inter-CU MCG LTM with intra-SN PSCell change doesn’t mean the intra-SN PSCell change is also triggered by LTM. Thus, simultaneous LTM for PCell change and LTM for PSCell change is still not supported based on the agreement that the NW implementation will ensure simultaneous execution for both MCG and SCG not to happen. To make it clear, we suggest RAN2 further clarify on this issue.

Proposal 2: RAN2 clarifies that simultaneous LTM for PCell change and LTM for PSCell change is not supported.

In previous RAN2 meeting, the following scenarios were agreed to be supported for inter-CU MCG LTM.

Agreements in RAN2#127

SCG configuration can be changed in inter-CU MN and leave how to handle SCG part up to NW implementation (e.g. release or reconfiguration).

Agreements in RAN2#128

RAN2 confirms that inter-CU MCG LTM with SCG addition is supported assuming no much specification effort is required. If there are much specification efforts, we will not have it.

RAN2 confirms that the inter-CU MCG LTM with intra-SN PSCell change is supported in Rel19.

Regarding the details on inter-CU MCG LTM in DC scenarios, further discussion might be needed. For inter-CU MCG LTM in DC, the interaction between the UE and the MN is the same as that in non-DC case and the stage-2 signalling flow in draftCR of TS 38.300 can be referred to. For the interaction between network nodes, the procedure of CHO with SN as illustrated in Figure 10.19.2-1 of TS 37.340 could be followed as baseline. In R2-2406386, the following Stage-2 signalling flow for inter-CU MCG LTM with SCG is provided as an example.

Fig.2 Procedure of inter-CU MCG LTM with SCG (Fig.4 in R2-2406386)

As illustrated above, for an inter-CU MCG LTM without SN change, the source SN and the target SN shown in Fig.2 are the same node. For an inter-CU MCG LTM with SN addition, the source SN and steps involving the source SN in Fig.2 are ignored. For the case of inter-CU MCG LTM with SCG release, it is also covered in Fig.2. Since similar procedure of Conditional Handover with SN could be followed, the spec impact seems not too much. Thus, we suggest RAN2 confirm to support inter-CU MCG LTM with SCG addition in Rel-19.

Proposal 3: RAN2 confirms to support inter-CU MCG LTM with SCG addition in Rel-19.

Although similar procedure as CHO with SN could be followed for inter-CU MCG LTM with SCG, it is better to have a separate stage-2 signalling flow and corresponding step descriptions in TS 37.340. The example flow chart in R2-2406386 could be used as a baseline and details can be discussed in CR drafting phase.

Proposal 4: Fig.2 (i.e. Fig.4 in R2-2406386) could be taken as baseline for stage-2 signalling flow of inter-CU MCG LTM with SCG. The details can be discussed in CR drafting phase.

- Conclusion

In this contribution, we discuss inter-CU LTM related aspects in Rel-19. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: RAN2 clarifies whether the support of subsequent LTM is mandatory or optional for a certain candidate gNB or candidate SN.

Proposal 2: RAN2 clarifies that simultaneous LTM for PCell change and LTM for PSCell change is not supported.

Proposal 3: RAN2 confirms to support inter-CU MCG LTM with SCG addition in Rel-19.

Proposal 4: Fig.2 (i.e. Fig.4 in R2-2406386) could be taken as baseline for stage-2 signalling flow of inter-CU MCG LTM with SCG. The details can be discussed in CR drafting phase.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

3GPP TS 38.300 18.1.0: " NR; NR and NG-RAN Overall Description; Stage 2".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB

R2-2406386	Discussion on inter-CU LTM	ETRI