3GPP TSG-RAN WG2 Meeting #129                                        R2-2500605

Athens, Greece, Feb 17th – 21th, 2025

Source:        ZTE Corporation

Title:           Discussion on LCM for positioning use case

Agenda item:   8.1.2.3

Document for:  Discussion and Decision

- Introduction

The revised WID [1] of Rel-19 NR_AIML_air related to AI/ML positioning is given as follows:

- Positioning accuracy enhancements, encompassing [RAN1/RAN2/RAN3]: - Direct AI/ML positioning: - (1st priority) Case 1: UE-based positioning with UE-side model, direct AI/ML positioning - (2nd priority) Case 2b: UE-assisted/LMF-based positioning with LMF-side model, direct AI/ML positioning - (1st priority) Case 3b: NG-RAN node assisted positioning with LMF-side model, direct AI/ML positioning - AI/ML assisted positioning - (2nd priority) Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning - (1st priority) Case 3a: NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning - Specify necessary measurements, signalling/mechanism(s) to facilitate LCM operations specific to the Positioning accuracy enhancements use cases, if any - Investigate and specify the necessary signalling of necessary measurement enhancements (if any) - Enabling method(s) to ensure consistency between training and inference regarding NW-side additional conditions (if identified) for inference at UE for relevant positioning sub use cases - Note: RAN1 will not discuss any proposals targeting 2nd priority issues in Q4 of 2024.

This contribution provides our view on LCM for AI/ML positioning use case, focusing on use case 1/2a/2b (high priority from RAN2 perspective) and use case 3a/3b (low priority from RAN2 perspective).

- Discussion

The inference procedure of 5 AI/ML positioning use cases described in the WID are briefly depicted as follows:

Figure 1. Use case 1, model inference is at UE side (1st priotrity)

Figure 2. Use case 2a, model inference is at UE side (2st priotrity)

Figure 3. Use case 2b, model inference is at LMF side (2st priotrity)

Figure 4. Use case 3a, model inference is at gNB side (1st priotrity)

Figure 5. Use case 3b, model inference is at LMF side (1st priotrity)

It can be seen that among the 5 use cases, only use case 1, use case 2a and use case 2b are involved with LMF-UE signaling, wherein use case 1 is the 1st priority.

Observation 1: Use case 1, use case 2a and use case 2b have clear RAN2 LPP specification impacts, wherein use case 1 is the 1st priority.

LCM of UE involved use case (use case 1, 2a and 2b)

Functionality/AI model activation

Based on the discussion so far, a comprehensive understanding of functionality and AI model is, the functionality can be recognized per use case, different functionality can be distinguished by different UE capability and different UE applicable functionality. Under each functionality, there can be multiple AI models, different models can be distinguished by different NW/UE additional conditions.

Figure 6. An example relationship between functionality and AI model

RAN2#125-bis [2] has made the following agreement for UE side model:

For UE-sided model, for the functionality management, the “network decision, network-initiated” AI/ML management is supported as a baseline. The following can be considered further “UE autonomous, decision reported to the network”, “Network decision, UE-initiated” (i.e. proactive approach). “UE-autonomous, UE’s decision is not reported to the network” is not considered for Rel-19.

Based on the above understanding of functionality and AI model, there can have following approaches to control the LCM:

- Approach 1: LMF activates/deactivates the functionality, and LMF activates/deactivates the AI model of the functionality.

  - Approach 1 means whether UE activates/deactivates use case 1, use case 2a or use case 2b is determined and indicated by LMF. For use case 1 and use case 2a where the AI model is at UE side, LMF should further determine which AI model under the indicated use case should be activated/deactivated. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE, and LMF should also know the relationship between UE side additional condition and AI model, so that LMF knows which AI model is available/unavailable to UE.

- Approach 2: LMF activates/deactivates the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality

- Approach 2 means whether UE activates/deactivates use case 1, use case 2a or use case 2b is determined and indicated by LMF. For use case 1 and use case 2a where the AI model is at UE side, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions. That is to say, LMF needs to get UE’s applicable functionality to know which functionalities is available/unavailable to UE.

- Approach 3: LMF allows UE to activate/deactivate the functionality, and LMF allows UE to activate/deactivate the AI model of the functionality, and UE reports the activated/deactivated functionality to LMF

- Approach 3 means LMF can allow UE to activate/deactivate use case1 or use case 2a according to UE’s applicable functionalities. That is to say, LMF can allow UE to report AI UE location (using use case 1), or to report AI intermediate feature (using use case 2a). Then UE should let LMF know which functionalities is activated/deactivated (e.g., using different measurement report). Further, LMF can allow UE to activate/deactivate the AI model of the indicated use case according to UE’s own additional condition and already provided NW side additional conditions.

- Or Approach 3 can be utilized when LMF configures some fallback functionalities to UE. That is to say, LMF configures more than one positioning methods to UE, wherein the AI positioning case 1 is the highest priority. But, LMF also allows UE to activate other configured positioning methods (e.g., non-AI positioning method) if AI positioning method is not available right now -- to acquire timely and accurate measurement result.

As we analysed above, NW side additional condition is already provided to UE. It is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE, i.e., approach 1 is not preferred in AI/ML positioning. Also as mentioned above, the relationship between additional condition and AI model is hard to specify, so LMF is hard to know such information. Since UE knows the UE side additional condition and NW side additional condition, it is more efficient to let UE decide the AI model under each functionality, i.e., go with approach 2 or approach 3.

Observation 2: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 1: For AI/ML positioning use case 1 and 2a, RAN2 to consider the following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality (e.g., when fallback positioning method is configured)

Proposal 2: For AI/ML positioning use case 1 and 2a, LMF allows UE to activate/deactivate the AI model of the activated functionality.

Functionality management as individual positioning method

For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

Proposal 3: For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

In legacy LPP specification, LMF can schedule different positioning methods to UE, wherein each positioning method has different kinds of reporting content. Different positioning method can be scheduled in parallel or in series, but they are scheduled independently from each other. Also each legacy positioning method has the type of UE-assisted positioning or UE-based positioning, where UE-assisted positioning means UE can report measurement and LMF is to calculate the UE location, UE-based positioning means UE can make the measurement and calculate the location by the UE itself. The table from TS38.305 [3] describes the mapping of the type and the positioning method:

Table 4.3.1-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, LMF-based | NG-RAN node assisted | SUPL Note 8 |
| --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | Yes |
| OTDOA Note1, Note 2 | No | Yes | No | Yes |
| E-CID Note 4, Note 7 | No | Yes | Yes | Yes for E-UTRA |
| Sensor | Yes | Yes | No | No |
| WLAN | Yes | Yes | No | Yes |
| BluetoothNote 9 | Yes | Yes | No | No |
| TBS Note 5 | Yes | Yes | No | Yes (MBS) |
| DL-TDOA | Yes | Yes | No | Yes |
| DL-AoD | Yes | Yes | No | Yes |
| Multi-RTT | No | Yes | Yes | Yes |
| NR E-CID | No | Yes | Yes | Yes (DL NR E-CID) |
| UL-TDOA | No | No | Yes | Yes |
| UL-AoA | No | No | Yes | Yes |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only OTDOA based on LTE signals is supported. NOTE 3: Void NOTE 4: This includes Cell-ID for NR method when UE is served by gNB. NOTE 5: In this version of the specification only for TBS positioning based on MBS signals. NOTE 6: Void NOTE 7: Enhanced Cell ID based on LTE signals. NOTE 8: This shows whether the positioning method is supported by SUPL ULP [16]. NOTE 9: UE-based positioning mode is supported only for Bluetooth AoD positioning method. |  |  |  |  |

In AI/ML positioning, it is similar that different functionality (i.e., different use case) has different kinds of reporting content, i.e., the reporting content of use case 1 is AI UE location, the reporting content of use case 2a is AI intermediate feature, the reporting content of use case 2b is PRS channel measurement. So from signaling structure perspective, AI/ML positioning functionality can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities.

Proposal 4: From RAN2 signaling structure perspective, AI/ML positioning functionality (use case) can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities (use cases).

There is a discussion on whether the AI function should reuse legacy positioning method, or a new positioning method should be introduced for AI/ML positioning. We support to introduce new positioning method for AI/ML positioning because of the following reasons:

- The UE’s capability is per-method designed. That is to say at least from spec perspective, there can be UEs that only support AI/ML positioning, not support legacy positioning method. From implementation perspective, some dedicated UE with specific kinds of usage such as AI/ML function may be implemented in the future, for specific scenarios or enhancements. Therefore such UE will not have legacy positioning capability, mixing the signaling will cause such UEs unable to decode the module;

- Currently legacy positioning method has mandatory reporting quantity, e.g., DL-TDOA method has mandatory reporting quantity RSTD (IE nr-RSTD), DL-AoD method has mandatory reporting quantity PRS-RSRP (IE nr-DL-PRS-RSRO-Result). If AI/ML reporting quantities are going to be included in the legacy positioning methods, how to deal with the legacy mandatory reporting quantity? That is to say, even if UE is only requested with AI/ML measurement reporting, UE has to make legacy measurement and report. This will also lead to a NW configuration restriction that LMF should always schedule the AI/ML positioning together with legacy positioning. Such positioning method-coupling is not aligned with the principle that different positioning methods should be isolated, from both UE capability and NW scheduling perspective.

- Not only usecase 1 but also usecase 2a and usecase 2b will be specified. The model output of usecase 1 is AI UE location, It is unclear that which positioning method should the usecase 1 reuse; The model output of usecase 2b is PRS channel measurement, which is totally different from the reporting of legacy positioning methods, so it is also unclear that which positioning method should the usecase 2b reuse;

- RAN1 will further design different measurement request signaling towards different usecases (usecase 2a and usecase 2b), it will be clearer and extendible to include them into a new method, rather than mix up with the legacy method.

Observation 3: For AI/ML positioning, if reusing legacy positioning method structure, the following disadvantages are observed:

- The UEs only supporting AI/ML positioning cannot decode the module of legacy positioning method;

- If only AI/ML positioning is requested by NW, UE cannot handle the mandatory reporting quantities in the legacy positioning methods;

- It is unclear that which legacy positioning method should the usecase 1 and usecase 2b reuse;

- Mixing AI/ML measurement request/report with legacy method’s measurement request/report causes poor readability and poor extendibility of the spec.

Therefore, a new positioning method is preferred for AI/ML positioning. The following options are most feasible to map the functionality to the new positioning method:

- Option 1: Introduce a new AI/ML positioning method for use case 1, 2a and 2b:

  - This is the most straightforward way of mapping. To be specific, UE-based positioning can be UE reporting AI UE location (use case1), UE-assisted positioning can be UE reporting AI intermediate feature (use case 2a) or PRS channel measurement (use case 2b), depending on LMF’s scheduling or UE’s decision.

- Option 2: Introduce a new AI/ML positioning method for use case 1 and 2a; and introduce another new AI/ML positioning method for usecase 2b:

- This is to differ UE-side model and LMF-side model. Use case 1 and 2a are UE-side model, so it is reasonable to introduce a new positioning method dedicated for the UE who has the AI model capability. To be specific, UE-based positioning can be UE reporting AI UE location (use case 1), UE-assisted positioning can be UE reporting AI intermediate feature (use case 2a).

Option 1 is the most straightforward way of mapping for AI/ML positioning. Option 2 requires two new positioning methods to be introduced which has larger spec impact. Therefore we slightly prefer option 1.

Proposal 5: Support to introduce a new AI/ML positioning method in LPP for use case 1, 2a and 2b, in which usecase 1 is UE-based positioning, usecase 2a and 2b are UE-assisted positioning.

Fallback from AI positioning method to non-AI positioning method

Legacy positioning only requires UE to support legacy/normal UE processing capability. However AI positioning method not only requires UE to report normal UE processing capability, but also requires UE to confirm AI availability at the time. It means even AI positioning has better performance than legacy positioning, AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

Observation 4: AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

The LCS client does not care whether UE uses AI positioning or legacy positioning, the LCS client only sends positioning request of the UE with a set of QoS requirement, including a requested maximum time for returning measurements/positioning estimates. When LMF thinks it is ok to activate AI positioning and activates AI, UE may report Error when the AI functionality becomes non-applicable. Then LMF has to activate another positioning method, and UE has to return the positioning measurement/positioning estimate of the scheduled another positioning method. Such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve.

Observation 5: If AI positioning fails and then UE is scheduled with another positioning method, such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve.

To solve the issue, LMF can schedule AI positioning together with other fallback positioning method(s) to UE, UE will take AI positioning as first priority. If it fails, UE can use the fallback positioning method to perform positioning and report positioning measurements/estimates on time. For usecase 1, UE needs to send AI-estimated location to LMF, so the fallback method can be DL-TDOA or DL-AoD which are UE-based positioning methods.

For the concern that LMF may transmit ProvideAssistanceData / RequestLocationInformation twice, since the ProvideAssistanceData / RequestLocationInformation of different positioning method can be embedded in one LPP message type via SEQUENCE structure, only one LPP message is enough. So the latency can also be reduced.

Figure 7. (a) LMF does not configure fallback method vs. (b) LMF configures fallback method

Proposal 6: LMF can schedule AI positioning method together with DL-TDOA or DL-AoD positioning method to UE, if AI positioning fails/non-applicable, UE can use the fallback DL-TDOA or DL-AoD to perform positioning and report positioning measurements/estimates on time.

LCM procedure in the new positioning method

In 37.355 logic, one positioning method contains the following 5 basic procedures:

Figure 8. LPP signaling in a positioning method in current spec

The signaling contained in the new AI/NL positioning method should be designed to support all of training, inference and performance monitoring.

Proposal 7: The signaling of the new AI/ML positioning method should support training, inference and performance monitoring function.

And this new AI/ML positioning method also have such baseline to contain the 5 basic procedures. Next we will discuss what signaling should be included in the newly introduced AL/ML positioning method, for each period, i.e., training/inference/monitoring.

Capability transfer procedure

RAN2#126 [4] has agreed to reuse Capability transfer procedure to indicate supported AI/ML positioning capabilities, and agreed to reuse Location Information transfer procedure to provide the results of the UE sided model inference:

RAN2#126 agreement The LPP Capability Transfer procedures (RequestCapabilities/ProvideCapabilities messages) are used to indicate supported AI/ML positioning capabilities. FFS how to handle dynamic capabilities, depending on further RAN1 progress and understanding of the functionality. At least for Case 1, existing LPP procedures related to Location Information Transfer (RequestLocationInformation/ ProvideLocationInformation messages) are used for providing the results of the UE sided model inference operation. FFS further details on signaling enhancements.

So the capability transfer procedure should be common for training, inference and monitoring.

RAN2#127-bis has agreed an example procedure for inference:

The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: - Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. - Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. - Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). - Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. - Step 5: The LMF requests the inferred location information using the LPP request location information message. - Step 6: UE reports the inferred location using LPP provide location information message. Whether the inference configuration is provided in step 3 or/and step 5 is FFS (to be revised based on RAN1 progress). Whether network side additional condition is needed and what it contains is FFS (to be revised based on RAN1 progress). FFS whether LMF controls the UE sending unsolicited LPP provide capabilities (i.e. whether step4 is sent reactively or proactively). FFS the signalling details.

RAN2 agreed that dynamic UE capability and static UE capability should be both contained in LPP Provide Capabilities message, wherein the dynamic UE capability can use unsolicited report, static UE capability can use solicited report.

In legacy mechanism, the unsolicited report is not controlled by LMF, i.e., UE is allowed to report dynamic UE capability at any time when the UE feels the need. In AI/ML positioning, LMF also do not need to control UE sending unsolicited LPP provide capabilities, similar like legacy mechanism. In addition, LMF does not know when/how the UE’s dynamic capability will change, and LMF does not know the frequency of change. It is hard for LMF to control such unsolicited report.

Proposal 8: Do NOT support LMF to control the UE sending unsolicited LPP provide capabilities using explicit signaling.

Assistance data transfer procedure

For usecase 1 and usecase 2a, UE needs to get training data set for training. The training data set can be UE’s own channel PRS measurement (with the label of UE’s own location), or can be other PRU’s PRS channel measurement (with the label of other PRU’s location/intermediate feature).

To acquire the UE’s own PRS channel measurement and the UE’s own location, UE needs to get PRS configuration and/or TRP information in advance. So, the LMF should be able to provide PRS configuration and/or TRP information to a UE (i.e., green arrows in the figure 8).

LMF should also be able to provide other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to a UE, for AI/ML training or monitoring (see green arrows in the figure 8).

Figure 9. LMF provides PRS configuration, TRP information and other UE/PRU’s PRS channel measurement with PRU/UE location/intermediate feature to target UE for AI training

The provision of such assistance data for AI/ML training can be embedded in ProvideAssistanceData message. And the request of such assistance data for AI/ML training can be embedded in RequestAssistanceData message.

Proposal 9: For usecase 1 and usecase 2a, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Different training data set associated with different TRP information will have different impact on UE AI model’s performance, for example, the model trained using DL-PRS measurement results from 20 TRPs will be more accurate than using DL-PRS measurement results from 3 TRPs. To suit the current UE’s condition or given QoS requirement for AI/ML positioning, UE can request some specific assistance data that will have impact on model training, e.g.:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

LMF may also provide other PRU’s measurement and PRU label to target UE for training. So UE is better to request the number of PRUs that the UE wants to receive the corresponding measurement and labels.

Proposal 10: UE can request some specific assistance data that will have impact on model training, as following:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Location information transfer procedure

Training

When UE is scheduled by the positioning method to do training, since the AI model is at UE side, UE only needs to get training data set via ProvideAssistanceData and then perform training, UE does not need to report model output to LMF. So when the AI positioning method is scheduled for training, location information transfer is not needed.

Proposal 11: For usecase 1 and 2a, if the AI/ML positioning method is scheduled for UE training, UE does not need to report location estimate or measurement in the AI/ML positioning method.

Monitoring

RAN1#116-bis [5] has the following agreement on performance monitoring:

Agreement For model performance monitoring of AI/ML positioning Case 1, for model performance monitoring metric calculation in label-based model monitoring, study the feasibility, benefits, and potential specification impact of the following options with regard to how to generate information on ground truth label: - Option A. The target UE side performs monitoring metric calculation. - Option A-1. At least information on ground truth label of the target UE is generated by LMF and provided to the target UE. - In one example, target UE and/or gNB sends measurement (e.g., legacy measurement) to LMF so that LMF can derive the information on ground truth label. - Option A-2. At least position calculation assistance data (e.g., existing information for UE-based positioning method) is provided from LMF to the target UE. - Option A-3. Reuse Rel-18 assistance data transfer framework from LMF to the target UE, where the PRU measurement (e.g., legacy measurement) and the corresponding PRU location are sent via LMF to the target UE. - Option A-4. PRU measurement (and the corresponding PRU location if not already known at the UE-side) are sent from PRU to the target UE side (e.g., target UE, OTT server). - Note: Option A-4 can be realized by implementation in a manner transparent to specification if the PRU sends information to the target UE side in a proprietary method. - Option B. The LMF performs monitoring metric calculation. - Option B-1. at least inference result (i.e., the model output corresponding to target UE’s channel measurement) of the target UE is sent by the target UE to LMF. - Option B-2. PRU’s channel measurement is sent via LMF to the target UE, and the inference result (i.e., the model output corresponding to PRU’s channel measurement) is sent by the target UE to LMF. Note: exact method to perform the monitoring metric calculation is up to implementation. Note: Other options are not precluded.

Based on the agreement, there are 4 kinds of performance monitoring control procedure under RAN1’s decision:

- Option 1: UE generates monitoring metric, UE makes monitoring decision;

- Option 2: LMF generates monitoring metric, LMF makes monitoring decision;

- Option 3: UE generates monitoring metric, LMF makes monitoring decision;

- Option 4: LMF generates monitoring metric, UE makes monitoring decision.

Option 1 and option 2 appear to have no LPP specification on performance monitoring.

If option 3 is adopted, LMF needs to request UE to perform monitoring, and UE needs to report monitoring metric to LMF.

If option 4 is adopted, LMF needs to indicate monitoring metric to UE.

For the above potential spec impact, RAN2 needs to discuss whether a new set of signaling/procedure dedicated for performance monitoring is necessary, or to embed it in other existing procedures (e.g. Location Information transfer procedure).

Proposal 12: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer, or to include the performance monitoring metric transfer in the ProvideLocationInformation message.

Abort procedure

In current LPP specification, the Abort message is to allow the UE or LMF to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from the UE).

Therefore, the abort procedure can be reused in this new AI positioning method for LMF to deactivate a functionality of UE. For example, if LMF makes the monitoring decision and decides the current AI/ML positioning result is bad, LMF can send the Abort message to abort the on-going PRS measurement/report of the AI positioning method.

Proposal 13: The Abort message should be used in the AI/ML positioning method to deactivate a functionality of UE.

Error procedure

In current LPP specification, the Error message is to indicate an error occurs at LMF or UE side during an on-going positioning method. For this new AI/ML positioning method, the Error message can be reused with some new error cause indication dedicated for AI/ML positioning use cases. RAN2#128 has already agreed to use Error message when the AIML based positioning method becomes non-applicable.

The error message can indicate error occurred during training, e.g., UE does not get enough model training data to train a model, or UE cannot download or acquire a AI model, etc.; The error message can also indicate error occurred during inference, e.g., UE cannot generate a AI UE location using a AI model, or UE cannot generate a AI intermediate feature using a AI model, etc.

Proposal 14: New error causes should be dedicated for specific AI/ML procedures (e.g. training or inference or monitoring).

Consistency between training and inference

There is a discussion on how to ensure the consistency between training and inference. The basic assumption is that, each AI model that UE trains should be under some certain condition, and the trained AI model should also be used for inference under the same or similar condition to achieve higher accuracy. From specification perspective, this means the set of attribute/parameter/condition of the training data input of an AI model should be same or similar as the set of attribute/parameter/condition of the inference data input.

For AI/ML positioning usecase 1, UE receives and measures TRP’s PRS for training and inference, so the set of attribute/parameter/condition of the training data input or inference data input can include NW additional conditions such as TRP location, TRP synchronization error, TRP number, PRS configuration, etc. It can be seen that for AI/ML positioning, the aforementioned NW additional conditions can already be indicated to the UE in current LPP spec since Rel-16.

Since detailed NW side additional condition is already provided to UE without any privacy concern, there is no need to use associated ID to represent a specific kind of NW side additional conditions. In addition, it is hard to set up explicit relationship between NW additional conditions and AI models, for example, an AI model can have good generalization and it can work well under several different additional conditions.

Observation 6: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

Proposal 15: For AI/ML positioning usecase 1 and 2a, ensuring the consistency between training and inference does not need an associated ID.

LMF is the node to provide inference configuration to UE, UE should strictly following NW’s configuration to perform inference. But LMF does not know which kind of inference configuration best suits the training configuration using which UE trained its AI model, e.g., how many number of TRPs used during training, which area the used TRPs locate on, the synchronization error range that the used TRP belongs to, etc. Therefore, UE should be able to request some specific TRP attributes/NW side additional conditions (for inference) that matches UE’s current well-trained AI model to ensure the training-inference consistency. The specific request can include the wanted number of TRPs, the wanted area of TRPs, the wanted synchronization error between TRPs for inference.

Since different AI model has different generalization and it is only known by UE, so only the UE knows how much difference the training and inference owns that will not have big impact to the inference performance, and to ensure the QoS requirement. The UE is able to ensure the requested value of TRP attribute to be consistent with the value used during training, and the UE ensures such difference can be covered by the good generalization performance.

The above request can reuse the current on-demand PRS request procedure.

Observation 7: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 16: UE should be able to request some specific NW side additional conditions that suits the UE’s current AI model, e.g., the wanted number of TRPs, the wanted area of TRPs, the wanted synchronization error between TRPs for inference.

LCM of gNB involved use case (use case 3a and 3b)

Similar like section 2.1, the gNB side functionality can also be recognized as use case 3a where gNB reports AI intermediate feature to LMF, and as use case 3b where gNB reports SRS channel measurement to LMF.

For use case 3a, gNB’s model input is SRS’s channel measurement derived from the SRS that UE sends, the ground truth label is the AI intermediate features. Therefore UE additional condition such as UE location, UE velocity, etc., can also have impact on gNB’s model performance. That is to say, different gNB-side AI models can correspond to different UE or gNB additional conditions;

For use case 3b, LMF’s model input is SRS’s channel measurement derived from the SRS that UE sends, the ground truth label is the UE’s location. Therefore UE additional condition such as UE location, UE velocity, etc., can also have impact on LMF’s model performance. That is to say, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Observation 8: For use case 3a, different gNB-side AI models can correspond to different UE or gNB additional conditions; for use case 3b, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Based on the above understanding of functionality and AI model, there can have following approaches to control the LCM:

- Approach 1: LMF activates/deactivates the gNB functionality, and LMF activates/deactivates the AI model of the functionality.

  - Approach 1 means whether gNB activates/deactivates use case 3a or 3b is determined and indicated by LMF. For use case 3a where the AI model is at gNB side, LMF should further determine and indicate to gNB that which AI model should be activated/deactivated. That is to say, LMF needs to get gNB’s applicable functionality to know which functionality is currently available/unavailable to gNB, and LMF should also know the relationship between gNB/UE side additional condition and the AI model, so that LMF knows which AI model is currently available/unavailable to gNB.

- Approach 2: LMF activates/deactivates the gNB functionality, and LMF can allow gNB to activate/deactivate the AI model of the functionality

- Approach 2 means whether gNB activates/deactivates use case 3a or 3b is determined and indicated by LMF. For use case 3a where the AI model is at gNB side, LMF can allow gNB to activate/deactivate the AI model according to gNB/UE’s additional condition. That is to say, gNB needs to get UE’s additional condition to know which AI model should be activated/deactivated.

- Approach 3: LMF can allow gNB to activate/deactivate the functionality, and LMF can allow gNB to activate/deactivate the AI model of the functionality, and gNB reports the activated/deactivated functionality to LMF

- Approach 3 means LMF can allow gNB to activate/deactivate use case3a or use case 3b according to gNB’s own applicable functionalities. That is to say, LMF can allow gNB to report AI intermediate feature (using use case 3a) or to report SRS channel measurement (using use case 3b). Then gNB should let LMF know which functionalities is activated/deactivated. Further, LMF can allow gNB to activate/deactivate the AI model according to gNB/UE’s additional condition.

- Letting LMF know gNB’s decision of functionality may not need new explicit signaling since in AI/ML positioning, different functionalities has different kinds of reporting content. LMF can differentiate different kinds of functionality according to different kinds of reporting content that gNB reports.

As mentioned above, the relationship between additional condition and AI model is hard to specify, so LMF is hard to know such information.

It can be seen that approach 2 and 3 both require gNB to know the UE’s additional condition to make AI model decision. If the UE is PRU then we think the problem can be easily solved, i.e., gNB can get the PRU’s current additional condition and get the SRS sent by the PRU, gNB can activate/deactivate the AI model according to PRU/gNB additional conditions.

Since gNB can know the PRU side additional condition and gNB side additional condition, it is more efficient to let gNB decide the AI model under each functionality, i.e., go with approach 2 or approach 3.

Proposal 17: For AI/ML positioning use case 3a and 3b, RAN2 assumes the following approaches can be used for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows gNB to activate/deactivate the functionality (e.g., when fallback positioning method is configured )

Proposal 18: For AI/ML positioning use case 3a, RAN2 assumes that LMF can allow gNB to activate/deactivate the AI model of the functionality.

Proposal 19: For use case 3a, RAN2 assumes that gNB should to know the PRU side additional condition to determine an AI model. FFS on how gNB gets such information.

This section should be low priority in RAN2 discussion as the main signaling is between gNB and LMF, which is RAN3’s scope. However RAN3 will start their discussion in October meeting with only 0.5 TU, it will be more efficient if RAN2 can make some recommendation or assumptions on use case 3a and 3b from RAN2 perspective, and send LS to them. Also, it is not hard to make such recommendation or assumptions, since the principle of LMF-UE and LMF-gNB use cases are very similar, we can make the recommendation or assumptions of LMF-gNB use cases according to RAN2’s agreement on LMF-UE use cases.

Proposal 20: Send LS to RAN3 about RAN2’s agreement of LCM for use case 3a and use case 3b.

Discussion on SA2’s LS

SA2 has sent a LS S2-2501133 to RAN2, asking us to provide to provide current progress and future work plans on data types and procedures defined for case 2b:

- 1 Overall description WID AI/ML_CN in SA2 has defined the new feature that LMF-based AI/ML Positioning in R19, covering Case 2b as studied in TR 38.843 by RAN WGs (Case 3b is also covered). The work done by SA2 includes at least the following aspects: - As defined in TS 23.273, the LMF may perform location calculation based on ML model, which was trained by NWDAF containing MTLF or the LMF. - AS defined in TS 23.288, the LMF may retrieve ML model for AI/ML Positioning from NWDAF containing MTLF. - AS defined in TS 23.273, the LMF collects input data from RAN and UE and may also expose to NWDAF to train or monitor the performance of the ML models for LMF-based AI/ML positioning. To RAN 1 and RAN 2: For case 2b, SA2 is working on some procedures in corresponding R19 specifications, but has also captured the following Editor’s Notes on the interaction between the LMF and UE which depend on RAN progress: Editor's note: The input data used for AI/ML based positioning will be decided by RAN WGs. Editor's note: What input data collected from UE and NG-RAN to LMF for LMF-based AI/ML Positioning will be determined by RAN WG1. How to collect the input data for LMF-based AI/ML Positioning calculation need coordination with RAN WGs. Editor's note: The collected input data for LMF-based AI/ML Positioning model training will be determined by RAN1. How to collect the input data for LMF-based AI/ML Positioning model training need coordination with RAN WGs. Q1: SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

Currently RAN2 is still working on the first priority usecase 1, and does not discuss any detail dedicated for usecase 2b yet. From our point of view, the model input for usecase 2b should be delivered using LPP protocol. Therefore, we suggest RAN2 to start discussing the procedure of usecase 2b since it has clear RAN2 LPP specification impact, and SA2 is developing it. See draft reply LS in Annex.

The procedure of usecase 2b should be similar like the procedure of usecase 1, i.e., LMF firstly schedules UE to derive the PRS channel measurement via LPP protocol, and then UE reports the PRS channel measurement (as LMF’s request) to the LMF via LPP protocol. The PRS channel measurement can be used for LMF to training or inference. As discussed in section 2.1.2, usecase 2b can be scheduled as a new AI/ML positioning method.

Proposal 21: Reply SA2 that RAN2 haven’t reach any agreement dedicated for usecase 2b yet, and RAN2 is going to discuss the procedure of usecase 2b.

- Conclusion

In this contribution, we propose the following observation and proposals:

Observation 1: Use case 1, use case 2a and use case 2b have clear RAN2 LPP specification impacts, wherein use case 1 is the 1st priority.

Functionality/AI model activation:

Observation 2: Since NW side additional condition is already provided to UE, it is not necessary and not efficient to let UE report UE additional condition to LMF, then LMF decides the AI model and feedback to UE.

Proposal 1: For AI/ML positioning use case 1 and 2a, RAN2 to consider the following approaches for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows UE to activate/deactivate the functionality (e.g., when fallback positioning method is configured)

Proposal 2: For AI/ML positioning use case 1 and 2a, LMF allows UE to activate/deactivate the AI model of the activated functionality.

Functionality management as positioning method:

Observation 3: For AI/ML positioning, if reusing legacy positioning method structure, the following disadvantages are observed:

- The UEs only supporting AI/ML positioning cannot decode the module of legacy positioning method;

- If only AI/ML positioning is requested by NW, UE cannot handle the mandatory reporting quantities in the legacy positioning methods;

- It is unclear that which legacy positioning method should the usecase 1 and usecase 2b reuse;

- Mixing AI/ML measurement request/report with legacy method’s measurement request/report causes poor readability and poor extendibility of the spec.

Proposal 3: For AI/ML positioning, the functionality can be recognized as use case 1, use case 2a and use case 2b.

Proposal 4: From RAN2 signaling structure perspective, AI/ML positioning functionality (use case) can be managed as positioning method, i.e., activating a positioning method means activating the corresponding AI/ML positioning functionalities (use cases).

Proposal 5: Support to introduce a new AI/ML positioning method in LPP for use case 1, 2a and 2b, in which usecase 1 is UE-based positioning, usecase 2a and 2b are UE-assisted positioning.

Fallback from AI positioning method to non-AI positioning method

Observation 4: AI positioning may fail/be unavailable more frequently than the legacy positioning methods.

Observation 5: If AI positioning fails and then UE is scheduled with another positioning method, such procedure requires two round of LPP message transfer which takes a lot of time, and the QoS of timing requirement may be hard to achieve.

Proposal 6: LMF can schedule AI positioning method together with DL-TDOA or DL-AoD positioning method to UE, if AI positioning fails/non-applicable, UE can use the fallback DL-TDOA or DL-AoD to perform positioning and report positioning measurements/estimates on time.

LCM procedure in the new positioning method

Proposal 7: The signaling of the new AI/ML positioning method should support training, inference and performance monitoring function.

Proposal 8: Do NOT support LMF to control the UE sending unsolicited LPP provide capabilities using explicit signaling.

Proposal 9: For usecase 1 and usecase 2a, UE can request assistance data for AI/ML training using RequestAssistanceData message, and LMF can provide assistance data for AI/ML training using ProvideAssistanceData message.

Proposal 10: UE can request some specific assistance data that will have impact on model training, as following:

- UE requests the number of TRPs that the UE wants to receive DL-PRS from;

- UE requests the area of TRPs that the UE wants to receive DL-PRS from;

- UE requests the number of PRUs that the UE wants to receive the corresponding measurement and labels from.

Proposal 11: For usecase 1 and 2a, if the AI/ML positioning method is scheduled for UE training, UE does not need to report location estimate or measurement in the AI/ML positioning method.

Proposal 12: RAN2 needs to discuss whether to introduce a new signaling procedure in LPP for performance monitoring metric transfer, or to include the performance monitoring metric transfer in the ProvideLocationInformation message.

Proposal 13: The Abort message should be used in the AI/ML positioning method to deactivate a functionality of UE.

Proposal 14: New error causes should be dedicated for specific AI/ML procedures (e.g. training or inference or monitoring).

Consistency between training and inference:

Observation 6: For AI/ML positioning, the NW additional conditions (e.g., TRP location, TRP synchronization error, TRP number, PRS configuration) can already be indicated to the UE in current LPP spec.

Observation 7: When providing inference configuration to UE, LMF may not know which kind of inference configuration best suits the training configuration that UE used to train an AI model.

Proposal 15: For AI/ML positioning usecase 1 and 2a, ensuring the consistency between training and inference does not need an associated ID.

Proposal 16: UE should be able to request some specific NW side additional conditions that suits the UE’s current AI model, e.g., the wanted number of TRPs, the wanted area of TRPs, the wanted synchronization error between TRPs for inference.

LCM of gNB involved use case (use case 3a and 3b):

Observation 8: For use case 3a, different gNB-side AI models can correspond to different UE or gNB additional conditions; for use case 3b, different LMF-side AI models can correspond to different UE or gNB additional conditions.

Proposal 17: For AI/ML positioning use case 3a and 3b, RAN2 assumes the following approaches can be used for functionality-LCM:

- LMF activates/deactivates the functionality

- LMF allows gNB to activate/deactivate the functionality (e.g., when fallback positioning method is configured )

Proposal 18: For AI/ML positioning use case 3a, RAN2 assumes that LMF can allow gNB to activate/deactivate the AI model of the functionality.

Proposal 19: For use case 3a, RAN2 assumes that gNB should to know the PRU side additional condition to determine an AI model. FFS on how gNB gets such information.

Proposal 20: Send LS to RAN3 about RAN2’s agreement of LCM for use case 3a and use case 3b.

Discussion on SA2’s LS:

Proposal 21: Reply SA2 that RAN2 haven’t reach any agreement dedicated for usecase 2b yet, and RAN2 is going to discuss the procedure of usecase 2b.

- Reference

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 2024-09

- Report of 3GPP TSG RAN WG2 meeting #125-bis, Changsha, China, 2024-04

- TS 38.305-i10, Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN, 2024-03

- Report of 3GPP TSG RAN WG2 meeting #126, Fukuoka, Japan, 2024-05

- R1-2403741, Summary of specification support for positioning accuracy enhancement, Changsha, China, 2024-04

- Annex: Draft reply LS to SA2

3GPP TSG-RAN WG2#129	R2-25XXXXX

Athens, Greece, February 17th–21st, 2025

Title:	Reply LS on LMF-based AI/ML Positioning for Case 2b

Response to:	S2-2501133

Release:	Rel-19

Work Item:	NR_AIML_air-Core

Source:	RAN2

To:	SA2

Cc:	RAN3, RAN1

Contact person:

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

- Overall description:

Regarding the question asked by SA2：

Q1: SA2 asks RAN1, RAN2 to provide current progress and future work plans on data types and procedures defined for case 2b. If there is any existing agreement in RAN, please provide a reference.

RAN2’s answer:

RAN2 haven’t reach any agreement dedicated for usecase 2b yet, and RAN2 is going to discuss the procedure of usecase 2b.

2 Actions:

ACTION:

To SA2

- RAN2 respectfully asks SA2 to take the above replied information into consideration for their future work.

3	Dates of next TSG SA WG 2 meetings:

TSG RAN WG2 Meeting #129bis    	7th – 11th April, 2025		China

TSG RAN WG2 Meeting #130       	19th – 23th May, 2025		Malta