3GPP TSG-RAN WG2 Meeting #129	R2-2500618

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:		8.9.2

Source:		Lenovo

Title:		Enhancements to support S&F operation

Document for:		Discussion

- Introduction

The latest RAN2#128 meeting made the following agreements regarding Store and Forward (S&F) operations in IoT NTN:

- Time information can be broadcasted to indicate when the current satellite operation mode will transit from “S&F operation” mode to real-time/normal mode. RAN2 assumes that a R19 UE could use this information at least to delay some NAS procedures until the feeder link is resumed (FFS what we specify in RAN2 specs for this and if we differentiate the behaviour for R19 UEs supporting and not supporting S&F mode, also considering the relation to the Wait Timer). FFS which SIB is used (SIB1 or SIB31). FFS if absolute or relative time is used. FFS on the opposite transition (i.e. whether to indicate when the “S&F operation” will start) - It is FFS whether we need to introduce a new release cause when the NW wants to release a R19 UE to RRC idle because the feeder link becomes unavailable - “S&F operation” indication is provided in SIB1 - RAN2 assumes that AS signalling is not needed to indicate the architecture option (Split MME or Full CN). - RAN2 waits for further progress in SA2, if any, before working on RAN2 aspects of a CIoT-UP solution for SF operation - RAN2 understands that the UE configured with a satellite ID list by MME is not prevented to camp on a satellite operating in normal IoT NTN mode (i.e. with feeder-link connection), and perform subsequent access and data/signalling communication with that satellite - RAN2 understands the UE configured with a satellite ID list by MME is not prevented to camp on, attempt to access to and communicate with a satellite which is not included in the MME-configured satellite list.

In this contribution we discuss some further enhancements for S&F operation.

- Discussion

The main different between S&F operation and normal operation in NTN is whether there is a continuous end-to-end connectivity path between the UE, the satellite and the ground network i.e., whether it requires the service and feeder links to be active simultaneously. Such differences have brought in some new requirements. Table 1 provides a general gap analysis between the current RAN2 progress and part of the consolidated requirements for S&F operation in TR22.865.

Table 1 Some consolidated requirements for S&F operation and the corresponding RAN2 progress

| Consolidated requirements | RAN2 progress | Potential enhancements |
| --- | --- | --- |
| CPR 6.2-1: support S&F operation for authorized UEs. | Mode transit time information is provided to UE. | Access control (e.g. cell barring) based on the mode transit time. |
| CPR 6.2-2: inform a UE whether S&F operation is applied. | S&F operation indication is introduced to indicate that the cell is operating in S&F mode. | Access control (e.g. cell barring) based on the indication. Cell reselection based on operation status. |
| CPR 6.2-3: allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data retention period. | No related progress. | Indication of S&F data retention period information to UE. |
| CPR 6.2-4: allow the operator or a trusted 3rd party to apply, on a per UE and/or satellite basis, an S&F data storage quota. | No related progress. | Indication of S&F data storage quota information to UE. |
| CPR 6.2-5: support a mechanism to configure and provision specific required QoS and policies for UE’s data subject to S&F operation (e.g. forwarding priority, acknowledgment policy) | No related progress. | Indication of S&F QoS or policy information to UE. |
| CPR 6.2-6: provide related information (e.g. estimated delivery time) to an authorized UE | No related progress. | Indication of S&F estimated delivery time information to UE. |

In the following we further discuss the listed potential enhancements.

## Access control

Based on current RAN2 discussion, cellBarred-NTN introduced in Rel-17 IoT NTN can be reused to restrict Rel-17 & Rel-18 NTN UEs from accessing an NTN cell operating in S&F. For Rel-19 NTN UEs, an indication that “the cell is operating in S&F mode” was agreed in RAN2#127bis and the following descriptions were decided:

- If the “S&F operation” indication is not broadcast, a Rel-19 UE (regardless whether supporting S&F or not) shall follow the legacy barring procedure

- When present, the “S&F operation” indication has two possible settings:

- ‘1’: the cell is operating in S&F mode for all UEs (Rel-19 UEs supporting S&F are allowed to access the cell)

- ‘0’: the cell is operating in S&F mode for UEs in Connected mode (which are not required to monitor the “S&F indication”), but idle Rel-19 UEs supporting S&F are barred (Rel-19 UEs not supporting S&F will follow legacy barring procedure).

Following the above description, the cell barring logic for S&F can be decided.

Proposal 1: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_IDLE ignores cellBarredNTN, and determines cell barring status based on its value (1 as not barred and 0 as barred).

The legacy cell barring schemes including cellBarred-NTN are solely based on the corresponding bit status due to that the network type (e.g., NTN or TN) is fixed, and for cell reselection the barring status is fixed to 300s. UE can only re-acquire the SIB1 to understand if the cell barring status has changed. Meanwhile, in Rel-19 IoT-NTN the access control could be time sensitive as the IoT-NTN cell may change between S&F mode and normal mode (real-time mode). Considering that the RAN2#128 meeting agreed to provide mode transit time information to UE, it is possible to determine the validity duration of a cell barring status based on the mode transit time.

Proposal 2: A cell barring status based on “S&F operation” indication has a validity duration till the mode transit time, if provided.

## Cell reselection

Based on the above discussions for S&F related cell barring, Rel-19 IoT-NTN UEs also face the similar issues during the cell reselection procedures. More specifically, if a neighbour NTN cell is operating in S&F mode, the corresponding cell barring scheme shall be considered by the UE when performing cell reselection. The determination of cell barring status for a neighboring NTN cell operating in S&F mode can either follow an option in Proposal 1 or an indicated list of neighbour cells operating in S&F mode.

Proposal 3: If a neighboring NTN cell operating in S&F mode is considered as not barred, an NTN-capable UE supporting S&F operation may prioritize the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in S&F mode is considered as barred, an NTN-capable UE supporting S&F operation may deprioritize the frequency of the cell for cell reselection.

## Additional indication for S&F operations

According to the consolidated requirements for S&F operation in TR22.865, UE may need some further information to determine whether an S&F cell can fulfill its service requirements or whether an S&F operation can be initiated, for example:

- UE may need to know whether the data retention period applied in the S&F cell can be tolerated for the service to be initiated;

- UE may need to know whether its data volume fulfils the data storage quota applied in the S&F cell;

- UE may need to know whether the QoS or policy (forwarding, acknowledgment) applied in the S&F cell fulfils the requirement of the service to be initiated;

- UE may need to know whether the estimated delivery time supported in the S&F cell f can be tolerated for the service to be initiated.

Therefore, it would be useful to indicate such information to UE for better S&F operation, and AS signaling solutions can avoid further coordination with other WGs.

Proposal 5: RAN2 to consider the following indications to support S&F operation:

- Indication of S&F data retention period information, e.g. for how long the data can be retained on the satellite eNB.

- Indication of S&F data storage quota information, e.g. the data volume threshold that can be stored and forwarded by the satellite eNB.

- Indication of S&F QoS or policy information, e.g. the store and forward priority or acknowledgement policy for user data applied at the satellite eNB.

- Indication of S&F estimated delivery time information, e.g. for how long the user data can be delivered to the core network.

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and operation information exchange for S&F operations in IoT NTN. The following proposal are provided:

Proposal 1: If the “S&F operation” indication is present, an NTN-capable UE supporting S&F in RRC_IDLE ignores cellBarredNTN, and determines cell barring status based on its value (1 as not barred and 0 as barred).

Proposal 2: A cell barring status based on “S&F operation” indication has a validity duration till the mode transit time, if provided.

Proposal 3: If a neighboring NTN cell operating in S&F mode is considered as not barred, an NTN-capable UE supporting S&F operation may prioritize the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in S&F mode is considered as barred, an NTN-capable UE supporting S&F operation may deprioritize the frequency of the cell for cell reselection.

Proposal 5: RAN2 to consider the following indications to support S&F operation:

- Indication of S&F data retention period information, e.g. for how long the data can be retained on the satellite eNB.

- Indication of S&F data storage quota information, e.g. the data volume threshold that can be stored and forwarded by the satellite eNB.

- Indication of S&F QoS or policy information, e.g. the store and forward priority or acknowledgement policy for user data applied at the satellite eNB.

- Indication of S&F estimated delivery time information, e.g. for how long the user data can be delivered to the core network.