3GPP TSG-RAN WG2 Meeting #129	R2-2500628

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.7.4.2

Source:		Lenovo

Title:			Enhanced delay status reporting for XR

Document for:	Discussion and Decision

- Introduction

In this contribution we discuss delay status reporting enhancements for XR traffic.

- Discussion

In RAN2#126 the following agreements were reached:

- Enhance DSR to report with multiple pairs of remaining time and buffer size for the LCG. - FFS whether DSR triggering is impacted - FFS whether PDU set importance needs to be included

In RAN2#127 the following agreements were reached:

- Network should be able to configure multiple remaining time thresholds for reporting for each LCG to report multiple pairs of remaining time and buffer sizes per LCG. For enhanced DSR: - There will be a single triggering threshold, as in Rel-18. FFS whether there are any constraints on how the NW configures DSR triggering and reporting thresholds - FFS whether there is any impact on delay critical data definition due to multiple reporting thresholds in the DSR - FFS whether to include non-delay critical data ahead of delay critical data in the buffer size calculation for DSR

In RAN2#127bis the following agreements were reached:

- For Rel-19 DSR, the buffered data is divided into multiple portions based on the multiple reporting time threshold levels configured for an LCG. The Rel-19 DSR indicates the following information for each portion for which BS>0: • Buffer size of data volume in each portion • Shortest remaining time among PDCP SDUs buffered in each portion. - There is no need to include PSI in the enhanced DSR MAC CE. - A one-bit indication may indicate whether a certain/further pair of remaining time information and buffer size information is present in the new DSR MAC CE for the associated LCG. - FFS whether old and new DSR can be configured/used at the same time or we always use a new DSR in case there is at least one LCG configured with multiple reporting thresholds

In RAN2#128 the following agreements were reached:

- Let the network configure the triggering and reporting thresholds without constraints. - RAN2 understanding is that the data that has been already reported in the DSR should not trigger another DSR - The existing cancelling and triggering of Rel-18 DSR is reused for the enhanced DSR. - The UE may also support including non-delay critical data ahead of delay critical data in the buffer size calculation for DSR, which is a capability indicated to the NW.

In RAN2#127 meeting, it was concluded to configure multiple remaining time thresholds for each LCG to report multiple pairs of remaining time and buffer size information. When multiple PDU sets in the LCG have different remaining times UE may report remaining time information on a PDU set level as in Rel-18 if DSR is triggered for an LCH and PDU set discard is configured.

Proposal 1: DSR reporting should be done on a PDU set level if PDU set discard is configured similar as Rel-18 principle.

RAN2 has agreed that no restrictions are introduced for the configuration of the DSR reporting and triggering thresholds, i.e., trigger threshold can be configured below or above reporting threshold. Here we assume, that the minimum remaining time reported in Rel-18 DSR is still reported in Rel-19, therefore reporting thresholds should include the Rel-18 reporting. UE accordingly reports a pair of remaining time information and corresponding buffer size based on the configured reporting threshold(s) for cases that a DSR has been triggered. RAN2 has agreed that UE may also support including non-delay critical data sitting ahead of delay critical data in the queue in the buffer size calculation for DSR. The data with a remaining time being above the DSR trigger threshold (remainingTimeThreshold) and less than a configured reporting threshold is considered as non-delay critical data, while the data with remaining time below the DSR trigger threshold is considered as delay critical data as in the legacy. It is beneficial from uplink resource allocation perspective for the case that there is non-delay critical data ahead of the delay critical data in the buffer, that the non-critical data is calculated and reported separately in a DSR based on the configured reporting threshold(s).

Proposal 2: RAN2 to define delay-critical data as data for which remaining time is below the DSR triggering threshold (as in Rel-18), non-delay critical data as data with a remaining time being below the DSR reporting threshold and above the DSR triggering threshold.

For the enhanced DSR MAC CE format, we think that NW should configure if and for which LCG the DSR based reporting threshold(s) should be considered. For example, only for specific LCG(s) NW may configure reporting thresholds. If one LCG is configured with reporting thresholds, UE should use a new DSR format for simplicity. For example, the new DSR format may use a one-bit field to indicate whether some further pair of remaining time information and buffer size information is present in the MAC CE for the associated LCG. For example, a one-bit field associated with a pair of remaining time information and buffer size information indicates whether a further pair of remaining time information and buffer size information follows, e.g. extension flag. If one LCG is only configured with a R18 DSR configuration, this new MAC CE format with extension flags is also compatible for the Rel-18 delay status reporting.

Proposal 3: If at least one LCG is configured with Rel-19 DSR reporting thresholds and there is data to be reported for an LCG in a triggered DSR, UE should use the new DSR format for all LCGs.

Proposal 4: A one-bit field associated with a pair of remaining time information and buffer size information indicates the presence of a further pair of remaining time information and buffer size information in the new DSR MAC CE for the associated LCG.

In Rel-18, when determining the data for triggering a DSR, the delay critical data should have not been transmitted or reported before in a DSR. In Rel-19, RAN2 revisits whether the same restriction should be applied to the non-delay critical data which has been reported in an DSR. For avoiding duplicated reporting, it is preferred to not trigger a DSR for non-delay critical data, which has been previously reported, when it becomes delay-critical.

Proposal 5: RAN2 confirms that UE does not trigger a DSR when non-delay critical data, which has been previously reported, becomes delay-critical.

- Conclusion

In this contribution, we discuss delay status reporting enhancements. We have the following proposals:

Proposal 1: DSR reporting should be done on a PDU set level if PDU set discard is configured similar as Rel-18 principle.

Proposal 2: RAN2 to define delay-critical data as data for which remaining time is below the DSR triggering threshold (as in Rel-18), non-delay critical data as data with a remaining time being below the DSR reporting threshold and above the DSR triggering threshold.

Proposal 3: If at least one LCG is configured with Rel-19 DSR reporting thresholds and there is data to be reported for an LCG in a triggered DSR, UE should use the new DSR format for all LCGs.

Proposal 4: A one-bit field associated with a pair of remaining time information and buffer size information indicates the presence of a further pair of remaining time information and buffer size information in the new DSR MAC CE for the associated LCG.

Proposal 5: RAN2 confirms that UE does not trigger a DSR when non-delay critical data, which has been previously reported, becomes delay-critical.

- References

### TS 38.321 v18.0.0