3GPP TSG-RAN WG2 Meeting #129	R2- 2500636

Athens, Greece, Feb. 17th – 21st, 2025

Agenda item:	8.12.3

Source:	Lenovo

Title:	Discussion on UEIBR

WID/SID:	NR_MIMO_Ph5-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

It is listed in the WID RP-242394 to specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency in Rel-19. And the related discussion has been started since RAN1#116. Many agreements have been achieved and the latest agreements from RAN1 on UEIBM related to RAN2 including:

RAN1#119

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, Alt-2 is supported for both mode-A and mode-B: the first PUCCH channel is a new UCI type

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - It is RAN1’s understanding that the RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission (if supported) for mode A and mode B will be continue to be separately discussed in RAN1.

Agreement

Confirm the following working assumption in RAN1#117:

On beam report transmission procedure for UE-initiated/event-driven beam reporting

- For mode-A, at least support one-bit indication in the first PUCCH channel to request a resource for a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- For mode-B, at least support one-bit indication in the first PUCCH channel to notify a second UL channel to carry beam report.

  - In such case, a periodic PUCCH resource (with PUCCH format 0/1) is configured by dedicated RRC signaling.

- FFS: Whether/how to support multi-bit indication in the first PUCCH for mode-A and mode-B, e.g., when multi-event(s) are approved.

- FFS: details on the dedicated RRC signaling

- Above applies at least for the single CC case.

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported:

- Option-2: Type-1 CG PUSCH can NOT be transmitted

Agreement

On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding first PUCCH channel configuration, down-select one of Alt-1 and Alt-2 for both mode-A and mode-B.

- Alt-1 (dedicated SR): Introduce RRC parameter, e.g., reportResourceRequest-UEIBR/reportNotification-UEIBR, corresponding to the one-bit indication in the first PUCCH channel

  - The RRC parameter is associated with the dedicated SchedulingRequestId.

    - Note: The detailed signaling is up to RAN2.

  - FFS: whether/how to enhance legacy SR to support notification of beam report transmission in Mode-B

- Alt-2 (new UCI type): Introduce RRC parameter, e.g., firstPUCCHResourceConfig- UEIBR, for the periodic PUCCH resource configuration.

  - Note: The RRC parameter is NOT associated with SchedulingRequestId.

  - 1-bit to PUCCH resource is encoded by reusing the encoding mechanism of positive/negative SR.

  - The dedicated RRC parameter at least comprises the following:

    - periodicityAndOffset

    - PUCCH-ResourceID

- Above applies at least for the single CC case.

- Reuse multiplexing/dropping rule(s) of SR as baseline

  - FFS: overlapping with SR/LRR or PUSCH

Note: Further details on first PUCCH retransmission for mode A and mode B will be separately discussed.

In this document, we discuss RAN2 impacts of the WID and the RAN1 progress.

# 2		Discussion

RAN1 is discussing two procedures for actually sending back the UEIBM report:

- Mode A, where the second UL channel for the UEIBM report is dynamically scheduled by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to request to the gNB resources in a second UL channel to carry the UEIBM report;

  - The gNB indicates via DCI to the UE a resource in a second UL channel to carry the UEIBM report;

  - The UE sends the UEIBM report on the second UL channel.

- Mode B, where the second UL channel for the UEIBM report is pre-configured by the gNB; in such procedure the following steps are implemented:

  - The UE sends in a first PUCCH channel an UL indication to notify to the gNB that a UEIBM report will be transmitted in a second UL channel;

  - UE sends the UEIBM report on the second UL channel.

It has been agreed that UL-SCH may be transmitted via the second PUSCH for the case the pre-configured Type-1 CG PUSCH carries the beam report. While for the overlapping case, RAN1 agreed to reuse multiplexing/dropping rule(s) of SR as baseline and FFS on overlapping with SR/LRR or PUSCH.

It seems to be RAN2 scope to discuss on the case of second PUSCH overlapping with PUSCH. In legacy, lch-basedPrioritization is configured to prioritize the urgent traffic when overlapping occurs. Considering the reason to enhance beam report by UEIBR, it is because UE has better and more-timely knowledge of beam quality changes, UE-initiated beam reporting procedure can lead to more timely beam reports yet with reduced reporting overhead. Under such a procedure, if the UE determines that e.g., current beam(s) quality becomes poor, UE can trigger beam reporting without the network needing to configure or trigger frequent reporting. To some extent, UEIBR is also an urgent traffic. The base station misses the first PUCCH and schedules a dynamic resource which overlaps with the second PUSCH in mode-B. If there is not rule similar with lch-basedPrioritization applied to UEIBR, the beam report may be dropped which is unexpected.

Proposal 1: lch-basedPrioritization is enabled when mode-B UEIBR is configured.

On the other hand, the CG type 1 configured for UEIBR is not same with the legacy CG type1. The UE can consider the legacy CG type 1 as available for data transmission once the CG type 1 is configured and the related mapping restrictions e.g. configuredGrantType1Allowed or allowedCG-List is satisfied. While it may not work for the CG type 1 configured for UEIBR because whether a UEIBR being carried becomes a crucial factor to determine the availability of the CG type1 resource. Considering the UEIBR is triggered by L1. MAC may determine the CG type1 for UEIBR as available resource based on L1 indication of the trigger of UEIBR.

Proposal 2: MAC determines whether the CG type1 for UEIBR is available for UL-SCH based on L1 indication.

Correspondingly, there is a scenario that new data arrives before a SR occasion and the SR occasion is ahead of the CG type 1 resource. In legacy, the SR may not be triggered because the CG type 1 resource is considered as available resource. If the CG type 1 resource is for UEIBR. SR may be triggered if CG type 1 is not transmitted for no beam report being carried. This procedure needs to take L1 indication into account.

Proposal 3: MAC determines whether to trigger a SR considering the L1 indication of no CG type 1 transmitted.

# 3	Conclusion

In summary, we have following proposals for UEIBR:

Proposal 1: lch-basedPrioritization is enabled when mode-B UEIBR is configured.

Proposal 2: MAC determines whether the CG type1 for UEIBR is available for UL-SCH based on L1 indication.

Proposal 3: MAC determines whether to trigger a SR considering the L1 indication of no CG type 1 transmitted.

# 4	References

[1] RP-242394	WID revision: NR MIMO Phase 5	Samsung

[2] RAN1#119 chairman notes.